3GPP TSG-RAN WG4 #113	R4-2418004

Orlando, US, November 18-22, 2024

Agenda item:	5.32.4		Demodulation performance and CSI requirements

Source: 	Qualcomm Incorporated

Title: 	On missing performance requirements for LTE-based 5G terrestrial broadcast

Document for:	Decision

# Introduction

In [1] new bands and bandwidths for LTE-based 5G terrestrial broadcast have been introduced but no performance requirements are defined so far for these new channel bandwidths. In this paper we propose how the missing requirements can be added to Rel-18.

# Discussion

In [1] new bands and bandwidths for LTE-based 5G terrestrial broadcast have been introduced but no performance requirements are defined so far. The new channel bandwidths are 6, 7 and 8 MHz. However, existing for LTE-based 5G terrestrial broadcast only define demod requirements for 10 MHz channel bandwidths which is not directly applicable for the new bandwidths. Therefore, the Rel-18 WID is incomplete in our opinion.

Hence, we propose to add the missing requirements for 6, 7 and 8 MHz by following the RMCs for 10 MHz as closely as possible such that the SNR requirements can be re-used. This just requires that new RMCs for the new bandwidth with the same code rates are introduced.

Table 2-1 below shows simulation results for the adaptation of R.107 to 6, 7 and 8 MHz. Here it is possible to keep the code rate for the new bandwidths the same as for R.107 that is defined for 10 MHz. There are some small bandwidth dependencies of the SNR to achieve 1% BLER but performance is still significantly better than the SNR requirement for R.107 in 10 MHz. Hence, we think that the SNR requirement can be reused without running new simulations.

Table 2-2 below shows simulation results for the adaptation of R.106-1 and R.106-2 to 6, 7 and 8 MHz. Here it is not exactly possible to keep the code rate unchanged but again the performance results are almost unchanged for 6, 7 and 8 MHz compared to 10 MHz. Again, there is a large margin to the requirements for the reference measurement channels for 10 MHz. Hence, we think that the SNR requirement can be reused without running new simulations.

The CR in [2] defines the details of the new reference measurement channels to extend the demod requirements for LTE-based 5G terrestrial broadcast from 10 MHz to the new channel bandwidths of 6, 7 and 8 MHz.

Observation: The reference measurement channels for LTE-based 5G terrestrial broadcast for 10 MHz can be easily extended to 6, 7 and 8 MHz while keeping the channel code rate (almost) constant. That allows reusing the existing SNR requirements without running new simulations.

Proposal: We propose to adapt the enhancements of the RMCs for 10 MHz as defined in [3] to complete the work item on LTE-based 5G terrestrial broadcast part 2 [1].

Table 2-1: Results for R.107

Table 2-2: Results for R.106-1 and R.106-2

# Conclusions

In this paper we shared our view how the work item on LTE-based 5G terrestrial broadcast part 2 can be completed in an easy way without running new simulations. We have the following observation and proposal:

Observation: The reference measurement channels for LTE-based 5G terrestrial broadcast for 10 MHz can be easily extended to 6, 7 and 8 MHz while keeping the channel code rate (almost) constant. That allows reusing the existing SNR requirements without running new simulations.

Proposal: We propose to adapt the enhancements of the RMCs for 10 MHz as defined in [3] to complete the work item on LTE-based 5G terrestrial broadcast part 2 [1].

# References

- RP-223378, Revised WID on new bands and bandwidth allocation for LTE based 5G terrestrial broadcast – part 2, RAN #98e, December 2022

- RP-193050, Revised WID for LTE-based 5G terrestrial broadcast, RAN #86, December 2019

- R4-2417773, Adding missing performance requirements for 6/7/8 MHz channel bandwidths for LTE-based 5G terrestrial broadcast, RAN4 #113, November 2024