3GPP TSG-RAN WG4 Meeting#113							         		R4-2418015

Orlando, Florida, USA, 18 – 22 November 2024

Title: 	Redefining allocation types for improved MPR with BS BW larger than UE CBW

Source: 	Skyworks Solutions, Inc.

Agenda Item:	7.1.1.2.1 Power domain enhancements for single carrier [NR_ENDC_RF_Ph4-Core]

Document for:	Approval

# 1	Background

In RAN#112bis [1], we discussed how allocation types and associated MPR could be redefined by considering the BS advertised BW and UE RB position. The different options from different companies related to contiguous and non-contiguous ULCA were captured in way forward [2]. In this contribution, we further develop our proposal to address some of the corner cases discussed in RAN4#112bis.

# Discussion

## 2.1 Way Forward input

#### The agreements and way forward from [2] relevant to improved MPR scenarios are copied below in italic.

#### Issue 1-1-1: Approaches for scenarios 1-1 and 1-2 (see Reference for description of scenarios)

WF: Strive for a unified solution which can accommodate both scenario 1 and scenario 2 under the framework of converting outer RB allocation to inner RB allocation.

- It does not preclude separate solutions for each applicable scenario

- No relaxation of ACLR/SEM/SE values

Issue 1-2-1: Clarification of multi-CC scenario

WF: The approach of converting outer RB allocation to inner RB allocation for MPR reduction should consider multi-carrier spectrum scenario from NW perspective while the UE is configured for single CC operation.

Issue 1-2-2: Approaches of converting outer RB allocation to inner RB allocation

WF: The approach should consider the following aspects

- unified solution could be applicable for both scenarios 1 and scenario 2

- unified solution considering the spectrum allocation status of operators, e.g multi-CC case from NW perspective, to better leverage the feature of MPR reduction

- Solution could benefit RedCap and non-RedCap with UE CBW less or even identical to BS CBW

- Solution could include both symmetric and asymmetric CBW extension

- Whether the extended UE CBW could exceed the BS CBW for different scenarios

- Signalling overhead is considered for the proposed approach(s)

#### Issue 1-2-3: Where to use IBE in the larger BS CBW

WF: IBE should be used between edges of UE CBW and extended UE CBW

- FFS whether IBE could be used in the guard band of UE CBW

- FFS the full RB allocation scenario

- FFS the impact to testability

#### Issue 1-2-4: Boundary to apply ACLR and SEM

Proposals:

- Proposal 1: ACLR and SEM should be applicable from the edge of extended UE CBW instead of the BS CBW.

- Proposal 2: ACLR, SEM and spurious emissions would be defined based on BS channel bandwidth.

WF: FFS in next meeting

- ITU regulation of 250% necessary bandwidth should be considered in further analysis

#### Issue 1-2-5: Boundary to apply SE

Proposals

- Proposal 1: The application range of SE should be altered with the shifting of the edge of the UE CBW.

- Proposal 2: SE is applied at BS channel bandwidth.

WF: FFS in next meeting

- ITU regulation of 250% necessary bandwidth should be considered in further analysis

#### Issue 1-2-6: Which CBW is utilized as the basis for the OOBE requirement and the applicable boundary

Proposals

- Proposal 1: The OOBE requirement and applicable boundary should be based on UE CBW.

- Proposal 2: The OOBE requirement and applicable boundary is based on BS CBW.

WF: FFS in next meeting

#### Issue 1-2-7: Ratio of extended CBW to UE CBW and to larger BS channel BW

WF: To discuss the following aspects for the extended UE CBW:

- Whether the extended CBW with new boundary of FOOB should comply with the recommendation by ITU-R on necessary bandwidth in terms of SE for the original UE BW

- Whether default or fixed extension, e.g. 1/2 UE CBW could be stipulated in the spec or the extension ratio could be a UE capability to fulfil the conversion of outer to inner

- Whether the extended UE CBW could belong to the regular defined channel bandwidth

- Whether some premise should be established for the extension size, e.g.

  - minimum extended CBW compared to original UE CBW

  - minimum BS CBW compared to UE CBW

  - frequency separation of the RB allocation to the BS edge

- Other identified issues are not precluded

#### Issue 1-2-8: Asymmetrical extended CBW approach

WF: Asymmetrical extension is considered, FFS the solution.

#### Issue 1-2-10: Signaling aspects

WF: To further discuss the signalling aspects in conjunction with the solution to be adopted for MPR reduction.

- Companies are encouraged to provide thinking on signalling impact together with the proposed solution.

## 2.2 A flexible solution covering multiple scenario or NW constraints

## A flexible solution is needed to cover any scenario or constraints that the NW may want to impose on the UE emissions and still achieve reduced MPR.

Observations: When looking at the list of issues in [2], it can be observed the following:

- For issue 1-1-1: With no relaxation of ACLR/SEM/SE values, the improved MPR can only come from extending the UE bandwidth appropriately to still meet regulations (including ITU) and, other band protections (UE meets the spurious emission level). But the extended UE BW has no relation to the BS CBW.

- For Issue 1-2-1: The UE extended bandwidth needs to be larger than the BS CBW.

- For Issue 1-2-4, 1-2-5 and 1-2-6: Meeting ITU regulation of 250% necessary bandwidth or BS OOBE boundary can be obtained by signalling the appropriate extended BW to the UE which may or may not correspond to the BS CBW.

- For issues 1-2-7 and 1-2-8, the UE should not only know what the extended UE BW is but also the position of the UE CBW within it.

- A flexible solution is needed in terms of providing the UE extended BW and UE should know its CBW position within this BW.

The above cases can all be covered by the solution discussed in [1] where the UE does not need to be aware of the BS BW but only be signalled the BW that defines its emission limits for SEM, ACLR and SE and still respect regulation and network constraints as needed (ITU 250% rule, BS OOBE, etc.). The solution is flexible enough to meet any of the above constraints by receiving the advertised larger BW in number of RBs and its RB0 position within those RBs. This provides a flexible solution directly to Issue 1-2-2.

For issue 1-2-3, since IBE is met with the inner and outer allocations MPR, the IBE requirement should apply within the entire UE extended BW.

For issue 1-2-10 we already provided an input in [1] that would cover all the cases up to 200MHz of extended BW (two contiguous CCs with a 1RB granularity.

Even if the extended CBW may be larger than the maximum applicable UE CBW in a band, the UE will never transmit a larger BW.

Based on the above, we can propose again the solution in [1] which is illustrated in Figure 1 for a case where the UE extended BW is < UE CBW as an example:

- On the left side, the classical allocation triangle is based on the UE CBW where the inner allocations (green) region has LCRB limited to half the UE CBW and their position is at half the allocation (LCRB) size from the channel edge; other allocations are outer (yellow), except for PC2 and PC1.5 where edge (red) allocations are needed.

- On the right side, the allocation triangle is now based on the BS CBW and a new UE allocation triangle can be defined by shifting the UE based triangle within the BS defined inner, outer, and edge regions by its RB0 position within the BS RBs.

Figure 1: Illustration of UE based, and BS based allocation triangle defining inner, outer and edge allocations. UE extended BW < 2xUE maximum CBW valid in the band.

Proposal on a mechanism to redefine allocation type based on advertised BS transmission BW:

- A UE capability is introduced for UEs that can re-assess the inner, outer, and edge allocation definition based on existing equations for single CC MPR but based on BS advertising a larger transmission BW than the UE CBW with:

- Using BS NRB (BS_NRB) instead of UE NRB.

- Adding the amount of RB by which the UE RB0 is shifted (RB0shift) within the BS RBs to the UE RBstart value.

- The base station shall signal the extended NRB (BS_NRB) and RB0shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR/boost power for allocations that have changed their type.

Proposal on inner/outer/edge allocation equations: For UEs supporting this capability, inner/outer/edge allocations equations in section 6.2.2 are modified in the TS based on BS_NRB and RB0shift parameters signaled by the BS in the following way:

- Replace NRB by BS_NRB in inner allocation equations.

- Replace RBstart by RBstart+RB0shift in inner allocation equations and for edge allocations.

Proposal on additional emission requirements: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function with:

- A-MPR based on the LCRB and RBstart and frequency based on the UE CBW (NRB and RBstart parameters)

- MPR based on the extended UE BW and position (BS_NRB and RB0shift parameters).

Proposal on advertised BS transmission BW signalling and constraints:

- So that more use cases are covered in the future, the BS advertised transmission BW may be smaller than, equal to or larger than the BS CBW, depending on where the ACLR/SEM and spurious emission limits need to be met by the UE.

- Except for additional emission requirements that the UE meets autonomously, the BS advertised transmission BW should ensure that the UE can meet:

  - ITU regulation of 250% necessary bandwidth

  - BS OOBE limit

  - UE co-existence with other bands not subject to NS (-30dBm/MHz spurious emission level boundary stays before the protected band boundary with the UE extended BW)

  - Any other BW constraints that can be applicable in each deployment

  - The minimum lower and upper guard bands applicable to the UE CBW closest to the UE extended BW.

- It is advertised by means of:

  - BS_NRB parameter that can be up to 566 as it is limited to twice the largest UE CBW applicable in the band.

  - RB0shift parameter to signal where the UE RBs are placed within the BS RBs. This parameter should be able to place any UE CBW within twice the largest UE CBW including for RedCap and eRedcap UEs.

  - Where 10bit seems sufficient to code both parameters.

# Conclusion

In this contribution, based on studying the issues raised in ay forward [2] we propose again the solution presented in [1] with some additions to address some of the specific cases in [2].

Proposal on a mechanism to redefine allocation type based on advertised BS transmission BW:

- A UE capability is introduced for UEs that can re-assess the inner, outer, and edge allocation definition based on existing equations for single CC MPR but based on BS advertising a larger transmission BW than the UE CBW with:

- Using BS NRB (BS_NRB) instead of UE NRB.

- Adding the amount of RB by which the UE RB0 is shifted (RB0shift) within the BS RBs to the UE RBstart value.

- The base station shall signal the extended NRB (BS_NRB) and RB0shift for the UE to calculate the new inner, outer, and edge regions and reduce MPR/boost power for allocations that have changed their type.

Proposal on inner/outer/edge allocation equations: For UEs supporting this capability, inner/outer/edge allocations equations in section 6.2.2 are modified in the TS based on BS_NRB and RB0shift parameters signaled by the BS in the following way:

- Replace NRB by BS_NRB in inner allocation equations.

- Replace RBstart by RBstart+RB0shift in inner allocation equations and for edge allocations.

Proposal on additional emission requirements: In cases where additional emission should be met, the UE performs the max(MPR, A-MPR) function with:

- A-MPR based on the LCRB and RBstart and frequency based on the UE CBW (NRB and RBstart parameters)

- MPR based on the extended UE BW and position (BS_NRB and RB0shift parameters).

Proposal on advertised BS transmission BW signalling and constraints:

- So that more use cases are covered in the future, the BS advertised transmission BW may be smaller than, equal to or larger than the BS CBW, depending on where the ACLR/SEM and spurious emission limits need to be met by the UE.

- Except for additional emission requirements that the UE meets autonomously, the BS advertised transmission BW should ensure that the UE can meet:

  - ITU regulation of 250% necessary bandwidth

  - BS OOBE limit

  - UE co-existence with other bands not subject to NS (-30dBm/MHz spurious emission level boundary stays before the protected band boundary with the UE extended BW)

  - Any other BW constraints that can be applicable in each deployment

  - The minimum lower and upper guard bands applicable to the UE CBW closest to the UE extended BW.

- It is advertised by means of:

  - BS_NRB parameter that can be up to 566 as it is limited to twice the largest UE CBW applicable in the band.

  - RB0shift parameter to signal where the UE RBs are placed within the BS RBs. This parameter should be able to place any UE CBW within twice the largest UE CBW including for RedCap and eRedcap UEs.

  - Where 10bit seems sufficient to code both parameters.

# References

[1] R4-2415274 Reduced MPR by redefining allocation type based on BS signalled transmission bandwidth, Skyworks Solutions Inc., RAN4#112bis

[2] R4-2417204 WF on power enhancement for UE Huawei, RAN4#112bis