3GPP TSG RAN WG4 #113	R4-2418018

Orlando, USA, Nov 18 – 22, 2024

Agenda item:		7.11.3

Source:	Nokia

Title:	On OTA test enhancements

Document for:		Approval

# Introduction

A Way Forward [1] was agreed in RAN4#112bis regarding OTA test enhancements. This contribution further elaborates Nokia’s views on the items related to OTA test enhancements.

# Discussion

## CLTA definition

It was agreed in RAN4#112bis:

Way forward: Potential outcomes of discussion on co-location reference antenna concept to be considered as alternatives to CLTA, rather than replacing it in Rel-19 OTA test specifications.

We have proposed in our earlier contributions that using wideband horn antennas as CLTA’s could be very beneficial concerning cost and test efforts. Single column passive antennas are not readily available for all of the required frequency bands, and they typically meet the CLTA requirements only at relatively narrow frequency ranges. Currently a lot of custom-made antennas are needed in many cases. This makes testing especially in higher and lower frequencies become highly complex and time-consuming. However, there is a need to ensure that wideband horn antennas deliver correct results. To see how wideband horn antennas behave when used as CLTAs with the same setup was tested. The spurious emissions measurements for 3GPP frequency bands were done for two AAS and it was measured with side-by-side distance of 10cm for CLTA and the horn antenna on both sides as depicted in setup A and B, respectively. Three measurements were repeated in order to secure some average result for the spurious emissions. The horn antenna vertical alignment was in the center of the DUT and both +45° and -45° polarizations were measured for each setup. The setups are shown in Figure 1 and Figure 2. Two DUTs were measured with each of them having different results.

Figure 1. Measurement setup for the wideband horn antenna measurements.

Similar setup was used for the CLTAs as depicted in Figure 2.

Figure 2. Measurement setup for the CLTA

The results show that when optimizing the setup to minimize noise, there is no difference in the noise level between a wideband horn antenna and the CLTA. If the DUT has no spurs above the noise level, the results are very similar. When there are spurs visible above the noise level, they are both seen by the wideband horn antenna and the CLTA. However, the wideband horn antenna is able to detect the spurs better in the lower frequencies, most likely due to wider beamwidth, and shows higher levels of spurious emissions. This is beneficial in a way that it shows wideband horn antennas have better performance than CLTAs and they can detect low signals more efficiently. However, in the higher frequencies we can see that horn antennas show weaker spurs than the CLTA. In some cases, this might lead to more failures to comply with the current limits than with CLTAs. The level of the spurious emissions is fluctuating in time, which makes it hard to get justified results. We can see that three measurements are not enough to get correct level result and there could be 10dB or more difference between the measurements. More study with better averaging is needed.

Note: In figures below positive difference indicates horn antenna gives higher value vs. CLTA

Figure 3. DUT1 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup A

Figure 4. DUT1 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup B.

Figure 5. DUT1 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup A.

Figure 6. DUT1 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup B.

Figure 7. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup A.

Figure 9. DUT2 spurious emissions differences between CLTA and MVG SH400 horn antenna per 3GPP band, setup B.

Figure 8. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup A.

Figure 10. DUT2 spurious emissions differences between CLTA and A-INFO LB-2080 horn antenna per 3GPP band, setup B.

As it is visible from the results, wideband horn antennas do not give exactly same results as CLTAs. But at this stage it is impossible to say which one gives more correct results. The spurs tend to fluctuate over time and three measurements are not enough to secure enough averaging to find the correct value of the peak of the spur. More measurements are needed to make a justified decision if wideband horn antennas are suitable to use as CLTAs. One thing to note here is that the SH400 receives different frequency bands in different parts of the antenna. The higher frequencies are received in the back part of the antenna, and it might be needed to move the horn antenna forward according to measured frequency band. During the measurements, the front part of the horn antenna was aligned with the radome of the DUT, which might not give correct results. More averaging is needed to ensure the level of spurs is on correct level. Other option would be to use a predefined spurs that could be altered in power level and frequency. If the power level and the frequency of the spurs were known, then the comparison measurement would have a great meaning.

The power level of the spurs in these measurements are fluctuating in time.

More measurements are needed to ensure the usability of wideband horn antennas as CLTAs.

## BS-to-BS isolation analysis

It was agreed in RAN4#112bis:

Way forward: BS-to-BS isolation analysis can be further discussed in this WI, with no additional objectives needed.

It has been repeatedly discussed over multiple work items that there is a need to study further if the current BS-to-BS isolation assumption is still valid with AAS deployments on higher frequencies. There have been proposals that the higher frequencies would be subject to higher isolation and just by considering the physical aspect, that is true. However, there are certain other aspects that need to be considered as well. The beam steering concept that is described in more detail in 2.3 is one of the most critical impacting factors. Secondly there needs to be agreement on the deployment scenarios that are seen as viable for further study. Also, the method of deriving different isolation levels needs to be agreed. Frequency bands where to apply the different isolation levels need to be agreed and we should not limit this to FR1 only. New bands are being introduced and the work done should cover all needed bands that are now foreseen. All these aspects need to be agreed on before starting the work.

There are many factors to be taken into consideration in BS-to-BS isolation analysis.

RAN4 to focus on agreeing on the methods how to analyze BS-to-BS isolation.

Frequency bands should not be limited to FR1.

## Beam steering consideration for co-location scenario

It was agreed in RAN4#112bis:

Way forward: Evaluation of the beam steering impacts is considered as applicable to the co-location scenarios analyses. Applicable frequency range within FR1 is FFS.

Beam steering in AAS is a crucial technology that enables improved coverage, increased capacity, enhanced signal quality, dynamic resource allocation, and reduced power consumption. These benefits contribute to a more efficient and reliable wireless network. Considering a side-by-side deployment with horizontal beam steering, where the majority of beam steering occurs and the side lobes are the worst, then the coupling loss might be completely different when the main beam is steered in the direction of the victim BS. Simulations were run with different distances between antennas to study the effect the beam steering has to the coupling between antennas.

Beam steering is a technology of paramount importance for AAS.

We did some further isolation simulations for two generic antenna arrays with simplified radome and chassis structure. Beam steering in antenna array A was done towards antenna array B for 0°, 50° and 65° for the 3800MHz frequency. Distance d between the edge of the antenna array was changed between 120mm, 300mm, 400mm and 500mm. The simulation setup is shown in Figure 4.

Figure 4. Simplified picture of the simulation setup for isolation between antenna arrays.

Table 1 demonstrates the simulation results for the worst case for each steering angle with different values for d.

Table 1. Simulation results for different beam steering values for different distances.

| Horizontal distance d (mm) | Isolation (dB) with AZ Steering = 0° | Isolation (dB) with AZ Steering = 50° | Isolation (dB) with AZ Steering = 65° |
| --- | --- | --- | --- |
| 120 | 57 | 43 | 39 |
| 300 | 59 | 47 | 41 |
| 400 | 60 | 48 | 42 |
| 500 | 60 | 48 | 43 |

Beam steering has a significant effect on the isolation level.

- It is clearly seen that the coupling is much less with beam steered to 50° or 65° than for 0°. The effect seems to be around 17dB for 65°, which seems to be the same for different distances too. It is blatantly obvious that the whole beam steering concept together with the side-, back- and grating lobes needs to be considered with respect to the deployment scenario.

Beam steering need to be considered as a parameter of major importance.

## TX IMD co-location

It was agreed in RAN4#112bis:

Way forward: Continue analysis of potential consequences due to TX IMD co-location requirement removal from Rel-19 specification. Applicable frequency boundary for TX IMD requirement removal (within FR1) is FFS.

We think it is still premature to remove TX IMD from Rel-19 specification. As the TX IMD is a regulatory requirement, a lot of caution should be considered when removing the requirement. The frequency boundary for the requirement removal should be clearly demonstrated by simulations rather than just a pen and paper exercise.

Frequency boundary for the TX IMD requirement removal needs to be clearly demonstrated by simulations before any agreement can be made.

# Conclusion

This document shares Nokia’s views on OTA test enhancements. Following observations and proposals were introduced:

- The power level of the spurs in these measurements are fluctuating in time.

- More measurements are needed to ensure the usability of wideband horn antennas as CLTAs.

- There are many factors to be taken into consideration in BS-to-BS isolation analysis.

- Beam steering is a technology of paramount importance for AAS.

- Beam steering has a significant effect on the isolation levels.

- RAN4 to focus on agreeing on the methods how to analyze BS-to-BS isolation.

- Frequency bands should not be limited to FR1.

- Beam steering need to be considered as a parameter of major importance.

- Frequency boundary for the TX IMD requirement removal needs to be clearly demonstrated by simulations before any agreement can be made.

# References

- R4-2416574 “Way Forward for [112bis][303] NR_BS_RF_Part2_CLTA”, RAN4#112bis, Hefei, China 14th – 18th October, 2024.