3GPP TSG RAN WG4 #113	R4-2418024

Orlando, USA, Nov 18 – 22, 2024

Agenda item:		7.20.1

Source:	Nokia

Title:	On SBFD general aspects and system parameters

Document for:		Approval

# Introduction

A Way Forward was agreed in RAN4#112bis [1] regarding evolution of NR duplex operation. There were not so many agreements made regarding SBFD general aspects and system parameters. Also, the open items are listed in the WF. The most important topic is related to subband configurations and guardbands. This contribution displays Nokia’s view on SBFD general aspects and system parameters.

# Discussion

## CLI handling impact on RF requirement

The following was agreed in RAN4#112 and remained open for RAN4#112bis:

- Agreement: - RAN4 shall define RF requirements for the frequency outside of wanted carrier based on no gNB-to-gNB CLI handling as baseline - FFS Revisit particular RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

Regarding the FFS, our preference is to define the BS RF requirements without considering the potential impact of gNB-to-gNB CLI handling mechanisms, i.e., the FFS should not be considered. First, for the sake of progress we should not risk slowing down the work in RAN4 with revisiting some requirements pending on RAN1 progress. Secondly, the CLI handling schemes discussed in RAN1 are mainly applicable to co-channel CLI and their applicability to adjacent channel CLI is questionable. In fact, RAN1 discussions on gNB-to-gNB CLI handling currently focuses on enabling time/frequency coordination and spatial coordination (beam nulling and beam pairing). All the discussed mechanisms require, at least,

i) exchange of CLI-RS configurations across gNBs,

ii) identification of aggressor gNBs via measurements,

and iii) exchange across gNBs of measurement results and additional information to enable the aggressor-victim coordination.

All the above rely on signaling between gNBs over the Xn interface (intra-operator signaling). However, there is no standardized inter-operator interface and therefore exchange of information between operators is not possible. Moreover, the discussed CLI handling schemes are only optional features that are not expected to be supported by all the base stations in the field. Lastly, the impact of each CLI handling on the RF requirements is quite challenging to measure due to the fact that it is quite dependent on the specific deployment, load conditions, frequency of the information exchange across nodes, etc. The arguments above support the following proposal:

RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

## Specification implementation for band/channel bandwidth support for SBFD

The following was agreed in RAN4#112bis:

Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported

As RAN4 has not agreed yet if SBFD would have a new dedicated specification for SBFD-capable BS RF requirements or sub-clauses for TS 38.104 would be used, we believe that there cannot be agreement to include the text in Clause 5.2 in TS 38.104. If a new specification will be introduced, then this text is not needed in TS 38.104, it should be included in the new specification.

On the FFS the band for NR-U shall be supported, our view is that NR-U bands should be excluded. First of all, NR-U (unlicensed) bands are varied from other licensed bands and are standardized in RAN4 under specific conditions. One of the crucial differences is usage of LBT (Listen Before Talk) mechanism in unlicensed spectrum which is often mandated by spectrum regulators. Also, after introduction of NR-U bands into NR specifications, many features exclude unlicensed bands (for example IAB, Repeaters, Network controlled-repeaters etc.). It should be noted that one of the use cases of NR-U is to compete with Wi-Fi standards in unlicensed bands where cost, size and complexity matters. Adding to NR-U products SBFD capability (even assuming technical possibility of implementatin) would complex these products significantly.

NR-U bands should be excluded from SBFD supported bands.

## Transmission bandwidth configuration for SBFD

The following was agreed in RAN4#112bis:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

We feel that the baseline for the figures does indeed well describe the transmission bandwidth configuration. Revisiting the DUD figure based on the updated definition of NRB,SBFD,DL could be done for clarification depending on the used acronyms. Lower DL sub-band could be changed e.g. to NRB,SBFD,DL1 and the upper DL sub-band could be changed to NRB,SBFD,DL2. These changes should also be introduced the following way if there is a need for distinguishing them:

- NRB,SBFD,DL1: Transmission bandwidth configuration for SBFD lower DL subband for DUD configuration, expressed in resource blocks.

- NRB,SBFD,DL2: Transmission bandwidth configuration for SBFD upper DL subband for DUD configuration, expressed in resource blocks.

If needed to distinguish, define lower and upper DL sub-bands the following way:

- NRB,SBFD,DL1: Transmission bandwidth configuration for SBFD lower DL subband for DUD configuration, expressed in resource blocks.

- NRB,SBFD,DL2: Transmission bandwidth configuration for SBFD upper DL subband for DUD configuration, expressed in resource blocks.

## DL/UL subband configurations support for SBFD

The following was agreed in RAN4#112bis:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

We agree with Alt-1. Alt-2 is merely just the option 2b without any real restriction. The UL being in the middle is really not a compromise at all as there really is no benefit of having it anywhere else. The reasons for going for restricted configurations have been justified already in earlier contributions. One thing to note is that we have only studied the 80:20 split of DL/UL. Other configurations for DUD have not been studied during the study item phase. We propose to follow the traditional safe approach of RAN4 and start with limited set of configurations and add more in the future if there is a need. One must not forget that SBFD is a very controversial technique to be introduced, and extreme caution is needed when defining the specifications to ensure functionality. RAN4 cannot follow the RAN1 approach where it is natural to have as much flexibility as possible. RAN4 must ensure that what is specified will result in a functional system when deployed in the field. For testing purposes, we need to limit the tested configurations. If we know the configurations, it is feasible to limit testing to the worst case to cover all other cases. Without this knowledge, there is no such possibility. The required number of needed test models would explode, making it impossible to cover all. Testing complexity would increase and testing time as well. Instead of adding numerous possibilities for manufacturer declarations, RAN4 should do the exact opposite and reduce the number of declarations. For regulators, the manufacturer declarations are becoming unacceptable. For example, when ETSI is doing harmonized standards, the manufacturer declarations are not accepted by the European Commission. On top of all the above, adding flexibility in configurations will lead to a situation that every single vendor might have a different configuration and ensuring the functionality in multi-vendor environment could be impossible. Considering all of the above, our proposal is Alt-1 with defining fixed guardbands and choosing the UL to be ~20% of the channel bandwidth.

For clarification, the process would be as follows with simplified example for 100MHz DUD case:

- UL ratio of ~20% would realize in ~20MHz UL subband that would be placed in the middle of the channel bandwidth.

- Fixed guard bands (start with the assumption of 5RBs as in the SI) would be allocated.

- Rest of available bandwidth would be allocated to lower and upper DL subbands.

This would lead to only one DL subband size that will be declared and tested.

Alt-2 is basically just option 2b without any additional restrictions.

There are multiple reasons why RAN4 should not allow too much flexibility.

Fixed guardbands should be used to ensure functionality in the field.

Additional manufacturer declarations are not desired, and they should be avoided.

RAN4 to agree on Alt-1 as the configuration method with fixed guard bands.

One DL subband size to be declared and tested.

In our view Alt-1 would be also applicable for FR1 MR BS. There could be added flexibility for FR1 LA BS as it is seen as more feasible for SBFD use due to different deployments, lower power levels etc. For FR2-1, there is still need for further study.

Apply Alt-1 also for FR1 MR BS subband configuration.

More flexibility for subband configuration can be considered for FR1 LA BS.

Subband configurations for FR2-1 is still FFS.

The same method for configuring UD and DU would be utilized as for DUD with the exception of placing the UL in the bottom of the channel bandwidth for UD and top of the bandwidth for DU. Example for 100MHz UD case:

- UL ratio of ~20% would realize in ~20MHz UL subband that would be placed in the bottom of the channel bandwidth.

- Needed guard band would be allocated according to what is required by the implementation.

- Rest of available bandwidth would be allocated to upper DL subband.

This would lead to only one DL subband size that will be declared and tested.

For UD/DU configuration, RAN4 to apply Alt-1 with modification of placing the UL subband to either bottom for UD or top for DU. Otherwise configuration follows Alt-1.

# Conclusion

In this contribution, our views on the SBFD BS RF requirements are presented. The following proposals were listed.

- Alt-2 is basically just option 2b without any additional restrictions.

- Alt-1 will solve the issue of different guard bands for different manufacturer implementations.

- There are multiple reasons why RAN4 should not allow too much flexibility.

- Additional manufacturer declarations are not desired, and they should be avoided.

- RAN4 to not revisit RF requirements pending on RAN1 progress on the gNB-to-gNB CLI handling work.

NR-U bands should be excluded from SBFD supported bands.

If needed to distinguish, define lower and upper DL sub-bands the following way:

- NRB,SBFD,DL1: Transmission bandwidth configuration for SBFD lower DL subband for DUD configuration, expressed in resource blocks.

- NRB,SBFD,DL2: Transmission bandwidth configuration for SBFD upper DL subband for DUD configuration, expressed in resource blocks.

RAN4 to agree on Alt-1 as the configuration method with fixed guard bands.

One DL subband size to be declared and tested.

Apply Alt-1 also for FR1 MR BS subband configuration.

More flexibility for subband configuration can be considered for FR1 LA BS.

Subband configurations for FR2-1 is still FFS.

For UD/DU configuration, RAN4 to apply Alt-1 with modification of placing the UL subband to either bottom for UD or top for DU. Otherwise configuration follows Alt-1.

# References

- R4-2416584 “Way Forward for [112bis][306] NR_duplex_evo_BSRF”, RAN4#113, Hefei, China, 14th-18th October, 2024