3GPP TSG-RAN WG4 Meeting #113	R4-2418043

Orlando, Florida, US, 18th – 22nd November 2024

Source:	Nokia

Title:	Discussion on Spatial Channel Model for Demodulation Performance Requirements

Agenda item:	7.14.2

Document for:	Discussion

Introduction

The study item on Spatial Channel Model (SCM) for demodulation performance requirements (FS_NR_demod_SCM) progressed satisfactorily in RAN4#112bis, resulting in an agreed WF [1].

This contribution addresses the open issues captured in the last WF [1], in particular the comparison setups and initial results. Furthermore, we update and extent our SU/MU SCM observations from the prior contributions [2] and [4].

We want to further highlight our companion simulation results contribution [6].	
Please also refer to our companion contribution [6] for an up-to-date TP-style description of the 827 CDL channel model, and implementation help.

Only section 4 contains proposals for the ongoing discussion, the prior sections contain background information and observations from our comparison campaign.

CDL based SCM

In this section we extend the results and observations presented in [2] and [4]. The observations will prove to be useful to advance open issues and clarify some topics in the study item.	
We will not repeat the background on TDL and CDL, please see  [2] for TDL. However, we will extend some observation about CDL in this section.

Virtualizers

Virtualizer concept

During RAN4#112bis we have introduced the general concept of “virtualizers” to the CDL discussion. With the full-term being Antenna Array Virtualisation (AAV), which was based on the “Antenna virtualization” used in TR 38.901.

Their use it to:

Antenna Array Virtualisation defines the connections and weights between BB/RF ports (e.g., CSI-RS or SRS ports) and antenna elements of an antenna array or panel.

Fully and partially connected virtualizers

For more background on fully and partially connected virtualizers, we recommend TR 36.897.	
Concerning the issues at hand, we distinguish two cases:

TR 38.827 Fully connected TX virtualizer + directional steering

In TR 38.827 a fully connected virtualizer (per polarization) is defined via the formulation

α1,1…αMe,Ne=1MeNeexp-j2πrtx,max T ∙ dtx,me,neλ0∈C1×MeNe.

This is applied to each BB/RF port, with different rtx,max  per port pair. Hence, each port pair has a separate directional steering (direction of departure, DoD) using the full antenna array. The virtualizer is not changed dynamically during the channel emulation, and rather set to match DoDs that result in maximized received power at the receiver in the given large-scale environment.	
The connection framework can be quickly visualized in Figure 1.

Figure 1: TR 38.827 fully connected TX virtualizer (+ directional steering).

It is currently unclear, why 38.827 has chosen this specific virtualizer. It remains unknown if this constitutes a limitation in the MPAC minimum test setup assumption.

TR38.901 Partially connected (subarray partition) TX virtualizer

In TR 38.901 it is most common to assume subarray partition virtualizers with directional steering/tilting on a per antenna array column basis. As we aim to represent in Figure 2, each BB/RF port is only connected to a subarray of the TX panel(s) and less weights are chosen. Often the weights are found using URA electrical downtilt formulas, e.g., as in the next subsection.

Using the matrix representation of the fully connected virtualizer it is straightforward to apply the same framework and judicious use of “0” weights, albeit that an implementation would not use zero weights, but rather not process these parts of the time domain channel coefficient generation.

Figure 2: Partially connected (subarray partition) TX virtualizer.

Summary

From the reference above we observe:

Both fully and partially connected virtualizer have precedence in TR 38.901 and TR 38.827.

Legacy downtilt subarray partition virtualizers in 38.827 framework

During RAN4#112bis, a new option for virtualizer weights was brought up. It consists in using the TR 38.901 legacy downtilt subarray partition virtualizers in the TR 38.827 framework, meaning to

- Use single column subarrays of AEs, one for each BB/RF port pair, in a partially connect configuration.

- Use TR 38.901 (section 7.3.1) antenna port mapping for each subarray:	
 	
Note that TR 36.897 defines such port mappings for general URA subpanel cases.

- Continue to use the randomness reducing framework from TR 38.827, i.e., SSF changes, fixed subpaths, weak clusters, etc.

These weights can easily be used in the framework of Figure 2.

Legacy no virtualizer

Yet another option discussed in RAN4#113 was to use the 38.827 framework, but to forgo the virtualizer completely, and replace it with direct BB/RF port to ULA AE mapping (meaning usage of less AEs overall). See Figure 3.

Figure 3: No AAV/no TX virtualizer.

Precoder selection and DoA for 827 CDL

One discussion that has been had repeatedly since considering 38.827 CDL, was whether the precoding vector selection (and following the typeI CB PMIs in particular) impacts the spatial properties of the RE-domain channel matrix seen at the receiver.

To answer this question, we look at the directions of arrival (DoAs) that can be measured at the receiver using DM-RS i.e., capturing both precoder and channel. As in our prior contribution [2], the DoAs are estimated using Bartlett analysis.

In Figure 4 it can be readily seen that changing the precoding direction of a layer, result in that layer being received from different DoAs (same as AoA in ULA).

In 38.827 CDL the precoding vector selection impacts the spatial properties of the RE-domain channel matrix seen at the receiver, i.e., changing the precoding direction of a layer, changes its receive direction.

| typeI PMI | Port selection precoder |
| --- | --- |

Figure 4: TR 38.827 based CDLC Uma Bartlett DoA analysis vs. precoding vector/layer (x-axis is AoA).
4x1 Xpol ULA receiver assumption.

Many more results can be found in our companion simulation paper [6].

Convergence speed/testing time

To gauge potential testing-time increase issues with the CDL channel models, we have run a trial simulation that uses a single draw/seed of 10000 slots and calculates the TPUT using 10/100/250/…/10000 slots.

To enable fair comparison, we tried to choose a 50%TPUT operating point for all channel models, however for TDLC (medA) the channel is very rank deficient, and we could not go above 32% even with MCS reduced to 7:

CDLC @ 22dB (52% TPUT) / TDLClow @ 19dB (50% TPUT) / TDLCmedA @ 50dB/MCS7 (32% TPUT)

Figure 5: Simulation convergence trials: 8T8R 2CW (MCS17 both) full rank, SU-MMSE receiver, Random precoding vs. simulated slots (at 50% TPUT operating point target).

Numerical stability for 827 CDL

To gauge potential issues with numerical stability of 38.827 CDL, we have run trial simulations with

- Different starting seeds

- Redrawing scalar random initial phase terms Φn,m~Uni(-π,π) every X slots and comparing to no redraw.

Different starting seeds

10000 slots for each SNR point with differing starting seeds.

Initial random seeds have a negligible impact on the TPUT performance scaling of 837 CDL.

Figure 6: CDLC 365-100 differences between randomness seeds.

Redrawing scalar random initial phase terms

Redrawing scalar random initial phase terms Φn,m~Uni(-π,π) every 100 slots, or never redrawing. 10000 slots for each SNR point and two different initial random seeds.

Redrawing scalar random initial phase terms does have a systematic impact on 827 CDL, that reduces with less redraw frequency (not redrawing for 1000 slots starts to be fine).

| Seed 1 | Seed 2 |
| --- | --- |

Figure 7: CDLC 365-100 differences from redrawing scalar random initial phase terms.

Note: Unpublished results show that 100 slots are not enough for the performance to settle after an initial phase redraw. Starting from 1000 performance has converged, i.e., 10x1000 would be acceptable and close to 1x10000.

Spatially consistent MU extension for 827 (and 901) CDL

Background of spatial consistency and 38.901

To evaluate many MU related TPUT and PMI impacts, multi cell MU configurations are needed for the requirement setup. In legacy TDL configurations, such a setup is created by drawing independent realizations of the TDL channel model and applying them to the tx-rx links independently. In TDL low this creates links that are perfectly statistical independent, no matter if the receivers should be spatially correlated or not. In TDL+antCorr the small-scale fading is still independent, but the AoDs and AoAs are the precisely same for all links, i.e., signal and interference are sent and received from the same directions.

TR 38.827 CDL is currently only described for TR 38.827 setups, where the signals are transmitted and received in spatially separable directions. To extend this to the needs of multi-cell MU, the created links need to have different spatial supports, but the differences between supports need to be spatially consistent with respect to the relative positions of target and interference receivers.

Since CDL is a SCM, it is in principle sufficient to obtain spatial consistency by offsetting et the azimuth and zenith angles of departure and arrival (AoD, AoA, ZoD, ZoA) between UEs, to achieve spatially separated UEs with correct large scale channel similarities.

However, traditionally channel models supporting inter-UE interference are drop based system simulators: A drop represents a channel realization at some instant, when the channel properties obey a specified distribution but are statistically independent between drops, and therefore lack spatial consistency.

Consequently, as a user moves along a trajectory, channel realizations will be discontinuous, and do not represent real world behaviour. The same is true for placing users in spatial relations. Since beam tracking and beamforming algorithms rely on the gradual evolution of the channel, drop-based models are therefore not suitable.

TS 38.901 7.6.3 describes two different spatial consistency procedures:

Method A (preferred): Is a deterministic procedure based on linear extrapolation, to predict the change in the geometrical properties of the channel between locations, where the amount of change is proportional to the distance travelled between the locations. Specifically, the procedure updates the AoD, AoA, and delay at given intervals (usually less than a meter) as the UE travels along a trajectory, based on the properties from the previous interval. The small-scale parameters are then updated along between the first and last cluster locations, for which the large-scale parameters are assumed to be constant.

Method B: Unlike the fully deterministic Method A procedure, which generates the channel properties for the first location and then updates them incrementally for the other locations, Method B generates the channel properties for all locations at once. To do this, all locations are correlated, with correlation decreasing with distance. The benefit of this procedure is that SC is imposed on all locations before the channel properties are generated, meaning the channel properties are correlated a priori and need not be updated incrementally.

Steps of the procedure

Figure 10 illustrates the general MU set-up with to two additional UEs having low and high spatial correlation with the target user:

Figure 10: Modelling of inter-user interference with high (Close) and low (Far) spatial correlation with the target UE.

Thus, whilst the initial focus of the FS_NR_demod_SCM SI might be based upon baseline CDL-C UMa, it is possible to modify the AODs, AOAs and ZODs to provide different levels of spatial correlations for different receivers in MU cases.

RAN4 to consider how SCM candidates can be modified to allow separately, but spatially consistently, correlated channels for different AODs, AOAs and ZODs corresponding to different multiple UEs.

Using the spatial consistency procedures from TS 38.901, the modelling of inter-user interference in a MU-MIMO environment, i.e., between two or more users, while maintaining spatial consistency can then be done as follows:

Step 1: Define a target user, UE_target, using the CDL channel model parameter tables from TR 38.827 (e.g. for the channel parameters for CDL-A, B, C, D, and E for UMa and Umi).

An example of a UE_target CDL channel model parameter table from TR 38.827 is show below:

TR 38.827 Table 7.2.1-6: Channel model parameters for UMa CDL-A at 3.5 GHz

| Cluster # | Absolute Delay [ns] | Power in [dB] | AOD in [°] | AOA in [°] | ZOD in [°] | ZOA in [°] |
| --- | --- | --- | --- | --- | --- | --- |
| 1 | 0 | -13.4014 | -63.5923 | 70.1754 | 89.1998 | 90 |
| 2 | 139.3935 | 0 | -2.804 | -154.231 | 96.5746 | 90 |
| 3 | 146.9125 | -2.2185 | -2.804 | -154.231 | 96.5746 | 90 |
| 4 | 214.182 | -3.9794 | -2.804 | -154.231 | 96.5746 | 90 |
| 5 | 168.265 | -5.9799 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 6 | 196.1875 | -8.1984 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 7 | 244.842 | -9.9593 | 30.1944 | 92.1659 | 101.5139 | 90 |
| 8 | 209.875 | -10.5014 | 41.1356 | -23.0699 | 106.3504 | 90 |
| 9 | 278.057 | -7.5014 | -29.8948 | -57.9245 | 90.0573 | 90 |
| 10 | 561.1875 | -15.9014 | 54.0343 | 107.4637 | 85.1179 | 90 |
| 11 | 692.697 | -6.6014 | -30.3493 | 70.6969 | 102.2686 | 90 |
| 12 | 811.833 | -16.7014 | 45.7847 | -122.245 | 110.0206 | 90 |
| 13 | 792.707 | -12.4014 | -54.8184 | 48.7064 | 106.5562 | 90 |
| 14 | 910.383 | -15.2014 | -61.46 | 92.1659 | 107.551 | 90 |
| 15 | 916.8435 | -10.8014 | -46.7436 | -27.5897 | 88.6853 | 90 |
| 16 | 1116.243 | -11.3014 | -48.8759 | -41.4968 | 87.519 | 90 |
| 17 | 1489.565 | -12.7014 | 56.4812 | -62.5312 | 88.0164 | 90 |
| 18 | 1627.1335 | -16.2014 | 50.5387 | 121.5446 | 108.3399 | 90 |
| 19 | 1667.8675 | -18.3014 | 45.0506 | 151.184 | 82.4424 | 90 |
| 20 | 1750.759 | -18.9014 | -42.7936 | 160.5712 | 83.4543 | 90 |
| 21 | 1827.409 | -16.6014 | -55.2029 | 114.2434 | 110.0378 | 90 |
| 22 | 1936.0695 | -19.9014 | 42.8834 | -153.449 | 84.4834 | 90 |
| 23 | 3525.389 | -29.7014 | -20.9811 | 73.5652 | 105.4414 | 90 |

Figure 8: Example UE_target Channel model parameters for UMa CDL-A from TR 38.827.

Step 2: Define an additional inter-user UE_close closely located (e.g. 5 m) to UE_target

This user is intended to have a high spatial correlation with the target user. It is therefore placed in close vicinity to the target user.

Optionally chose the location in relation to UE_target spatial support, meaning that for scheduling with highest interference the distance between UE_target and UE_close is taken in the direction of UE_target support. For lowest interference it is taken orthogonally. Choices in between are valid.

Step 3: Define an additional inter-user UE_far located far away (e.g. 100 m) from UE_target

This user is intended to have a low spatial correlation with the target user. It is therefore placed far away from the target user.

Optionally chose the location in relation to UE_target spatial support, meaning that for scheduling with highest interference the distance between UE_target and UE_far is taken in the direction of UE_target support. For lowest interference it is taken orthogonally. Choices in between are valid.

Step 4 (only for multi-cell): Channel models need to be created for all UE with respect to each cell. I.e., UE_close and UE_far may become UE_target in another cell. In this case distances and relation to spatial support are to be chosen reciprocally.

Step 5: Generate new channel model parameter tables for UE_close and UE_far using one of the spatial consistency procedures as described in TS 38.901 section 7.6.3.

Either Method A or Method B can be used to generate new spatially consistent channel model parameter tables for UE_close and UE_far. However, based on actual channel measurements in various indoor and outdoor environments, it has recently been shown that the fully deterministic procedure, Method A, provides a much better approximation to the measured channel in all measured channel environments than Method B, which is a fully stochastic procedure [8]. Therefore, Method A is the preferred spatial consistency procedure for MU extension of the TR 38.827 tables.

Resulting CDLC UMa tables for MU SCM

A practical example of selected new channel model parameter tables for UE_close and UE_far using the spatial consistency procedure Method A as described in TS 38.901 7.6.3. is shown in Figure 9:

|  | UE Target |  |  | UE Close |  |  | UE Far |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Cluster # | AOD [°] | AOA [°] | ZOD [°] | AOD [°] | AOA [°] | ZOD [°] | AOD [°] | AOA [°] | ZOD [°] |
| 1 | -63.5923 | 70.1754 | 89.1998 | -64.03 | 70.373 | 89.202 | -62.52 | 69.604 | 89.193 |
| 2 | -2.804 | -154.231 | 96.5746 | -3.9398 | -153.74 | 96.691 | -10.939 | -150.14 | 97.497 |
| 3 | -2.804 | -154.231 | 96.5746 | -3.8466 | -153.78 | 96.681 | -0.45606 | -155.22 | 96.339 |
| 4 | -2.804 | -154.231 | 96.5746 | -3.4147 | -153.96 | 96.637 | 8.8481 | -159.71 | 95.353 |
| 5 | 30.1944 | 92.1659 | 101.5139 | 29.897 | 93.001 | 101.73 | 31.274 | 89.14 | 100.72 |
| 6 | 30.1944 | 92.1659 | 101.5139 | 29.954 | 92.84 | 101.69 | 49.72 | 37.962 | 87.153 |
| 7 | 30.1944 | 92.1659 | 101.5139 | 30.014 | 92.673 | 101.65 | 30.773 | 90.577 | 101.09 |
| 8 | 41.1356 | -23.0699 | 106.3504 | 41.076 | -23.848 | 106.59 | 41.533 | -17.813 | 104.76 |
| 9 | -29.8948 | -57.9245 | 90.0573 | -30.465 | -58.499 | 90.057 | -36.103 | -64.43 | 90.059 |
| 10 | 54.0343 | 107.4637 | 85.1179 | 54.076 | 107.7 | 85.095 | 53.7 | 105.66 | 85.296 |
| 11 | -30.3493 | 70.6969 | 102.2686 | -30.56 | 70.789 | 102.28 | -34.136 | 72.276 | 102.53 |
| 12 | 45.7847 | -122.245 | 110.0206 | 45.787 | -122.28 | 110.08 | 45.82 | -122.88 | 110.88 |
| 13 | -54.8184 | 48.7064 | 106.5562 | -55.008 | 48.718 | 106.55 | -57.045 | 48.83 | 106.5 |
| 14 | -61.46 | 92.1659 | 107.551 | -61.621 | 92.283 | 107.54 | -64.26 | 94.066 | 107.36 |
| 15 | -46.7436 | -27.5897 | 88.6853 | -46.902 | -27.741 | 88.685 | -44.419 | -25.318 | 88.68 |
| 16 | -48.8759 | -41.4968 | 87.519 | -49.005 | -41.626 | 87.519 | -43.61 | -36.207 | 87.495 |
| 17 | 56.4812 | -62.5312 | 88.0164 | 56.5 | -62.623 | 88.013 | 55.654 | -58.981 | 88.157 |
| 18 | 50.5387 | 121.5446 | 108.3399 | 50.548 | 121.63 | 108.37 | 50.171 | 118.4 | 107.24 |
| 19 | 45.0506 | 151.184 | 82.4424 | 45.051 | 151.27 | 82.431 | 45.044 | 145.06 | 83.377 |
| 20 | -42.7936 | 160.5712 | 83.4543 | -42.875 | 160.64 | 83.454 | -65.432 | -179.59 | 83.471 |
| 21 | -55.2029 | 114.2434 | 110.0378 | -55.284 | 114.32 | 110.03 | -8.4316 | 70.437 | 113.8 |
| 22 | 42.8834 | -153.449 | 84.4834 | 42.881 | -153.43 | 84.476 | 42.637 | -151.47 | 83.845 |
| 23 | -20.9811 | 73.5652 | 105.4414 | -21.019 | 73.584 | 105.45 | -22.074 | 74.119 | 105.56 |

Figure 9: Example CDLA UMa AoD/AoA/ZoD for target and close/far receivers. ZoA is 90° as per TR 38.827.

If deemed necessary by RAN4,the tables in Figure 9 can be extended to include additional Absolute Delay and Power scaling in addition to the AOD, AOA and ZOD angle scaling.

Measured PDP and RMS DS

To confirm the RMS DS of channel model candidates, it is instructive to calculate the PDP of measured DM-RS symbols and calculate the RMS DS from the measured PDP. It is possible to mathematically transform perfect channel coefficient (in the RE domain) to the time domain, however we opted for measurements at very high SNR to reduce the number of sources for systematic errors.

Channel models with principal beam direction steering modifications, can also use such measurement to gauge the impact of this steering on the DS. It is understood that a fixed directional receive antenna setup in a channel model (e.g., array factors or antenna correlations of a fixed ULA setup), coupled with changing DoAs from directional steering, changes the powers received from different angles/clusters/delays. 	
Hence, in such a channel model, the RMS DS is changing over the course of sample generation, in accordance with the principal direction steering.

The CDL models from 38.901 and 38.827 anticipate this effect and have delay scaling procedures to be applied with changing DoAs and DoDs to keep the DS matched with the desired value (see, for example TR 38.827 section 7.2).

We show this exercise here for TDL low with and without DoD beamsteering. 	
Note that this case does not show impact from principal direction steering, since (1) only DoD steering is applied in (2) a channel model that does not have spatial selectivity in either DoD or DoA, i.e., the effective per tap/cluster powers do not change with changing directions:

| a) Without beam steering [22.7355 18.0173 18.9006 17.6107 17.1727 16.9964 20.2687 22.1507 ] * 16e-9 b) With DoD beam steering [22.7355 18.0173 18.9006 17.6107 17.1727 16.9964 20.2687 22.1507] * 16 e-9 Average RMS DS is 310ns. |  |
| --- | --- |

Figure 11: Calculated RMS DS based on PDP measurement for TDLC300-100 low.

CDL comparisons and observations

In WF [1] an initial set of used cases and configurations were agreed for SCM comparison case (w.r.t. legacy and SCM candidates). We discuss all requested baseline comparisons, as well as voluntary extensions, in this section. It will be of use for TR drafting decisions in RAN4#113 and to answer open issues.

SU PDSCH

We define the baseline comparison as per WF, including all FFS.

Baseline: 2CW

For the 2CW, 8T8R, full rank, comparison case we first compare (827) CDL with legacy TDL (low and MedA correlation).

Figure 12 shows that only CDL can reproduce the expected inter-CW performance difference at feasible SNR levels, which is needed to fully test a 2CW SDM receiver and correctly model performance scaling in deployment.	
TDL low does not show any CW performance differences, while TDL non-low cannot achieve reasonable SNR levels for 8T8R systems due to deficiencies in the correlation model (but it would likely have shown inter-CW differences).

For 2CW features, only 827 CDL can create the expected inter-CW performance difference at feasible SNR levels, which is needed to fully test a 2CW SDM receiver and correctly model performance scaling in deployment.

|  | Cannot achieve meaningful TPUT |  |
| --- | --- | --- |
| TDLC300-100 low (MCS17) | TDLC300-100 MedA (MCS17) | 827 CDLC UMa (MCS17) |
|  | Cannot achieve 70% TPUT |  |
| TDLC300-100 low (MCS13) (Fixed PMI from histogram max.) | TDLC300-100 XP high (MCS13) |  |
|  | TDLC300-100 MedA (MCS7) | 827 CDLC UMa (MCS7) |

Figure 12: 2CW Legacy vs. 827 CDL: (MCS17 top, MCS13 mid, MCS7 bottom) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR.

When comparing different CDL variants, we have chosen “vanilla 827 CDLC Uma with UE antenna assumptions from Maastricht” (827 CDLC UMa), 827 CDL with partially connected array (one column per CSI-RS port), and 827 with no AAV. Please see prior sections for details on these virtualizers.

Figure 13 shows our results and it must be noted that we chose the fixe PMI choice following the PMI profiles of hypothetical follow_PMI choices (most common index), given each virtualizer choice. See coming sections for details on such PMI histograms. 	
The figure in question re-used the 827 AAV most frequent PMI for all channel variants, but the random precoding curves and serve as a measure for the expected SNR requirements. It shows that using “827 fully connected AAV” results in the least SNR requirements, hence the largest testable range.

It is feasible to make a fixed PMI choice based on the most frequently chosen follow_PMI under a given channel model.

All three evaluated virtualizer options (827 AAV, mech. downtilt AAV, no AAV) show the expected impact of a large-scale environment, with 827 AAV having the largest testable range/least SNR requirements, followed by no AAV at +6dB and downtilt AAV at +12dB.

| 70% TPUT Fixed PMI CW0 16.7 dB Fixed PMI CW1 24.0 dB Random PMI CW0 11.1 dB Random PMI CW1 20.2 dB | 70% TPUT Fixed PMI CW0 30.9 dB Fixed PMI CW1 15.3 dB Random PMI CW0 23.4 dB Random PMI CW1 32.0 dB | 70% TPUT Fixed PMI CW0 26.5 dB Fixed PMI CW1 15.2 dB Random PMI CW0 20.6 dB Random PMI CW1 26.8 dB |
| --- | --- | --- |
| 827 CDLC UMa (827 AAV) | 827 CDLC UMa (downtilt AAV) | 827 CDLC UMa (no AAV) |

Figure 13: CDL Candidates: (MCS13) full rank relative TPUT, SU-MMSE receiver, Fixed and random precoding vs. pre-EQ SNR.

SNR operating points in AAV alternatives

Our explanation for the differences in SNR budgets for the AAV alternatives is as follows, and it is not as evident as one might expect, due to RAN4 SNR measurement/setting definitions.

Any SNR budget gains cannot stem from exploiting the gain of strong channel directions, as the RAN4 SNR definition normalizes average receive branch power, so SNR budget gains arise from the channel model (a) providing spatially diverse environments that the SDM receiver can efficiently equalize, and (b) providing a less uneven per layer signal quality.

Additional: Sensitivity to receiver implementations

We compare the TPUT sensitivity of CDL, TDL, and TDL+antCorr with respect to receiver implementation differences, i.e., we look at whether the legacy TPUT demod requirements are fit to detect issues with SDM receiver implementations.

In particular, we look at covariance matrix estimation implementations specifically, and thus, we test two versions of SU MMSE (“IRC” in RAN4 demod terminology) for TPUT performance. We denote with “MMSE CovD” and “MMSE CovDI”, MMSE receivers with covariance matrix averaged between REs, and re-use of instantaneous value between REs, respectively.	
Results are given in Figure 15. See observe that for legacy TDL channel models, there is insufficient TPUT performance difference to distinguish between different SCM receiver implementations.

For SU TPUT use cases, legacy TDL channel model-based demodulation performance requirements are neither fit to test receiver functionality, nor fit to set a stable minimum performance requirement. Functionally differing SDM receiver implementation show same performance under legacy TDL channel requirements, but SCMs with spatially typical properties (in this case 827 CDL) can meaningfully differentiate between implementations, even under random precoding.

| TDLC300-100 low | TDLC300-100 MedA (MCS 7) |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 15: 4T4R, 1CW (MCS17) full rank, random precoding, Differing SDM receiver implementation. 
Relative TPUT vs. pre-EQ SNR.

SU PMI

We define the baseline comparison as per WF, including all FFS.

Additional: PMI profiles

As we previously noted, our fixed precoder choices are based on follow PMI profiles/histograms, i.e., the frequency with which a certain PMI is chosen for a given channel model, assuming our UE CSI/PMI selection algorithm.

The following figures we present the histograms of TypeI PMI choices (max TPUT criterion) in a varying BB branches and layer configurations, and under different channel models. 	
“Randomization of the principal beam direction” is not applied as it was arguably never implemented in many TEs and many legacy simulation campaigns (as was discussed during the Demod maintenance sessions in RAN4#112 [7]). Additionally, it has implications on the RMS delay spread correctness during the test.

4Tx4Rx 4Layers

| TDLC300-10 low SNR=15dB => CW Throughput = 96.07% | TDLC300-10 XP high SNR=25dB => CW Throughput = 91.38% |
| --- | --- |
| 827 CDLC UMa, DS 365ns, 10 Hz Doppler, ULA Rx SNR=14dB => CW Throughput = 92.35% |  |

Figure 16: Histograms of PMI feedback choices under different channel models (2000 CSI reports).
4T4R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=2, N2=1, O1=4, O2=1.

8Tx8Rx 8Layers

| TDLC300-10 low SNR=16dB => Throughput = CW0 90.3% CW1 90.3% | TDLC300-10 XP high SNR=40dB => Throughput = CW0 2%, CW1 73.6% |
| --- | --- |
| 827 CDLC UMa ULA, 10Hz SNR=22dB => Throughput = CW0 74.9%, CW1 78.15% | 827 CDLC UMa ULA, 10Hz SNR=25dB => Throughput = CW0 87.4%, CW1 98.5 |

Figure 17: Histograms of PMI feedback choices under different channel models (2000 CSI reports).
8T8R, 1CW, MCS13, full rank, PMI chosen based on (non-precoded) CSI-RS, N1=4, N2=1, O1=4, O2=1.

Summary

The results may be summarized as follows:

- TDL low

  - The capacity optimal PMI choice is seemingly random, reflecting the outwardly random spatial properties of TDL low.

  - This explains the observation that PMI requirements with TDL low have a gamma ratio (follow PMI tput vs. random tput) of about 1, i.e., the precoding direction does not matter even for low 10Hz Doppler configurations.

- TDL+antCorr

  - The capacity optimal PMI choice is randomizing between ULA broadsight (PMI 0) and its co-phasing variant (PMI 1), and the ULA symmetric variants (PMI 8+9).

  - For 8Rx/8layers, the TPUT goal is not achievable, so PMI choices are more random and not useful for statistics.

- 827 CDL

  - The capacity optimal PMI choice is switching between the defined and stable strong directions of the channel model, with variation introduced by the small scall fading component.

Thus, we reach the observations of:

The capacity optimal TypeI PMI choice is random in TDL low, fixed to broadsight in TDL+antCorr, and following defined and stable strong directions in CDL. Thus, only SCM model-based requirements can provide evidence for correct algorithmic function and set reasonable minimum performance expectations.

It is possible to derive a reasonable fixed precoder setting for all tested channel models, by choosing the most frequent choice of the follow_PMI histogram for each channel model.

Baseline: 4Rx and 2/4 layer

CDL AAV variations

The baseline results are mostly interesting from a CDL alternatives pov.	
We have chosen to implement “vanilla 827 CDLC Uma with UE antenna assumptions from Maastricht” (827 CDLC UMa), and 827 with no AAV. Please see prior sections for details on these virtualizers.

| Follow PMI 90% TP 5.3dB Random PMI TP @5.3 dB 76.6% Γ = 1.18 | Follow PMI 90% TP 4.82dB Random PMI TP @4.81 dB 58.3% Γ = 1.54 | Follow PMI 90% TP 4.81dB Random PMI TP @4.81 dB 55.8% Γ = 1.61 |
| --- | --- | --- |
| 827 CDLC UMa (rank 2) | 827 CDLC UMa (downtilt AAV, rnk2) | 827 CDLC UMa (no AAV, rank 2) |
| Follow PMI 90% TP 13.24dB Random PMI TP @13.24 dB 66.6% Γ = 1.35 | Follow PMI 90% TP 13.92dB Random PMI TP @13.39 dB 52.6% Γ = 1.711 | Follow PMI 90% TP 13.39dB Random PMI TP @13.39 dB 53.2% Γ = 1.69 |
| 827 CDLC UMa (rank 4) | 827 CDLC UMa (downtilt AAV, rnk4) | 827 CDLC UMa (no AAV, rank 4) |

Figure 18: Follow vs. random PMI in AAV alternatives.
8T4R, 1CW, MCS13, 10Hz/3kph, rank2 (top) and rank 4 (bottom), typeI CB with N1=4, N2=1, O1=4, O2=1.

Notes on Figure 18: 	
For the 2 layers random PMI case there is 0.6dB gap between with and without AAV at 70%TPUT (AAV being better). The gap increases with number of layers.	
For the 2 layers follow PMI case there is no (0.2dB) gap between with and with AAV at 70%TPUT (no AAV being better). The “gap” is decreasing further with number of layers.

From Figure 18 it is observed that

- CDL with 827 fully connected AAV

  - The gain of follow precoding increases with rank and is small for low ranks.

  - This observation makes sense to us, when we consider the RAN4 SNR operating point definition at the receiver input side. The AAV creates CSI-RS ports that “see” strong and spatially diverse directions. The strong, or gain part, is normalized away by the SNR definition, what remains is the spatially diverse aspect (see prior sections to see link between Tx and Rx diversity in CDL), which can be exploited by a SDM receiver to improve performance.	
For rank2 there is not much spatial diversity, so the SDM receiver cannot improve performance significantly. This changes with increasing number of ranks (and DoDs/DoAs).

- CDL with no AAV

  - The gain of follow precoding increases with rank and is already significant for low ranks.

- CDL with downtilt AAV

  - More extreme version of “no AAV”.

- All

  - Scale as expected with rank and are feasible to be used in this use case.

Hence,

For SU PMI requirements, both 827 CDL with and without fully connected AAV are feasible.

Legacy channel models

It is also instructive to repeat the legacy channel model results for the above used configurations.

| Follow PMI 90% TP 5.33dB Random PMI TP @5.33 dB 68.8% Γ = 1.31 | Follow PMI 90% TP 7.08dB Random PMI TP @7.08 dB 26.9% Γ = 3.35 |
| --- | --- |
| TDLC300-10 low (rank 2) | TDLC300-10 XP high (rank 2) |
| Follow PMI 90% TP 13.73dB Random PMI TP @13.73 dB 67.3% Γ = 1.34 | Follow PMI 90% TP 31.36dB Random PMI TP @31.36 dB 52.2% Γ = 1.72 |
| TDLC300-10 low (rank 4) | TDLC300-10 XP high (rank 4) |

Figure 18: Follow vs. random PMI in legacy channels.
8T4R, 1CW, MCS13, 10Hz/3kph, rank2 (top) and rank 4 (bottom), typeI CB with N1=4, N2=1, O1=4, O2=1.

As expected, the TDL non-low channel models run into issues due to being rank deficient.

More interesting are the results for TDL low, that produce gammas larger than 1. This contrasts with all current TDL low requirements in TS 38.101-4, which have gamma around 1.	
This observation is explained by TS 38.101-4 using 35Hz of doppler, which results in significantly changed channel spatial properties in the 6/8 slots later, when the PMI is applied. The 10Hz we chose here, are just within the window, where the channel does not “evolve away to look completely decorrelated” with the earlier chosen follow PMI.

We aim to finish the 100Hz comparison cases during the review week, where we expect for the TDL gamma to be more aligned with current specifications.

Additional: eTypeI vs. eTypeII

Due to the short time between meetings, we didn’t manage to run the eTypeII comparison in CDL. 	
Hence, we maintain the observations from last meeting [4] with the discussion and references given therein.

In legacy TDL channel models TypeI and eTypeII PMI feedback performs nearly identical, raising serious concerns whether such performance requirements are reasonable from an implementation correctness pov and from the perspective of absolute values for performance requirements. Performance requirements based on CDL channels behave as expected.

Notes on PMI numbering and PMI spatial interpretation

Sometimes we use a single number to refer to a PMI, which is derived from the PMI bits as follows:

- PMI indexes are translating to PMI bit fields as follows:

  - PMI 0: i1_1 = 0, i2=0, (i1_2 = 0, i1_3=n/a).

  - PMI 1: i1_1 = 0, i2=1, (i1_2 = 0, i1_3=n/a).

  - PMI 2: i1_1 = 1, i2=0, (i1_2 = 0, i1_3=n/a).

  - etc.

  - PMI 15: i1_1 = 7, i2=1, (i1_2 = 0, i1_3=n/a).

  - I.e., even and odd indices only differ w.r.t. co-phasing factors, but have the same direction.

Concerning the spatial interpretation of PMIs we also note the following:

- TypeI PMIs exhibit a symmetry or repeating patterns around the 0-7 vs. 8-15 indices.

  - This is caused by the one beam (a) for layers 1 and 3, switching roles with the one beam (b, c, or d) for layers 2 and 4.	
Beams b, c, and d, have a fixed relation to beam a:

MU PDSCH

We define the baseline comparison as per WF, including all FFS.

Background: SU SDM reduces inter-UE interference in SCM

Before presenting and interpreting results, we want to clarify how even a SU SDM receiver used in a MU context indirectly reduces inter-UE interference. This effect is known from theory and shows up in SCMs, but not in legacy channels where both signal and interference are always received from same DoA.

Figure 19 graphically illustrates how a SU SDM receiver that focuses on the direction of signal energy, will indirectly mitigate interference coming from different directions.

Figure 19: SU SDM reduces inter-UE interference in SCM.

SU SDM receivers used in MU systems, indirectly reduce inter-UE interference coming from directions that are not aligned with the signal DoA.

Baseline: 2+2 layers with IRC, E-IRC, and R-ML

Here we look at the behaviour of differing advanced receivers in MU TPUT setups. Here we assume that single cell MU configurations (and corresponding precoder based interference modelling) are sufficient to model interference profiles and patterns in the presented channel models. This assumption should be revisited in the future.

In Figure 20 we compare the demodulation performance of two advanced IRC receivers (R-ML and E-IRC) to baseline SU MMSE (“IRC” in RAN4 terminology). We use a configuration that is based on the Rel-18 advRx requirements (2+2 layers, MCS13+MC4 for target and co-scheduled UE). We observe:

- In legacy TDL,

  - E-IRC (MU MMSE) and IRC (SU MMSE):

    - Have significantly different performance, for non-orthogonal precoding/non-negligible inter-user interference.

    - SNR requirements, for non-orthogonal precoding/non-negligible inter-user interference, are greater than 20dB.

  - R-ML and (E-)IRC: Have a very large performance gap.

- In 827 CDL,

  - E-IRC (MU MMSE) and IRC (SU MMSE)

    - Perform about equally well.

    - All SNR operating points are feasible.

  - R-ML and (E-)IRC: Have a medium performance gap.

| TDLC300-100 low | TDLC300-100 XP Med |
| --- | --- |
| 827 CDLC UMa ULA |  |

Figure 20: Differing advanced receivers (R-ML, E-IRC, IRC) in presence of co-scheduled UE.
4T4R, 1CW (MCS13+MCS4), rank 2+2, MCS4 interference, orthogonal and random MU precoding.

In our understanding only the observed scaling in 827 CDL is aligned with practical deployment expectations:

- SU and MU MMSE perform equally well in presence of MU interference

  - See Figure 19 on how SU SDM reduces inter-UE interference in SCM.

  - Note that this expectation only holds true for DL, where the DUT and the SS precode and receive with the intent to improve the layers of the target UE only.	
In UL, the receiver goal is different. The receiver wants to detect all layers from all transmitters, thus the detection channel is an aggregation of all transmitters’ channels. UL SDM focusing on a single transmitter (i.e., SU SDM) is expected to perform significantly worse.

- R-ML and MMSE perform within a reasonable margin

  - MMSE receivers perform worse than R-ML, but the gap is within 5dB for high interference scenarios and less for low interference.

  - As the complexity of ML increases with co-scheduled modulation order (MO), the performance gap between R-ML and liner receivers is largest for low co-scheduled MOs.

Note that SU and MU MMSE in the RAN4 DMD baseline implementation are statistically equal (only) for Gaussian interference [9]. 	
Our results are with QPSK for the co-scheduled users, which is the farthest MO from Gaussian signaling, but the implicit spatial rejection is independent of this assumption, as is seen from SU and MU MMSE still performing equally well at this MCS.

For single cell MU TPUT is it expected that SU and MU MMSE perform about equally well in typical co-scheduling scenarios, and the R-ML to linear receiver gap is within a reasonable margin.

If a SCM was used in Rel-18 advanced receivers instead of TDL, RAN4 would not have needed to spend a lot of time unnecessarily analyzing and debating SU vs. MU MMSE in DL.

Using 827 CDL in MU TPUT scenarios significantly reduces SNR requirements for all advanced receivers and allows to introduce non-orthogonal precoder requirements, even for non-R-ML requirements.

Note on ortho vs. random gap:	
The question may arise why there is a performance gap between orthogonal and random precoding. If any SDM receiver can separate and cancel interference, then shouldn’t they be equal?	
The ortho/random precoder gap comes from part of the signal being unrecoverable, when using non-orthogonal precoding. The most pathological case being to send two layers using the same precoder; the symbols just merge into one and cannot be recovered. Similar overlapping directions are found in random precoding, and signal parts sent in exactly the same subspace cannot be recovered. 	
Note that in some circumstances (low SNR with MRT) the loss of some signal parts is outweighed by receive power gains.

MU PMI

No baseline comparison cases have been captured in the last WF.

There are at least two issues that have previously been highlighted when using legacy TDL models in MU PMI performance requirements:

- eTypeII PMI selection algorithm functionality and performance may have never been reliably tested, as TypeI and eTypeII perform similarly and TypeI is a subset of eTypeII.

- Gains of MU MIMO are also hidden behind the similar TypeI and eTypeII PMI feedback performance, leading to unreasonable baseline decisions.

To evaluate these impacts, multi cell MU configurations are needed that are backed by SCMs with spatial consistency between near and far receivers, which provides both typical interference levels and detectable spatial profiles of interference sources.

MU interference mitigation via PMI choice

It will be instructive to determine, whether the PMI feedback of a receiver (should) change in the presence of a receiver, that interferes with and contributes to measurements for PMI derivation.

In our understanding, there are at least some environments, where the PMI selection algorithm shall take spatial information about interference into account to achieve reasonable performance. For example, the (single and multi) cell MU setup presented in Figure 21.

Figure 21: MU interference mitigation via PMI choice.

In section 2.2 we show how precoder selection influences the signal arrival directions at the receiver in reliable SCMs.

There are some environments, where the PMI selection algorithm shall take spatial information about interference via DM-RS and/or CSI-RS into account to achieve reasonable performance. The link between interference and PMI selection is given by the SDM receiver spatial selectivity towards signal arrival directions that are influenced by PMI selection in SCMs.

CDL with spatial consistency

For both single and multi-cell MU SCM setups, the channel model needs to have a spatial consistency property. This means that the channels of UEs that are close together have (generally) similar large-scale properties that are correlated. UEs that are more spatially separated should have (generally) less similar large-scale properties.

The spatially consistent MU extension of 38.827 based CDL (and CDL in general) has been presented in section 2.5, and refer to there for our proposal on 38.827 CDL for spatially consistent single/multi-cell MU setups.

Other

We want to highlight that many further SCM comparison cases are of interested in RAN4 demod, even though they are not discussed in the context of this SI. For example:

CQI requirements

In the current specifications, we find multiple CQI requirements with fading channels (TDL) that have a gamma ratio of around 1. The issue with these requirements is as previously highlighted for PMI requirements with gamma=1, and the same solutions using SCMs apply.

FDD reciprocity CB requirements (Rel-17 feTypeII-PS)

As of now (end of Rel-18), no CSI requirements for feTypeII-PS codebooks exist, as it was judged untestable using current test setups.	
Following the SCM study insights this is not surprising, as the FDD reciprocity intent of this CB relies on UL to DL spatial consistency to compress/predict the DL spatial support from UL measurements (the transmitter needs to estimate the same spatial support from UL signals, as the receiver estimated from DL signals). TDL neither offers such a UL/DL spatial consistency, nor does it offer a diverse and stable enough spatial support. SCMs on the other hand are expected to solve both these problems.

Doppler compression CB (typeII-doppler-r18)

In Rel-18 is has been exceedingly difficult to find configurations that all to define reasonable CSI requirements for this CB.	
Following the SCM study insights this is not surprising, as the Doppler compression/prediction relies on a physical meaning and process behind the Doppler changes to be able to predict it. TDL based models only contain statistical Jakes distribution max Doppler effects (speed based), which cannot be used for prediction. An SCM with velocity and physical environment Doppler evolution is expected to be much more amenable for requirements for this CB.

Channel condition

The legacy channel models cannot be used to evaluate the rank, or rather condition, of a channel with corresponding antenna assumptions. TDL low is statistically always perfectly conditioned, i.e., overestimated the channel condition. TDL+antCorr is rank deficient/ill-conditioned by (antenna correlation matrix) design, which underestimates the channel condition. (827) CDL is deriving channel condition based on geometrical considerations that are typical for deployment (thus also resulting typical interference levels for physical distance/positioning).

Spatial Channel Modelling Methodology

This section follows the last WF [1] to advance open issues and add new topics, where found necessary.

Common for all Methodologies

Test cases for SU-MIMO

The WF from RAN4#112bis [1] sets the current of SU-MIMO test cases as follows:

Cases for SU-MIMO Agreement: Single-User PDSCH: 8Rx: 8 Layer, MCS 13 on both codewords (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting (FFS Whether it is already covered in PMI) 4Rx: 4 Layer, MCS 13 (Table 1) (type I codebook) – PMI Choice (FFS Fixed, Random) Companies encouraged to bring Fixed PMI choice to next meeting 4 CSI-RS Ports (2,1) for 4 Layer 8 CSI-RS Ports (4,1) for 8 Layer Single-User PMI 4Rx: 4 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 4 Layer (eType II) – Full Throughput Curves (PMI Follow) 4Rx: 2 Layer (type I) – Full Throughput Curves (PMI Follow, PMI Random) 4Rx: 2 Layer (eType II) – Full Throughput Curves (PMI Follow) Interested companies can use initially 8 CSI-RS Ports, more ports can be used to identify and show relevant impacts. Note: Several companies would prefer to focus on PDSCH until the modelling is stable

SU PDSCH

Concerning the fixed precoding choice, we showed in section 3 that it is feasible to make a fixed PMI choice based on the most frequently chosen follow_PMI under a given channel model. If this value is the same for

RAN4 shall allow each contributor to make a fixed PMI choice based on the most frequently chosen follow_PMI under a given channel model.

For alignment purposes of the SCM implementation, it would be advantageous for all contributors to choose the same PMI.

RAN4 to agree one common PMI choice for each channel model and configuration, with the following starting point for discussion:
- 8T8R-8Layer CDLC Uma 365-100 with 827 virtualizer: (i1_1=6, i1_2=0, i2=0)
- 8T8R-8Layer CDLC Uma 365-100 no AAV: (i1_1=6, i1_2=0, i2=0)
- 4T4R-4Layer CDLC Uma 365-100 with 827 virtualizer: (i1_1=2, i1_2=0, i2=0)
- 4T4R-4Layer CDLC Uma 365-100 no AAV: (i1_1=2, i1_2=0, i2=0)

The SCM evaluation relevant aspect about SU TPUT comparisons is whether different receiver implementations perform differently under SCM, but equally under legacy channel models. However, this comparison requires for the SCM candidate implementations to be aligned in a reference architecture, otherwise it is not clear whether performance differences are due to SCM implementation misalignment, or due to different receiver implementations.	
The reference architecture may either be specified, or the 2 CW case is used for alignment, which is less ideal but may be practicable.

RAN4 to use TR 38.833 IRC as reference receiver with a common fixed precoder, to align SCM candidate implementations in. Different additional, and potentially undisclosed, receiver implementations to be useable in comparison test cases.

Concerning 4Rx PMI and PDSCH comparison case overlap, we see the main difference between the two cases being the fixed precoder in PDSCH. This fixed setting will allow for the receiver performance to be evaluated without any dependencies on the PMI selection algorithm, or the KPI equalizing effect of random precoding

Request results for 4Rx / 4layer case with fixed precoding, to allow for receiver performance scaling to be evaluated without dependency on the PMI selection algorithm or the KPI equalizing effort of random precoding.

SU PMI

In section 3, we have presented our results for all typeI CB PMI comparison cases. Additionally, we show the PMI histograms for the comparison cases under both legacy channels and various CDL candidates.

The impact of SCMs on SU PMI selection may require less implementation pre-alignment than the PDSCH case. Interesting insights can be obtained from simply comparing follow vs. random PMI scaling, as well as eTypeI vs. eTypeII scaling.

RAN4 to discuss whether random vs. follow PMI TPUT should overlap for low rank cases in a SCM, and how the gap is expected to scale with different numbers of layers and Rx branches.

SU PDSCH and PMI

Concerning the comparison results themselves, all contributions should be captured in the TR (while allowing for future updates). This can either be done in image format, or by creating a spreadsheet with SNR vs. relative TPUT automatic plotting, merging all contributions.

RAN4 shall capture all comparison cases results (from all contributors and with respect to all legacy and SCM candidates) in the TR, to allow for evaluation of fitness for purpose of the channel models.

Test cases for MU-MIMO

The WF from RAN4#112bis [1] sets the current state of MU-MIMO test cases as follows:

Cases for MU-MIMO Agreement: Interested companies to bring to RAN4#113 views on the following cases: PDSCH 2+2 layers with IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE) Interested companies are encouraged to assess 2+2 layers with R-ML / E-IRC (type I orthogonal and random precoding, to distinguish target and co-scheduled UE) Proponents to highlight the difference between a SCM and TDL for MU-MIMO.

MU PDSCH

Concerning the precoder configuration, it is likely useful to distinguish the two choices as “random orthogonal” and “fully random” precoding. This Rel-17/18 implementation was assumed by us, when creating the results presented in section 3.

In this section we also highlighted how SCMs are needed to show the indirect MU interference mitigation effect of any SDM receiver; even SU SDM receivers. To see this effect in SCM candidates over legacy channel models, it is required to have comparisons including both SU and MU receiver algorithms.

RAN4 to necessarily include MU SDM receiver algorithms in the comparison test cases and their evaluation, e.g., E-IRC and R-ML. PMI choices as in SU PMI.

MU PMI

In section 3 we have explained our understanding concerning how it is possible and desirable for the PMI choice to impact MU interference mitigation.

Since the most straightforward comparison test setups of this effect involve a secondary interference cell and spatially consistent CDL modelling (see section 2.5), we expect first results in RAN4#114.

RAN4 to include comparison cases for eTypeI vs. eTypeII in a MU PMI setup, at least for single cell MU.

RAN4 to aim to bring initial multi cell MU PMI results in RAN4#114.

MU PDSCH and PMI

Concerning the comparison results themselves, all contributions should be captured in the TR (while allowing for future updates). This can either be done in image format, or by creating a spreadsheet with SNR vs. relative TPUT automatic plotting, merging all contributions.

RAN4 shall capture all comparison cases results (from all contributors and with respect to all legacy and SCM candidates) in the TR, to allow for evaluation of fitness for purpose of the channel models.

CDL Based Methodologies

This sub-topic was heavily worked on during RAN4#112bis, and we will subdivide the topic along the following lines:

- CDL transmitter and receiver antenna related configuration

- CDL modelling modifications

- Other setup parameters

These categories will be treated in this section.

CDL transmitter and receiver antenna related configuration

We reunite the following issues from the WF [1] in this sub-section

Radiation Pattern Proposals: - Option 1: For the purposes of CDL alignment, disregard radiation pattern parameters included in TR 38.827 Section 7.2 and consider Omnidirectional antenna pattern - Option 2: The antenna pattern of a single BS antenna element could use the radiation power pattern defined in 38.901 Table 7.3-1 Antenna Coordinates, Angles, and other parameters for gNB (for UMa-CDL-C) Proposals: - Option 1: For the gNB, use the following parameters for alignment with UMa CDL-C model: - GCS Location Coordinates for the gNB: - Height = 25m (based on UMa); - Azimuth = 0; (placement on the x axis); - GCS to LCS Conversion angles (α,β,γ = bearing, downtilt, slant) for the gNB; - α,β,γ=(0,0,0); - Option 2: - Frequency Fc = 3.5 GHz, height BS = 25m, and D2D = 100m. - BS orientation: (alpha = 0 degrees, beta = 0 degrees, gamma = 0 degrees). Intention is that BS points towards horizon along x-axis. - Other options not precluded Antenna Coordinates, Angles, and other parameters for UE (for UMa-CDL-C) Proposals: - Option 1: For the UE, use the following parameters for alignment with UMa CDL-C model: - Height = 25m (same as gNB) - Azimuth = 0; (placement on the x axis); - α,β,γ=(90,0,45); - Option 2: - Frequency Fc = 3.5 GHz, height UE = 1.5m and D2D = 100m. - UE orientation: (alpha = -90 degrees, beta = -13.2 degrees, gamma = 45 degrees). Intention is that UE points towards BS and uses "[+ + ... +]" crosspol elements. - Other options not precluded.

Following the pre-RAN4#113 email discussion, we propose to follow the CDL transmitter and receiver antenna related configuration, as described in Figure 22.

Figure 22: SU SDM reduces inter-UE interference in SCM.

Or in text proposal form:

RAN4 to set CDL transmitter and receiver antenna related configuration as follows:
BS
    (α,β,γ = 0, 10, 0), 
        I.e., intent of 10deg mechanical rotation around the y-axis, downtilted with respect to the horizon, resulting in θ_BS=100° and ϕ_BS=0°.
    Polarization slant (+45, -45).
    AE radiation pattern from 38.901 Table 7.3-1
    Informative: BS height is assumed 25m and d_2D=100m.
UE
    (α,β,γ = 180, 0, 0), 
        I.e., intent is pointing towards BS along x-axis with no tilt, resulting in θ_UE=90° and ϕ_UE=180°.
    Polarization slant (0, 90).
    AE radiation pattern omnidirectional
    Informative: UE height is assumed 1.5m and d_2D=100m.

CDL modelling modifications

We reunite the following issues from the WF [1] in this sub-section

Subcluster Correlation Proposals: - Option 1: RAN4 should discuss and clarify whether the potential correlation between subclusters is an issue, and whether there should be mechanisms to avoid it. Angles of Departure and Angles of Arrival Proposals: - Option 1: Replace the mean angle in the last position of departure and arrival angles generated equation by the desired angle as . Time Varying Beam Direction Proposals: - Option 1: Further evaluate the method for time variant beam directions in CDL model. BS Antenna Virtualiser choice Proposals: - Option 1: Down Tilt Virtualiser : -10 degrees - Option 2: Broadside - Option 3: Max Receive Power - Option 4: DFT Virtualiser according to TR 36.897 with application of panning and tilting angles (as per Table 7.8-1 of 38.901) Assumptions for CDL based modelling: Proposals: - Option 1: - For companies submitting 38.901 CDL based results, it is encouraged to base themselves on 38.901 CDL-C (i.e., on denormalized 38.901 table 7.7.1-3) and bring final CDLC table (delays, AoDs, ASDs, etc.) in the next meeting for alignment. - It is further encouraged to also consider the 38.827 "randomness reduction framework" on top of 901 modelling. - Other options not precluded.

Assuming agreement on the CDL transmitter and receiver antenna related configuration issues above, many of the CDL modelling modifications will start to self-align. The issues remaining after are:

Concerning BS Antenna Virtualizer choice, we propose

RAN4 to use “Max Receive Power” for standard 827 fully connected BS Antenna Virtualizers, and to use “Broadside” for 827 partially connected BS Antenna Virtualizers with 10 degrees of mechanical downtilt.

Concerning 827 randomness reduction framework, we propose

All CDL based approaches to use the 827 randomness reduction framework, i.e., small scale fading changes, fixed subpaths, weak clusters, etc.

Other setup parameters

We reunite the following issues from the WF [1] in this sub-section

Frequency Bands Proposals: - Option 1: RAN4 to study if the same CDL channel model can be defined for all carrier frequencies/ bands in given frequency range UE Speed Agreement Interested companies to use 30 km/h and/or 3 km/h for UE speed/velocity. TDL doppler values to be used for simulation: - 100 Hz for 30 km/h - 10 Hz for 3 km/h Assumption: 3.5 GHz carrier to be used for simulations (aligned with FR1)

We don’t see the need for further agreements in this sub-topic. The UE speed was agreed to be both 3 and 30 kph, and the applicability of the CDL model to all CC of (all bands) of a FR, is not different from TDL, given similar RMS DS.

Comparison of methodologies

The WF from RAN4#112bis [1] sets the current state of MU-MIMO test cases as follows:

TDL and CDL Methodologies Agreement: Interested companies to use the following candidate SCM options: - (Option A) CDL (as defined in R4-2415283) with Antenna Array Virtualisation (AAV) using subarray configuration - (M,N,P,Ms,Ns) = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y) - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y) - (Option B) CDL (as defined in R4-2415283), i.e., with AAV without subarray (fully connected) - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports] - (Option C, for interested companies) CDL (as defined in R4-2415283) without AAV with the following configuration: - [(M,N,P,Ms,Ns) = (1,4,2,1,1) for 8Tx CSI-RS Ports] - [(M,N,P,Ms,Ns) = (1,2,2,1,1) for 4Tx CSI-RS Ports] - (Option D, for interested companies) CDL from 38.901 with AAV using subarray - (M,N,P,Ms,Ns) = (8,8,2,8,4) for 4Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,8,2,8,2) for 8Tx CSI-RS Ports (option X) - (M,N,P,Ms,Ns) = (8,2,2,8,1) for 4Tx CSI-RS Ports (option Y) - (M,N,P,Ms,Ns) = (8,4,2,8,1) for 8Tx CSI-RS Ports (option Y) - (Option E, for interested companies) CDL from 38.901 with AAV without subarray (fully connected) - [(M,N,P) = (8,8,2) for 8Tx and 4Tx CSI-RS Ports] - (Option F, for interested companies) TDL extensions as defined in R4-2412762) - (Option G for interested companies) TDL extensions as defined in R4-2415382)

TDL and CDL Methodologies

To reduce the implementation, simulation, and analysis load, we recommend to downselect the SCM candidates:

RAN4 to focus on the following SCM candidates
(a) CDL (as defined in R4-2415283) with AAV using subarray configuration (partially connected with broadside steering+mechanical tilt), 
(b) CDL (as defined in R4-2415283), i.e., with AAV without subarray (fully connected), and 
(c) CDL (as defined in R4-2415283) without AAV.

Additionally, we showed in section 3 that 827 AAV has the largest testable range/least SNR requirements, followed by no AAV at +6dB and downtilt AAV at +12dB. Hence, priority should be given to 827 AAV.

RAN4 to consider expected SNR levels for selection of virtualizer, and hence to give priority to the 827 fully connected AAV.

Conclusion

In this contribution the TS 38.827 based CDL spatial channel model is once more found to be a reasonable demod (i.e., baseband branches referenced) channel model “black box”, that can be aligned between implementations of different contributors. It also exhibits MIMO effects and properties that we expect and observe in deployment, it is spatially consistent, predicable, and explainable. It can be directly applied to SU use cases and was extended to spatially consistent MU setups (both single and multi-cell).

The first alignment and comparison results have been provided in this contribution, to be compared to other contributors’ results in RAN4#113.	
More explication about CDL models (both 827 and 901), as well as virtualizers, has been shared to facilitate alignment of implementations.

The following observations and proposals were made:

Chapter === CDL based SCM

== Virtualizers

- Antenna Array Virtualisation defines the connections and weights between BB/RF ports (e.g., CSI-RS or SRS ports) and antenna elements of an antenna array or panel.

- Both fully and partially connected virtualizer have precedence in TR 38.901 and TR 38.827.

== Precoder selection and DoA for 827 CDL

In 38.827 CDL the precoding vector selection impacts the spatial properties of the RE-domain channel matrix seen at the receiver, i.e., changing the precoding direction of a layer, changes its receive direction.

== Numerical stability for 827 CDL

Initial random seeds have a negligible impact on the TPUT performance scaling of 837 CDL.

Redrawing scalar random initial phase terms does have a systematic impact on 827 CDL, that reduces with less redraw frequency (not redrawing for 1000 slots starts to be fine).

== Spatially consistent MU extension for 827 (and 901) CDL

- RAN4 to consider how SCM candidates can be modified to allow separately, but spatially consistently, correlated channels for different AODs, AOAs and ZODs corresponding to different multiple UEs.

Chapter === CDL comparisons and observations

== SU PDSCH

= Baseline: 2CW

For 2CW features, only 827 CDL can create the expected inter-CW performance difference at feasible SNR levels, which is needed to fully test a 2CW SDM receiver and correctly model performance scaling in deployment.

It is feasible to make a fixed PMI choice based on the most frequently chosen follow_PMI under a given channel model.

All three evaluated virtualizer options (827 AAV, mech. downtilt AAV, no AAV) show the expected impact of a large-scale environment, with 827 AAV having the largest testable range/least SNR requirements, followed by no AAV at +6dB and downtilt AAV at +12dB.

Any SNR budget gains cannot stem from exploiting the gain of strong channel directions, as the RAN4 SNR definition normalizes average receive branch power, so SNR budget gains arise from the channel model (a) providing spatially diverse environments that the SDM receiver can efficiently equalize, and (b) providing a less uneven per layer signal quality.

For SU TPUT use cases, legacy TDL channel model-based demodulation performance requirements are neither fit to test receiver functionality, nor fit to set a stable minimum performance requirement. Functionally differing SDM receiver implementation show same performance under legacy TDL channel requirements, but SCMs with spatially typical properties (in this case 827 CDL) can meaningfully differentiate between implementations, even under random precoding.

== SU PMI

= Additional: PMI profiles

The capacity optimal TypeI PMI choice is random in TDL low, fixed to broadsight in TDL+antCorr, and following defined and stable strong directions in CDL. Thus, only SCM model-based requirements can provide evidence for correct algorithmic function and set reasonable minimum performance expectations.

It is possible to derive a reasonable fixed precoder setting for all tested channel models, by choosing the most frequent choice of the follow_PMI histogram for each channel model.

= Baseline: 4Rx and 2/4 layer

For SU PMI requirements, both 827 CDL with and without fully connected AAV are feasible.

= Additional: eTypeI vs. eTypeII

In legacy TDL channel models TypeI and eTypeII PMI feedback performs nearly identical, raising serious concerns whether such performance requirements are reasonable from an implementation correctness pov and from the perspective of absolute values for performance requirements. Performance requirements based on CDL channels behave as expected.

== MU PDSCH

= Background: SU SDM reduces inter-UE interference in SCM

SU SDM receivers used in MU systems, indirectly reduce inter-UE interference coming from directions that are not aligned with the signal DoA.

= Baseline: 2+2 layers with IRC, E-IRC, and R-ML

For single cell MU TPUT is it expected that SU and MU MMSE perform about equally well in typical co-scheduling scenarios, and the R-ML to linear receiver gap is within a reasonable margin.

If a SCM was used in Rel-18 advanced receivers instead of TDL, RAN4 would not have needed to spend a lot of time unnecessarily analyzing and debating SU vs. MU MMSE in DL.

Using 827 CDL in MU TPUT scenarios significantly reduces SNR requirements for all advanced receivers and allows to introduce non-orthogonal precoder requirements, even for non-R-ML requirements.

== MU PMI

= MU interference mitigation via PMI choice

There are some environments, where the PMI selection algorithm shall take spatial information about interference via DM-RS and/or CSI-RS into account to achieve reasonable performance. The link between interference and PMI selection is given by the SDM receiver spatial selectivity towards signal arrival directions that are influenced by PMI selection in SCMs.

Chapter === Spatial Channel Modelling Methodology

== Common for all Methodologies

= Test cases for SU-MIMO

RAN4 shall allow each contributor to make a fixed PMI choice based on the most frequently chosen follow_PMI under a given channel model.

RAN4 to agree one common PMI choice for each channel model and configuration, with the following starting point for discussion:
- 8T8R-8Layer CDLC Uma 365-100 with 827 virtualizer: (i1_1=6, i1_2=0, i2=0)
- 8T8R-8Layer CDLC Uma 365-100 no AAV: (i1_1=6, i1_2=0, i2=0)
- 4T4R-4Layer CDLC Uma 365-100 with 827 virtualizer: (i1_1=2, i1_2=0, i2=0)
- 4T4R-4Layer CDLC Uma 365-100 no AAV: (i1_1=2, i1_2=0, i2=0)

RAN4 to use TR 38.833 IRC as reference receiver with a common fixed precoder, to align SCM candidate implementations in. Different additional, and potentially undisclosed, receiver implementations to be useable in comparison test cases.

Request results for 4Rx / 4layer case with fixed precoding, to allow for receiver performance scaling to be evaluated without dependency on the PMI selection algorithm or the KPI equalizing effort of random precoding.

RAN4 to discuss whether random vs. follow PMI TPUT should overlap for low rank cases in a SCM, and how the gap is expected to scale with different numbers of layers and Rx branches.

RAN4 shall capture all comparison cases results (from all contributors and with respect to all legacy and SCM candidates) in the TR, to allow for evaluation of fitness for purpose of the channel models.

= Test cases for MU-MIMO

RAN4 to necessarily include MU SDM receiver algorithms in the comparison test cases and their evaluation, e.g., E-IRC and R-ML. PMI choices as in SU PMI.

RAN4 to include comparison cases for eTypeI vs. eTypeII in a MU PMI setup, at least for single cell MU.

RAN4 to aim to bring initial multi cell MU PMI results in RAN4#114.

RAN4 shall capture all comparison cases results (from all contributors and with respect to all legacy and SCM candidates) in the TR, to allow for evaluation of fitness for purpose of the channel models.

== CDL Based Methodologies

RAN4 to set CDL transmitter and receiver antenna related configuration as follows:
BS
    (α,β,γ = 0, 10, 0), 
        I.e., intent of 10deg mechanical rotation around the y-axis, downtilted with respect to the horizon, resulting in θ_BS=100° and ϕ_BS=0°.
    Polarization slant (+45, -45).
    AE radiation pattern from 38.901 Table 7.3-1
    Informative: BS height is assumed 25m and d_2D=100m.
UE
    (α,β,γ = 180, 0, 0), 
        I.e., intent is pointing towards BS along x-axis with no tilt, resulting in θ_UE=90° and ϕ_UE=180°.
    Polarization slant (0, 90).
    AE radiation pattern omnidirectional
    Informative: UE height is assumed 1.5m and d_2D=100m.

RAN4 to use “Max Receive Power” for standard 827 fully connected BS Antenna Virtualizers, and to use “Broadside” for 827 partially connected BS Antenna Virtualizers with 10 degrees of mechanical downtilt.

All CDL based approaches to use the 827 randomness reduction framework, i.e., small scale fading changes, fixed subpaths, weak clusters, etc.

== Comparison of methodologies

RAN4 to focus on the following SCM candidates
(a) CDL (as defined in R4-2415283) with AAV using subarray configuration (partially connected with broadside steering+mechanical tilt), 
(b) CDL (as defined in R4-2415283), i.e., with AAV without subarray (fully connected), and 
(c) CDL (as defined in R4-2415283) without AAV.

RAN4 to consider expected SNR levels for selection of virtualizer, and hence to give priority to the 827 fully connected AAV.

- References

- R4-2416591, “Way Forward Minutes for [112bis][318] NR_SCM”, Moderator (Nokia), RAN4#112bis.

- R4-2411300, Discussion on Spatial Channel Model for Demodulation Performance Requirements, Nokia, RAN4#112.

- R4-2411301, “Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112.

- R4-2415282, “Discussion on Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112bis.

- R4-2415283, Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#112bis.

- R4-2418044, Simulation Results and CDL Implementation for Spatial Channel Model for Demodulation Performance Requirements”, Nokia, RAN4#113.

- R4-2413531, Ad-hoc meeting minutes for [112][317] Demod_Maintenance, Nokia, RAN4#112.

- W. Sloane et al., "Measurement-Based Validation of the 3GPP Spatial Consistency Procedures," in IEEE Transactions on Vehicular Technology, vol. 73, no. 4, pp. 4787-4800, April 2024.

- R4-2309351, On MU-MIMO advanced receiver, Qualcomm, Inc., RAN4#107.