3GPP TSG RAN WG4 #113	R4-2418068

Orlando, US, November 18 – 22, 2024

Agenda item:		7.20.3.1

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements for UE-to-UE CLI handling

Document for:		Discussion

# Introduction

At RAN4#112bis meeting, it was agreed to define the RRM requirements for L1-SRS-RSRP and L1-CLI-RSSI measurement [1]. While the side conditions were still open due to different understandings on the scenarios, we agreed on the simulation assumptions to facilitate the discussion. In this contribution, we are discussing the open issues and providing our initial views on the RRM impacts due to co-channel CLI handling mechanisms based on the progress in other WGs.

# RRM requirements for co-channel CLI handling

The following were agreed in last RAN4 meeting on the RRM requirements for CLI handling. We will discuss the details on the UE-to-UE CLI measurement requirements in this section.

Issue 1-1-1: Measurement quantities - Agreements (from online session) - RAN4 to define requirements for L1-SRS-RSRP and L1-CLI-RSSI. - Requirements for other measurement quantities can be considered based on further RAN1 agreement. Issue 1-1-6: Side condition - Agreements (from online session) - Send LS to RAN1: in addition to intra-cell scenario, whether inter-cell scenario is considered in this WI. Further discuss the details in the LS. - For SRS configuration, re-use Rel-16 side condition as baseline. Issue 1-1-14: Measurement timing for L1-SRS-RSRP - Agreements (from online session) - As baseline, for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. - This can be revisited if it proves to be conflicting with further RAN1 agreements.

CLI measurement scenarios

In R19, the CLI measurements are mostly discussed in SBFD, i.e., the victim UE is aware of the SBFD configuration. During RAN1#116, four methods were identified for how to measure the co-channel UE-to-UE CLI in SBFD as depicted in Figure 1 and the following was agreed in RAN1#118 meeting:

Agreement - For L1 UE-to-UE CLI measurement and reporting, CLI measurements is performed within the active DL BWP and the following are supported​ - Method#1: UE measures RSSI within DL subband​ - Method#2: UE measures RSRP of aggressor UE within UL subband​ - FFS: Method#3: UE measures RSSI within UL subband

Figure 1. Methods for UE-to-UE CLI measurements in SBFD

If SRS-RSRP is configured for CLI measurement, UE needs to measure the RSRP of aggressor UE within UL subband, i.e., Method #2. This SRS-RSRP CLI measurement indicates the interference on the UE’s “UL transmission” due to the UL transmission from aggressor UE which is different from the R16 legacy CLI measurement where the interference is on the DL reception of the victim UE.

The SRS-RSRP CLI measurement in R19 indicates the interference on the UE’s “UL transmission” due to the UL transmission from aggressor UE.

For CLI-RSSI measurement, the main difference between Method#1 and Method#3 is whether the UE measures the CLI over the DL subband(s) or over the UL subband of the victim UE where SBFD is configured. In other words, the UE measures the leakage interference in Method#1 and the fundamental UL signal in Method#3. Our main concern with Method#1 is the fact that the CLI-RSSI measurement likely include legacy inter-cell interference making it difficult for the network to conclude on whether the reported CLI-RSSI value corresponds to high UE-to-UE CLI or high legacy interference. On the other hand, Method#3 ensures that the measurements capture only the UL signals being transmitted over the UL subband.

Method#1 is not able to distinguish whether the CLI-RSSI measurement corresponds to UE-to-UE CLI or legacy inter-cell interference.

To define the CLI measurement requirements in SBFD as the following:

- SRS-RSRP measurement requirements for Method#2.

- CLI-RSSI measurement requirements for Method#1 and Method#3.

Rx beam assumption

Rel-16 UE-to-UE CLI measurements for FR2 UEs assume that the Rx beam used for the UE-to-UE CLI measurements is the same as the latest beam used for the reception of the PDCCH/PDSCH:

TS 38.133 clause 9.7.1: For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET.

There is an on-going discussion on RAN1 about allowing the gNB to provide an indication of the QCL-type D assumptions as part of the L1 UE-to-UE CLI measurement configuration. In RAN1#118-bis, the following agreement on this topic was reached:

Agreement For aperiodic L1 CLI-RSSI/CLI-SRS-RSRP reporting on PUSCH, support the following in the IE CSI-AperiodicTriggerStateList: - In CSI-AssociatedReportConfigInfo a list of TCI state(s) with qcl-Type set to 'typeD' is optionally configured where the TCI state(s) correspond to the resource(s) in the set of CLI measurement resources indicated by CSI-AssociatedReportConfigInfo o The list of TCI states can be configured (present) only if unifiedTCI-StateType is configured and the CLI measurement resource(s) are aperiodic - If the list of TCI states is not configured (absent), the UE assumes the following for each CLI measurement resource: o If unifiedTCI-StateType is not configured § Same assumption as for L3-based CLI measurement as in 38.133: “For performing CLI measurement in FR2, UE can assume the configured CLI measurement resources are QCL-ed with TypeD to one of the latest received PDSCH and the latest monitored CORESET” o If unifiedTCI-StateType is configured § “Indicated” DL only/joint TCI state as specified in TS 38.214 o Note: Above default rules apply to aperiodic reporting based on aperiodic, semi-persistent, and periodic CLI measurement resources FFS: details of configuration of TCI state(s) for aperiodic reporting based on periodic and semi-persistent CLI measurement resources

Based on the agreement above, the network can configure the UE to perform measurements with a TCI state different than from the current TCI state used for the PDCCH and PDSCH reception. Therefore, the Rel-16 assumptions for Rx beam are not always applicable to all cases in R19. Note that this is currently only possible for aperiodic measurements and reporting, and it is applicable to both CLI-RSSI and SRS-RSRP measurements. It is still FFS for periodic and semi-persistent measurements. Given the differences with existing Rel-16 assumptions on Rx beam in FR2, RAN4 should discuss the CLI measurement requirements considering the measurements using different Rx beams than the Rx beam used for DL reception.

The Rel-16 assumptions for Rx beam used for CLI measurement in FR2 are not applicable in all cases in R19, provided the latest RAN1 discussion.

RAN4 to discuss the CLI measurement requirement considering the different Rx beams.

CLI measurement periods

In R16, the CLI measurement periods for SRS-RSRP and CLI-RSSI are defined in the way of L3 measurements for dynamic TDD. For instance, 3 samples are assumed for deriving the SRS-RSRP measurement period and 1 sample RSSI is assumed for the CLI-RSSI measurement period.

In R16, 3 samples are assumed for deriving the SRS-RSRP measurement period and 1 sample RSSI is assumed for the CLI-RSSI measurement period.

In last RAN4 meeting, it was agreed to define the measurement periods for L1 based UE-to-UE CLI measurement but no conclusion on the number of samples. As the WID has indicated L1 based UE-to-UE CLI measurement and reporting is based on existing CSI framework, the SRS-RSRP CLI measurement requirement can be specified following L1-RSRP measurement. According to TS 38.133 clause 9.5, L1-RSRP measurement period is defined based on the high layer parameter timeRestrictionForChannelMeasurement and both 1 and 3 samples are considered. We think the same assumption can be applied to L1-SRS-RSRP measurement period in R19 up to network configuration. And for L1-CLI-RSSI measurement, the CLI measurement period is specified assuming 1 sample the same as the legacy CLI-RSSI measurement requirement.

-	M=1 if higher layer parameter timeRestrictionForChannelMeasurement is configured, and M=3 otherwise

For L1-SRS-RSRP measurement, the CLI measurement period can be specified following L1-RSRP measurement and both 1 and 3 samples are considered based on network configuration.

For L1-CLI-RSSI measurement, the CLI measurement period is defined assuming 1 sample.

Side conditions

Another aspect on the measurement requirement is the side conditions. Similar as for R16 CLI measurement, since the UE performs L1-SRS-RSRP measurement based on the DL timing in victim cell, there is time difference between the DL timing and the SRS arrival timing from the aggressor UE. This will impact the accuracy performance of CLI measurement.

Issue 1-1-6: Side condition - Agreements (from online session) - Send LS to RAN1: in addition to intra-cell scenario, whether inter-cell scenario is considered in this WI. Further discuss the details in the LS. - For SRS configuration, re-use Rel-16 side condition as baseline. Issue 1-1-14: Measurement timing for L1-SRS-RSRP - Agreements (from online session) - As baseline, for L1 SRS-RSRP measurement, UE will apply a constant offset relative to the downlink reference timing in the serving cell, and the constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset. - This can be revisited if it proves to be conflicting with further RAN1 agreements.

In R16 CLI measurement discussion, the time offset between DL timing and SRS arrival timing was derived as below. It was assumed UE would apply a constant offset to the DL reference timing in the serving cell to compensate the UL timing difference. And the residual timing error Tother is defined as a side condition for the CLI measurement accuracy requirement. In last meeting it was agreed to adopt the same methodology in R19 to define the time offset side condition for L1 CLI measure accuracy.

R16 CLI measuerment discussion: Definition for DL timing error (TCLI) in R16 CLI: - Consider cell phase synchronization error for TCLI derivation and for residual timing error assumptions for deriving measurement accuracy - TCLI = Tc*NTA_offset + Tother - Tother = Tpropagation_delay + Tcell_phase_synchronization - Tother = 4.67usec for FR1 and 3.67usec for FR2 - under the assumption that ISD=500m for FR1 and ISD=200m for FR2 - Residual timing error for measurement accuracy - 4.67usec for FR1 and 3.67usec for FR2 TS 38.133 clause 9.7: When the UE measures SRS-RSRP and CLI-RSSI, a constant offset relative to the downlink reference timing in the serving cell shall be applied. The constant offset value is derived by UE implementation and shall be at least Tc*NTA_offset.

Regarding to the residual timing error Tother, it has been determined by the propagation delay difference and the cell phase error between the cells for victim and aggressor UEs. While the same rule can be applied to R19 discussion, the SRS-RSRP measurement is being discussed on the UL subband with SBFD operation where the aggressor UE mainly comes from the same cell of victim UE. We believe this is the most likely scenario to configure the L1-SRS-RSRP measurement hence the side condition can be defined assuming intra-cell i.e. aggressor and victim UEs are in the same cell. The main reason to focus the SRS-RSRP measurements applicability on the intra-cell scenario is the lack of exchange of SRS information across gNBs. As per agreement on RAN1#117, “The measurement resources for SRS-RSRP are based on the existing legacy RS patterns. No specification impact for SRS configuration information exchange between gNBs.”, and thus, it is not possible for a cell to configure the SRS-RSRP resources that match the SRS configuration of an aggressor cell in the neighbour cell.

To define the residual timing error Tother in intra-cell scenario as one side condition for the L1-SRS-RSRP measurement accuracy requirement.

If the intra-cell scenario is agreeable as the starting point, the cell phase error can be zero as victim and aggressor UEs are assumed in the same cell. As the propagation delay difference is derived assuming the victim and aggressor UEs are located in the cell center and cell edge respectively, it remains the same irrespective of intra-cell or inter-cell. Hence the residual timing error can be derived assuming cell phase error is zero and same cell size as in R16.

For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

In last meeting, there was different views if the L1 CLI measurement in inter-cell scenario shall be considered in addition and an LS was sent to RAN1. In our understanding, if inter-cell scenario is considered, it will be very similar to the scenario we have been assuming in R16 CLI measurement except 1 sample is considered for L1 CLI measurement. We would need check the accuracy performance based on the simulation results and define the requirement only if sensible accuracy can be achieved.

For inter-cell scenario, RAN4 shall define the requirement only if sensible accuracy performance can be achieved based on the simulation results.

As for SRS configuration, we agreed to use the same SRS configuration as in R16 CLI measurement. But the SRS Es/Iot value may be higher considering the R19 SRS-RSRP intends to measure the interference on UL subband where there is no other DL transmission hence less interference can be expected in the victim UE. RAN4 should define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

For L1-SRS-RSRP measurement, RAN4 to define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

CLI measurement reporting

With the updated WID, it informs the L1 based CLI measurement and reporting are based on existing CSI framework and at least aperiodic reporting is supported. This means the CLI measurement reporting will be handled similar as L1-RSRP reporting, hence at least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

At least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

Sub-band CLI-RSSI reporting

Due to the frequency dependency of the CLI, RAN1 is currently discussing whether to support finer reporting granularity for the CLI-RSSI measurements. This topic is denoted as “subband CLI-RSSI reporting” as it is different from the wideband reporting and per-DL subband reporting discussed as part of the CLI-RSSI measurement resource type #2. In this case, subband CLI-RSSI reporting allows multiple CLI-RSSI values to be reported with a single CLI-RSSI measurement resource. This can be applicable to CLI-RSSI measurement resource type #1 and type#2. Reporting with finer granularity is already supported in CSI framework, where a UE can report CQI and PMI on a CSI subband basis. However, now the interest is on L1-CLI-RSSI measurements. Additionally, we would like to raise the issue that the subband sizes for the subband CQI and PMI measurements might be too large for the CLI-RSSI measurements (current minimum value is 16 RBs for a BWP size between 145 and 275 RBs) and, therefore, new subband measurement sizes could be introduced. If this feature is supported by RAN1, RAN4 should define new requirements for the sub-band CLI-RSSI reporting accordingly.

RAN4 should define new requirements for the sub-band CLI-RSSI reporting if this feature is supported by RAN1. Sub-band CLI-RSSI reporting is applicable for both measurement resource type #1 and type #2.

Measurement/scheduling restriction

As discussed in CLI measurement scenarios, different CLI measurement methods are being discussed in RAN1 based on if the CLI measurements are configured on DL subbands or UL subband. In particular, if the UE is configured with CLI measurements on UL subband i.e Method#2 and Method#3, no further DL reference signals are expected to be measured other than the CLI measurement resources hence there will not be measurement restriction. Otherwise, the measurement restriction shall be defined.

Similarly, when the UE is performing CLI measurements on serving cell, the scheduling availability also depends on if the CLI measurements are configured on DL subbands or UL subband i.e. the respective CLI measurement methods. RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

Report mapping

Issue 1-1-12: Report mapping

- Proposals

  - Proposal 1 (CATT):

    - RAN4 to reuse the R16 report mapping tables for R19 CLI measurements.

  - Proposal 2 (E///):

    - For measurement report mapping the Rel-19 CLI measurement reporting should be based on the simulation outcome.

    - No differential SS-RSRP for L1 reporting and L3 reporting is needed.

  - Proposal 3 (MTK):

    - Wait for RAN1 to define the new measurement resources and framework before defining reporting mapping.

  - Proposal 4 (HW):

    - RAN4 not to discuss measurement report mapping for L1 CLI measurement unless triggered by RAN1.

- Recommended WF

  - RAN4 not to discuss measurement report mapping unless requested by other WGs.

RAN1 has already agreed on re-using the CLI-RSSI and SRS-RSRP as reporting quantities, while it is still FFS whether other metrics will be supported. The SRS-RSRP and CLI-RSSI mapping tables defined in Table 10.1.22.1.2 and Table 10.1.22.2.2 can be used as baseline.

Rel-16 CLI-RSSI and SRS-RSRP mapping tables can be used as baseline for absolute L1 UE-to-UE CLI reporting.

RAN1 is also discussing the number of CLI measurement resources that can be configured for a single report. Latest proposal from RAN1#118-bis indicated that the UE can be configured with {1, 2, 3, 4, 8} measurement resources. This is similar to the current L1-SINR measurement resource configuration where the UE can be configured up to 4 measurement resources. For such cases, the UE can use differential reporting, where the UE uses 7 bits for reporting the absolute power of the strongest measurement and 4 bits to indicate the difference of the remaining measurement resources with respect to the strongest. Similar reporting methodology can be used for per-DL-subband CLI-RSSI measurements if the UE is configured with measurement resource type #2. Current Rel-16 report mapping table doesn’t consider the differential mapping and always assume 7 bits for each measured resource.

RAN4 to discuss how to support differential reporting for L1-based SRS-RSRP and CLI-RSSI measurements.

Requirement applicability

In R16 CLI measurements, the measurement requirements apply only when the measurement resource is fully confined within active BWP. While SBFD operation is considered in R19, the SRS-RSRP and CLI-RSSI in Method#3 will be configured in UL subband. However, the UL subband is also considered within DL active BWP. Hence the existing applicability conditions in R16 are still applicable to R19 L1 CLI measurement.

TS 38.133 clause 9.7.1:

CLI measurements are only applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

# Conclusion

In this contribution, our views on the RRM requirements are presented. The following proposals were listed.

- The SRS-RSRP CLI measurement in R19 indicates the interference on the UE’s “UL transmission” due to the UL transmission from aggressor UE.

Method#1 is not able to distinguish whether the CLI-RSSI measurement corresponds to UE-to-UE CLI or legacy inter-cell interference.

- To define the CLI measurement requirements in SBFD as the following:

- SRS-RSRP measurement requirements for Method#2.

- CLI-RSSI measurement requirements for Method#1 and Method#3.

The Rel-16 assumptions for Rx beam used for CLI measurement in FR2 are not applicable in all cases in R19, provided the latest RAN1 discussion.

RAN4 to discuss the CLI measurement requirement considering the different Rx beams.

In R16, 3 samples are assumed for deriving the SRS-RSRP measurement period and 1 sample RSSI is assumed for the CLI-RSSI measurement period.

For L1-SRS-RSRP measurement, the CLI measurement period can be specified following L1-RSRP measurement and both 1 and 3 samples are considered based on network configuration.

For L1-CLI-RSSI measurement, the CLI measurement period is defined assuming 1 sample.

To define the residual timing error Tother in intra-cell scenario as one side condition for the L1-SRS-RSRP measurement accuracy requirement.

For L1-SRS-RSRP measurement in intra-cell scenario, the residual timing error is derived assuming cell phase error is zero and same cell size as in R16.

For inter-cell scenario, RAN4 shall define the requirement only if sensible accuracy performance can be achieved based on the simulation results.

For L1-SRS-RSRP measurement, RAN4 to define a higher value of SRS Es/Iot as side condition for the L1 CLI SRS-RSRP measurement on the UL subband in SBFD operation.

At least the L1-RSRP measurement reporting requirement for aperiodic reporting can be reused for the L1 based CLI measurement reporting.

RAN4 should define new requirements for the sub-band CLI-RSSI reporting if this feature is supported by RAN1. Sub-band CLI-RSSI reporting is applicable for both measurement resource type #1 and type #2.

RAN4 shall wait for RAN1 conclusion on the CLI measurement methods before discussing the measurement and scheduling restriction.

Rel-16 CLI-RSSI and SRS-RSRP mapping tables can be used as baseline for absolute L1 UE-to-UE CLI reporting.

RAN4 to discuss how to support differential reporting for L1-based SRS-RSRP and CLI-RSSI measurements.

The L1 CLI measurement requirements are applicable for RRC_CONNECTED intra-frequency:

-	when SRS-RSRP measurement resource is fully confined within BW of DL active BWP

-	when CLI-RSSI measurement resource is configured within active BWP

# References

- R4-2416858, WF on SBFD RRM, Huawei