3GPP TSG-RAN WG4 Meeting #113	R4-2418071

Orlando, US, November 18 – 22, 2024

Source:	Nokia, Nokia Shanghai Bell

Title:	RRM requirements maintenance for network energy saving

Agenda item:	5.27.1

Document for:	Discussion

Introduction

At RAN4#112 meeting, the core part of the WI was further discussed but there are still some open issues as listed in the WF [1]. In this paper, we will continue the discussion on the leftover issues.

SSB-less operation

Neighbor cell measurement on SSB-less SCell carrier

In RAN4#112 meeting, the neighbor cell measurement on the carrier of SSB-less SCell was discussed but could not conclude in terms of intra-frequency vs. inter-frequency and gaps vs. without gaps issues.

Issue 1-1-4: Neighbour cells on carrier of SSB-less SCell FFS: · Intra-frequency/Inter-frequency definition: o Inter-frequency based on existing definition. No impact to spec about the intra-f and inter-f definition. · No spec impact in terms of inter- or intra- frequency, and with or without gap.

Once an SCell has been configured (added), the SCell is a serving cell. The configured SCell is a serving cell no matter if the SCell is activated, dormant or deactivated. For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies.

Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies.

According to current definition of intra-frequency measurement, it is defined based on the center frequency of the SSBs in the serving cell and neighbor cell. As there is no SSB in the serving cell (SSB-less SCell), the current intra-frequency measurement definition does not very well cover the SSB-less SCell scenario. Hence, it was considered whether the measurement requirements applicable for an SSB-less serving should be intra-frequency or inter-frequency measurement requirements.

Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133.

However, the SSB-less SCell is a serving cell hence the carrier of SSB-less SCell is a serving CC. Although there is no SSB transmission on the SCell, it would be beneficial to consider neighbor cell measurements in the serving cell carrier to be intra-frequency ones – Anyway UE is able to measure those without interruptions.

Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell.

Signaling allows UE to be configured with servingCellMO on the carrier of SSB-less SCell and the neighbor cell measurement in this MO shall be considered as neighbors on a serving carrier and therefore possibility to be measured according to intra-frequency measurement requirements. In particular, in the MO indicated by servingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine the intra-frequency or inter-frequency measurement based on the ssbFrequency. As there is no actual SSB being transmitted, the ssbFrequency is configured indicating an ARFCN value not associated with any SSB.

Observation #4:  The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC.

If the SSB-less SCell configuration include servingCellMO , the neighbor cell measurement in this MO shall be considered as intra-frequency measurement.

In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

NES-specific CHO

Measurement behaviour

In current specification, the measurement time delay for NES-based conditional handover is defined based on if the DCI 2-X command comes before or after TEvent_DU + Tidentify_intra_with_index.

TS 38.133:

##### 6.1.4.2.2	Measurement time

The measurement time delay is defined from the end of TEvent_DU until UE executes a handover to a target cell and interruption time starts.

For conditional intra-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify intra with index or Tidentify_intra_without_index defined in clause 9.2.5.1 or clause 9.2.6.2.

For conditional inter-frequency handover, the measurement time delay measured without Time To Trigger (TTT) and L3 filtering shall be less than Tidentify_inter_with_index or Tidentify_inter_without_index defined in clause 9.3.4 or clause 9.3.9.

For NES-based conditional intra-frequency handover:

- If UE successfully decodes DCI 2-9 command earlier than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equal to Tidentify_intra_with_index or Tidentify_intra_without_index

- If UE successfully decodes DCI 2-9 command later than the time at the end of TEvent_DU + Tidentify_intra_with_index or TEvent_DU + Tidentify_intra_without_index, then the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

For the case DCI 2-X command comes after TEvent_DU + Tidentify_intra_with_index, the assumption behind existing NES-based condition handover delay is that the CHO condition remains fulfilled/met from the end of Tevent_DU until UE successfully decodes DCI 2-X command. However, the CHO condition may vary from time to time and the NES-based CHO condition may or may not be met when receiving the DCI 2-9 command. As the arrival of DCI 2-9 command is uncertain relevant to the end of Tevent_DU, the possibility of channel variation needs to be considered when defining the NES-based conditional handover delay.

Observation #5: Existing NES-based condition handover delay assumes that the CHO condition remains fulfilled/met from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

Observation #6: For the case DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index, the NES-based CHO condition may or may not be met when receiving the DCI 2-9 command.

The NES-based conditional handover delay shall be defined considering the possible channel variation when DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index.

Based on the latest RAN1/2 agreement, the NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-9 command. As shown in Fig.1-(a), the measurement time delay equals to the time from the end of Tevent_DU until UE successfully decodes DCI 2-9 command. This is the same as the formulation in existing spec.

Otherwise, if the CHO condition has been met but is not fulfilled when receiving the DCI 2-9 command, the UE shall not execute NES-based conditional handover immediately. Instead, the UE keeps the measurement until the condition exists after DCI 2-9 command which will trigger the NES-based conditional handover. Afterwards, the UE will measure till the NES-based CHO is met i.e. another measurement time delay is needed. As the cell has been detected, the measurement time delay is equal to TSSB_measurement_period_intra or TSSB_measurement_period_inter..

The NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-X command after TEvent_DU + Tidentify_intra_with_index.

      - NES-based CHO condition is met when receiving DCI 2-9

      - NES-based CHO condition is not met when receiving DCI 2-9

Fig.1 NES-based handover delay

While there was consensus understanding on the expected UE behaviour, there was no conclusion how to formulate the CHO delay. Based on the discussion last meeting, if the condition of NES-based CHO is not met when receiving the DCI 2-X command as in Fig.1(b), we may consider defining Tevent_DU as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until a condition exists after receiving DCI 2-9 which will trigger the NES-based conditional handover. And the measurement time delay Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter. We provide a tentative CR to clarify the conditions for measurement time delay in R4-2407741[2].

If the condition of NES-based CHO is not met when receiving the DCI 2-9 command after TEvent_DU + Tidentify_intra_with_index.,

- Tevent_DU is defined as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until “a condition exists at the measurement reference point after receiving DCI 2-9 which will trigger the NES-based conditional handover”, and

- Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter.

Conclusion

In this paper we have made the following proposals and observations:

Observation #1: For serving cells, the basic rule is that intra-frequency measurement requirements in section 9.2 applies.

Observation #2: The current intra-frequency measurement definition does not account when the serving cell is an SSB-less SCell. Hence, it is not clear if it does fulfill the conditions for intra-frequency measurement as defined in TS 38.133.

Observation #3: It would be beneficial to consider serving cell frequency measurements as intra-frequency to allow fluent mobility for SCell.

Observation #4:  The SSB-less SCell is a serving cell and the carrier of SSB-less SCell is a serving CC.

- If the SSB-less SCell configuration include servingCellMO, the neighbor cell measurement in this MO shall be considered as intra-frequency measurement.

In the MO indicated by ServingCellMO, network may configure ssbFrequency to indicate a carrier frequency for the SSB-less SCell so that UE can determine measurement as intra-frequency or inter-frequency measurement based on the ssbFrequency.

Observation #5: Existing NES-based condition handover delay assumes that the CHO condition remains fulfilled/met from the end of Tevent_DU until UE successfully decodes DCI 2-9 command.

Observation #6: For the case DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index, the NES-based CHO condition may or may not be met when receiving the DCI 2-9 command.

The NES-based conditional handover delay shall be defined considering the possible channel variation when DCI 2-9 command comes after TEvent_DU + Tidentify_intra_with_index.

The NES-based CHO shall be executed only if the condition of NES-based CHO is met when receiving the DCI 2-X command after TEvent_DU + Tidentify_intra_with_index.

If the condition of NES-based CHO is not met when receiving the DCI 2-9 command after TEvent_DU + Tidentify_intra_with_index.,

- Tevent_DU is defined as the delay uncertainty which is the time from when the UE successfully decodes a conditional handover command until “a condition exists at the measurement reference point after receiving DCI 2-9 which will trigger the NES-based conditional handover”, and

- Tmeasure equals to TSSB_measurement_period_intra or TSSB_measurement_period_inter.

References

[1] R4-2414102, WF on RRM requirements for NR network energy saving, Huawei

[2] R4-2418071, 38133CR on handover delay for NES-based handover, Nokia, Nokia Shanghai Bell