3GPP TSG-RAN WG4 Meeting #113                                                              R4-2418096

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Further discussion on AI/ML two-sided model logistical issues

Agenda item:			7.17.2.2

Document for:	Discussion

# 1. Introduction

Based on the study outcome of Rel-18 SI on the Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface [1], the corresponding normative work item is approved to introduce the specification support for the aspects of AI/ML general framework and two use cases (i.e., beam management and positioning accuracy enhancements) [2].

Over Rel-18/19, RAN4 has been currently conducting a feasibility study for testing the feature of CSI compression. It was agreed that this feasibility study will be performed over multiple steps and data sharing (ML models of encoder or decoder or both, and/or datasets for training/testing/inference) could be required. Accordingly, the data sharing is a new process for 3GPP and several logistical issues will need to be solved.

In RAN#104, based on the RAN4 offline discussion on AI/ML logistical issues, it was concluded to require RAN4 email discussion and “report back to next RAN meeting” (a.k.a. RAN#105), that is:

RP-241615 Moderator's summary for offline discussion on AI/ML logistical issues Qualcomm considered Tdoc: RP-2412220 Oppo: wants to add a note into way forward, can remove the examples Apple: there are a lot of moving targets, maybe there is nothing to be shared DT: we might develop something very specific for RAN, in SA they also discuss this so a reporting to SA would be useful RAN4 chair: does not need to have meeting time for this Nokia: supports the way forward from Qualcomm ZTE: some aspects from background could be moved to way forward and e.g. aspects can be removed CMCC: examples could raise some concerns from RAN1 Qualcomm: it is important to understand that the trigger for this was the question how RAN4 could do testing for this conclusion: RAN4 email reflector can be used to continue discussion on this matter, please report back to next RAN meeting

In RAN#105, based on post-RAN4#112 email discussion concluded in [3], the logistical issues were further discussed thus resulting in the endorsed way forward [4, RP-242350] as follows:

<RP-242350> - RAN Plenary agrees to formally start implementing the infrastructure/platform needed by RAN4 for data sharing - MCC to work out the details of the data sharing platform with input from RAN4 experts - Discussion on the platform setup with MCC to be triggered as soon as possible - Inputs to MCC based on previous RAN4 discussions (data size, etc) - RAN4 to discuss the technical aspects (data formats, model formats etc. for purposes of studying the two-sided CSI compression) in the next RAN4 meeting

Since RAN4 has conducted two rounds of conference calls with MCC to discuss the logistical issues, and in this contribution, we would like to provide our further viewpoints for remaining logistical issues for sharing AI/ML model/dataset.

# 2. Remaining Issues

## 2.1 Model Conversion between ONNX and PyTorch

Our conversion process is listed below for reference.

1) Trained our own encoder and decoder models in PyTorch 1.12.0 GPU environment.

2) Imported ONNX 1.17.0 with greatest opset version 22 and used torch.onnx.export() API to convert PyTorch model to ONNX format. UnsupportedOperatorError occurred because ‘_convolution_mode to ONNX opset version 16 is not supported’, which came from nn.Conv2d(…, padding='same', …).

3) Loaded the saved PyTorch model in PyTorch 2.5.1, which supports greatest ONNX opset version 20, and resolved the _convolution_mode Error.

4) When converting PyTorch encoder model with scalar quantization, multiple errors occurred, such as ‘IndexError: Argument passed to at() was not in the map’ related to autograd.Function inline export and ‘Type 'tensor(uint8)' of input parameter of operator (Pow) is invalid’ related to ONNX operator limitation.

5) Finally, quantization layer was refactored to match ONNX opset requirements. And ran the saved ONNX model using onnxruntime to verify its validness.

Based on our experience, it is recommended to use the latest version PyTorch and ONNX to conduct the conversion, while PyTorch model may need to be redesigned to match the ONNX opset limitation.

Observation 1: Based on our experience, it is recommended to use the latest version PyTorch and ONNX to conduct the conversion.

Observation 2: PyTorch model may need to be redesigned to match the ONNX opset limitation.

Given that the final selected model is ONNX model, another question shall be further studied, that is how to train a PyTorch encoder model concatenating an imported ONNX decoder model in PyTorch framework. The key issue is how to delivery gradients between PyTorch model and ONNX model in backwards propagation. To save the effort and also more importantly to avoid any mismatch from companies’ model conversion, we suggest the company of final adopted decoder shall provide the common decoder in both ONNX and PyTorch versions, by guarantee the consistency between these two versions.

Observation 3: The company of final adopted decoder shall provide the common decoder in both ONNX and PyTorch versions, by guarantee the consistency between these two versions.

## 2.2 Rule of “lifetime” for data/model sharing

Based on the two rounds of conference call discussion with MCC, the importance of setting “lifetime” for different shared data and/or model was mentioned and emphasized. The relevant conclusion is given in [3], i.e., “The “lifetime” of the data needs to be further discussed, some data used in intermediate steps in the RAN4 study might not have to be stored for a long time”. Accordingly, the following different categories of lifetimes for data/model sharing are provided, which can be used as baseline for further discussion.

Proposal 1: For different categories of lifetimes for data/model sharing, RAN4 can further study based on the following ones as baseline:

|  | AI/ML Model |  |  | AI/ML Dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cat-1 | Cat-2 | Cat-3 | Cat-a | Cat-b | Cat-c |
| Lifetime (i.e., minimum storage time in 3GPP server) | 3 months (i.e., to be deleted 3 months after model submission) | 1 release (i.e., to be deleted after the current release ASN.1 is frozen) | Forever (i.e., to be kept in 3GPP server forever) | 3 months (i.e., to be deleted 3 months after model submission) | 1 release (i.e., to be deleted after the current release ASN.1 is frozen) | Forever (i.e., to be kept in 3GPP server forever) |
| Typical sharing purpose | e.g., temporarily shared model for cross-company evaluation purpose etc. | e.g., reference model (e.g., reference encoder) used to derive RAN4 requirements | e.g., Standardized reference model (e.g., test model used in RAN4 spec.) | e.g., temporarily shared dataset for cross-company evaluation purpose etc. | e.g., the common dataset to derive RAN4 requirement | FFS the necessity |

# 3. Conclusion

In this contribution, we provide our further viewpoints for remaining logistical issues to share AI/ML model/dataset, accordingly the following observations and proposals are obtained:

Observation 1: Based on our experience, it is recommended to use the latest version PyTorch and ONNX to conduct the conversion.

Observation 2: PyTorch model may need to be redesigned to match the ONNX opset limitation.

Observation 3: The company of final adopted decoder shall provide the common decoder in both ONNX and PyTorch versions, by guarantee the consistency between these two versions.

Proposal 1: For different categories of lifetimes for data/model sharing, RAN4 can further study based on the following ones as baseline:

|  | AI/ML Model |  |  | AI/ML Dataset |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Cat-1 | Cat-2 | Cat-3 | Cat-a | Cat-b | Cat-c |
| Lifetime (i.e., minimum storage time in 3GPP server) | 3 months (i.e., to be deleted 3 months after model submission) | 1 release (i.e., to be deleted after the current release ASN.1 is frozen) | Forever (i.e., to be kept in 3GPP server forever) | 3 months (i.e., to be deleted 3 months after model submission) | 1 release (i.e., to be deleted after the current release ASN.1 is frozen) | Forever (i.e., to be kept in 3GPP server forever) |
| Typical sharing purpose | e.g., temporarily shared model for cross-company evaluation purpose etc. | e.g., reference model (e.g., reference encoder) used to derive RAN4 requirements | e.g., Standardized reference model (e.g., test model used in RAN4 spec.) | e.g., temporarily shared dataset for cross-company evaluation purpose etc. | e.g., the common dataset to derive RAN4 requirement | FFS the necessity |

# 4. Reference

[1] TR 38.843, v18.0.0, “Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface”.

[2] RP-242399 (revised from RP-234039, RP-240774), “Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface”, Qualcomm, RAN#105, Sep. 2024.

[3] PostRAN4-112EmailDiscussionOnAIML_conclusions_moderator.docx

[4] RP-242350, “WF on AI/ML Logistical Issues”, CAICT, RAN#105, Sep. 2024.

[5] R4-2416496, “Further discussion on AI/ML two-sided model logistical issues”, Samsung, RAN4#112bis, Oct. 2024.

[6] R4-2417207, “WF on data sharing for AI/ML air interface”, Qualcomm, RAN4#112bis, Oct. 2024.

# 5. Appendix: Existing Agreements

Before and after RAN4#112, there were two rounds of email discussion on this topic in which companies provided inputs regarding data size and possible formats to be used for ML model sharing. The following summary is provided [3]. Much progress has been achieved in RAN4#112bis, and the relevant agreement has been captured in the approved WF [6]:

Agreement: All agreements for data sharing are only for the CSI compression use case feasibility study. These agreements can be revisited in an eventual WI phase. - AI/ML model format Agreement for AI/ML formats: Use at least ONNX format for sharing. Companies can also share PyTorch model The models in different formats from one company shall be the same Use version v1.16 or later ONNX version and opset version number to be included in the file - Dataset format Agreement: Use NumPy for dataset sharing Do not use pickled data(compression mechanism) Use npy – single array in each file - Training Data file format Agreement: Dataset/model input file format: - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers) - In other words, dataset should content following info: - 1st dimension: Number of samples - 2nd dimension: Real and imaginary - 3rd dimension: Number of sub-bands - 4th dimension: Number of antenna ports - 5th dimension: Number of layers – present only if >1 layer - Note: Each element of the dataset will be a float32 real number The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447 Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization Dataset files can be split into multiple files to enable easier upload. - Use the 2 digits for split files starting from 00, increment for each additional file Split files and then archive each file - File naming scheme Agreement: - Folders for AI/ML data sharing and current WI/use case to be created under “RAN4 folder” - Subfolder created for each meeting File naming scheme( ML model file and dataset file) - a unique identifier for the company (4 characters – list to be maintained by RAN4 secretary) - meeting number - differentiate model and dataset by identifier (2 characters ml and ds) - additional identifier with 2 characters (TBD) - ec for encoder - dc for decoder - ei for encoder input - do for decoder output - lt for latent (decoder input or encoder output) - others to be added as needed - dataset can be split in multiple files – 2 digits - files to be compressed to zips and uploaded Example: Folders to be created under https://www.3gpp.org/ftp/tsg_ran/WG4_Radio/ Folder for models: /Data_sharing/ NR_AIML_air/CSI_compression/Models/R4_XXX Folder for datasets: /Data_sharing/ NR_AIML_air/CSI_compression/Datasets/R4_XXX Example for model file: vivoR4_113mlec.onnx Example for dataset file: vivoR4_113dsei00.npy - Files to be shared Agreement: Files to be shared, to be decided based on option under study - Dataset containing encoder input per subband (as described in Section 5-3) - Encoder and/or decoder model (as described in Section 5-1)