3GPP TSG-RAN WG4 Meeting #113                                                               R4-2418097

Orlando, US, 18th – 22nd November, 2024

Source:	Samsung

Title:	Discussion on RAN4 aspects for SBFD system parameters

Agenda item:			7.20.1

Document for:	Approval

# 1. Introduction

Based on Rel-18 study item on evolution of NR duplex operation, the support of subband non-overlapping full duplex has been studied, which is targeted to provide enhanced UL coverage, reduced latency, improved system capacity, and improved configuration flexibility for NR TDD operation [2].

In RAN#102, the work item on evolution of NR duplex operation (SBFD) has been approved, with WID further revised in the follow-up RAN plenary [1]. According to the objectives in WID, RAN1 is tasked to specify the mechanisms to support SBFD, including semi-static indication of time/frequency location, random access in SBFD symbols, and other transmission, reception and measurement behavior and procedures for SBFD aware UE. Furthermore, the enhancement for CLI handing, including gNB-to-gNB and UE-to-UE CLI handling, will also be specified in RAN1. Accordingly from RAN4 perspective, it is tasked to specify BS RF, necessary RRM and demodulation requirements.

In this contribution, we would like to provide our further analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters.

# 2. RAN4 aspects for SBFD system parameters

### 2.1 Band/channel bandwidth support for SBFD

Based on the discussion in RAN4#111, SBFD was recognized as “a feature with requirements of which can potentially be applied to all TDD band”, and the declaration-based method for the SBFD-support in a particular TDD band was agreed in RAN4#112. As follow-up, in RAN4#112bis, the following agreements was provided [9]:

<From RAN4#112bis> Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead. Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported

Based on RAN1 work, the SBFD operation is not designed and studied for NR-U bands, therefore we have the following proposal.

Proposal 1: SBFD operation shall not be applied in NR-U bands.

Based on the above agreement for specification implementation for band/channel bandwidth support for SBFD, we provide the following proposal which gives the detailed text proposal in Clause 5.2 in TS 38.104:

Proposal 2: RAN4 shall adopt the following text proposal in Clause 5.2:

=================== Start of Text Proposal ===================

## 5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in all NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96 and n101, and all NR operating bands in FR2 which are defined in Table 5.2-2.

=================== End of Text Proposal ===================

For all supported channel bandwidth, except the channel bandwidth values < 20MHz in band n39 and the channel bandwidth values < 40MHz in other bands, there is no necessity to preclude other supported channel bandwidth values for each band from SBFD deployment. Accordingly the following Note can be added in Table 5.3.5-1.

Proposal 3: RAN4 shall introduce the following note in Table 5.3.5-1:

- “Note 9: Subband full duplex (SBFD) can be applied to all supported channel bandwidth values except the ones smaller than 20MHz in band n39 and ones smaller than 40MHz in other TDD bands except bands n34, n39, n46, n51, n53, n54, n96 and n101.”

### 2.2 Transmission bandwidth configuration for SBFD

In RAN4#112, much progress has been achieved on the definition clarification of BS channel bandwidth and BS transmission bandwidth configuration for SBFD, transmission bandwidth configuration for SBFD DL subband NRB,SBFD,DL, transmission bandwidth configuration for SBFD UL subband NRB,SBFD,UL and guardband between SBFD UL and DL subband NRB,SBFD,GB, as follows:

Agreement: - BS channel bandwidth for SBFD-capable BS: - BS channel bandwidth for SBFD-capable BS: RF bandwidth supporting a single NR RF carrier with the transmission bandwidth configured in the uplink or downlink or SBFD operation. Transmission bandwidth configuration for SBFD Agreement: - BS transmission bandwidth configuration for SBFD: - BS transmission bandwidth configuration for SBFD NRB,SBFD: set of resource blocks located within the BS channel bandwidth which may be used for transmitting or/and receiving by the BS. - The transmission bandwidth configuration for SBFD NRB,SBFD for each BS channel bandwidth and subcarrier spacing shall reuse the values for BS transmission bandwidth configuration in table 5.3.2-1 for FR1 and table 5.3.2-2 for FR2-1. - FFS a corresponding figure for SBFD is needed Agreement: - Transmission bandwidth configuration for SBFD UL/DL subband: - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband, expressed in resource blocks. - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks. - Guardband between SBFD UL and DL subband: - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband, expressed in resource blocks.

In the existing specification TS 38.104, the figure 5.3.1-1 is used to clarify the definition of channel bandwidth and transmission bandwidth configuration for one NR channel. For SBFD operation, the variable for transmission bandwidth configuration is changed to NRB,SBFD. Additionally, the illustrations of transmission bandwidth configuration for SBFD DL subband, UL subband and guardband between SBFD UL and DL subband shall also be provided.

In RAN4#112bis, the illustrative figures are discussed and the group agree to use the figures from Ericsson [R4-2416134] used as the baseline for further study:

Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

Firstly, as given in the WF [9], it is suggested that the definition of NRB,SBFD,DL can be updated for DUD case. Considering the fact that even mandating the UL subband always in the middle of transmission bandwidth configuration for SBFD, it is still possible the transmission bandwidth configurations of two SBFD DL subbands are different. Therefore, we suggest to firstly refine the definition of NRB,SBFD,DL and NRB,SBFD,GB for SBFD DUD case.

Proposal 4: For SBFD DUD case,

- the transmission bandwidth configurations for two SBFD DL subbands are denoted by NRB,SBFD,DL,1 and NRB,SBFD,DL,2

- the guardbands between SBFD UL and DL subbands are denoted by NRB,SBFD,GB,1 and NRB,SBFD,GB,2

Accordingly, the existing symbol explanation for NRB,SBFD,DL and NRB,SBFD,GB can be updated and the new ones for DUD case can be added.

Proposal 5: The existing symbol explanation for NRB,SBFD,DL and NRB,SBFD,GB can be updated and the new ones for DUD case can be added as:

  - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

  - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

  - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU or UD case, expressed in resource blocks.

  - NRB,SBFD,GB,1: The size of guardband between SBFD UL subband and the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,GB,2: The size of guardband between SBFD UL subband and the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

Furthermore, based on our observations on three figures given in R4-2416134, the following issues are identified:

Observation 1: The following issues are identified in the illustrative figures for SBFD DUD, DU and UD patterns:

  - Transmission bandwidth configuration for SBFD NRB,SBFD shall be used instead of  NRB

  - In SBFD DUD case, the values of NRB,SBFD,DL,1 and NRB,SBFD,DL,2 are used instead, which can be different

  - In SBFD DUD case, the two values of NRB,SBFD,GB,1 and NRB,SBFD,GB,2 are used instead, which can be different

  - WGuard is not defined which should not be used

  - “Channel Edge” can be added to illustrate channel bandwidth edge, the same as Figure 5.3.1-1 in TS38.104.

  - “Resource Block” can be added to illustrate the resource block represented by a rectangular, the same as Figure 5.3.1-1 in TS38.104.

Proposal 6: To solve the issues in the baseline figure for DUD, DU and UD case, the issues given in Observation 1 shall be solved, and the following revised figure for DUD case is given as example:

Figure 1: Definition of channel bandwidth and transmission bandwidth configurations for one NR channel for SBFD DUD case

### 2.3 DL/UL subband configurations support for SBFD

As requested in WID, to support SBFD operation at gNB side within a TDD carrier, the mechanism for semi-static indication of frequency location of SBFD subbands to UE shall be specified.

Based upon previous RAN4 discussion, it is “within RAN4 scope to study/specify the limitation or restriction on the size of subband/guardband”. In RAN4#112, the following three options with different levels of configuration flexibilities have been provided [7]:

Agreement: - To specify DL/UL subband configuration in BS RF requirements, the following 3 options for DL/UL subband configuration are identified: - Option-1 (with the least flexibility): - Only a number of selected DL/UL subband configurations are specified for RAN4 BS RF requirement. - Example (given in Nokia Tdoc R4-2413238): - For FR1 DUD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <106, 51, 5> - For FR1 DU or UD configuration in 100 MHz channel and 30 kHz SCS: < NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB > = <217, 51, 5> - Option-2 (with the middle flexibility): - Option-2a: - DL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - UL subband size and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and NRB,SBFD,DL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,UL and NRB,SBFD,GB. - Option-2b: - UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. - DL subband size(s) and guardband size(s) depends on BS implementation - Example (FR1 DU or UD in 100MHz and 30kHz SCS, 80-20MHz case): As long as BS transmission configuration for SBFD shall be 273 (NRB,SBFD for 100MHz and 30kHz SCS) and the value of NRB,SBFD,UL is selected from Table 5.3.2-1 (according to the 30kHz SCS), from BS RF requirement perspective, no limitation on the values NRB,SBFD,DL and NRB,SBFD,GB. - Option-3 (with the highest flexibility): - No restriction on the subband sizes, i.e., RF requirement shall be specified to enable all DL and UL subband configurations (and implicitly guardband configurations). - Example (FR1 DU or UD in 100MHz and 30kHz SCS): From BS RF requirement perspective, no limitation on the values of NRB,SBFD,DL, NRB,SBFD,UL, NRB,SBFD,GB - FFS how to implement BS RF requirements based on the above options for different levels of flexibility.

Among the three levels of flexibilities, we firstly identify Option-1 as being not only unreasonable but also against RAN1 agreements. Specifically, simply setting a number of selected DL/UL subband configurations is actually not attainable, since it is not possible to determine the size of subbands and guardband(s) in the standardization stage. If RAN4 is tasked to only pick up few configurations, companies will end up by proposing lots of configurations to make sure future products are standard-compatible, while lots of them could not be used in practice. Even that, the completed RAN4 standard may still miss some of demanded configuration in the future, because of the least flexibility by Option 1. Technically, the following techniques shall be considered in the system design, from RF and digital domains respectively:

- In the field of RF, to address interference from the DL and ensure accurate reception of the desired UL signal, an analog filter can be employed prior to the ADC. This filter serves to attenuate DL interference and protect the dynamic range of the ADC. An active approach can also involve an analog interference canceller, although in mMIMO systems, the complexity of the analog canceller increases as the number of Tx and Rx chains grows. All these factors could potentially impact the determination of the size of subband and guardband.

- In the digital domain, various techniques can be utilized to reduce the impact on the sensitivity of the BS receiver, aiming to keep it below 1 dB. One such technique is digital interference cancellation (IC), which aims to eliminate interference from the desired UL signal and reduce any remaining self-interference (SI) to an acceptable level. Digital IC involves synthesizing interference from baseband Tx samples with accurate modeling of Tx chain non-linearity or capturing interference from the RF chain on the transmit side and converting it to baseband. This process requires estimating the SI channel taps for each Tx chain to each Rx chain. Additionally, other digital techniques can help mitigate interference. For example, Digital Pre-distortion (DPD) can reduce leakage into the UL subband. Other techniques, such as beam nullification to lessen the impact of clutter reflections or optimizing digital Tx beamforming to decrease SI, can also be employed. All these digital domain techniques could be relevant in determination of the size of subband and guardband.

From RAN1 perspective, it is observed that RAN1 agree to support the flexible frequency domain configuration with the cell-specific configuration, by explicitly configuring frequency locations of UL subband and DL subband(s) thus making the guardband(s) if any can be derived implicitly. If only few configurations are specified for RAN4 RF requirements, we see no reason for RAN1 to define such flexible configuration:

<From RAN1#117> Agreement For cell-specific indication of SBFD subband frequency location, adopt the following option. - Option 1: Frequency locations of UL subband and DL subband(s) are explicitly configured. Guardband(s) if any are implicitly derived as the RBs which are not within UL subband or DL subband(s).

Observation 2: To specify DL/UL subband configuration in BS RF requirements, Option-1 (i.e., only a number of selected DL/UL subband configurations are specified) is impracticable for standardization and against RAN1 agreements.

Based on our understanding, the good tradeoff between configuration flexibility and standardization complexity can be achieved by Option 2, i.e., DL (or UL) subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. Among Option-2a and 2b, we prefer Option-2b because selecting UL subband configuration from the existing values of BS transmission bandwidth configuration will simplify the standardization procedure for receiver requirement.

Proposal 7: To specify DL/UL subband configuration in BS RF requirements, Option-2b is adopted as baseline while Option-1 should be precluded.

As one of the most controversial topics in RAN4#112bis, the group further discussed the Option 2b with the additional restrictions, as provided in the way forward [9]:

Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

Potential restriction-1: For DUD case, the UL subband should be placed in the middle of channel bandwidth

During last meeting, it is proposed by some companies that UL subband should be placed in the middle of channel bandwidth for SBFD DUD case. In general, this restriction provides additional restriction to place UL subband. Furthermore, based on the definition of Option-2b, the UL subband configuration shall be selected from the existing values of BS transmission bandwidth configuration, as provided in Table 5.3.2-1 for FR1 and Table 5.3.2-2 for FR2-1. For instance, taking 100MHz channel bandwidth with 30kHz SCS (NRB,SBFD=273) in FR1 as example, it is possible to place 20MHz (NRB,SBFD,UL=51) UL subband in the middle of channel bandwidth, while it is not possible to putting 10MHz (NRB,SBFD,UL=24) in the exact middle of channel bandwidth because the number of remaining RBs are odd number.

Table 5.3.2-1: Transmission bandwidth configuration NRB for FR1

| SCS (kHz) | 3 MHz | 5 MHz | 10 MHz | 15 MHz | 20 MHz | 25 MHz | 30 MHz | 35 MHz | 40 MHz | 45 MHz | 50 MHz | 60 MHz | 70 MHz | 80 MHz | 90 MHz | 100 MHz |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB | NRB |
| 15 | 15 | 25 | 52 | 79 | 106 | 133 | 160 | 188 | 216 | 242 | 270 | N/A | N/A | N/A | N/A | N/A |
| 30 | N/A | 11 | 24 | 38 | 51 | 65 | 78 | 92 | 106 | 119 | 133 | 162 | 189 | 217 | 245 | 273 |
| 60 | N/A | N/A | 11 | 18 | 24 | 31 | 38 | 44 | 51 | 58 | 65 | 79 | 93 | 107 | 121 | 135 |

Observation 3: For restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD), it is possible that UL subband can’t be located at the exact middle of channel bandwidth.

Proposal 8: The proponent of restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD) shall solve the issue in the Observation 3.

Potential restriction-2: The number of DL subband sizes to be declared and tested

There are two alternatives of this potential restrictions, in which the difference comes from whether several DL subband sizes shall be supported. Based on our understanding, it is possible that given a certain UL subband filtering implemented, gNB can fulfill the RAN4 minimum requirements given multiple DL subband configurations, e.g., with the relaxed Q-value subband filter centered at 20MHz UL subband (NRB,SBFD,UL=51) within 100MHz channel bandwidth (NRB,SBFD=273), the supported DL subband size can be a range, e.g., NRB,SBFD,DL,1 = NRB,SBFD,DL,2 can be 106 (with 5 PRB as guardband), 105 (with 6 PRB as guardband) etc. Based on our understanding, as long as the SBFD configuration has been tested against RAN4 requirement, the corresponding configuration shall be allowed.

Proposal 9: For the number of DL subband sizes to be declared and tested,

  - For FR1 WA BS, all DL subband sizes declared to support SBFD shall be tested;

  - This restriction is not applicable to other FR1 BS classes and FR2 BS.

Potential restriction-3: Restriction on UL subband size based on the specific channel bandwidth

Based on existing RAN4 agreement, the supported channel bandwidth shall be no less than 40 MHz for all bands except band n39, and it is reasonable to preclude the very small UL subband size considering the ratio of UL subband size over channel bandwidth is no less than 20%. Consequentially, the following proposal is provided:

Proposal 10: For UL subband size based on the specific channel bandwidth, the following restriction can be considered

  - For DUD and DU case, the ratio for UL subband size over channel bandwidth shall be no less than 20%

  - The restriction can be applied to both FR1 and FR2.

### 2.4 Restriction on guardband size

By adopting Option-2b which allows flexible DL subband size(s) and guardband size(s) depending on BS implementation. Accordingly, we suggest no restriction is required for both maximum and minimum number of RBs for guardband.

Proposal 11: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

# 3. Other general aspects for SBFD operation

### 3.1 BS RF specification structure for SBFD requirements

In previous RAN4 discussion, two options, i.e., creating new sub-clauses and embedding the SBFD RF requirement in corresponding section, were listed on how to incorporate SBFD requirements in the BS RF core specification (i.e., TS 38.104). Based on the discussion outcome from RAN4#112bis, the following way forward is achieved:

<From RAN4#112bis> Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

By considering the readability of the specification for SBFD-capable BS, we think the way of creating new and standalone sub-clauses (e.g., with a suffix) in TS 38.104 can be followed for SBFD-specific existing or new gNB RF requirements. As mentioned in previous RAN4 discussion by some companies, one comparable precedent is using suffix-B for Mobile IAB-node in the core and conformance specification for IAB nodes (TS 38.174, TS 38.176-1 and TS 38.176-2).

Observation 4: Setting new sub-clauses with a suffix for a new feature is not only a common practice used in UE RF requirement, but also in the specification for network nodes like IAB.

Furthermore, in the updated WID [1], the impacted existing TS (including TS 38.104 and TS 38.141-1/2 for BS requirements) are listed while there is no new specification listed for BS RF requirement dedicated for SBFD operation or SBFD-capable BS.

Observation 5: In WID of Rel-19 duplex evolution, there is no new specification expected, and TS 38.104 is listed as impacted existing TS by SBFD feature introduced.

Another drawback for setting up the new specification shall be noted: it could bring confusion to the reader the scope of this new specification, whether it is the specification for new SBFD-capable gNB or the specification for SBFD operation within SBFD symbols. If the former one is followed, we expect the specification should copy/paste lots of existing BS RF requirements which bring lots of specification implementation issues to sync-up future text proposals on the similar requirements in two specifications. If the latter is followed, we also expect lots of referring to TS 38.104 are required, which greatly impact the specification readability.

Considering all the above reasons, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B, for both new and modified existing requirements.

Proposal 12: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B, i.e.,

- For the SBFD BS RF requirements specified by modifying existing requirements:

  - Set up new sub-clause with suffix-B

- For the new BF RF requirements dedicated to SBFD BS

  - Set up new sub-clause with suffix-B

During the last meeting discussion, companies proposed to combine this discussion with the transmission bandwidth configuration discussion, which we think is not appropriate because the technical discussion should be separated from the specification implementation discussion.

Proposal 13: The issue of BS RF specification structure (or potentially new specification) is standardization problem for specification implementation, which shall be separated from other technical discussion.

# 4. Conclusion

In this contribution, we would like to provide our analysis and viewpoints on the general aspects including RAN4 aspects for SBFD system parameters, accordingly with the following observations and proposals obtained:

Proposal 1: SBFD operation shall not be applied in NR-U bands.

Proposal 2: RAN4 shall adopt the following text proposal in Clause 5.2:

=================== Start of Text Proposal ===================

## 5.2	Operating bands

NR is designed to operate in the operating bands defined in table 5.2-1 and 5.2-2.

NR operating band n1, which is defined in Table 5.2-1, can be applied for HAPS operation.

NR operating bands n1, n3, n34, n39, n41, n78, n79, which are defined in Table 5.2-1, can be applied for ATG operation.

NB-IoT is designed to operate in the NR operating bands n1, n2, n3, n5, n7, n8, n12, n13, n14, n18, n20, n25, n26, n28, n31,  n41, n65, n66, n70, n71, n72, n74, n85, n90, n106 which are defined in Table 5.2-1.

Subband full duplex (SBFD) is designed to operate in all NR TDD operating bands in FR1 which are defined in Table 5.2-1 except bands n34, n46, n51, n53, n54, n96 and n101, and all NR operating bands in FR2 which are defined in Table 5.2-2.

=================== End of Text Proposal ===================

Proposal 3: RAN4 shall introduce the following note in Table 5.3.5-1:

- “Note 9: Subband full duplex (SBFD) can be applied to all supported channel bandwidth values except the ones smaller than 20MHz in band n39 and ones smaller than 40MHz in other TDD bands except bands n34, n39, n46, n51, n53, n54, n96 and n101.”

Proposal 4: For SBFD DUD case,

- the transmission bandwidth configurations for two SBFD DL subbands are denoted by NRB,SBFD,DL,1 and NRB,SBFD,DL,2

- the guardbands between SBFD UL and DL subbands are denoted by NRB,SBFD,GB,1 and NRB,SBFD,GB,2

Proposal 5: The existing symbol explanation for NRB,SBFD,DL and NRB,SBFD,GB can be updated and the new ones for DUD case can be added as:

  - NRB,SBFD,DL: Transmission bandwidth configuration for SBFD DL subband in SBFD DU or UD case, expressed in resource blocks.

  - NRB,SBFD,DL,1: Transmission bandwidth configuration for the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,DL,2: Transmission bandwidth configuration for the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,UL: Transmission bandwidth configuration for SBFD UL subband, expressed in resource blocks.

  - NRB,SBFD,GB: The size of guardband between SBFD UL subband and DL subband in SBFD DU or UD case, expressed in resource blocks.

  - NRB,SBFD,GB,1: The size of guardband between SBFD UL subband and the lowest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

  - NRB,SBFD,GB,2: The size of guardband between SBFD UL subband and the highest assigned SBFD DL subband in SBFD DUD case, expressed in resource blocks.

Observation 1: The following issues are identified in the illustrative figures for SBFD DUD, DU and UD patterns:

  - Transmission bandwidth configuration for SBFD NRB,SBFD shall be used instead of  NRB

  - In SBFD DUD case, the values of NRB,SBFD,DL,1 and NRB,SBFD,DL,2 are used instead, which can be different

  - In SBFD DUD case, the two values of NRB,SBFD,GB,1 and NRB,SBFD,GB,2 are used instead, which can be different

  - WGuard is not defined which should not be used

  - “Channel Edge” can be added to illustrate channel bandwidth edge, the same as Figure 5.3.1-1 in TS38.104.

  - “Resource Block” can be added to illustrate the resource block represented by a rectangular, the same as Figure 5.3.1-1 in TS38.104.

Proposal 6: To solve the issues in the baseline figure for DUD, DU and UD case, the issues given in Observation 1 shall be solved, and the following revised figure for DUD case is given as example:

Proposal 7: To specify DL/UL subband configuration in BS RF requirements, Option-2b is adopted as baseline while Option-1 should be precluded.

Observation 3: For restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD), it is possible that UL subband can’t be located at the exact middle of channel bandwidth.

Proposal 8: The proponent of restriction-1 (the UL subband should be placed in the middle of channel bandwidth for DUD) shall solve the issue in the Observation 3.

Proposal 9: For the number of DL subband sizes to be declared and tested,

  - For FR1 WA BS, all DL subband sizes declared to support SBFD shall be tested;

  - This restriction is not applicable to other FR1 BS classes and FR2 BS.

Proposal 10: For UL subband size based on the specific channel bandwidth, the following restriction can be considered

  - For DUD and DU case, the ratio for UL subband size over channel bandwidth shall be no less than 20%

  - The restriction can be applied to both FR1 and FR2.

Proposal 11: For guardband size, no restriction shall be introduced on maximum/minimum number of RBs.

Observation 4: Setting new sub-clauses with a suffix for a new feature is not only a common practice used in UE RF requirement, but also in the specification for network nodes like IAB.

Observation 5: In WID of Rel-19 duplex evolution, there is no new specification expected, and TS 38.104 is listed as impacted existing TS by SBFD feature introduced.

Proposal 12: For introducing BS RF requirements for SBFD-capable BS, RAN4 shall adopt the way of creating new and standalone sub-clauses in TS 38.104 by setting up new sub-clauses with suffix-B, i.e.,

- For the SBFD BS RF requirements specified by modifying existing requirements:

  - Set up new sub-clause with suffix-B

- For the new BF RF requirements dedicated to SBFD BS

  - Set up new sub-clause with suffix-B

Proposal 13: The issue of BS RF specification structure (or potentially new specification) is standardization problem for specification implementation, which shall be separated from other technical discussion.

# 5. Reference

[1] RP-240789 (revision from RP-234035), “WID revision: Evolution of NR duplex operation: Sub-band full duplex (SBFD)”, Huawei, RAN#103.

[2] 3GPP TR 38.858 v18.0.0, “Study on Evolution of NR Duplex Operation (Release 18)”.

[3] R4-2406107, “Way Forward for [110bis][313] NR_duplex_evo”, Samsung, RAN4#110bis.

[4] R4-2409763, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#111.

[5] R4-2409958, “Way Forward for [111][313] NR_duplex_evo”, Samsung, RAN4#111.

[6] R4-2411637, “Discussion on SBFD general aspects”, Samsung, RAN4#112.

[7] R4-2413514, “Way Forward for [112][307] NR_duplex_evo_General”, Samsung, RAN4#112.

[8] R4-2416497, “Discussion on RAN4 aspects for SBFD system parameters”, Samsung, RAN4#112bis.

[9] R4-2416584, “Way forward for [112bis][306] NR_duplex_evo_General”, Samsung, RAN4#112bis.