3GPP TSG-RAN WG4 Meeting # 113	R4-2418107

# Orlando, USA, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Agenda item:	7.23.4.2

Document for:	Discussion

Introduction

In this contribution, we provide our views on issues of RRM core requirements for NR LP-WUS/WUR based on listed issues in approved WF [1].

Discussion

- Topic #1: RRM core requirements for LP-WUS/WUR

General aspects

### Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

RAN4 has been agreed to support case#1 and #3. But regarding case#2, RAN4 still be on going. We have same view with the moderator’s recommendation in the last meeting’s moderator summary as below.

Recommendations: Figure for illustration different cases, only for information purpose

If the signal quality is high enough, then the UE is not required to measure the neighbor cell. And if LR measurement is performed, the UE can be in case#1 where is serving cell measurement fully offloading region. But we think the high signal quality region can be divided to two area. Very high signal quality area and near the no neighbor cell measurement boundary area. So, if the UE is in the near the no neighbor cell measurement boundary area, the UE can measure serving cell with RRM relaxation reguirements.

Proposal 1: Support case2 scenario.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | On |

Issue 1-1-4: Threshold for switch between case 1 and case 3

This issue depends on whether case 2 will be considered or not.

Proposal 2: Need to discuss whether case 2 will be considered or not first.

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation

Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring

We think RAN4 does not need to consider about criteria which is up to RAN1/2’s decision.

Proposal 3: Up to RAN1/2’s decision. Support P1 for issue 1-1-5/1-1-6.

### Issue 1-1-7: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7

The LP-SS or PSS/SSS LR-based measurement threshold can be different from LP-WUS monitoring LR-based threshold. We think that for LP-WUS monitoring, at least the LP-WUR meausement should be activated concurrently. So, the LP-WUR measurement threshold can be less than or equal to the LP-WUS monitoring threshold.

Proposal 4: The LR-based RRM measurement threshold can be less than or equal to the LP-WUS monitoring threshold.

Issue 1-1-9: LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS

Before entering LP-WUS monitoring or after exiting LP-WUS monitoring, we think the MR measurement should be ON with RRM measurement on serving cell and neighbor cell(if any). And it doesn’t matter whether the RRM measurement with relaxtion or not. It depends on the threshold. And the LP-WUR can be ON or OFF. We think it depends on the threshold. At least Th1 should be higher than or equal to the Th2.

Proposal 5: The LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring

- MR is ON with RRM measurement on serving cell and neighbor cell (if any) and LP-WUR can be ON or OFF for serving cell measurement. If the threshold for LP-WUS monitoring and LP-WUR RRM measurement are same the LP-WUR can be OFF, else if the threshold for LP-WUS monitoring is higher than for the LP-WUR RRM measurement the LP-WUR can be ON.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

### Issue 1-2-1: Accuracy requirements

In the RAN4#110bis meeting, RAN4 discussed whether/how to define the accuracy requirements for LP-WUR in Idle/Inactive state and some agreements and information were made below.

RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: - RAN4 to discuss the simulation assumptions considering the target SNR, accuracy and number of samples. - RAN4 will define the delay requirement in core part, and further discuss whether/how to define the accuracy requirement. For information only: RAN4 requirement for LP-WUR RRM measurement in Idle/inactive states: Options for further discussion (the options are only for information purpose) - Option 1): Define the dedicated accuracy requirement in the performance section, FFS whether/how it can be verified in the test case. - Option 2): No dedicated accuracy requirement in the performance section, and reflect the accuracy performance as a margin in the core requirement. Option 2a): FFS accuracy requirement, and reflect the accuracy performance in delay requirement in core part. - Other options are not precluded.

Basically, in Idle/Inactive state, there is no measurement reporting, so core requirement for Idle/Inactive state is measurement delay based on the number of samples to meet measurement accuracy at certain side condition. Therefore, no dedicated accuracy requirement is needed.

Proposal 6: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section.

### Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

In the RAN4#112bis meeting, RAN4 discussed whether the requirements for entry/exit critera evaluation for WUS monitoring/LP-WUR measurement/MR RRM relaxation. And the proposals and moderator’s recommendatios are as below:

- Proposals - P1: Define evaluation period for evaluating entry/exit threshold for LP-WUS monitoring; Define evaluation period for evaluating entry/exit threshold for fully offloading (case 1) or MR RRM relaxation (case 3). (vivo CATT LG MTK) - P1-1: The evaluation period is the same as that of the corresponding measurement period. (vivo) - P1-2: The evaluation period is based on LP-WUR measurement period, and further discuss whether to define measurement filtering (e.g., number of samples). (CATT) - P1-3: RAN4 to discuss the delay requirements to evaluate the entry criteria by using MR measurement results only and to evaluate exit criteria by using LR measurement results only. (MTK) - P2: RAN4 specify following requirements for MR RRM relaxation and offloading and existing requirements for S criteria or relaxed measurement criterion can be considered as starting point (Samsung) - LR measurement period and accuracy - MR measurement relaxation and the evaluation period for entry/exit criteria threshold evaluation under different states (case 1 fully offloading, and case 3 MR relaxation) - P3-1: Do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition). Use the MR measurement period and/or LR measurement period to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; (Apple) - P3-2: RAN4 to define requirements for serving cell evaluation against the entry/exit condition for LP-WUS monitoring, including Measurement delay (number of samples); Measurement accuracy (margin for evaluation); Measurement filtering (number of samples and separation between samples) (Huawei) - P4: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring: (QC) - The number of measurement samples needed for filtering. - The number of consecutive times the UE needs to meet the entry/exit criteria Recommendations: For both MR and LP-WUR, for checking whether the thresholds for entry/exit WUS paging monitoring/LP-WUR measurement/MR RRM relaxation are fulfilled or not, Option 1: Use the measurement period of LR or MR (Note: the measurement period of LR has already been agreed to be specified as issue 1-1-2) Option 2: Define another evaluation delay period. Evaluation is longer or the same as that of measurement period.

We think the WUS paging monitoring and LP-WUR measurement entry/exit criteria can be based on LR threshold. But in case of the entry of LP-WUR measurement, there is no LR results. So, only MR results can be used for check the entry of LP-WUR measurement. But the higher than some level of MR results can not guarantee good enough of LR performance. So, additionally using the LR results evalution requirement also can be necessary. From MR offloading state to MR RRM relaxation transition is also similar situation. There is no MR results, so LR based checking is necessary. But additionally using the MR results evalution requirement can be necessary.

Proposal 7: Define evaluation period for evaluating entry threshold for LP-WUS monitoring or LP-WUR measurement; Define evaluation period for evaluating exit threshold for fully offloading (case 1).

### Issue 1-2-2-1: Detail on LP-WUR/MR evaluation/measurement requirement

Proposal 8: We support detail evaluation periods as below:

- For the evaluation period for evaluating entry threshold for LP-WUS monitoring or LP-WUR measurement

  - LP-SS based LP-WUR measurement period

- For the evaluation period for evaluating exit threshold for fully offloading

  - The MR serving cell measurement period

### Issue 1-2-2-2: LR measurement metrics used for entry/exit threshold evaluation for LP-WUS monitoring/RRM relaxation

As mentioned previous, for the WUS paging monitoring and LP-WUR measurement entry/exit criteria can be based on LR threshold. And as the reference signal for LR-based requirement, LP-SS can be considered. Regarding the measurement metrics, following the RAN2’s decision, the UE can use (LP-)RSRP and optional (LP-)RSRQ.

Accoring to RAN2 agreement,:

- RAN2#127: Procedure and configuration of LP-WUS in RRC_IDLE/INACTIVE The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ.

Proposal 9: As the reference signal for LR-based requirement, LP-SS can be considered. Regarding the measurement metrics, following the RAN2’s decision, the UE can use (LP-)RSRP and optional (LP-)RSRQ.

MR RRM relaxation

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

As mentioned above section 2.1, MR based measurement relaxation for serving cell and/or nighbor cells should be considered for UE supporting LP-WUR. Based on the simulation results, to get enough power saving gain, the relaxation factor should be at least 8. We support P1-1 that the relaxation factors within the range from 8 to 16 as starting posint for the relaxation factor. This can be adapted using multiple threshold.

Proposal 10: Support P1-1 that the relaxation factors within the range from 8 to 16 as starting posint for the relaxation factor. This can be adapted using multiple threshold.

LP-WUR CONNECTED mode

Issue 1-4-1: LP-WUR at CONNECTED mode

Based on the objectives for LP-WUS/WUR, RAN4 needs to clarify the overall scope of RRM core requirements. LP-WUS design commonly applicable to both IDLE/INCATIVE and CONNECTED modes, but RAN4 needs to only focus on IDLE/INACTIVE mode related core requirements.

Proposal 11: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19.

Conclusion

In this contribution, we provide views on issues listed in WF [1], and we propose

Proposal 1: Support case2 scenario.

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | On |

Proposal 2: Need to discuss whether case 2 will be considered or not first.

Proposal 3: Up to RAN1/2’s decision. Support P1 for issue 1-1-5/1-1-6.

Proposal 4: The LR-based RRM measurement threshold can be less than or equal to the LP-WUS monitoring threshold.

Proposal 5: The LP-WUR status before entering LP-WUS monitoring or after exiting LP-WUS monitoring

- MR is ON with RRM measurement on serving cell and neighbor cell (if any) and LP-WUR can be ON or OFF for serving cell measurement. If the threshold for LP-WUS monitoring and LP-WUR RRM measurement are same the LP-WUR can be OFF, else if the threshold for LP-WUS monitoring is higher than for the LP-WUR RRM measurement the LP-WUR can be ON.

Proposal 6: Do not define measurement accuracy requirements for LP-WUR in Idle/Inactive state in performance section.

Proposal 7: Define evaluation period for evaluating entry threshold for LP-WUS monitoring or LP-WUR measurement; Define evaluation period for evaluating exit threshold for fully offloading (case 1).

Proposal 8: We support detail evaluation periods as below:

- For the evaluation period for evaluating entry threshold for LP-WUS monitoring or LP-WUR measurement

  - LP-SS based LP-WUR measurement period

- For the evaluation period for evaluating exit threshold for fully offloading

  - The MR serving cell measurement period

Proposal 9: As the reference signal for LR-based requirement, LP-SS can be considered. Regarding the measurement metrics, following the RAN2’s decision, the UE can use (LP-)RSRP and optional (LP-)RSRQ.

Proposal 10: Support P1-1 that the relaxation factors within the range from 8 to 16 as starting posint for the relaxation factor. This can be adapted using multiple threshold.

Proposal 11: Focus on RRM core requirements only for IDLE/INACTIVE mode in Rel-19.

Reference

[1] R4-2413899, “WF for RRM requirements for LP-WUSWUR,” vivo