3GPP TSG-RAN WG4 Meeting #113		R4-2418109

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.19.3.1

Source:		Samsung

Title:	Discussion on RRM requirements of UE-initiated beam management in Rel-19 MIMO phase 5

Document for:		Discussion

- Introduction

The latest revised WID [1] has been approved in the RAN plenary RAN#105 meeting. The objectives of Rel-19 MIMO Phase 5 are described in [1] including:

The detailed objectives are as follows: RAN1: - Specify enhancement to facilitate UE-initiated/event-driven beam management for reducing overhead and/or latency, assuming the unified TCI while leveraging (as much as possible) legacy CSI measurement and reporting configuration frameworks, targeting FR2 and sTRP with intra- and inter-cell beam management - UL signaling content(s) (and procedure(s) as required) for UE-initiated/event-driven beam reporting facilitating fast beam switching - UL signaling medium/container considering the UE-initiated/event-driven nature of the UL transmission, designed primarily for the purpose of beam reporting - Specify CSI support for up to 128 CSI-RS ports, targeting FR1 - Type-I codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks - Type-II codebook refinement supporting up to a total of 128 CSI-RS ports across all resources, assuming legacy CSI-RS resources (with up to 32 CSI-RS ports per resource), based on extension of legacy codebooks, without modifying any codebook parameter other than introducing additional values for the number of ports codebook parameter(s) - Extension of CRI(s)-based CSI reporting (CQI/PMI/RI calculated per CRI for ≥1 CRIs) for hybrid beamforming supporting up to a total of 128 CSI-RS ports across all resources, with up to 32 CSI-RS ports per resource, without new codebook design - SRS port grouping and its association to the two codewords for the 6/8Rx low complexity receiver supporting more than 4 layers, with legacy codebook - No enhancement on codeword-to-layer mapping, DL resource allocation, CSI feedback, and DCI format - Note: Whether to support 6Rx with more than 4 layers is to be decided in RAN4 Rel-19 RF enhancements WI - Specify UE reporting enhancement for CJT deployments under non-ideal synchronization and backhaul, targeting FR1, both FDD and TDD - Inter-TRP time misalignment and frequency/phase offset measurement and reporting, assuming legacy CSI-RS design, with stand-alone aperiodic reporting on PUSCH - Specify non-coherent UL codebook to facilitate 3-antenna-port codebook-based transmissions, enhancement(s) to enable 3T6R SRS antenna switching, as well as UE capability signaling for 3T3R antenna switching and 3-antenna-port non-codebook-based transmissions, without enhancement on UL full power transmission and without enhancement on SRS resource Note: UL full power transmission mode 1 and 2 are not supported. Note: Other than UE capability signaling, no other enhancement is specified for 3T3R SRS antenna switching. - Specify enhancement for asymmetric DL sTRP/UL mTRP deployment scenarios, assuming intra-band intra-DU non-co-located mTRP scenarios, without changing existing cell definition or defining a new cell (e.g. UL-only cell), assuming the Rel-17/18 unified TCI framework and fully reusing the legacy QCL/UL spatial relation rules, targeting FR1 and FR2 - Two closed-loop PC adjustment states for SRS, both separate from PUSCH; and pathloss offset configurations for pathloss calculation to UL TRP(s), when the pathloss RS is from DL sTRP. - Two TAs through reusing Rel-18 specification of two TAs for multi-DCI-based multi-TRP and removing the restriction that coresetPoolIndex needs to be configured, assuming legacy PRACH resources Acronyms: - sTRP: single TRP (transmit-receive point) - mTRP: multiple TRP (transmit-receive point) - CJT: coherent joint transmission - DU: distributed unit - CRI: CSI-RS resource indicator RAN2: Specify higher layer support of the enhancements listed above. RAN4: Specify necessary core requirements for the enhancements listed above.

In RAN4#112bis meeting, we have already discussed the RRM requirements of UE-initiated/event-driven beam management and achieved some agreements in WF [2]. In this contribution, we continue to discuss the open issues of RRM requirements for UE-initiated/event-driven beam management.

- Discussion

In RAN4#112bis meeting, the agreements and open issues are captured in WF [2]. We will discuss the open issues.

Issue 1-2: How to define measurement period in Event triggered measurement reporting delay? <Way forward > - Option 1: - If multiple beams are measured with different measurement period, RAN4 to discuss how to consider the total measurement time. - Option 2: - It is proposed to take measurement period defend for Rel-18 LTM as baseline - Option 3: - RAN4 should investigate whether those measurement periods can be reused for defining measurement reporting delay requirements for UE-initiated/event-driven beam management - Option 4: - Use legacy measurement period for TL1-RSRP_Measurement_Period_SSB for SSB based intra-cell , inter-cell, TL1-RSRP_Measurement_Period_CSI-RS for CSI RS based. - Option 4a: - Remove Treport or set to 0 in legacy requirements. - Option 4b: - Remove considerations for HST enhancements/ scenarios - From legacy measurement period, for new beam report further discuss if M is needed or can be removed/assumed 1. - From legacy measurement period, for new beam report further discuss if P can be the same or modified for simplification. - Option 4c: - For FR2, the T_L1_measurement_period is scaled by N. - Other options are not precluded.

In legacy L3 event triggered measurement reporting delay, the measurement time is one L3 measurement period.

The event triggered measurement reporting delay, measured without L3 filtering shall be less than Tidentify intra with index or T identify intra without index defined in clause 9.2.5.1 or clause 9.2.6.2.

At the beginning, we want to reuse the similar principle as one L3 identify time (including sync time and measurement period).

However, according to the definition in RAN1, it is different from that in L3. The condition needs to be met to consider a configurable number of M as below:

RAN1#117 Agreement Regarding the triggering event determination for Event 2: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam Above feature is subject to UE capability. - Basic feature: Once the L1-RSRP of the new beam becomes a threshold value better than the current beam, UE initiated beam report occurs

The procedure is shown as:

Proposal 1: In the event triggered measurement reporting delay, the L1 evaluation time is included at least. However, as figure shown above, it cannot reuse one-shot identify time like L3 event triggered measurement reporting delay.

RAN4 to discuss and specify the evaluation time inside of event triggered measurement reporting delay.

Issue 1-4: How to consider the time window of triggering event determination in Event triggered measurement reporting? <Way forward > - Option 1: - RAN4 needs to consider the impact of triggering window defined in RAN1. - Option 2: - It is proposed to define requirements for both the case without filtering and the case with filtering (i.e. timer and counter M). - It is proposed that same reporting requirements are applied to the case without filtering and the case with filtering (i.e. timer and counter M) - Option 3: - No additional time-window-based filtering or other filtering should be applied for the L1-RSRP measurements - Option 4: - Discuss the necessity of filtering based on the two trigger-event detection solutions approved in RAN1. - For the basic solution, one-shot measurement is defined by RAN1. No room to consider filtering. - For the optional solution, multi-shot measurement with at least M instance is defined by RAN1. Filtering is feasible. - Option 5: - RAN4 to discuss how to define one Event-2 instance for following Event-2 case: - If within a time window (which is configurable), the number of Event-2 instance(s) for at least one same new beam is greater than or equal to a configurable number M, UE initiated beam report occurs. - Note: Event-2 instance for a new beam is determined if the L1-RSRP of the new beam becomes a threshold value better than the current beam - Option 6: - Wait further RAN1 progress. - Other options are not precluded.

For the time window of triggering event determination, as shown in the figure, it is the condition of event is met. Therefore, it cannot be ignored for the delay. Only after the evaluation, UE detects the event condition is met, then UE is ready and trigger L1 reporting.

In RAN1 latest discussion in last RAN1#118bis meeting:

Proposal 1.3.1A (offline consensus): Regarding the triggering event determination for Event 2, the event instance(s) counting is per new beam. Further study candidate condition(s) of resetting the counting.

The time window and counting of resetting are still open. However, it can be observed it will impact the evaluation which is related to RAN4 reporting delay.

Proposal 2: For event trigger reporting delay, RAN4 need to consider the time window and M defined in RAN1. Need further RAN1 progress.

In addition, in last RAN1#118bis meeting, there is discussion related to the evaluation for current beam and new beam:

Agreement Regarding the triggering event determination for Event 2, down-select among the following alternatives for the evaluation periodicity for determining Event-2 instance [at least when DRX is not configured] - Alt-1: The periodicity of the current beam RS should be the same as that of the new beam RS(s). - The evaluation periodicity is the same as the periodicity of the current and new beam RS(s) - Alt-2: The periodicity of the current beam RS can be different from that of the new beam RS(s) - Alt-2_1: The evaluation periodicity is the same as the periodicity of the current beam RS; - Alt-2_2: The evaluation periodicity is the same as periodicity of the new beam RS; - Alt-2_3: The evaluation periodicity is the same as shortest periodicity of the current beam RS and new beam RS(s): - Alt-2_4: The evaluation periodicity is the maximum of {X ms, shortest periodicity of the current beam RS and new beam RS(s)}: - Alt-2_5: The evaluation periodicity is the same as largest periodicity of the current beam RS and new beam RS(s): FFS: Whether the periodicity of the current beam RS should be the same as that of the new beam RS(s). Note: There is the same periodicity for the new beam RS(s).

In addition, for the type of RS, the working assumption is shown as:

Agreement Regarding RS measurement for the current beam for Event 2, for Option-2a, the following working assumption in RAN1#117 is confirmed with modification: - (Working Assumption) Enabling of either Scheme-1 or Scheme-2 should ensure the same RS type for RS measurement for current beam and new beam. Note: In such case, the RS type comprises SSB and CSI-RS configured in a CSI-RS resource set configured with repetition.

Although there is no agreement but WA, the current assumption is the RS types for current beam and new beam are the same. It is not the same as that in event triggered L1 reporting in LTM.

However, for the RS periods of current beam and new beams can be different.

In RAN1 discussion, it used the terminology of “evaluation periodicity” but not RS periodicity. The evaluation is from UE’s perspective. This part will also affect the event trigger reporting delay. RAN4 should consider it in evaluation time in event trigger reporting delay.

Proposal 3: Need further RAN1 progress for RSs relationship between current beam and new beams.

Issue 1-8: CSI-RS based L1-RSRP for inter-cell BM? <Way forward > - Option 1: - RAN4 should not introduce in Rel-19 CSI-RS based L1-RSRP measurements for inter-cell beam management as long as RAN1 has not identified a clear use case - Option 2: - For CSI-RS based inter-cell L1-RSRP measurement, since no specific discussion before, RAN4 needs to identify whether additional clarifications on the following points are necessary: - Known condition for the NSC, especially for the time offset assumption between serving cell and NSC; - The prerequisite on L3 measurement on NSC. - Option 3: - RAN4 to define CSI-RS based L1-RSRP measurements requirements for inter-cell mTRP scenario in FR2. - RAN4 to use principles of sharing agreed for SSB based L1-RSRP for inter-cell CSI-RS based L1-RSRP measurements. - Other options are not precluded.

The scenario in the WID is defined as FR2 intra-cell and inter-cell scenario.

In Rel-17, the beam management for inter-cell are based on SSB only. There is no beam management for CSI-RS which is for additional PCI Cell. The TRP in additional PCI is only from another SSB which is different from serving cell. For CSI-RS L1 measurement, it is still for serving cell.

Proposal 4: For inter-cell mTRP in FR2, the beam management is only based on SSB L1-RSRP. RAN4 do not specify CSI-RS based L1-RSRP measurement for additional PCI Cell (inter-cell).

Issue 1-9: Whether to specify new unified TCI switching delay requirements? <Way forward > - Option 1: - RAN4 should investigate whether any new required TCI state switch delays need to be defined or existing requirements are sufficient - Option 2: - Wait further RAN1/RAN2 progress. - Option 3: - No need to define new TCI state switching delay requirements. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching.This can be revisited if the RS in the RS resource set can be updated by MAC-CE in further RAN1 conclusion. - Other options are not precluded.

Issue 1-10: Impact of beam on known/unknown conditions for TCI states for TCI states switching based on UEIBM? <Way forward > - Option 1: - Reuse known/unknown conditions for Rel-17 unified TCI states when defining requirements for TCI-switching based on UEIBM. - Option 2: - No impact of UE triggered beam report to known/unknown conditions for TCI state. - The known/unknown condition would be determined by the same criteria as legacy. - Other options are not precluded.

After the UE sent L1 report to gNB, gNB can use the L1 report results to indicate UE to switch to a new beam or not. Until now, there is no any difference whether the L1 reports are UE triggered or NW based. The only difference is when the target TCI state is unknown in the TCI state switching. In legacy requirements, L1-RSRP time is added in the TCI state switching delay. However, in event-triggered L1 measurement, the target TCI is always known if it remains detectable. For unknown case of the switching, it has less usage in event trigger L1 reporting.

Proposal 5: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy.

Proposal 6: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM.

Issue 1-11: Whether to enhance beam sweeping in event triggered L1 beam reporting? <Way forward > - Option 1: - Discussions on beam sweeping enhancements should be postponed till the baseline requirement for delay and measurement period have taken place - Option 2: - Reuse the current assumption of beam sweeping for requirements for UE/ event triggered beam reporting. No enhancement. - Option 3: - Specify RAN4 requirements for a UE supporting Rel-18 multi-Rx fast beam sweeping and configured with UEIBM reporting. - Other options are not precluded.

The fast beam sweeping in rel-18 multi-RX is applicable when gNB configures groupBasedBeamReporting and within the single carrier.

In the WID, the UEIBM is targeting FR2 and sTRP communication.

Proposal 7: No enhancement for fast beam sweeping.

Issue 1-12: Whether to define new accuracy requirements by Enhancement for UE-initiated/event-driven beam management? <Way forward > - Option 1: - The existing accuracy requirements for L1 measurements can be applied. No new accuracy requirements. - Option 2: - For SSB based beam report, Rel-18 LTM accuracy requirements can be used as baseline. - For CSI-RS based beam reprt, FFS whether to use legacy accuracy requirements which only cover serving cell, or to define new accuracy requirements. - Option 3: - Discuss the accuracy for the two trigger-event solutions respectively. - For the basic trigger-event detection, tight the existing L1-RSRP accuracy requirement; - For the optional trigger-event detection, reusethe existing L1-RSRP accuracy requirement. - Other options are not precluded.

Proposal 8: Focus on core requirements firstly, FFS on accuracy requirements.

Issue 1-13: Capabilities for Support of Event Triggering and Reporting Criteria? <Way forward > - Option 1: - Define a capability for supported number of UE-initiated/event-driven beam reporting for intra-frequency. - Other options are not precluded.

Issue 1-14: Compatibility of capabilities with LTM <Way forward > - Option 1: - Events configured for mobility and MIMO should be considered as different events even when they are configured for the same beams and the same cell with the same threshold. - Define the single capability for total number of L1 events shared by mobility L1 events and MIMO L1 events on intra-frequency. - Other options are not precluded.

Event triggered L1 measurement reporting will be introduced in Rel-19 MIMO and Rel-19 mobility separately. However, the motivation and procedures are different. For example, one is for FR2 different beams, while the other is for FR1 and FR2 beams for candidate cells. The Events are also different. In addition, there might be different UE capabilities to support either of them or both of them.

Proposal 9: Define separate capabilities for MIMO and mobility.

- Conclusion

In this contribution, we provide our proposals for RRM requirements for UE-initiated/event-driven beam management in Rel-19 MIMO phase 5:

Proposal 1: In the event triggered measurement reporting delay, the L1 evaluation time is included at least. However, as figure shown above, it cannot reuse one-shot identify time like L3 event triggered measurement reporting delay.

RAN4 to discuss and specify the evaluation time inside of event triggered measurement reporting delay.

Proposal 2: For event trigger reporting delay, RAN4 need to consider the time window and M defined in RAN1. Need further RAN1 progress.

Proposal 3: Need further RAN1 progress for RSs relationship between current beam and new beams.

Proposal 4: For inter-cell mTRP in FR2, the beam management is only based on SSB L1-RSRP. RAN4 do not specify CSI-RS based L1-RSRP measurement for additional PCI Cell (inter-cell).

Proposal 5: For known conditions for TCI states for TCI states switching based on UEIBM, it can use as legacy.

Proposal 6: No difference by using UE triggered L1 report or NW based L1 report. Existing unified TCI state switching delay requirements are applicable for event triggered measurement reporting based TCI state switching if known case. No unknown case is applied if the TCI states switching is based on UEIBM.

Proposal 7: No enhancement for fast beam sweeping.

Proposal 8: Focus on core requirements firstly, FFS on accuracy requirements.

Proposal 9: Define separate capabilities for MIMO and mobility.

- Reference

[1] RP-242394, WID revision: NR MIMO Phase 5, Samsung

[2] R4-2416856, WF on RRM requirements for NR_MIMO_Ph5_Part1, Samsung