3GPP TSG-RAN WG4 Meeting #113		R4-2418112

Orlando, US, 18th – 22nd November, 2024

Agenda item:		7.24.2.2

Source:		Samsung

Title:	Discussion on RRM requirements of CSI-RS based L1 measurement for LTM

Document for:		Discussion

- Introduction

In previous RAN4 meetings, the RRM impacts of CSI-RS based L1 measurement for LTM has been discussed. The agreements in RAN4#112bis are captured in WF [1]. In this contribution, we continue to discuss the RRM requirements of CSI-RS based L1 measurement for LTM.

- Discussion

CSI-RS based L1 measurement for LTM

#### Issue 2-2: known cell definition in CSI-RS based L1 measurement For CSI-RS based L1-RSRP measurement, the cell is considered as known if the following conditions are met: - Agreement: - The UE has performed L3 measurement on the target cell during the last 5 seconds, and - Further check whether to consider SSB-based L3 measurement only - The SSB from the target cell configured for L3[/L1] measurement remains detectable according to the cell identification requirements in clause 9.2. - L1 measurement in the 2nd bullet is not needed for FR1. - Further discuss whether to keep or remove L1 measurement for FR2-1 in the 2nd bullet. - The CSI-RS from the target cell configured for L1 measurement remains [detectable/measurable]. - Further discuss whether to use detectable or measurable. - Further discuss the side condition

In Rel-18 LTM Intra-frequency L1-RSRP measurements for neighbor cell and NR inter-frequency L1 measurement， the cell is considered as known if:

- The UE has performed L3 measurement on the target cell during the last 5 seconds, and - The SSB from the target cell configured for L1 measurement remains detectable according to the cell identification requirements specified in clause 9.2.ment remains detectable according to the cell identification requirements specified in clause 9.2.

SSB-based L3 measurement are widely used. For CSI-RS based L3 measurement, firstly, it is an optional capability. Secondly, the CSI-RS L3 measurement is based on its associated SSB. From network implementation, it may set it to associate SSB. In Rel-18 LTM measurement, it doesn’t mention what is the type of L3 measurement which means any of them should be fine. However, CSI-RS L3 measurement is valid based on it’s associated SSB.

Proposal 1: A cell is considered as known if the conditions are met:

- The UE has performed L3 measurement on the target cell during the last 5 seconds (no need to have both SSB-based L3 measurement and CSI-RS based L3 measurement; only SSB-based L3 measurement)

#### Issue 2-3: CSI-RS based L1 measurement outside active BWP or with measurement gap - Candidate solutions - Option 1: Deprioritize CSI-RS based L1-RSRP measurement outside active BWP (or with measurement gap) in R19. (MTK, QC, vivo, OPPO, Nokia) - Option 1a: RAN4 can first discuss measurement within active BWP for CSI-RS based L1 measurement and wait for progress in other WG about whether measurement gap will be supported. (Xiaomi) - Option 1b: RAN4 to define intra-frequency CSI-RS measurement requirements first. Support inter-frequency L1-RSRP CSI-RS measurements without gap. FFS for inter-frequency with gap. (Nokia) - Option 2: RAN4 at least support the following measurement types for CSI-RS based L1 measurement. (CATT) - Without gap - With gap - Option 2a: for CSI-RS based LTM, it is proposed to consider intra-frequency measurement, inter-frequency measurement without gap and inter-frequency measurement with gap, which are aligned with Rel-18 SSB based LTM. (CMCC) - Option 2b: prioritize intra-frequency measurement without GAP, inter-frequency measurement with GAP. (Apple, Samsung, HW, OPPO, CATT) - Option 2c: CSI-RS based L1 measurements for candidate cells shall support - Measurement within active BWP, Measurements outside active BWP. (E///, ZTE)

#### Issue 2-14: definition of intra-frequency and inter-frequency for CSI-RS based L1 measurement (already discussed during online session) - Candidate solutions: - Option 1: Following CSI-RS based L3 measurement (CTC, CMCC, Apple, Samsung, OPPO, [Nokia?]) - A measurement is defined as a CSI-RS based intra-frequency L1 measurement provided that: - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and - It is applied for SCS = 60KHz - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement - Option 2: For definition of intra-frequency and inter-frequency, suggest to wait for more progress in other WGs. (Xiaomi) - Option 3: For backward compatibility, RAN4 better to have a definition for intra- and inter-frequency CSI-RS based L1-RSRP measurement as legacy. (CATT) - One possible way is to revise the legacy definition and clarify that the reference centre frequency can be any of centre frequencies of the CSI-RS resources of the serving cell. - Option 4: Considering the introduction of CSI-RS based LTM L1 measurement, whether an LTM cell switch is intra-frequency or inter-frequency may depend on whether the associated SSB reported in the CSI-RS-based measurement result is intra-frequency or inter-frequency. (vivo) - Whether the associated SSB is intra-frequency or inter-frequency follows the R18 rules - No additional definition of intra-F/inter-F for CSI-RS based L1 measurements. - Option 5: Intra-frequency CSI-RS based L1 measurement can be defined that the NZP-CSI-RS resource(s) of a candidate cell is within the DL active BWP of the serving cell and the SCS and CP of NZP-CSI-RS resource(s) of a candidate cell is same as those of the DL active BP of the serving cell. (HW) - It should be further discussed when the DL active BWP of the serving cell is within the bandwidth of NZP-CSI-RS resource(s) of a candidate cell.

From RAN1 perspective, the periodic CSI-RS are supported for candidate cells. There is no restriction for the frequency location of the CSI-RS resources. In Rel-18 LTM, intra-frequency L1-RSRP measurements for neighbor cell and inter-frequency L1-RSRP measurement based on SSB are specified.

From the flexibility of configuration of CSI-RS resources, it can have same/different of SCS/CP type/Bandwidth. Therefore, for different candidate cells, they can be different for SCS/CP type/bandwidth in practical.

In L3 measurement, it is assumed that all CSI-RS resources in the same MO are with the same measurementBandwidth. If they are different, no requirements.

In L1 measurement, if the measurementBW of serving cell and candidate cell are different, it is assumed as inter-frequency measurement.

Proposal 2: Support both intra-frequency and inter-frequency CSI-RS measurement.

For the definition of intra-frequency, reuse the definition:

      - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

      - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

        - It is applied for SCS = 60KHz

      - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

      - The measurementBW are the same.

#### Issue 2-4: whether SSB based L3 measurement is a pre-requisite of CSI-RS based L1 measurement Tentative Agreement during online session: further discuss the two alternatives Alternative 1: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 2: For FR2-1, SSB-based L1 measurements and reports on the same candidate cell - Further discuss whether Step 2 can be skipped under certain conditions. FFS the details of the conditions. - Note: Step 2 is removed for FR1. - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1 or 2. - Note: For FR1, the CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1. Alternative 2: - RAN4 to consider the following measurement procedure for CSI-RS based LTM candidate cell measurements: - Step 1: SSB based L3 measurements [and reports] - Step 3: CSI-RS based L1 measurements on the same candidate cell - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Compare Alternative 1 and Alternative 2, the only difference is whether Step 2 is needed in FR2.

From the QCL type of candidate cell of CSI-RS, there is no restriction to only associated to SSB. And for SSB beam management, it is unnecessary to perform CSI-RS L1 measurement only after SSB L1 measurement. The CSI-RS can be separate from the same SSB beam in the candidate cell. L3 measurement is enough.

Proposal 3: No need to keep SSB L1 measurement and reports before CSI-RS L1 measurement, which is:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

#### Issue 2-10: CSI-RS resource type - Candidate solutions: - Option 1: RAN4 prioritize CSI-RS based L1 measurement requirements for periodic CSI-RS. (vivo)

According to current RAN1 working assumption in [2], periodic and semi-persistent CSI-RS is supported.

Proposal 4: RAN4 define CSI-RS based L1 measurement for periodic and semi-persistent.

#### Issue 2-12: RTD assumption - Further discuss based on the following assumption: - RTD is the received time difference among CSI-RS and SSB from different cells. - Alt 1: define requirements for both RTD<CP and RTD>CP. Introduce an optional UE capability for RTD>CP. - Alt 2: only define requirements for RTD<CP. - NW shall only enable this when RTD<CP is guaranteed.

In the neighbor cell measurement, the RTD between cells depends on the different propagation delays. For different neighbor cells, they are typical with the condition of RTD>CP in real deployments. In Rel-18 L1-RSRP measurement for neighbor cells, RTD>CP is supported. The real RTD will not be changed by changing type of measurement RS.

Proposal 5: RAN4 define the requirements for RTD>CP. Need further clear definition of RTD.

#### Issue 2-18: accuracy and side condition for CSI-RS based L1-RSRP measurement - Candidate solutions: - Option 1: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point. (Apple) - Option 1a: CSI-RS based L1-RSRP accuracy of the serving cell can be applied for CSI-RS based L1-RSRP of the candidate neighbour cells. (HW) - Option 2: RAN4 to discuss the CSI-RS accuracy requirements during performance part. (E///)

For the accuracy and side condition for CSI-RS based L1-RSRP measurement, we are fine with option 1. However, the legacy requirements are applied with 48PRBs and density is 3. Need further progress in core to check whether update is needed.

Proposal 6: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point. Need further progress to check whether any update is needed (if bandwidth of CSI is not 48PRBs, etc. )

#### Issue 2-19: CSI-RS based L1-SINR measurement - Candidate solutions: - Option 1: RAN4 need to discuss whether to define the requirements for CSI-RS based L1-SINR measurement. (CATT) - RAN4 to wait for more RAN1 input. - Option 2: RAN4 focus on CSI-RS based L1-RSRP measurement and FFS on L1-SINR measurement (Apple, Nokia)

According to conclusion in last RAN1 118b meeting,

Conclusion There is no consensus in RAN1 on the support L1-SINR measurement based on CSI-RS for candidate cells.

Proposal 7: RAN4 focus on CSI-RS based L1-RSRP measurement.

- Conclusion

In this contribution, we provide our proposals for RRM requirements of CSI-RS based L1 measurement for LTM:

Proposal 1: A cell is considered as known if the conditions are met:

- The UE has performed L3 measurement on the target cell during the last 5 seconds (no need to have both SSB-based L3 measurement and CSI-RS based L3 measurement; only SSB-based L3 measurement)

Proposal 2: Support both intra-frequency and inter-frequency CSI-RS measurement.

For the definition of intra-frequency, reuse the definition:

      - the SCS of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the SCS of the CSI-RS resource on the serving cell indicated for L1 measurement, and

      - the CP type of the CSI-RS resource of neighbour cell configured for L1 measurement is the same as the CP type of the CSI-RS resource of the serving cell indicated for L1 measurement, and

        - It is applied for SCS = 60KHz

      - the centre frequency of the CSI-RS resource of the neighbour cell configured for L1 measurement is the same as the centre frequency of the CSI-RS resource of the serving cell indicated for L1 measurement

      - The measurementBW are the same.

Proposal 3: No need to keep SSB L1 measurement and reports before CSI-RS L1 measurement, which is:

      - Step 1: SSB based L3 measurements [and reports]

      - Step 3: CSI-RS based L1 measurements on the same candidate cell

      - Note: The CSI-RS in Step 3 should be QCL’ed with the SSB in Step 1.

Proposal 4: RAN4 define CSI-RS based L1 measurement for periodic and semi-persistent.

Proposal 5: RAN4 define the requirements for RTD>CP. Need further clear definition of RTD.

Proposal 6: The existing side condition for CSI-RS based L1-RSRP measurement for serving cell is used as the starting point. Need further progress to check whether any update is needed (if bandwidth of CSI is not 48PRBs, etc. )

Proposal 7: RAN4 focus on CSI-RS based L1-RSRP measurement.

- Reference

[1] R4-2416862, WF on RRM requirements for NR_Mob_Ph4_Part1, Apple

[1] R1-2409283, LS on the support of semi-persistent CSI-RS resource for LTM candidate cells, RAN1