3GPP TSG-RAN WG4 Meeting # 113	R4- 2418162

Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements of on-demand SSB Scell operation for Rel-19 NES

Agenda item:	7.22.2.1

Document for:	Discussion

Introduction

In agreed WID for Rel-19 NES operation [1], following objective was captured for RAN4 RRM core requirements:

- Specify procedures and signaling method(s) to support on-demand SSB SCell operation for UEs in connected mode configured with CA, for both intra-/inter-band CA. [RAN1/2/3/4] - Specify triggering method(s) (select from UE uplink wake-up-signal using an existing signal/channel, cell on/off indication via backhaul, Scell activation/deactivation signaling) - Note1: On-demand SSB transmission can be used by UE for at least SCell time/frequency synchronization, L1/L3 measurements and SCell activation, and is supported for FR1 and FR2 in non-shared spectrum. - Specify the corresponding core requirements, for the above features [RAN4].

In this contribution, we provide our views on RRM core requirements for RRM requirements of on-demand SSB Scell operation for Rel-19 NES.

Discussion

On-demand SSB in SCell (Case 1)

L1 measurement for SCell(Case 1)

In previous RAN4 meeting, L1 measurement for deactivated SCell was discussed, but not captured in WF[2]. The issue in the summary[3] is as follows:

Issue 1-1-4: L1 measurement for deactivated SCell - Proposals - Option 1: Xiaomi, Samsung, China Telecom, Intel - RAN4 shall define both L1 and L3 measurement requirements based on OD-SSB. - Option 2: Qualcomm, OPPO, Nokia - L1 measurements are not supported for deactivated SCell - Option 3: Nokia - Wait further RAN1 clarification on the applicable scenario - Recommended WF - Notes: - From moderator’s understanding, RAN1’s agreement is applied to the L1 measurement after receiving SCell activation command other than before SCell activation. - RAN4 only needs to clarify the L1 related requirement can also applied to OD-SSB in SCell. - Suggest to further discuss the scenarios below. The proponent companies need to clarify which of the following scenarios are supported by your proposal. - L1 measurement before SCell activation: - L1 measurement during SCell activation: - L1 measurement after SCell activation:

It seems that L1 measurement scenario needs to be clarified. In our understanding, L1 related measurement can be happended during SCell activation and after SCell activation. During SCell activation, L1 measurement can be performed based on SSB or semi-persistenet or periodic CSI-RS for CSI reporting. After SCell activation, L1 related measurement includes L1-RSRP, L1-SINR, BFD, CBD purpose. Since we are focusing measurements using SSB, we think L1 measurement during and after SCell activation is valid scenario for OD-SSB.

Regarding L1 measurement in Case 1, after SCell activation, a UE can perform L1 measurement such as L1-RSRP measurement using OD-SSB. In the current specification, the periodicity of the SSB-Index in ssb-periodicityServingCell configured for L1-RSRP measurement is used for the requirements. One simple approach is to replace SSB periodicity to activated OD-SSB periodicity in Case 1 as shown in Table 1.

Table 1 Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 (OD-SSB in Case1)

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = the periodicity of the SSB-Index in activated OD-SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Proposal 1: RAN4 to consider L1 measurement during and after SCell activation as below

Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 (OD-SSB in Case1)

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = the periodicity of the SSB-Index in activated OD-SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Cell identification(Case 1)

In Case1, always-on SSB (AO-SSB) is not expected in SCell. A UE can be indicated multiple OD-SSB activation/deactivation as illustrated in Figure 1. In Figure 1, the UE receives OD-SSB indication1 after activation time, which is deactivated after 4 SSB bursts. After some time after OD-SSB deactivation, the UE receives another OD-SSB indication2. In the legacy SSB measurement, the time between each SSB bursts are always same (e.g. period). However, in OD-SSB operation, it is not guaranteed the time between multiple OD-SSB activation/deactivation is always same (e.g. time between OD-SSB deactivation and OD-SSB indication2), which may lead additional efforts for UE to perform cell identification. For example, if the time is long enough to lose the sync, the UE may need to detect PSS/SSS and/or acquire the index of the SSB being measured. For another example, if the time is short enough to keep the sync, the UE can skip PSS/SSS detection and/or SSB index acquisition.

Observation 1: If the time between multiple OD-SSB activation/deactivation is short enough to keep the sync, PSS/SSS detection and/or SSB index acquisition can be skipped

From the observation, the benefit is clear when the time between multiple OD-SSB activation/deactivation is ‘short’. One way to define whether the time is short or not is to define a threshold in order to decide which requirements to apply. If the time is shorter than the threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition, and perform measurements.

Proposal 2: RAN4 to specify cell identification requirements based on the time between multiple OD-SSB activation/deactivation

- If the time is shorter than a threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition

Figure 1 OD-SSB transmission in Case1

OD-SSB based deactivated SCell measurement(Case 1)

Slow mode

In the previous meeting, RAN4 introduced the time window in which a UE is required to measure OD-SSB. The motivation of the time window is to reduce the UE power consumption, since NW may not guarantee OD-SSB deactivation. According to the agreement, the UE performs ‘fast L3 measurement’ during the time window, otherwise ‘slow L3 measurement’ is expected. The definition of fast and slow L3 measurent, and the configuration of the time window including the relationship with OD-SSB deactivation indication is still open issue. We would like to touch upon the open issue one by one.

Agreements(RAN4#112bis): Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

Regarding the slow mode or slow L3 measurement, Figure 2 shows the example of slow mode in the red circle. UE expects to operate in slow mode after the expiration of the time window before OD-SSB deactivation indication. Three options are captured in WF[2]. The first option is that the UE performs measurement with legacy requirement, which is measCycleSCell or the period of SMTC. The second option is to stop performing measurements, and the last options is no requiremens, which is up to UE implementation. From the NW view, no requirements implies stop performing measurement. In our view, in order to minimize the UE power consumption stop measuring is reasonable. If NW requires measurement report from the UE, it can indicate another OD-SSB activation indication. So our preference is the second or third option.

Figure 2 slow L3 measurement for deactivated SCell

Proposal 3 : For a slow mode for deactivated SCell L3 measurements in Case 1, prefer Stop measurement (Option 2-2) or No requirement (Option 2-3)

The relation between time window and OD-SSB deactivation command in the time domain

In the agreement, RAN4 defined the starting point of the time window as the receving time of OD-SSB activation indication, and the expiration time or time window duration needs to be specified. In the meantime, following OD-SSB deactivation indication candidates are captured in RAN1:

Agreement(RAN1#118bis) For a cell supporting on-demand SSB SCell operation, deactivation of on-demand SSB transmission is supported. In order to deactivate on-demand SSB transmission from a UE perspective, support at least one of the following options. - Option 1: Explicit indication of deactivation for on-demand SSB via MAC-CE for on-demand SSB transmission indication - Option 1A: Explicit indication of deactivation for on-demand SSB via RRC for on-demand SSB transmission indication - Option 2: Configuration/indication of the number N of on-demand SSB bursts to be transmitted after on-demand SSB is indicated - Option 3: Configuration/indication of the duration of on-demand SSB transmission window - Option 4: On-demand SSB transmission, if any, is deactivated when UE receives SCell deactivation MAC-CE for the activated SCell - Option 4A: On-demand SSB transmission, if any, is deactivated when the timer for SCell deactivation is expired - Option 5: On-demand SSB transmission, if any, is deactivated when SCell activation is completed - Option 6: Explicit indication of deactivation for on-demand SSB via [group-common] DCI - FFS: Each option is applicable to which Cases or Scenarios - FFS: Details related to each of the above options

In our understanding, above options can be organized into two categories. The one is explicit deactivation indication, which includes Option 1, 1A, 4, 4A, 5 and 6. It implies OD-SSB is activated until explicit deactivation indication. The other is implicit deactivation indication, which includes Option 2, 3. The deactivation time of OD-SSB is already configured/indicated by NW. We think the expiration time or time window duration relates to OD-SSB deactivation indication, which is illustrated in Figure 3.

In explicit indication of Figure 3, OD-SSB deactivation indication deactivates OD-SSB. We can think two approaches for the expiration time or time window duration. One approach is that the time window always expires at OD-SSB deactivation indication. The other approach is that the time window can be configured/indicated regardless of OD-SSB deactivation indication. From UE power consumption perspective, UE consumes power until OD-SSB deactivation indication in the first apporach. Since NW may not indicate OD-SSB deactivation, it may lead to unnessary UE power consumption. On the other hand, the second approach provides pre-configured time window. Subsequently, the UE can transfer to slow mode after sufficient measurements. From above analysis, we think second approach is more aligned with the motivation, in case of explicit OD-SSB deactivation indication.

Unlike explicit indication, OD-SSB deactivation time is already configured or indicated for implicit indication. We oberserved that the UE can know the exact OD-SSB deactivation time in case of implicit indication, which implies that NW already guarentee the end time of OD-SSB. One thing that is not clear is whether implicit indication is cell-specific or UE-specific. Note that we assume the time window as UE-specific feature. If implicit indication is cell-specific, NW can configure proper time window based on status of UE. On the other hand, if implicit indication is UE-specific, the time window may not be meaningful. Therefore, in our view, the time window depends on cell-specific or UE-specific signaling of implicit indication.

Proposal  4: Consider the expiration time or the duration of the time window related to OD-SSB deactivation indication as follows:

- In case of explicit OD-SSB deactivation indication, the expiration time or the duration of the time window is configured regardless of OD-SSB deactivation indication

- In case of implicit OD-SSB deactivation indication, the expiration time or the duration of the time window depends on whether signaling of implicit indication is cell-specific or UE-specific

Figure 3 OD-SSB deactivation indication and time window

Duration of the time window

If the time window is configured independently, the duration of the time window can be configured/indicated. Since RAN4 fixed the start time of the time window as receiving time of OD-SSB activation indication, we think the duration of time window can be divided into two section as illustrated in Figure 4. The one is processing time section and the other is actual measurement section.

For the processing time section at least MAC-CE processing time should be included in the duration. We think at least Tmin from the RAN1 LS can be applied. In addition, the time of actual transmitted OD-SSB can be included, if unavailable OD-SSB location is occurred. The intention is to guarantee the time window to reach at the location of actual OD-SSB. After the MAC-CE processing time or the time of transmitted actual OD-SSB, actual measurement duration can be started.

For the actual measurement section, we think at least the time window needs to be consisted of OD-SSB period to include OD-SSB. One may think if a UE provided enough measurements report, UE can change to slow mode for reduceing power consumption. We think such execption rule can help the UE save more power.

Figure 4 Duration of the time window

Proposal 5: Support duration of the time window as below:

- Twindow = Tprocess + Tactual_measure

  - Tprocess = Tmin +Tpossible_SSB

    - where, Tprocess is the processing time, Tmin is OD-SSB activation processing time, and Tpossible_SSB is the time of actual transmitted OD-SSB after OD-SSB activation processing

  - Tacutal_measure = X*TOD_SSB

    - where, Tacutal_measure is actually measured time, X is the number of OD-SSB period, and TOD_SSB is the period of OD-SSB

  - The time window can be terminated when the UE provided valid Y reports

    - Y is the number of valid report

Relationship between the time windows

According to the above agreement, RAN4 defined the starting point of the time window at the receiving time of OD-SSB activation indication. In general, the UE measures activated OD-SSB in the time window, if there is single activated OD-SSB. However, if there are multiple activated OD-SSB, the measurement behaviour is not clear.

For example in Figure 5a, Time window1 starts from OD-SSB activation1 indication, and measures OD-SSB1. Meanwhile, Time window 2 starts from OD-SSB activation2 indication, and measures OD-SSB2. Note that OD-SSB1 is not deactivated during Time window2. From UE perspective, it is clear that OD-SSB1 is measured during Time window1, and OD-SSB2 is measured during Time window2. However, if two time windows are overlapped in Figure 5b red circle, the measurement behaviour is not clear. In this case, the UE may need to follow different measurement behaviour due to RF retuning time at the overlapping area in the time domain.

Simple approach to address the issue is that the UE only measures activated OD-SSB indicated by the activation command that initiated the time window during the time window. For example in Figure 5b, the UE terminates Time window1 when the UE receives.

Proposal 6: RAN4 to specify the relationship between the time windows

- In case of overlapping of time windows, the UE measures the OD-SSB indicated by the activation command that initiates the time window during the time window

Figure 5 Multiple OD-SSB activation indication and the time window

Fast L3 measurement requirements

In the moderator summary [3], following discussion regarding fast measurement was captured.

- Option 1: Ericsson, CATT, CMCC, Nokia, Samsung, ZTE, China Telecom, Huawei, Qualcomm - Apply with fast measurement - Option 1-1: OD-SSB periodicity - Option 1-2: SMTC or measurement period in MeasObject for OD-SSB - Option 1-3: fast measurement period is FFS - Option 2: Apple, , NTT - Follow legacy measurement (measCycleSCell)

In our understanding ‘Fast L3 measurement’ refers to measurements using OD-SSB within the time window. Given the fact that only OD-SSB can be recievd in Case 1, it is reasonable to perform measurement with OD-SSB period. It can be applied to measurement period for intra-frequency measurements w/wo gaps for deactivated SCell.

Proposal 7: Apply OD-SSB period for Fast L3 measurements

Starting point of the time window

In the previous meeting, RAN4 introduced the time window starting from receiving OD-SSB activation indication. Based on the agreement, in our understanding, it seems that the starting point of the time window is the slot or the symbol of the PDSCH containing the MAC-CE indicating OD-SSB activation. We have a concern that there can be different understanding between NW and UE regarding the starting point of the time window. From NW perspective, staring of the time window is clear. However, from UE perspective, a UE can acknowledge the OD-SSB activation after decoding the PDSCH and proceeding MAC-CE process in the higher layer. Technically, UE acknowledges OD-SSB activation inciation after Tmin, which implies that the time window starts after Tmin. In order to address the issue, we would like to revise the agreement as follows:

Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

Proposal 8: Revise the agreement regarding the time window as follows:

- Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

On-demand SSB in SCell (Case 2)

Deactivatd SCell L3 measurement(Case2)

Following agreements have been achieved in the perivious meeting for Case 2:

Agreement: - In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement. - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities. - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities. - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies. - Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS. - RAN4 will inform RAN1 if any further agreement. Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

In the previous meeting, RAN4 discussed scenarios of relationship of AO-SSB and OD-SSB. According to the agreement, Scenario1, which assumes that AO-SSB and AO-SSB are with same carrier frequency, and same offset, but different periodicities is supported.

In case of Scenario1, AO-SSB and OD-SSB are transmitted with time/frequency offset. They could be overlapped at least in time or frequency domain. Note that whether spatial filter and the position within SSB burst of AO-SSB and OD-SSB are same is not defined yet. Taken into account different spatial filter, a single SSB can be received with different multi-path from UE perspective. So it is able to combine AO-SSB and OD-SSB. Regarding different position within SSB burst, at least for the same position or SSB index, AO-SSB and OD-SSB can be combined, otherwise it seems not possible.

For the other scenarios, OD-SSB can be configured with different time/frequency offset from AO-SSB. We think different approach can be applied, e.g. defining a gap to take into account RF retuning time. Whether to define requirements for other scenarios can be discussed once reply LS is received.

Proposal 9: Specify OD-SSB based deactivated SCell L3 measurement requirements(Scenario1 - Case 2) as follows:

- Combine AO-SSB and OD-SSB at least for the same position within SSB burst or same SSB index

- Otherwise, apply OD-SSB period

Cell identification(Case 2)

In Case 2, it can be assumed that a UE has already detected PSS/SSS and acquired the index of the SSB being measured using AO-SSB before OD-SSB activation indication. At least for Scenario 1 in Case 2, OD-SSB can be in-sync because of same time/frequency relation with AO-SSB. So, we think cell identification procedure can be skipped at least for Scenario 1 in Case 2.

Proposal 10: Skip cell identification at least for Scenario 1 in Case 2

OD-SSB based SCell activation

Following agreements have been achieved in the perivious meeting:

Issue 1-4-1: General Agreement: - RAN4 focuses on OD-SSB based SCell activation known and unknown requirement for both Case 1 and Case 2 and not specifically differentiate Scenario 2/2A scenarios. Issue 1-4-3: known cell condition design – principle Agreement: - The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command. - When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met. - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB - Other principles are not precluded. Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

Known/unknown cell condition

In the previous meeting, the principle of known cell and unkown cell using legacy principle has been agreed. In our understanding, as speficied in 38.133 - 8.3.2, Scell in FR1 is considered as ‘known’ when the UE has sent a valid measurement report for the SCell being activated, and the SSB measured remains detectable during the period max(5*measCycleSCell, 5*DRX cycles) before the reception of the SCell activation command. For SCell in FR2 is considered as ‘known’ when the UE has sent a valid L3-RSRP measurement report with SSB index, and SCell activation command is received after L3-RSRP reporting and no later than the time when UE receives MAC-CE command for TCI activation during the period equal to 4s or 3s before UE receives the last activation command, which depends on the UE power class. Otherwise, the SCell is considered to be unknown.

One issue to be resolved is whether to use legacy known/unknown cell condition. As defined in the legacy requirement, measCycleSCell is applied in the legacy known/unknown condition. The motivation of indicating OD-SSB activation is to perform measurements frequently. In order to align the motivation, we think the period of OD-SSB can be used instead of measCycleSCell. Furthermore, it is already agreed that OD-SSB is used as baseline during SCell activation. To guaranteed synchronization and AGC, it is reasonable to use the period of OD-SSB for known/unknown condition.

Proposal 11: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*(period of OD-SSB), 5*DRX cycles)

The other issue is that the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication. As illustrated in Figure 6, multiple OD-SSB can be activated at SCell activation command. Scenario in Figure 6 can be valid, since the time window can be initiated only when OD-SSB activation is indicated. NW may activate multiple times to renew the time window. In this case, a UE may confuse which activated OD-SSB should be used for determining known/unknown cell condition. From NW perspective, the latest measurements report is valid for activating SCell. Therefore, the OD-SSB provided the latest valid measurement report can be used for known/unknown cell condition.

Proposal 12: For the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication, the OD-SSB provided the latest valid measurement report can be used for known/unknown cell condition

Figure 6 Known/unknown cell condition with multiple OD-SSB activation indication

Restriction during SCell activation

In general, during SCell activation, the UE measures SSB for synchronization, AGC, and time/frequency tracking. We think OD-SSB should not be changed during the SCell activation to keep the same characteristics. In addition, it is reasonable that the time window is activated during the SCell activation. RAN4 needs to specify the restriction during SCell activation.

One possible way is that UE does not expect OD-SCell activation/deactivation indication during SCell activation, and The UE can assume the time window is always activated during the SCell activation regardless of the configuration of the time window.

Proposal 13: During the SCell activation following should be specified:

- Whether the OD-SSB can be indicated during the SCell activation

- Whether the time window can be expired or not existed during the SCell activation

SCell activation delay requirement

We would like to point out the issue for OD-SSB measurement during and after SCell activation. RAN4 agreed to reuse the legacy known cell condition principle. If we apply the legacy known condition principle, sending ‘valid measurement report or L3-RSRP measurement report with SSB index’ may depend on the OD-SSB indication timeline. For example in Figure 7, a UE received OD-SSB activation indication, and SCell activation command. If the UE have enough time to provide valid measurement report, UE may decide SCell as known. However, as shown in Figure 7, the UE cannot provide valid measurement report due to the lack of enough time. In this case, SCell can be remained unkown, and it may lead wasting measured OD-SSB.

In order to avoid wasting resources, additional measurement and reporting after SCell activation command can be provided in Figure 7. For example the UE measures OD-SSB using OD-SSB before SCell activation command and after SCell activation command, and provides valid measurement reports. The UE can apply shorter delay requirements precluding synchronization, AGC, or time/frequency tracking( the UE can also apply known condition requirements). We are not proposing to modify the known condition period, but having impact on SCell activation delay. In other words, additional measurement can be included in SCell activation delay requirements.

Figure 7 Additional measurement for SCell activation

Proposal 14: For SCell activation delay requirement, propose to add additional measurement delay

LS reply

In previous RAN1 meeting, RAN1 agreed to send LS[4] on timeline for on-demand SSB operation on SCell. In LS, RAN1 asks RAN4 to confirm the timeline between on-demand SCell indication via MAC-CE and on-demand SSB burst(s) is not less than existing timeline required for UE’s MAC-CE processing for SCell activation such as T_min=. In our understanding it is reasonable that the UE receives on-demand SSB burst(s) after transmitting UL transmission with HARQ-ACK and MAC-CE processing timeline. Since MAC-CE processing timeline is already applied in current requirements, reusing the same processing timeline seems reasonable. On the other hands, some companies propose to add additional delay requirement to consider unavailable cases such as occupied searcher or RF retuning time.

In the meantime, RAN1 revised previous agreement as follows:

Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

According to revised agreement, it seems that a gNB shall configure the location of the first “possible” OD-SSB burst and SSB position in the time domain. The “possible” means that it is actually transmitted OD-SSB location, which is already taken into account unavailable cases. The gNB may avoid the location where the UE cannot receive SSB, and inform the location. Therefore, UE can acknowledge the first actually transmitted OD-SSB location.

Observation 2: UE can acknowledge the first actually transmitted OD-SSB location

Since the UE knows actual OD-SSB location, we are not sure additional delay is required. We see that some example cases that companies proposed for additional delay are valid. However, if we are to include all the cases into the requirements, we need to define delay requirements case by case, which is not preferred. In consequence, we prefere RAN4 to confirm the working assumption of T_min.

Proposal 15: RAN4 to confirm working assumption of T_min

Miscellaneous

RRC based signaling

RAN4 agreed to focus on at least for MAC-CE based single SCell activation case as staring point. Meanwhile, in RAN1#118, RAN1 confirms supporting both RRC based signaling and MAC-CE signaling as follows:

Agreement For a cell supporting on-demand SSB SCell operation, - Support RRC based signaling to indicate on-demand SSB transmission on the cell at least for the case where this RRC also configures the SCell, activates the SCell, and provides on-demand SSB configuration. - FFS: Whether to support RRC based signaling for other cases. - Support MAC CE based signaling to indicate on-demand SSB transmission on the cell for Scenarios #2 and #2A. Note: Deactivation and adaptation of on-demand SSB transmission can be separately discussed.

Generally, for RRC based signlaing to indicate on-demand SSB transmission, the NW can provide SCell configuration and on-demand SSB configuration before SCell activation command. When the UE receives SCell activation command, UE can measure OD-SSB after decoding MAC-CE.

In our understanding, the difference between RRC based signlaing and MAC-CE based signaling is applicapable scenario and processing delay for receiving on-demand SSB indication. For applicapable scenario, we think RRC based signaling can only be applied for Scenario #2A. For the processing delay, we think additional RRC processing delay can be expected. Since it was agreed to introduce RRC based signlaing, we think RAN4 needs to specify RRC based signlaing procedure requirement in addition to MAC-CE signaling procedure requirement.

Proposal 16: Specify RRC based signaling procedure in addition to MAC-CE signaling procedure

Conclusion

In this contribution, we provide views on RRM requireiments for on-demand SSB SCell operation, and we propose

Proposal 1: RAN4 to consider L1 measurement during and after SCell activation as below

Measurement period TL1-RSRP_Measurement_Period_SSB for FR1 (OD-SSB in Case1)

| Configuration | TL1-RSRP_Measurement_Period_SSB (ms) |
| --- | --- |
| non-DRX | max(TReport, ceil(M*P)*TSSB) |
| DRX cycle ≤ 320ms | max(TReport, ceil(K *M*P)*max(TDRX,TSSB)) |
| DRX cycle > 320ms | ceil(M*P)*TDRX |
| Note 1: TSSB = the periodicity of the SSB-Index in activated OD-SSB configured for L1-RSRP measurement. TDRX is the DRX cycle length. TReport is configured periodicity for reporting. Note 2: K = 1 when TSSB ≤ 40 ms and highSpeedMeasFlag-r16 or highSpeedMeasCA-Scell-r17 are configured; otherwise K = 1.5. Note 3: When highSpeedMeasFlag-r16 is configured, the requirements apply only to UE supporting either measurementEnhancement-r16 or intraNR-MeasurementEnhancement-r16. or measurementEnhancementCA-r17 |  |

Observation 1: If the time between multiple OD-SSB activation/deactivation is short enough to keep the sync, PSS/SSS detection and/or SSB index acquisition can be skipped

Proposal 2: RAN4 to specify cell identification requirements based on the time between multiple OD-SSB activation/deactivation

- If the time is shorter than a threshold, the UE can skip PSS/SSS detection and/or SSB index acquisition

Proposal 3 : For a slow mode for deactivated SCell L3 measurements in Case 1, prefer Stop measurement (Option 2-2) or No requirement (Option 2-3)

Proposal 4: Consider the expiration time or the duration of the time window related to OD-SSB deactivation indication as follows:

- In case of explicit OD-SSB deactivation indication, the expiration time or the duration of the time window is configured regardless of OD-SSB deactivation indication

- In case of implicit OD-SSB deactivation indication, the expiration time or the duration of the time window depends on whether signaling of implicit indication is cell-specific or UE-specific

Proposal 5: Support duration of the time window as below:

- Twindow = Tprocess + Tactual_measure

  - Tprocess = Tmin +Tpossible_SSB

    - where, Tprocess is the processing time, Tmin is OD-SSB activation processing time, and Tpossible_SSB is the time of actual transmitted OD-SSB after OD-SSB activation processing

  - Tacutal_measure = X*TOD_SSB

    - where, Tacutal_measure is actually measured time, X is the number of OD-SSB period, and TOD_SSB is the period of OD-SSB

  - The time window can be terminated when the UE provided valid Y reports

    - Y is the number of valid report

Proposal 6: RAN4 to specify the relationship between the time windows

- In case of overlapping of time windows, the UE measures the OD-SSB indicated by the activation command that initiates the time window during the time window

Proposal 7: Apply OD-SSB period for Fast L3 measurements

Proposal 8: Revise the agreement regarding the time window as follows:

- Time window means the time duration starting from receiving OD-SSB activation indication→ Time window means the time duration starting from Tmin after receiving OD-SSB activation indication

Proposal 9: Specify OD-SSB based deactivated SCell L3 measurement requirements(Scenario1 - Case 2) as follows:

- Combine AO-SSB and OD-SSB at least for the same position within SSB burst or same SSB index

- Otherwise, apply OD-SSB period

Proposal 10: Skip cell identification at least for Scenario 1 in Case 2

Proposal 11: RAN4 to specify known/unknown cell condition period as below:

- At least for FR1: max(5*(period of OD-SSB), 5*DRX cycles)

Proposal 12: For the OD-SSB used for known/unknown cell condition when there are multiple OD-SSB activation indication, the OD-SSB provided the latest valid measurement report can be used for known/unknown cell condition

Proposal 13: During the SCell activation following should be specified:

- Whether the OD-SSB can be indicated during the SCell activation

- Whether the time window can be expired or not existed during the SCell activation

Proposal 14: For SCell activation delay requirement, propose to add additional measurement delay

Observation 2: UE can acknowledge the first actually transmitted OD-SSB location

Proposal 15: RAN4 to confirm working assumption of T_min

Proposal 16: Specify RRC based signaling procedure in addition to MAC-CE signaling procedure

Reference

[1] RP-242354, “Revised WID: Enhancements of network energy savings for NR,” Ericsson

[2] R4-2416916, “WF on RRM requirements for Netw_Energy_NR_enh_Part1,” Ericsson

[3] R4-2415662, “Topic summary for [112bis][215] Netw_Energy_NR_enh_Part1,” Ericsson

[4] R1-2407565,” LS on timeline for On-demand SSB operation on SCell,” Apple