3GPP TSG-RAN4 Meeting #113	R4-2418165

Orlando, US, Nov 18 – 22, 2024

Agenda item:			5.19.1

Source:	vivo

Title:	Discussion on MSD testing of coverage enhancement

Document for:	Discussion

- Introduction

In RAN#111 meeting, the WF[1] has been agreed, and the remaining issue is as follows:

## 1.1 How to handle MSD for Rel-18 power boosting extension Agreement: - No new MSD test cases are introduced to reflect Rel-18 power boosting. - TS 38.101-1 shall explicitly state that no MSD requirements unique to power boosting operation are defined. How to capture is FFS.

- Discussion

According to the WF, it was agreed that no new MSD test cases need to be introduced to reflect power boosting, and no MSD requirements unique to power boosting operation will be defined. However, how to capture this part in TS 38.101-1 is FFS. In the #112 meeting, a contribution[2] has been proposed to reflect the change to receiving requirements but there is some ambiguity in the wording. According to the comments on the paper, there are some details about MSD which need to be clarified.

In the spec TS38.101-1, it can be seen that the MSD due to UL harmonic from PC3 or PC2 aggressor are specified separately in Table 7.3A.4-1, Table 7.3A.4-2a and Table 7.3A.4-2b. In the above tables there is no further clear TX power indication except the PC level of the aggressor in the header, which is similar in the tables of MSD caused by harmonic mixing and crossband isolation. There is no change about the MSD testing procedure with power boosting supported as the TX power itself is not so limited to a certain value but a dynamic range related to the corresponding power class. And the possibility of re-testing the MSD with TX power exceeding the PC level is unlikely. So for this part maybe we don’t need to consider adding new wording for power boosting feature.

However as for intermodulation interference, some issues still remain to be considered. Check the MSD for PC3 CA in Table 7.3A.5-1, it can be seen that the TX power of the two transmitters shall be set min(+20dBm, PCMAX_L,f,c), as shown in note 1 of fig 1. Similar indication of TX power could also be seen in the other MSD tables due to IMD, such as Table 7.3A.5-1a, Table 7.3A.5-1b, and so on. It seems that in the current stage the quantity min(.,.) will always be filtered to 20 in PC3 CA table and 23 in PC2 CA table. However, there is still potential risk in the future when there is some other change in format PCMAX, and the TX power would be related to ΔPpowerboost and become higher if the transmitter power is equal to the latter one, PCMAX_L,f,c. It is necessary to eliminate the relevant potential risks in the early stage, so some wording still needs to be added in the MSD requirements, at least for IMD part.

Fig 1. Screenshot from Table 7.3A.5-1

Based on [2], the wording of MSD with power boosting supported could be changed as proposal 1 and switched to chapter 7.3A.5.

Proposal 1: No dedicated MSD requirements nor MSD verification is needed when ΔPPowerBoost is a non-zero value, and the change of spec is captured as follows:

##### TS38.101-1: << Start of change >> # 7 Receiver characteristics ## 7.1 General Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity). <<Unchanged sections are omitted>> Unless otherwise stated, the receiver requirements of inter-band UL CA are applicable to UE with one Tx antenna connector in each of the two bands, or UE with one Tx antenna connector in one band and two Tx antenna connectors in the other band. Unless otherwise stated, the receiver requirements of single carrier or CA operation are applicable to UE with one Tx antenna connector or multiple Tx antenna connectors with UL MIMO or Tx diversity operation in the UL band(s). Unless otherwise stated, for single carrier/CA cases there is no dedicated MSD defined nor MSD verification when ΔPPowerBoost is a non-zero value. << End of change >>

##### TS38.101-3: << Start of change >> # 7 Receiver characteristics ## 7.1 General Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with E-UTRA or NR FR1 connecting to the network by OTA without calibration. <<Unchanged sections are omitted>> If a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 , it shall be verified with bothfour Rx antenna ports and two Rx antenna ports requirements. If a UE indicates interBandMRDC-WithOverlapDL-Bands-r16 but does not indicate requirementTypeIndication-r18, it shall be verified with two Rx antenna ports requirements.Unless otherwise stated, the receiver requirements of inter-band EN-DC are applicable to UE with one or two Tx antenna connectors in NR band. Unless otherwise stated, for CA/DC cases there is no dedicated MSD defined nor MSD verification when ΔPPowerBoost is a non-zero value. << End of change >>

- Conclusion

Proposal 1: No dedicated MSD requirements nor MSD verification is needed when ΔPPowerBoost is a non-zero value, and the change of spec is captured as follows:

##### TS38.101-1: << Start of change >> # 7 Receiver characteristics ## 7.1 General Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE. For UE(s) with an integral antenna only, a reference antenna(s) with a gain of 0 dBi is assumed for each antenna port(s). UE with an integral antenna(s) may be taken into account by converting these power levels into field strength requirements, assuming a 0 dBi gain antenna. For UEs with more than one receiver antenna connector, identical interfering signals shall be applied to each receiver antenna port if more than one of these is used (diversity). <<Unchanged sections are omitted>> Unless otherwise stated, the receiver requirements of inter-band UL CA are applicable to UE with one Tx antenna connector in each of the two bands, or UE with one Tx antenna connector in one band and two Tx antenna connectors in the other band. Unless otherwise stated, the receiver requirements of single carrier or CA operation are applicable to UE with one Tx antenna connector or multiple Tx antenna connectors with UL MIMO or Tx diversity operation in the UL band(s). Unless otherwise stated, for single carrier/CA cases there is no dedicated MSD defined nor MSD verification when ΔPPowerBoost is a non-zero value. << End of change >>

##### TS38.101-3: << Start of change >> # 7 Receiver characteristics ## 7.1 General Unless otherwise stated the receiver characteristics are specified at the antenna connector(s) of the UE for the bands operating on frequency range 1 and over the air of the UE for the bands operating on frequency range 2. The requirements for frequency range 1 and frequency range 2 can be verified separately. For the carrier in frequency range 1, requirements can be verified with NR FR2 link disabled. For the carrier in frequency range 2, requirements can be verified in OTA mode with E-UTRA or NR FR1 connecting to the network by OTA without calibration. <<Unchanged sections are omitted>> If a UE indicates both interBandMRDC-WithOverlapDL-Bands-r16 and requirementTypeIndication-r18 , it shall be verified with bothfour Rx antenna ports and two Rx antenna ports requirements. If a UE indicates interBandMRDC-WithOverlapDL-Bands-r16 but does not indicate requirementTypeIndication-r18, it shall be verified with two Rx antenna ports requirements.Unless otherwise stated, the receiver requirements of inter-band EN-DC are applicable to UE with one or two Tx antenna connectors in NR band. Unless otherwise stated, for CA/DC cases there is no dedicated MSD defined nor MSD verification when ΔPPowerBoost is a non-zero value. << End of change >>

- References

- R4-2410611, WF on NR_cov_enh2_R18, Huawei, HiSilicon, 3GPP TSG-RAN WG4 Meeting #111.

- R4-2411156, (NR_cov_enh2-Core) On remaining issues with Rel-18 coverage, Apple, 3GPP TSG-RAN WG4 Meeting #112.