# 3GPP TSG-RAN WG4 Meeting #113	R4-2418192

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Discussion on the subband configurations and guardbands for gNB SBFD

Agenda Item:	7.20.1

Document for:	Approval

Introduction

In the last meeting, we further discussed system parameters for gNB SBFD. The WF for gNB SBFD was captured in [1]. In this contribution, we focus on subband configurations and guard band issues for gNB SBFD.

Discussion

In the last meeting, we discussed operating bands and channel bandwidth support for SBFD operation. One big progress made in the last meeting is that we reached consensus that the minimum channel bandwidth for SBFD is 40MHz by default and 20MHz for band n39. However, there was no agreement on whether NR-U bands can support SBFD. The agreements are captured as follows:

Issue 1-1-1: Minimum channel bandwidth for SBFD (X MHz) Agreement: - Declaration-based method, and SBFD is possible to be used on any TDD band, with the following principle: - The supported channel bandwidth(s) is declared by gNB vendors - The supported channel bandwidth(s) is from existing channel bandwidth defined in TS 38.104 - The supported channel bandwidth shall be no less than X MHz - X = 40 as the default value - For band n39, X = 20MHz is used instead. Issue 1-1-3: Specification implementation for band/channel bandwidth support for SBFD Agreement: - To include the following text in Clause 5.2 in TS 38.104: subband full duplex can be applied to the following TDD bands given in Table 5.2-1, by giving the band numbers which support SBFD. - Detailed band list is provided based on the agreement for minimum channel bandwidth for SBFD - FFS the band for NR-U shall be supported

The minimum channel bandwidth for SBFD shall be no less than 40MHz, with an exception 20MHz is used for band n39. With this restriction, the band list can be decided: n38, n39, n40, n41, n46, n48, n50, n77, n78, n79, n90, n96, n102, n104. However, for NR-U bands n46, n96, n102, it needs further study whether to support SBFD for these bands. NR-U does have some special design compared with NR. For example, one subband for NR-U channel means 20MHz. If one NR-U channel is used for SBFD, the DL/UL subbands should be in the granularity of 20MHz. It may need different subband configurations with NR. One import impact we can think of is LBT mechanism if NR-U supports SBFD. BS needs to perform LBT before it transmits in DL subbands, in the meantime, BS will receive the signal of UL subbands. In this case, it will affect the LBT results. Anyway, these mechanisms may need further enhancement if NR-U supports SBFD operation. For NR-U bands support SBFD, we suggest more study is needed.

Proposal 1：NR operating bands n38, n39, n40, n41, [n46,] n48, n50, n77, n78, n79, n90, [n96, ][n102,] n104 which are defined in Table 5.2-1, can be applied for subband full duplex operation.

Observation 1: For NR-U supporting SBFD, these issues need further study:

- Granularity of one subband: 20MHz

- LBT threshold

- Issue 1-1-4: Transmission bandwidth configuration for SBFD Way Forward: - Illustrative figures from Ericsson will be used as the baseline for further study: - FFS the restriction of SCS for DL and UL subbands - Revisit the DUD figure based on the updated definition of NRB,SBFD,DL - FFS other potential issues.

For transmission bandwidth configuration for SBFD, we are OK to use the same SCS for DL and UL subbands in the illustrative figures. For example, we need to define Transmission bandwidth configuration for SBFD for 15, 30, 60kHz SCS in FR1. However, we do not want restrictions on different SCS cases for UL and DL subbands. In RAN1 design, frequency locations of SBFD subbands are separately configured for each SCS configuration in SCS-SpecificCarrierList. There is no restriction that DL and UL subbands are only configured in the same SCS. Different SCS for UL and DL subband can be configured from our understanding.

Proposal 2: Use the same SCS for DL and UL subbands in the illustrative figures for transmission bandwidth configuration for SBFD, while do not retrict different SCS for DL and UL subband.

If different SCS are configured for UL and DL subbands, then GB does’t have to be in the granularity of RB. We need to mention that in Transmission bandwidth configuration for SBFD.

Proposal 3：GB doesn’t have to be in the number of RB when different SCS are configured for UL and DL subbands.

Issue 1-1-5: DL/UL subband configurations support for SBFD Way Forward: - RAN4 to further study Option 2b by considering the below additional restrictions such as: - For DUD case, the UL subband should be placed in the middle of channel bandwidth - FFS the number of DL subband sizes to be declared and tested - Alt-1: For FR1 WA BS, one DL subband size is declared to support SBFD and tested, operation with other DL subband sizes than the tested one is not allowed. - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested - FFS the necessity of this restriction is applicable to other FR1 BS classes and FR2 BS. - Restriction on UL subband size based on the specific channel bandwidth - FFS to specify a range of ratio for UL subband size over channel bandwidth. - FFS the restriction is different for DU and DUD - FFS the restriction is applied for FR2 case.

We think Option 2b is a good compromise considering flexibility and test complexity. For the DL subband sizes, we prefer BS can declare more than one DL subband sizes. In the real field, only operating in one subband configuration restricts too much. In this case, Alt.2 is preferred by us.

As for the ratio for UL subband size over channel bandwidth, we think 20% can be used as a starting point.

For DUD and DU case, the only different is the location for UL subband, one is in the middle, one is in the left or right hand. Other than that, the same restrictions can apply for both DUD and DU cases.

For FR2, we think it allows more flexibility than FR1 since it is easier to implement SBFD in FR2.

Proposal 4: Option 2b (UL subband shall be selected from BS transmission channel bandwidth) can be used as a starting point for subband configuration.

- For DUD case, the UL subband should be placed in the middle of channel bandwidth

- the number of DL subband sizes to be declared and tested

    - For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested

    - No restriction is applicable to other FR1 BS classes and FR2 BS.

- Restriction on UL subband size based on the specific channel bandwidth

- [20%~] ratio for UL subband size over channel bandwidth

- UL subband position can be different for DU and DUD, other than that there is no different restrcitions.

- These above restrictions doesn’t apply to FR2.

##### Issue 1-2-1: BS RF Specification structure for SBFD requirements Way Forward: - FFS options of how to introduce BS RF new requirements for SBFD-capable BS: - Option-2: Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD - Option-3: A new specification for SBFD BS RF requirements

For BS RF specification structure for SBFD, we prefer to create new sub-clauses in TS 38.104.

SBFD-capable base stations do not only work in SBFD symbols, but also in UL or DL symbols; When operating in SBFD symbols, potentially new BS RF requirements for SBFD-capable BS apply; when working in UL or DL symbols, legacy BS Tx/ Rx requirements apply. The new SBFD requirements and legacy BS RF requirements together constitute the full set of specifications for SBFD-capable BS. In our view, defining them in one specification will not cause a sense of fragmentation, rather than defining a new specification. Furthermore, it is easy to edit in the same protocol for maintenance and so on.

Proposal 5: Option-2 (Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD) is preferred.

Conclusion

This contribution discusses SBFD configurations and guardbands for gNB SBFD. The following proposals are made:

Proposal 1：NR operating bands n38, n39, n40, n41, [n46,] n48, n50, n77, n78, n79, n90, [n96, ][n102,] n104 which are defined in Table 5.2-1, can be applied for subband full duplex operation.

Observation 1: For NR-U supporting SBFD, these issues need further study:

- Granularity of one subband: 20MHz

- LBT threshold

Proposal 2: Use the same SCS for DL and UL subbands in the illustrative figures for transmission bandwidth configuration for SBFD, while do not retrict different SCS for DL and UL subband.

Proposal 3：GB doesn’t have to be in the number of RB when different SCS are configured for UL and DL subbands.

Proposal 4: Option 2b (UL subband shall be selected from BS transmission channel bandwidth) can be used as a starting point for subband configuration.

- For DUD case, the UL subband should be placed in the middle of channel bandwidth

- the number of DL subband sizes to be declared and tested

    - Alt-2: For FR1 WA BS, the DL subband sizes declared to support SBFD shall be tested

    - No restriction is applicable to other FR1 BS classes and FR2 BS.

- Restriction on UL subband size based on the specific channel bandwidth

- [20%~] ratio for UL subband size over channel bandwidth

- UL subband position can be different for DU and DUD, other than that there is no different restrcitions.

- These above restrictions doesn’t apply to FR2.

Proposal 5: Option-2 (Creating new sub-clauses in TS 38.104 for RF requirements that applies for SBFD) is preferred.

- References

[1] R4-2413514, Way Forward for [112][307] NR_duplex_evo_General, Samsung, RAN4#112.