3GPP TSG-RAN WG4 Meeting # 113	R4-2418204

Orlando, US, 18th – 22nd November, 2024

Source: 	vivo

Title: 	Further discussion of UE RF impact for NR MIMO phase 5

Agenda Item:	7.19.2

Document for:	Approval

Introduction

In RAN#102, a new WID [1] was approved on NR MIMO Phase 5. There were already three RAN1 meetings and significant progress have been made, and this is the first meeting for RAN4 to discuss this.

In RAN4#112, an initial analysis and study [2] were made for the WI, and a WF [3] has been agreed. In RAN#105, the NR MIMO phase 5 WID has been revised to [4] to add some new scope.

In RAN4#112bis, the 3Tx issues and mTRP issues were discussed and document summary is in [6], a WF is agreed in [7] and an LS was sent out in [8].

In this paper, further discussion was provided based on last meeting’s WF. Considering that LS reply is not yet available, some of the topics would delayed.

Discussion

3Tx RF Impact

Power classes / UE types

In the agreed WF [7], the following agreements have been made.

<Agreement>: PC for 3Tx requirement

- PC for 3Tx requirement

  - PC3

  - PC2

  - FFS on PC1.5

    - FFS on UE type, i.e., FWA and/or handheld UE

    - FFS on support of full power mode 0

- RAN4 understanding is that TxD for 3Tx is not in the scope of this WI

- Full power mode 0 apply to PC2 and PC3 only.

  - FFS on for which UE types it should apply.

<Way forward>: FFS on New capability for power class per band per declared UL power mode for 3Tx

Now the main discussion point is PC1.5 and related features.

In our understanding, one of the embarrassing situation for PC1.5 with 3Tx is it use one more RF chain compared to PC1.5 2Tx architecture, but can not have more power. In addition, 23dBm RF chain still can not satisfy the need of PC1.5 3Tx, making 3x26dBm RF chain still needed for PC1.5 3Tx.

If no more power can be obtained, less number of RF chain is helpful to control the cost of front end, especially for handheld UEs. Based on these understanding, if PC1.5 is supported, we tend to let PC1.5 only support FWA UE type unless clear needs can be clarified otherwise.

Proposal 1: If PC1.5 is defined for 3Tx, it is proposed to be for FWA only, unless sufficient reason could be provided for handheld not to use 2Tx.

For the full power mode 0, it can not be implemented for PC1.5 in our understanding. However, since this issue is already on the outgoing LS [8] for further clarification, the final decision can be postponed.

Proposal 2: Wait until LS reply for confirmation that Full power mode 0 can not be applied to PC1.5.

There are some proposal for new capability for power class per band per declared UL power mode for 3Tx, but kept FFS since no sufficient discussion is taken place. It is believed that this is not needed. The full power mode signalling itself is a complete solution and no need to have any per power mode power class.

Proposal 3: Do not consider new capability for power class per band per declared UL power mode for 3Tx.

MPR

In the WF, the following WF and options are agreed is as follows for MPR dereviation.

<Way forward>: MPR derivation method for 3Tx

- Companies are encouraged to evaluate MPR based on existing MPR requirements, e.g., PC, 2/4Tx, UE type, considering 3PA configuration agreed above

  - Option 1: Reuse legacy MPR requirements for each power class (OPPO, Huawei, Nokia)

  - Option 2: Further evaluate with 3PA configuration (Apple)

Up till now, no real evaluation of 3Tx has been done yet. The most likely way is try to derive new requirements based on existing ones.

Currently, a lot of comparison were done between 1Tx and 2Tx MPR. In most cases an approximately 1dB R-IMD is expected. This value is unlikely to have large changes for 3Tx, and reusing legacy MPR requirements seems a choice.

Proposal 4: Considering using legacy MPR requirements for each power class with 2Tx as a starting point.

Supported layer for Mode 0

As discussed in [2][9], we repeated our proposal as following:

Proposal 5: Do not include 2-layer verification for ULFPTx mode 0.

Asymmetric DL sTRP/UL mTRP

This issue has been discussed for a few meetings. Since an the reply of outgoing LS [8] is still on the way, we propose to postpone the discussion until a feed back is received.

Proposal 6: Postpone the m-TRP issue discussion until a reply LS is received.

Conclusion

In this paper, more study on UE RF impact of MIMO phase 5 is provided:

3Tx RF Impact

Proposal 1: If PC1.5 is defined for 3Tx, it is proposed to be for FWA only, unless sufficient reason could be provided for handheld not to use 2Tx.

Proposal 2: Wait until LS reply for confirmation that Full power mode 0 can not be applied to PC1.5.

Proposal 3: Do not consider new capability for power class per band per declared UL power mode for 3Tx.

Proposal 4: Considering using legacy MPR requirements for each power class with 2Tx as a starting point.

Proposal 5: Do not include 2-layer verification for ULFPTx mode 0.

Asymmetric DL sTRP/UL mTRP

Proposal 6: Postpone the m-TRP issue discussion until a reply LS is received.

- References

[1] RP-234007, New WID: NR MIMO Phase 5, Samsung (Moderator), RAN#102

[2] R4-2412095, Discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112

[3] R4-2414310, WF on UE RF requirements for Rel-19 MIMO enhancement, Samsung, RAN4#112

[4] RP-242394, WID revision: NR MIMO Phase 5, RAN#105

[5] R4-2413224, On asymmetric connection scenarios for NR MIMO Phase 5, Qualcomm, RAN4#112

[6] R4-2415232, Topic summary for [112bis][128] NR_MIMO_Ph5_UE, Samsung, RAN4#112bis

[7]R4-2417116, WF on UE RF requirements for MIMO enhancement, Samsung, RAN4#112bis

[8] R4-2417117, LS on UE RF issues for Rel-19 MIMO enhancement, Huawei, RAN4#112bis

[9] R4-2415792, Further discussion of UE RF impact for NR MIMO phase 5, vivo, RAN4#112bis