3GPP TSG-RAN WG4 Meeting #113		R4-2418218

Orlando, US, 18th – 22nd November, 2024

Agenda item:	 7.23.2.4

Source:	 vivo

Title:	Further Discussions on LP-WUS Testability

Document for:	 Approval

- Introduction

This contribution shares our views on LP-WUS RF requirements testability issues.

- LP-WUS UE RF testability

It was agreed that RAN4 should further discuss high-level approach on how to achieve LP-WUS performance verification, however, there is no much progress on LP-WUS testability issue last meeting [1].

The fundamental issue is that the LP-WUS RF requirements can not be verified by the same way of MR. RAN4 should have common understanding that this is not the testing-time reduction issue but the basic aspect on how to verify the performance of the receiver which may only work at idle mode. This belongs to core requirements related issues, rather than detailed test procedure design which is RAN5 responsibility.

There are two different views, one way is to verify the RF performance based on a full-process of LP->MR wakeup procedure, this is close to real working scenario of LP-WUR, but drawback is repeatability and testing time of test system. No RF requirement was verified by this approach currently.

Another way around is a simple approach to verify the LP-WUS performance based on UE test mode without wake-up MR each time but just record the LP-WUS outcome and calculate MDR percentage.

In our understanding, to meet the statistics of test results, the repeatability and testing-time are not the only issues, more important, the LR to MR wakeup can be a functionality test rather than the RF performance testing. For REFSENS and other RX requirements, we just need to verify LR output results via test mode, no wake-up action is needed at each OOK-ON signal/sequence during testing period.

Observation 1: LR to MR wakeup can be a functionality test. For RF requirements testing, no need to wake up MR each time.

Observation 2: For REFSENS and other RX requirements, a UE test mode is needed to support the measurements, given there is no close-loop feedback to gNB by LP-WUR.

Therefore, before discussing detailed test procedures, RAN4 should align firstly the general understanding that the MDR should be counted without waking up the MR each time during LP-WUS RF requirements testing.

Proposal 1: RAN4 should align the common understanding that there is no close-loop feedback to gNB by LP-WUR, RAN4 should figure out a general solution to verify the RF performance. The MDR should be counted without waking up the MR each time during LP-WUS RF requirements testing.

Another thing is that LP-WUR UE capability may support idle and/or connection mode. Then, based on UE capability difference, the test conditions might also be different. For example, if UE only supports idle mode, then the connection-mode-based testing would not be applicable. RAN4 should also discuss whether a generic approach can be applied to all LP-WUR capabilities.

Proposal 2: RAN4 should also discuss whether the solution could be applied to all LP-WUR capabilities, i.e., supports both idle and connection mode.

- Conclusion

In this contribution, we share our views on LP-WUR testability issue.

Observation 1: LR to MR wakeup can be a functionality test. For RF requirements testing, no need to wake up MR each time.

Observation 2: For REFSENS and other RX requirements, a UE test mode is needed to support the measurements, given there is no close-loop feedback to gNB by LP-WUR.

Proposal 1: RAN4 should align the common understanding that there is no close-loop feedback to gNB by LP-WUR, RAN4 should figure out a general solution to verify the RF performance. The MDR should be counted without waking up the MR each time during LP-WUS RF requirements testing.

Proposal 2: RAN4 should also discuss whether the solution could be applied to all LP-WUR capabilities, i.e., supports both idle and connection mode.

# References

- R4-2417112, WF on UE RF requirements for LP-WUS, vivo, RAN4#112bis