3GPP TSG-RAN WG4 Meeting #113	R4-2418219

Orlando, US, 18th – 22nd November, 2024

Agenda item:	 7.23.3

Source:	 vivo

Title:	Discussions on LP-WUS BS RF requirements

Document for:	 Approval

- Introduction

In last RAN4 meeting, the BS RF requirements was discussed, and the key issue is how to specify the power boosting requirements [1].

This contribution shares our views on the concept of LP-WUS power boosting.

- LP-WUS power boosting

Regarding power boosting, the agreements in the WF [1].

#### Issue 1-2: Concept of LP-WUS power boosting

- Proposals

  - Proposal 1: Since EPRE ratio has the merit that power boosting declaration for LP-WUS does not vary with different CBWs, RAN4 to introduce and specify power boosting represented by EPRE ratio between LP-WUS and NR REs excluding LP-WUS. (CATT, Huawei, [vivo])

    - The LP-WUS power boosting is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration and the average power NR REs (the NR carrier excluding the LP-WUS REs).

  - Proposal 2: Adopt following definition rather than introduce EPRE ratio. (Ericsson, ZTE, Nokia)

    - The LP-WUS RB power dynamic range (or LP-WUS power boosting) is the difference between the average power of LP-WUS REs (which occupy certain REs within a NR transmission bandwidth configuration) and the average power over all REs (from both LP-WUS and the NR carrier containing the LP-WUS REs).

WF

  - Compare the Pros and Cons of the two definitions

  - At least the following aspects should be considered

    - Convenience for manufacturer’s declaration

    - Compatibility with other issues relevant to power boosting, e.g. the possible upper bound, applicable lowest CBW, etc.

    - Cases of power boosting with different CBW and LP-WUS / NR RE allocation should be provided

    - Comparison of power boosting with NB-IoT with the same definition should be provided

    - Test aspects of power boosting should be considered

    - Other aspects are not precluded

  - Try to define the power boosting concept next RAN4 meeting with one definition as baseline but allow flexibility to manufacturer declaration of boosting level for the supported CBW

In last RAN4 meeting, companies mixed two aspects during the LP-WUS power boosting discussion:

- One aspect is the power level difference between LP-WUS and NR, which is considered as the power boosting level.

- Another aspect is the RE power control dynamic range among all LP-WUS REs, this is similar to the traditional NR RE power control dynamic range, which is related to the signal PAPR and EVM performance. However, this is the characteristic of LP-WUS itself, and somehow partially belongs to the transmitted signal quality (e.g., EVM performance).

Therefore, we believe the discussion in RAN4 on LP-WUS power boosting requirement should be focused on the first aspect, i.e., average RB power level difference between LP-WUS and NR. The power boosting concept and RE dynamic range concept for LP-WUS should be separated discussion.

Proposal 1: RAN4 should use LP-WUS power boosting concept rather than “dynamic range” as definition for this requirement. The power boosting concept and RE dynamic range concept for LP-WUS be separated.

Regarding LP-WUS RE dynamic range, it can be considered further as part of Transmitted signal quality (similar to NR EVM requirements) to ensure the quality of LP-WUS signal.

- Conclusion

In this contribution, we share our views on LP-WUR BS RF:

Proposal 1: RAN4 should use LP-WUS power boosting concept rather than “dynamic range” as definition for this requirement. The power boosting concept and RE dynamic range concept for LP-WUS be separated.

# References

- R4-2416589, Way Forward for [112bis][308] NR_LPWUS, Huawei, RAN4#112bis