3GPP TSG-RAN WG4 Meeting #113	R4-2418252

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	Discussion on NR NTN system parameters for Ku band

Agenda item:	7.9.3

Document for:	Approval

Introduction

WI on the introduction of Ku Band for NR NTN has been approved in [1]. RAN4 has started the work since RAN4#112 meeting [3][4] and has continued the work in RAN4#112bis [6]. For frequency range definition and numerology for Ku band(s), in the past two meetings the following WF have been approved in [5][6].

Issue 3-1: Frequency range definition Following the scope of the WID, definition of a new frequency range e.g. FR3, is not in scope. Issue 3-2: Numerology - Decision criteria The following technical criteria will be used to determine the feasibility of FR1 and FR2 numerology - UL timing synchronization - Phase noise - Efficient channel utilization - Beam hopping granularity - Link budget UL/DL – as a function of SCS and channel bandwidth Issue 3-3: Numerology – timing aspects The impact of SCS on timing is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues. Issue 3-4: Numerology – phase noise The impact of SCS on susceptibility to phase noise is critical to reliable performance therefore decisions on FR1 and FR2 numerology feasibility should not be made without a thorough analysis of the issues. Issue 3-5: Numerology – efficient channel utilization The ability to efficiently support in future the required aggregate bandwidths [list] of NTN services for different SCS choices based on existing FR1 and FR2 channel bandwidths needs to be fully studied. Issue 3-6: Numerology– beam hopping granularity An analysis of the impact of different SCS on performance, although not critical, can influence the decision on FR1 vs FR2 numerology.

Sub-topic 3-4: Phase noise target SNR Further analysis on the impact of phase noise on FR1/FR2 numerology selection should assume a maximum of 12 dB SNR and maximum 16QAM modulation. This is just for analysis and is not intended to limit support of higher MCS. Sub-topic 3-5: Efficient use of spectrum for FR1 – native bandwidths Assume 10 MHz and 25 MHz native bandwidths are included in the analysis of spectrum use efficiency.

In this contribution, we discuss the numerology for Ku band according to the WF above. Furthermore, we discuss other remaining aspects of system parameters for Ku band.

Discussion

Frequency range

In TS 38.101-5, FR1-NTN frequency range is defined for 410-7125 MHz and FR2-NTN frequency range is defined for 17300-30000 MHz. Furthermore, in TS 38.101-1/2, FR1 is defined for 410-7125 MHz, FR2-1 for 24250 – 52600 MHz and FR2-2 for 52600 – 71000 MHz. There is no existing frequency range for Ku band in both the NTN and TN specifications.

Agreed in Issue 3-1, defining a new frequency range is out of scope of the WI. Thus, we reuse either FR1-NTN or FR2-NTN for Ku band by extending one of the existing NTN frequency ranges.

Analyzing the choice of numerology for the Ku-band

Priority 1 in the WI objective is to support 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. As mentioned in the WI objective, for FR1-NTN, 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing can be introduced, though it is not restricted to. For FR2-NTN, 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing can be introduced.

In other words, this WI intends to analyze whether FR1-NTN or FR2-NTN numerology should be chosen as applicable over Ku bands. There is currently an ongoing effort in this WI to identify relevant aspects to be considered for the evaluation of the feasibility of both numerologies. In the WF proposed in [5], a few technical items were identified as potential decision criteria:

- UL timing synchronization

- Phase noise

- Efficient channel utilization

- Beam hopping granularity

- Link budget UL/DL – as a function of SCS and channel bandwidth

In this contribution we’ll provide our view regarding the afore mentioned criteria.

UL Timing Synchronization

The UL timing synchronization mechanism chosen for the NTN feature differs from legacy timing synchronization in TN, but it is the same for FR1-NTN and FR2-NTN. It relies on timing advance pre-compensation performed by the UE based on its own GNSS position and satellite assistance information. In order to implement this new synchronization mechanism, it was needed to account for the imperfections and potential inaccuracy of the GNSS system or the ephemeris information, which caused the transmit timing error requirement to be relaxed for NTN when compared to the TN counterpart.

The impact of this relaxation is perceived differently in FR1-NTN and FR2-NTN numerologies, and this happens because the cyclic prefix used for the different subcarrier spacings (SCS) is inversely proportional to the SCS. As evidenced below:

Table 1. Comparison of UL Timing Requirements for FR1-NTN (30 kHz) and FR2-NTN (120 kHz)

|  | DL SSB SCS (kHz) | UL SCS (kHz) | Te_NTN (Ts) | CP length (Ts) | Te_NTN / CP length | Tp_NTN (Ts) | Tq_NTN (Ts) |
| --- | --- | --- | --- | --- | --- | --- | --- |
| FR1-NTN | 30 | 30 | 22 | 72 | 30.6% | 5.5 | 5.5 |
| FR2-NTN (fixed VSat) | 240 | 120 | 7.5 | 18 | 41.6% | 2.5 | 2.5 |
| FR2-NTN (mobile VSAT) | 240 | 120 | [10] | 18 | 55.5% | 2.5 | 2.5 |

In Table 1, it is possible to observe that the transmit timing accuracy in NTN (Te_NTN) is larger than half of CP when a mobile VSAT (which is in the scope of this WI) is considered under current provisory requirements (between square brackets). This is highly problematic, because Te_NTN is the maximum absolute error regardless of the direction of the inaccuracy, i.e., the UE transmission might be off the ideal transmission timing by any value in the interval [-Te_NTN, + Te_NTN]. In other words, it is not possible for the network to accommodate the receiving window and guarantee that the UE transmission will arrive within the CP: ISI cannot be fully avoided.  The current requirements adopted for 120 kHz SCS poses a risk on the feasibility of such SCS.

On the other hand, the Table also shows that FR1-NTN is more robust to UL Timing Synchronization inaccuracy than FR2-NTN.

For UL timing Synchronization, FR1-NTN is more robust to UE transmit timing inaccuracies, whereas ISI cannot be fully avoided in UL for FR2-NTN using 120 kHz SCS.

Moreover, an additional problem refers to the autonomous adjustment rate that can be performed by NTN UEs. The value of Tq_NTN imposes an upper bound on how much the UE is allowed to adjust its timing advance autonomously within 200 ms. For FR1-NTN, the value of 5.5 Ts allows for a total aggregate adjustment rate corresponding to a UE relative speed of 966 km/h, which is enough to cover for most airplane cases. However, the value of 2.5 Ts chosen for FR2-NTN would allow a maximum aggregate adjustment rate corresponding to a speed of 439 km/h, which is not enough to provide autonomous adjustment in flight conditions.

The maximum aggregate rate of pre-compensation enabled in FR2-NTN does not enable inflight connectivity, one of the exemplary use cases in this WID.

Based on the analysis above, we have the following proposal:

For the decision criteria related to UL timing synchronization, capture that FR1-NTN is more suitable whereas FR2-NTN cannot avoid ISI.

Phase Noise

It is well known that the wider SCS are more robust to phase noise than narrower SCS, which would tend to favour FR2-NTN numerologies, which adopts 120 kHz of SCS, compared to the 30 kHz offered by FR1-NTN. However, previous investigations that can be found in [8][9] and [10] have demonstrated that there is no significant difference between the two numerologies at the frequency ranges adopted in Ku band, although further evaluation might be required for the specific case of NTN deployments (which would require assistance of satellite companies).

Therefore, based on the previous analysis we propose the following:

For the decision criteria related to Phase Noise, capture that no significant differences were identified in the performance at FR1-NTN and FR2-NTN.

Efficient Channel Utilization

In terms of efficiency in the channel utilization, 3GPP specifications are quite flexible and enable different configurations that allow the system to adapt to different environments and conditions, normally no significant difference would be seen between the two numerologies.

However, this criterion was added to investigate which numerology would provide a better fit to the available transponders implemented in the satellite market. The efficient usage of 250 MHz (Downlink) and 125 MHz (Uplink) transponders is critically important, whereas, other legacy transponders (such as 36 MHz) are also considered important for some satellite operators according to [11].

As CA is not currently implemented in NTN (neither for FR1-NTN nor FR2-NTN), the following analysis will be performed only considering single carrier implementation.

Table 2. Analysis of channel efficiency

| Transponder bandwidth | FR1-NTN support | FR2-NTN Support |
| --- | --- | --- |
| 125 MHz | 100 MHz carrier + 20 MHz carrier (5 MHz not utilized) | 100 MHz carrier (25 MHz not utilized) |
| 250 MHz | 2 x 100 MHz carrier + 50 MHz carrier | 200 MHz carrier + 50 MHz carrier |
| 36 MHz | 35 MHz carrier | Not supported |

The first two rows in Table 2 are representative of the two BWs agreed in the WID (125 MHz UL and 250 MHz DL) as fundamental. Whereas, the third row is indicated for information, as it was raised as an important use case in [11].

Both numerologies can support 250 MHz efficiently, but for the UL BW (125 MHz), more unused spectrum is observed at FR2-NTN.

Therefore we propose to capture the following:

For the efficient channel usage criterion, capture that both numerologies can support 250 MHz efficiently. For 125 MHz (aggregated UL BW), FR1-NTN has better efficiency which would lead only to 5 MHz of “unused” spectrum compared to 25 MHz of unused spectrum in FR2-NTN.

Beam Hopping Granularity

Beam hopping granularity has been established as a technical aspect to be considered to influence the decision on the numerology to be adopted in the Ku band. Although such analysis was not considered critical, it could still have an impact over the decision.

This analysis is to be performed under the assumption that no extended periodicity of SSB (i.e. no enhancements on the beam hopping implementation) discussed in Rel-19 should be considered, only Rel-18 implementations.

In Rel-18, there is no cell DTX available, and one shall operate under the assumption that the cell is always active and transmitting a SSB every 20 ms. In this sense, the “beam hopping” possibility allowed depends on the number of logical beams supported per cell in the specification.

FR2-NTN would allow up to 64 beams per specification design, whereas FR1-NTN would reach a maximum of 8 beams.

On the other hand, FR1-NTN would allow more flexibility for beam hopping on the carrier domain, as it allows to split the available spectrum in smaller chunks (carriers) of bandwidth. Each carrier would be covered by a different logical cell, each with 8 beams on its own. This can mitigate the disparity between FR1-NTN and FR2-NTN.

Therefore we propose:

For the beam hopping granularity, capture that FR2-NTN would provide more flexibility with 64 available beams against 8 in FR1-NTN.

Link Budget

Considering that most of the factors that influence the link budget (coupling losses, MCS, maximum transmit power, etc.) will not be altered by the choice of numerology, not much differentiation is expected between the link budget using FR1-NTN and FR2-NTN (the link budget is not altered by the SCS). However, some specificities of each numerology might be impactful for the link budget.

For instance, FR2-NTN has the advantage that, in a single carrier scenario, the larger bandwidth provided in a single carrier would theoretically enable an increase in the “per UE throughput”. However, this is not a realistic assumption, as NTN link budget is severely impacted by higher coupling losses due to very large distances of propagation, operating most of the time in a “power limited” region (not MCS limited). In UL is unlikely that any UE would benefit from  a full-bandwidth allocation, as the number of actual PRBs to be allocated per UE is rather small. Moreover, the transmit timing error mentioned above will also impact the UL link budget, as the UEs will incur in poorer reception and more ISI due to time inaccuracies, therefore, it is unlikely that this will produce a significant throughput gain.

On the other hand, if FR1-NTN numerology is adopted, the throughput conditions under very extreme propagation conditions will be improved, as it allows for smaller and more robust bandwidth allocations. This is a real advantage, especially considering the likelihood of NTN scenarios working under extreme conditions, but may not be significative enough to decide the analysis of the link budget aspect in favor of FR1-NTN. Therefore, we propose:

For the link budget analysis, capture that no numerology has a significant advantage over the other.

Conclusion

The following Observations and Proposals were made:

Observation 1: For UL timing Synchronization, FR1-NTN is more robust to UE transmit timing inaccuracies, whereas ISI cannot be fully avoided in UL for FR2-NTN using 120 kHz SCS.

Observation 2: The maximum aggregate rate of pre-compensation enabled in FR2-NTN does not enable inflight connectivity, one of the exemplary use cases in this WID.

Proposal 1: For the decision criteria related to UL timing synchronization, capture that FR1-NTN is more suitable whereas FR2-NTN cannot avoid ISI.

Proposal 2: For the decision criteria related to Phase Noise, capture that no significant differences were identified in the performance at FR1-NTN and FR2-NTN.

Observation 3: Both numerologies can support 250 MHz efficiently, but for the UL BW (125 MHz), more unused spectrum is observed at FR2-NTN.

Proposal 3: For the efficient channel usage criterion, capture that both numerologies can support 250 MHz efficiently. For 125 MHz (aggregated UL BW), FR1-NTN has better efficiency which would lead only to 5 MHz of “unused” spectrum compared to 25 MHz of unused spectrum in FR2-NTN.

Proposal 4: For the beam hopping granularity, capture that FR2-NTN would provide more flexibility with 64 available beams against 8 in FR1-NTN.

Proposal 5: For the link budget analysis, capture that no numerology has a significant advantage over the other.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

- R4-2413414 Topic summary for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2413520 Ad-hoc meeting minutes for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2413519 Way Forward for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2416593 Way Forward for [112bis][313] NR_NTN_Ku_Band_General, RAN4 #112 bis

- TS 38.133 “NR; Requirements for support of radio resource management”, 3GPP, Release 18.7.0

- R4-1911831, “Discussion on parameterized phase noise model for example frequencies in 7-24 GHz”, Ericsson, RAN4 #92bis, Chongqing, China

- R4-2415823, “Discussion on system parameters for NR NTN Ku band “, ZTE Corporation, Sanechips, RAN4 #112bis,  Hefei, China

- R4-2416141, “NTN Ku-band – FR1-TN vs FR2-TN”, Ericsson, RAN4 #112bis,  Hefei, China

- R4-2415284, “Feasibility analysis of FR1 and FR2 Numerology”, Eutelsat Group,  RAN4 #112bis,  Hefei, China

- R4-2313986, “Collected calibration results for NTN co-existence study”, Samsung, RAN WG4 Meeting # 108	, Toulouse, France.

- R4-2411950 Discussion on NR NTN system parameters for Ku band, Nokia, RAN4#112