3GPP TSG-RAN WG4 Meeting #113	R4-2418253

Orlando, USA, 18th – 22nd November 2024

Source:	Nokia

Title:	Discussion on NR NTN UE RF requirement for Ku band

Agenda item:	7.9.4

Document for:	Approval

Introduction

WI on the introduction of Ku Band for NR NTN has been approved in [1]. RAN4 has discussed UE and SAN RF aspects since RAN4#112 meeting [3][4][6] and the WFs [5][7] has been so far agreed for NTN Ku band UE RF. In this contribution, we discuss some issues on UE RF requirement.

Discussion

NTN VSAT UE types are defined in TS 38.101-5 Table 9.2.1.0-1 for FR2-NTN and are currently applicable to Ka bands n512, n511 and n510. OTA-based UE RF requirements have been introduced to these VSAT UE types.

| UE class | UE type | Type description |
| --- | --- | --- |
| Fixed VSAT | 1 | Fixed VSAT communicating with GSO and LEO with mechanical steering antenna. |
|  | 2 | Fixed VSAT communicating with GSO and LEO with electronic steering antenna. |
|  | 3 | Fixed VSAT communicating with LEO only with electronic steering antenna. |
| Mobile VSAT | 4 | Mobile VSAT communicating with GSO with mechanical steering antenna. |
|  | 5 | Mobile VSAT communicating with GSO with electronic steering antenna. |
| Note 1: The UE types are assuming UE has only one antenna beam towards one satellite at a given time in this release. Note 2: The Mobile VSAT communicating with non-GSO is not considered in this release. |  |  |

The scope of the WI is to specify the requirement for NTN VSAT types in 38.101-5. All UE types 1-5 can be introduced to Ku band.

All UE types 1-5 in TS 38.101-5 can be introduced to Ku band.

A few satellite operators have indicated that Mobile VSAT with NGSO is also necessary and the WID is considered updated in December [4]. As the coexistence requirement may not differ much from Fixed VSAT to Mobile VSAT, this may be incorporated easily in the coexistence analysis if the WID scope is updated with a new VSAT type.

A new mobile VSAT type with NGSO may be later incorporated if the WID scope is updated.

The existing UE capability signaling, ntn-VSAT-AntennaType-r18, ntn-VSAT-MobilityType-r18 and ntn-ScenarioSupport-r17, can be reused for such a new UE type.

The existing capability signalling can be reused for a new mobile VSAT for NGSO.

During the online discussion RAN4#112, one satellite operator indicated both OTA and conducted requirements were of interest for Ku band. The conducted requirement in TS 38.101-5 has been introduced to L/S-bands, however, high directional antenna has not been considered in the earlier coexistence analysis as well as in RF requirement derivation [2]. In RAN4#112-bis, it was agreed at least OTA requirement was considered in the following.

#### Issue 1-1-1: The Ku-band UE RF requirements structures - At least the OTA requirement is considered for the Ku band, FFS on whether the conductive requirement is considered to allow efficient testing.

Only the isotropic UE antenna is assumed in conducted requirement for FR1-NTN bands and thus only the power class is defined instead of UE types for various VSAT implementations with high gain antennas. Therefore, the current conducted requirement specified for L/S-bands may not be simply reused for Ku band.

The existing conducted requirement for L/S-bands may not be simply reused for Ku bands.

In RAN4#112-bis, there was discussion that Ku band cannot have the specific maximum TRP and EIRP to support various VSAT implementations, for example, for different satellite altitudes in LEO. Therefore, the following issue is left for further study.

#### Issue 1-2-1: Maximum TRP and EIRP for the NTN Ku band - FFS on the following options - Option 1: No maximum TRP and EIRP defined - Option 2: Maximum TRP and EIRP need to be defined, same approach as the NTN Ka band in the current specification - Option 3: Define maximum TRP and EIRP based on declaration by manufacturer

Maximum TRP and EIRP as well as Minimum peak EIRP are needed to keep the configured transmitted power well defined in TS 38.101-5 clause 9.2.3. Thus, it cannot be simply left undefined. The possible solution is to introduce multiple UE types for different VSAT implementations with different TRP and EIRP values. It is expected not only Tx requirement but also Rx requirement (such as REFSENS, etc) depend on the UE types. Another option is leave them to manufacture declaration. However, this means there is no minimum standard requirement for most Tx and Rx requirement.

It is further discussed if we introduce multiple UE types with different TRP and EIRP levels or introduce manufacture declarations.

Conclusion

The following Observations and Proposals were made:

Observation 1: All UE types 1-5 in TS 38.101-5 can be introduced to Ku band.

Observation 2: A new mobile VSAT type with NGSO may be later incorporated if the WID scope is updated.

Observation 3: The existing capability signalling can be reused for a new mobile VSAT for NGSO.

Observation 4: The existing conducted requirement for L/S-bands may not be simply reused for Ku bands.

Proposal 1: It is further discussed if we introduce multiple UE types with different TRP and EIRP levels or introduce manufacture declarations.

- References

- RP-241690, New WID on Introduction of Ku Band for NR NTN, RAN#104, Shanghai, China, 17 – 20 June 2024.

- TR 38.863, Solutions for NR to support non-terrestrial networks (NTN): Non-terrestrial networks (NTN) related RF and co-existence aspects.

- R4-2413415 Topic summary for [112][315] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112

- DRAFT Meeting Report for TSG RAN WG4 meeting: 112

- R4-2413519 Way Forward for [112][314] NR_NTN_Ku_Band_General, RAN4#112

- R4-2416535	Topic summary for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112bis

- R4-2416563 Way Forward for [112bis][314] NR_NTN_Ku_Band_UE_SAN_RF, RAN4#112bis

- R4-2416568	Ad-hoc meeting minutes for [112bis][313] NR_NTN_Ku_Band_General, RAN4#112bis

- R4-2416593	Way Forward for [112bis][313] NR_NTN_Ku_Band_General, RAN4#112bis