3GPP TSG-RAN WG4 Meeting #113	R4-2418256

Orlando, USA, 18th – 22nd November 2024

Source:	Nokia

Title:	Discussion on less than 5 MHz NTN System/UE aspects

Agenda item:	7.8.3.1

Document for:	Approval

Introduction

In this paper, we provide our views on UE aspects for the support of less than 5 MHz channel bandwidth for NR NTN. Core parts of the objectives mentioned in the WID are found below [1].

- Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands Specify the necessary signaling to support the above objectives

In this contribution, we discuss the remaining open issues in the Wayforward on System parameters and UE RF aspects [5].

Discussion

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional?

- Way forward:

  - Option 1: Mandatory

  - Option 2: Optional

Issue 1-2: Clarification on maximum transmission bandwidth configuration

- Agreement:

  - Follow the previous agreement in WF R4-2413528 as:

Issue 1-1-3: Maximum transmission bandwidth configuration(NRB) - Agreement - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.

Issue 1-3: Signalling of SS raster

- Way forward:

  - Option 1: The exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations.

  - Option 2: Defer the UE capability discussion for 3MHz in FR1-NTN after all the requirements are clear.

  - Option 3: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

SSB design for 3 MHz channel bandwidth is different from other existing channel bandwidths. It cannot be supported by the legacy UE design. Therefore, 3 MHz channel bandwidth is an optional feature in UE in the TN specifications. There is dedicated UE capability signalling, e.g., support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 in the TN specifications, which is not mandatory present.

We can consider reusing the same signalling for NTN as Issue 1-3 option 2. Alternatively, if a dedicated capability signalling for NTN is needed, we can define a similar capability signalling specific to NTN.

Depending on operator demands on whether the signalling is mandatory present and whether the support of the feature is mandatory with signalling in Rel-19. Such signalling can be optional as an early implementable feature from an earlier release.

3 MHz channel bandwidth is optional feature in TN and dedicated UE capability signalling is defined.

The same UE capability signalling as TN can be reused for NTN.

The support of new feature can be made mandatory in the open release (i.e. Rel-19) but can be made optional from an earlier release if it is specified as an early implementable feature.

Currently 3 MHz channel bandwidth is not applied to RedCap/eRedcap in TN specifications. 3 MHz cannot be simply included in RAN4 specifications since impacts to RAN1 specifications are expected to include Redcap/eRedcap. As RAN1 is not involved in this WI, we cannot apply 3 MHz to RedCap/eRecap.

Including 3 MHz channel bandwidth to Redcap/eRecap NTN is not aligned with the scope of the WID since RAN1 impacts are expected.

Issue 2-1-9: General spurious emission

- Way forward

  - FFS

NTN UE transmitter architecture is in principle the same as TN UE, therefore, MPR, Minimum output power, Transmit OFF power, Occupied bandwidth, Spectrum emission mask, ACLR, general spurious emission, Configured transmitted power, Transmit ON/OFF time mask, Power control, Frequency error, Transmitter modulation quality, UTRA ACLR and Transit intermodulation for 3 MHz channel bandwidth shall follow the requirement in TS.38.101-1.

For general spurious emissions, the OOB boundary shall be 6 MHz for 3 MHz channel bandwidth in the following tables.

Table 6.5.3.1-1: Boundary between NR out of band and general spurious emission domain

| Channel bandwidth | OOB boundary FOOB (MHz) |
| --- | --- |
| 3 | 6 |
| 5, 10, 15, 20, 25, 30 | BWChannel + 5 |

Table 6.5.3.1-2: Requirement for general spurious emissions limits

| Frequency Range | Maximum Level | Measurement bandwidth | NOTE |
| --- | --- | --- | --- |
| 9 kHz ≤ f < 150 kHz | -36 dBm | 1 kHz |  |
| 150 kHz ≤ f < 30 MHz | -36 dBm | 10 kHz |  |
| 30 MHz ≤ f < 1000 MHz | -36 dBm | 100 kHz |  |
| 1 GHz ≤ f < 5th harmonic of the upper frequency edge of the UL operating band in GHz | -30 dBm | 1 MHz |  |

For general spurious emissions OOB boundary is 6 MHz for 3 MHz channel bandwidth.

Issue 2-2-1: Reference sensitivity

- Way forward

  - Option 1: 2.2dB improvement compared with 5 MHz CBW;

  - Option 2: 1.7dB improvement compared with 5 MHz CBW;

In TS 38.101-1, as far as UL/DL separation is enough (without significant desense), the Rx sensitivity is 2.2 dB better than 5 MHz. As the NTN spectrum utilization for 3 MHz is the same as TN, the same scaling can be applied to NTN bands n256, n255 and n254.

REFSENS for 3 MHz can be scaled from 5 MHz by 2.2 dB.

Issue 2-2-2: Maximum input level

- Way forward

  - FFS on the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm)

In the current TS 38.101-1 and 38.101-5, the maximum input level for 20 MHz channel bandwidth or less is common (including 3 MHz for TN). We propose to keep the maximum input level for NTN UE 3 MHz is the same as for 5 – 20 MHz.

The maximum input level for 3 MHz is the same as 5, 10, 15 and 20 MHz, i.e., -40 dBm.

Issue 2-2-3: ACS

- Way forward

  - Option 1: Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE.

  - Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

NTN UE receiver architecture is in principle the same as TN UE and therefore ACS, In-band blocking, Out-of-band blocking and Narrow band blocking for 3 MHz channel bandwidth can be reused from TN requirement in TS 38.101-1. For ACS, option 2 should be considered, i.e.,

Table 7.5-1: ACS for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20 |
| ACS | dB | 33 | 30 | 27 |

Table 7.5-2: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm | REFSENS + 45.5 dB |  | REFSENS + 42.5 dB | REFSENS + 39.5 |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset)2 | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | -71.5 |  | -68.5 | -65.5 |
| Pinterferer | dBm | -40 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

For ACS, 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

Conclusion

The following Observations and Proposals were made:

Observation 1: 3 MHz channel bandwidth is optional feature in TN and dedicated UE capability signalling is defined.

Proposal 1: The same UE capability signalling as TN can be reused for NTN.

Observation 2: The support of new feature can be made mandatory in the open release (i.e. Rel-19) but can be made optional from an earlier release if it is specified as an early implementable feature.

Observation 3: Including 3 MHz channel bandwidth to Redcap/eRecap NTN is not aligned with the scope of the WID since RAN1 impacts are expected.

Proposal 2: For general spurious emissions OOB boundary is 6 MHz for 3 MHz channel bandwidth.

Proposal 3: REFSENS for 3 MHz can be scaled from 5 MHz by 2.2 dB.

Proposal 4: The maximum input level for 3 MHz is the same as 5, 10, 15 and 20 MHz, i.e., -40 dBm.

Proposal 5: For ACS, 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

- References

- RP-240857 New WID: Enhanced requirements and test methodology for NR and IoT NTN, RAN#103

- R4-2413528 WF on for [112][312] NR_IoT_NTN_less_than_5MHz_UERF, Xiaomi, RAN4#112

- ETSI EN 301 441 Satellite Earth Stations and Systems (SES); Harmonised Standard for Mobile Earth Stations (MES), including handheld earth stations, for Satellite Personal Communications Networks (S-PCN) operating in the 1,6 GHz/2,4 GHz frequency band under the Mobile Satellite Service (MSS) covering the essential requirements of article 3.2 of the Directive 2014/53/EU

- Radio Regulations, Edition of 2024, ITU-R

- R4-2416587 Way Forward for [112bis][311] NR_IoT_NTN_less_than_5MHz_UERF, RAN4#112bis