3GPP TSG-RAN WG4 Meeting #113	R4-2418278

Orlando, US, 18th – 22nd November, 2024

Agenda item:			7.18.4

Source:	Moderator (Nokia)

Title:	Topic summary for [213][220] FS_NR_AIML_Mob

Document for:	Information

# Introduction

This summary thread addresses the issues related to AI/ML mobility SI in RAN4. The document is split into 3 high-level topics which are further split into sub-topics and issues.

Moderator’s recommendations are to discuss following issue, listed in the order of priority.

- Issue 1-1-1 Scenario priorities

- Issue 1-1-2 Sub-use case priorities

- Issue 2-1-1 Absolute and/or Relative RSRP Prediction Accuracy

- Issue 2-1-2 Impacts on beam/cell level measurements

- Issue 2-1-3 Impacts of measurement error on prediction accuracy

- Issue 3-1-2 Testing goals

- Issue 3-1-1 Testing setup

- Issue 3-1-2 Prediction consistency in time domain

- Issue 3-1-3 Considering multiple cells in the testing setup

- Issue 3-1-4 Ground Truth derivation

- Issue 2-1-4 inter-frequency prediction scenario

# Topic #1: General Aspects

This section addresses general aspects and priorities of different scenarios and sub use-cases identified in the draft TR 38.744.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418529 | Apple | General aspects on AI/ML for mobility in NR Observation 1: RRM measurement predictions provide several advantages, including reducing measurement gaps, increasing the periodicity of SMTC windows, minimizing scheduling and measurement restrictions, decreasing measurement delays, and shortening measurement gap lengths Observation 2: Further discussion is needed in RAN4 regarding future requirements for RLF design: - Should the indication be reported from the UE to the network, or only from the UE's lower layers to higher layers? - If a direct or indirect indication is sent from the UE to the network: - To be determined (FFS): How should such an RLF indication be triggered to the network? - For a direct indication, should the UE report a predicted RLF to the network when all measured and/or predicted downlink radio link quality values on all configured RLM-RS resources fall below a threshold (Qout)? - For an indirect indication, should the UE report all predicted SINR values with corresponding RS indices to the network when the measured and/or predicted downlink radio link quality on all configured RLM-RS resources is worse than Qout? Alternatively, should the UE indicate the predicted SINR with the relevant RS indices as long as at least one configured RLM-RS resource shows measured and/or predicted downlink radio link quality? Proposal 1: RAN4 should evaluate the combined benefits of RRM measurement predictions across multiple domains (such as spatial beam/cell, frequency, and temporal) and dimensions (across various cells and frequencies). Additionally, a working plan should be established to investigate scenarios that maximize these potential advantages. Proposal 2: For measurement event prediction, leverage AI/ML for temporal prediction of {TX, RX} Beam Pair Changes to optimize RRM measurement efficiency. To minimize RRM measurement overhead, the AI/ML model will predict the timing of beam pair changes, enabling the system to conserve resources by focusing measurements only during periods when beam changes are predicted. This approach eliminates the necessity for constant beam monitoring. |
| R4-2419176 | Nokia | - On General Aspects of AIML Mobility - RAN2 agreed that L3 filtered beam level prediction cases are lower priority. - RAN4 to focus initially the prediction of L3 cell-level measurement results and consider L3 filtered beam level prediction following RAN2 progress. For cell-level predictions, the device will use beam-level measurements configured in the test even when the prediction model uses L3 measurements as an input. Then, only L3 cell-level measurements will be reported from the device even when L1 measurements are predicted. In this respect all sub-use-cases are similar, and it will be beneficial to down-scope the number of (scenarios x sub-use-cases) combinations based on better performance. RAN4 to consider the sub-use case 2 (L3 Cell-level measurement result(s) are predicted based on actual L3 cell-level measurement result(s)) as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction. Intra-frequency intra-cell spatial domain prediction scenario is expected to have different RAN2 requirements from time domain prediction scenarios. This scenario provides high prediction accuracy without introducing complex models. RAN4 to include FR2 to FR2 intra-frequency spatial domain scenario 6 into the scope of the Rel-19 study. |
| R4-2419362 | Ericsson | General discussion on AI/ML for mobility Observation 1: Focus/scope of setting the requirement should not be on the details of the AI/ML model deployed to support mobility functionality Proposal 1: RAN4 to not define requirements for prediction as a process. Observation 2: Measurements defined for legacy L3-based mobility are used for prediction in RAN2 evaluations. Observation 3: For the measurements used in RAN2 evaluation, requirements and the side-conditions are well defined in the existing RAN4 specifications. Observation 4: Impact of the error in the measurements on the predicted measurements/event is important issue that needs to be evaluated. Proposal 2: Study whether the existing side conditions are still applicable when the legacy measurements are used for further predictions. Proposal 3: RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event. Proposal 4: Discuss and identify RAN4 specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility. |
| R4-2418673 | Huawei, HiSilicon | Discussion on general aspects in AIML mobility Proposal 1: RAN4 can discuss the following two options: - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement. - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters. - FFS: The value of MRRT. - FFS: Other parameters and the values, e.g., the prediction window. Observation 1: AI based prediction may help reduce the power consuming due to the measurement reduction. Observation 2: AI based prediction may help reduce the delay of data transmission and improve the throughput due to the reduction of measurement gap. Proposal 2: When defining measurement prediction requirements, the gain derived from power consumption reduction and measurement gap reduction shall also be taken into account. Proposal 3: RAN4 will study the following two potential testing options for defining the requirement of prediction accuracy. Pros and Cons need to be analyzed. - Option 1: Use system level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in link level channel model to define the requirement in RAN4. - Option 2: Use link level channel model to generate the training dataset and train the AI prediction model, align the prediction accuracy in the same link level channel model to define the requirement in RAN4. Proposal 4: RAN4 will study the following two potential options for defining the testing and training dataset. - Option 1: Use the channel model and the sampling rules. - Option 2: Specify or partially specify the testing dataset in a sample-by-sample manner |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 1-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about priorities for AI/ML Mobility scenarios and sub-use cases are summarized

#### Issue 1-1-1: Scenario priorities

- Prioritization of evaluation scenarios in RAN2 (Table 5.2.1-1 in TR 38.744 draft) for measurement prediction use-case.

| Scenario number | Priority | Evaluation scenario | Target study goal | Methodology |
| --- | --- | --- | --- | --- |
| 1 | Low | FR1 to FR1 intra-frequency temporal domain case A | 2nd goal | TBD |
| 2 | High | FR1 to FR1 intra-frequency temporal domain case B | 1st goal | Intra-cell |
| 3 | High | FR1 to FR1 inter-frequency (frequency domain) | 1st goal | Inter-cell |
| 4 | High | FR2 to FR2 intra-frequency temporal domain case A | 2nd goal | Intra-cell |
| 5 | Low | FR2 to FR2 intra-frequency temporal domain case B | 1st goal | TBD |
| 6 | Middle | FR2 to FR2 intra-frequency spatial domain | 1st goal | Intra-cell |

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to include FR2 to FR2 intra-frequency spatial domain scenario 6 into the scope of the Rel-19 study.

- Recommended WF:

  - Companies to discuss if Proposal 1 is agreeable.

#### Issue 1-1-2: Sub-use-case priorities

- RRM measurement prediction including intra and inter-frequency (UE sided and NW sided model) At RAN2#125bis, the following sub-use cases were agreed for RRM measurement prediction:

  - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

  - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

  - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

- Proposals and Observations:

  - Proposal 1 (Nokia): RAN4 to consider the sub-use case 2 (L3 Cell-level measurement result(s) are predicted based on actual L3 cell-level measurement result(s)) as a baseline for the study of scenarios 2, 3, and 4 for cell-level prediction.

  - Proposal 2 (Ericsson): For intra-frequency intra-cell temporal prediction scenarios (scenario 2 and 4 agreed in RAN4#112bis), RAN4 to prioritize evaluation of impact of sub-use case 2 on RAN4 requirements.

  - Proposal 3 (Ericsson): Update agreement from RAN4#112bis on sub-use cases for cell level measurement prediction as:

    - Scenario 2, 3 and 4, the 3 sub-cases can be discussed:

      - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated

      - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s)

      - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s)

      - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed.

  - Proposal 4 (Qualcomm): RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3.

- Recommended WF:

  - Companies to further discuss above listed proposal.

#### Issue 1-1-3: Requirement on prediction

- Proposals and Observations:

  - Proposal 1 (Ericsson):

    - RAN4 to not define requirements for prediction as a process.

- Recommended WF:

  - Companies to further discuss Proposal 1.

# Topic #2: RAN4 requirements

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418296 | NTT DOCOMO, INC | Discussions on RAN4 requirement impacts of AIML for mobility Observation 1: RAN4 requirements for intra/inter-frequency measurement, cell identification is needed and it requires PSS/SSS sync and SSB time index acquisition. Proposal 1: RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML. Proposal 2: For PSS/SSS sync, - In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results. - In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases. Proposal 3: For SSB time index acquisition is need more discussions for all the scenarios. |
| R4-2417622 | OPPO | Study of impacts on RAN4 requirements for AI mobility Observation 1: For Sub-use case 1/2/3, differences primarily pertain to various implementation methodologies for transitioning from L1 measurement to L3 prediction. For RAN4 testing, RAN4 can treat this without excessive fragmentation into different AI/ML implementation strategies and the synergy between AI and traditional algorithms. Sub-use case 1 Sub-use case 2 Sub-use case 3 Proposal 1: All three sub-use cases should use L3-level measurements as test objects for RAN4 testing (Point C ), rather than L1/L3 beam-level results(e.g. point A or E). - Concerning sub-use case 1 and sub-use case 2, some additional parameters(if needed) may be included as part of the test conditions in the form of side conditions (e.g., L1/L3 filter). Proposal 2: In RAN4, reuse the measurement prediction accuracy that defined in RAN2 as candidate performance metrics for all RRM sub-use cases, - Metrics for measurement prediction accuracy: Measurement prediction accuracy for cell-level RRM measurement prediction is defined as Average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell for all RRM sub-use cases, and the RSRP difference values should be an absolute value before they are averaged. Proposal 3: In RAN4, reuse the assumptions on measurement reduction rate that defined in RAN2: MRRT = skipped measurement time instances / total measurement time instances MRRS = skipped beams to be measured/ total beams to be measured Proposal 4: Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 discussions, do not need to let RAN2 do extra works. Proposal 5: To avoid redundant work of these assessments, not to introduce similar evaluations/works in the research of RAN4 AI mobility study at this stage. Proposal 6: Initiating evaluations on measurement error is not the most critical task within the current 0.5 TU framework. Proposal 7: For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window. Proposal 8: For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4: - the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5} - the window length should be multiple times of sampling period or measurement period of corresponding frequency range. Proposal 9: For FR1 intra-frequency temporal domain case B, following recommended value range have been suggested in RAN2 and could be also considered in RAN4 discussion: - MRRT: {50%~80%} - OW: {40ms ~ 2000ms} - PW: {40ms~800ms} |
| R4-2419363 | Ericsson | On requirements for AI/ML based mobility Proposal 1: Update agreement from RAN4#112bis on sub-use cases for cell level measurement prediction as: - Scenario 2, 3 and 4, the 3 sub-cases can be discussed: - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - Follow the latest definition of the sub use-cases in RAN2. Further clarification on the sub use-cases from RAN4 perspective can be discussed. Proposal 2: For intra-frequency intra-cell temporal prediction scenarios (scenario 2 and 4 agreed in RAN4#112bis), RAN4 to prioritize evaluation of impact of sub-use case 2 on RAN4 requirements. Observation 1: The impact of error in prediction of L1 beam-level measurement predicted at point A1 on L3 cell level measurement predicted at point C may need to be evaluated. Proposal 3: RAN4 to not consider studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E to align with RAN2 prioritization. Proposal 4: Measurement delay for temporal prediction cases is impacted by the duration of the observation window (OW). Proposal 5: Study the relationship between the OW length and the PW length to understand the impact of the OW length on the accuracy of the predicted measurements for the instances inside the PW and the impact of PW length on the OW length. Proposal 6: RAN4 to evaluate impact on core requirement for frequency domain prediction by at least considering the time taken by the UE to perform measurements on the measured cells. Proposal 7: RAN4 to prioritize evaluation of impact on requirement for event prediction for in-direct prediction use-case over direct prediction use case. |
| R4-2418500 | ZTE Corporation, Sanechips | Discussion impacts on requirements of AI/ML mobility Observation 1: Both cell level measurement prediction and beam level measurement prediction are within the scope of RRM measurement prediction. Observation 2: according to RAN2’s agreements, L3 filtered beam level prediction cases are lower priority and companies should focus on FR2-to-FR2 intra-frequency temporal domain prediction caseA (scenario 4). Proposal 1: RAN4 should consider whether to discuss L3 filtered beam level prediction cases. If yes, for scenario 4, the 3 cases can be discussed: Case 1: To predict L1 filtered beam level results, then generate L3 filtered results based on the predicted L1 beam results. Case 2: To directly predict L3 filtered beam level results based on the L3 beam level measurement results. Case 3: To directly predict L3 filtered beam level results based on the L1 beam level measurement results. Observation 3: As to the KPI of cell-level measurement prediction, focus on measurement prediction accuracy and RSRP difference between predicted L3 cell level measurement result and actual L3 cell level measurement result of the same cell should be considered. Proposal 2: RAN4 should guarantee the preformance of RRM measurement prediction, and define measurement prediction accuracy for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered. Proposal 3：For measurement prediction accuracy of cell-level measurement prediction, metrics for measurement prediction accuracy defined in RAN2 can be used as start point. Proposal 4: RAN4 should study the RRM impact of MRRT and MRRS, which involve the number of skipped measurement instances and total measurement instances. Observation 4: compared with legacy measurement delay requirements, number of actual measured samples and frequency layers are different, which will impact measurement requirements. Proposal 5: RAN4 should discuss whether/how to update measurement delay requirements for AI/ML mobility. Proposal 6: Prefer to study the prediction accuracy performance with consideration of the impact of measurement error in RAN4. Proposal 7: due to L1-RSRP is for beam management, it’s ok for us not to consider point A1 in AI Mob. as to point E, we prefer to consider in RAN4, which is useful for beam level measurement prediction. |
| R4-2418460 | CMCC | Discussion on impacts on RAN4 requirements for AI/ML for mobility Proposal 1: for the study on the impacts on requirements, except L3 cell level RSRP (Point C) prediction, it is proposed to consider L1 beam-level (Point A1). L3 beam-level (Point E) can be deprioritizedbasedon RAN2 agreements. Proposal 2: it is proposed to study the impact of measurement error on prediction accuracy performance. Proposal 3: For RAN2 agreement that measurement prediction accuracy for cell-level RRM measurement prediction is defined as average L3 RSRP difference between predicted L3 filtered cell-level measurement result and actual L3 filtered cell-level measurement result of the same cell, from RAN4 requirements point of view, it is proposed to further clarify the detail of actual L3 level measurement results. Proposal 4: the cell level RSRP prediction accuracy is proposed as following: - Absolute cell level RSRP accuracy = predicted L3-RSRP of cell i – ideal measured L3-RSRP of cell i - For FR2, the ideal measured L3-RSRP is the approximate as the reported RSRP measurement under a certain SNR (the SNR is high enough for sufficient accuracy of reported RSRP) - For FR1, the ideal measured L3-RSRP is the transmitted power, which is obtained as the legacy way of conducted test Proposal 5: it is proposed to discuss whether to introduce RAN4 requirements about MRRT and MRRS. Proposal 6: for RRM measurement prediction, the measurement delay requirements need to be defined/updated - For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements |
| R4-2418674 | Huawei, HiSilicon | Discussion on impacts of RAN4 requirements in AI/ML mobility Proposal 1: If new measurement quantity is introduced in AI/ML mobility, new requirements for data collection may be specified accordingly, otherwise no RRM requirement is observed. Proposal 2: For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified. Proposal 3: For intra-frequency temporal domain prediction case A, the RSRP prediction accuracy would be specified. Proposal 4: For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT with limited PW length may need to be specified. Proposal 5: For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified. Proposal 6: The processing time of inference function may need to be considered in measurement reporting delay. Proposal 7: For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C. Proposal 8: Predication accuracy is the RSRP difference between predicted L3 cell level measurement and the ideal L3 cell level RSRP measurement. The ideal L3 cell level RSRP measurement does not include any channel estimation noise. Proposal 9: RAN4 to discuss whether to define sperate prediction accuracy for each sub-use case or define one prediction accuracy requirement to cover 3 cell level sub-use cases. Observation 1: Measurement impairment may impact the prediction accuracy. Observation 2: The RSRP with measurement impairment can be obtained in reality, rather than the ideal RSRP. Observation 3: In RAN2 assumption, the ideal RSRP is assumed both for model input and the label of model output. Proposal 10: RAN4 to discuss the measurement impairment considered for verifying the prediction accuracy enabled by AI. Proposal 11: The methodology of prediction accuracy from the link level simulation perspective is, Ideal RSRP at point C: - Input is at point A and output is at C (i.e., Input from point A, experiencing channel without noise=> ideal RSRP at point A1=> ideal RSRP at point B=> ideal RSRP at point C). - Ideal RSRP does not include any channel estimation noise. Estimated RSRP at point C: - Sub-use case 1 Input from point A, experiencing channel with noise=> measured RSRP at point A1=>predicted RSRP at point A1=> generated RSRP at point B=> generated RSRP at point C - Sub-use case 2: Input from point A, experiencing channel with noise=> measured RSRP at point A1 => measured RSRP at point B=> measured RSRP at point C=>predicted RSRP at point C - Sub-use case 3: Input from point A, experiencing channel with noise=> measured RSRP at point A1 =>predicted RSRP at point C Proposal 12: If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified. Proposal 13: If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements. Proposal 14: Wait for more progress on performance monitoring from RAN1 R19 AI/ML air interface. |
| R4-2418530 | Apple | Study of impacts on RAN4 requirements for AIML Mobility in NR Observation 1: Scenario #2 for AI/ML in Release-19 Mobility is similar to Release-18 AI/ML beam prediction (Case-2), except that BM Case-2 allows set B to differ from set A. They are identical when set B = set A - Proposal 1: RAN4 will need to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E) Proposal 2: For beam-level L1 requirements and given that L1 reporting is not supported within the L3 mobility framework, bypass L3 filtering and utilize a single Tx beam for testing purposes. (point A1 equivalent to C) Proposal 3: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy requirements to assess the feasibility of the RAN2 study. This translation is essential for aligning performance metrics and ensuring that mobility optimization goals are met effectively. Additionally, explore the impact of RRM measurement requirements on prediction accuracy across various RRM measurement prediction cases and evaluation scenarios given side conditions of OW and PW, SNR. Proposal 4: The proposed cell-level RSRP prediction accuracy is defined as follows: - Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy. - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test) Proposal 5: To support the increase of MGRP, RAN4 should investigate the feasibility of predicting measurement results for multiple cells per frequency layer. Additionally, it should examine the impact of adjusting the measurement gap period or selectively skipping gap occasions to reduce measurement gaps. Proposal 6: In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells. Proposal 7: Investigate the translation of RAN2 KPI requirements into RAN4 RRM measurement accuracy and prediction measurement accuracy requirements to assess the feasibility of the RAN2 study. The evaluation of the benefits provided by AI/ML-assisted mobility should consider (KPIs) related to handovers (e.g., ping-pong handovers, handover failures/radio link failures, time of stay, handover interruptions, prediction accuracy, and measurement reduction). These factors will determine the formulation of RAN4 requirements, as the primary objective is to enhance the robustness of mobility procedures and minimize failures. - Proposal 8: RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility. Proposal 9: RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility. |
| R4-2418242 | Qualcomm Incorporated | Impact of AI based mobility on RRM requirements Input to RAN2 Observation 1: According to the objective of the study item, evaluation the potential benefits and gains of AI/ML-aided mobility should be studied by RAN2. As the item is still in a study phase, and given the limited TU allocated to RAN4 for the study item, RAN4 should focus on discussing potential impacts on core/performance requirements and testability/interoperability issues. Additionally, RAN4 should try to leverage the outcomes of the ongoing parallel work item (AI/ML air interface, particularly AI/ML-based beam management, LCM, and performance monitoring). Observation 2: The impact of measurement error on the AI/ML-based prediction results is under evaluation in AI/ML-based BM. Although the exercise is limited to FR2, the analysis of the impact of measurement error on the inference result will likely show a similar trend. Scenario/sub-use-case priorities Proposal 1: RAN4 to consider all three sub-use cases, with higher prioritization given to sub-use case 2 and 3. - Sub-use case 1: L1 beam-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) and then L3 cell-level measurement result is generated - Sub-use case 2: L3 Cell-level measurement result(s) is predicted based on actual L3 cell-level measurement result(s) - Sub-use case 3: L3 Cell-level measurement result(s) is predicted based on actual L1 beam-level measurement result(s) - FFS on whether and how to differentiate Sub-use case 2 and 3, e.g. one of or both sub-use case can be considered in defining RRM core and performance requirements RRM prediction performance and impact of measurement error Proposal 2: RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM, e.g. - Whether and how to accommodate the impact of measurement inaccuracies for a cluster-based approach. - Whether and how to model the impact of imperfect RF calibration for inter-frequency RRM prediction. - Whether different aspects need to be considered for Case A and Case B in intra-cell/intra-frequency RRM prediction. - Note that the effective differences between Case A and Case B, from a UE measurement and prediction perspective, might be limited to the measurement rate within the observation window and the prediction range into the future. Case A and Case B may have different impacts on RRM requirements, particularly about measurement/scheduling restrictions. Potential impact on RRM requirements Proposal 3: RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g. - Common aspects: - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc. - Measurement and scheduling restrictions in the prediction window. - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation. - Side conditions such as requirement applicability rules. - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements. - Scenario 2: FR1 to FR1 intra-frequency temporal domain case B, Intra-cell - Uniform measurement approach for cells in the same frequency layer - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell - CSSF - MG - Scenario 4: FR2 to FR2 intra-frequency temporal domain case A, Intra-cell Uniform measurement approach for cells in the same frequency layer |
| R4-2417951 | Xiaomi | Discussion on impacts on RAN4 requirement for AI mobility Observation 1: L3 filtered beam level prediction sub-use cases are lower priority. Proposal 1: RAN4 to deprioritize L3 beam-level(Point E) measurement prediction. Proposal 2: Prefer RAN4 to study L1 beam level RSRP prediction accuracy requirement. Proposal 3: RAN4 to discuss which part of procedure agreed in AI BM can be re-used for measurement error analysis of AI mobility. Proposal 4: Observation window and prediction window defined in RAN2 will have RRM impact: - Observation window will have impact on measurement duration - Prediction window will have impact on scheduling restriction Observation 2: Observation window and prediction length is dependent on different L1/L3 filtering method. Observation 3: Observation window and prediction length is dependent on different sub-cases. Observation 4: Observation window and prediction length is dependent on different patterns. Proposal 4: For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window requirement. Observation 5: Inter-frequency inter-cell RSRP prediction will have impact on CSSF reduction in measurement period. Proposal 5: For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period. |
| R4-2419188 | Samsung | Discussion on AI mobility general aspects Observation 1: For RRM measurement prediction, at least 5 dimensional discussions can be involved in AI mobility, for each dimension, multiple elements are contained. The 5 dimensions can be: - Scenarios - Sub use-cases - Including: Cell-specific and/or cluster approaches - Sliding/Non-sliding filtering - Observation and prediction pattern - Including: OW length/PW length; MRRT ratio - Speed Proposal 1: For RRM measurement prediction requirement definition, suggest RAN4 to discuss the priorities of, at least, scenarios, sub use-cases, sliding/non-sliding filtering, observation and prediction pattern, and speed. - How to combine all the dimensions and down select the scope should takes RAN2’s processes, agreements and the prediction performance gain analysis into consideration Proposal 2: The general aspects part of RAN4 RRM work on AI/ML based beam prediction in Rel-18 could be reused for AI/ML based RRM measurement prediction as much as possible - RAN4 to discuss what general aspects could be reused to avoid the duplicate study Proposal 3: Based on RAN2’s progress and agreements to determine what kind of AI model (i.e., UE side model or NW side model) could be considered to define the AI/ML based requirement for RRM measurement prediction in RAN4 Proposal 4: The performance Metrics/KPIs for Measurement Prediction use case considered in RAN4 shall subject to RAN2’s processes and agreements Proposal 5: - RAN4 to first study the impacts on requirements for L3 cell level RSRP (Point C) prediction - How to define the accuracy requirements, e.g., RSRP prediction accuracy/performance degradation threshold, the applicability of the accuracy, i.e., percentage in L3 AI mobility prediction evaluation - If RAN4 to study the impacts on requirements for L1 beam-level (Point A1) - Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C - RAN4 to deprioritize studying the impacts on requirements for L3 beam-level (Point E) prediction Proposal 6: For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for the Sub use-cases Proposal 7: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error - For FR2 temporal domain prediciton, suggest to wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility - For FR1 temporal and frequency domain predictions, suggest RAN4 to discuss how to evaluate the impacts of measurement errors on the prediction accuracy performance for AI mobility Proposal 8: The simulation parameters for verifying the feasibility of requirement/testing in RAN4 should follow RAN2’s progress Proposal 9: RAN4 to discuss the RRM impact of OW and PW for scenarios 2 and 4 (FR1/FR2 intra-frequency temporal domain case A/B) - The principle of determining OW and PW based on measurement period defined in RAN2 can be applied in RAN4 Proposal 10: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility Proposal 11: RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction for inter-frequency prediction Proposal 12: LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases - The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures - RAN4 study should be aligned with the agreements in RAN1 and RAN2 |
| R4-2417895 | MediaTek Inc. | Discussion on RRM requirements of AI mobility Proposal 1: For RRM measurement prediction, only use cell level RSRP as the performance metric. Proposal 2: For scenario 2 and 4 of RRM measurement prediction, define the absolute accuracy requirements of predicted/generated cell level RSRP. Proposal 3: For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated cell level RSRP. Proposal 4: RAN4 should study the impact of measurement error on the prediction accuracy requirements for AI mobility. A method is proposed above for further discussion. Proposal 5: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility. Proposal 6: Wait for more progress in AI/ML BM case 2 to study the impact on measurement delay requirements for AI mobility Scenario 2. Proposal 7: For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved. Proposal 8: Scenario 4 has no impact on measurement delay requirements. |
| R4-2419186 | Nokia | On RRM Requirement Impacts of AIML Mobility Observation 1: In all 3 agreed sub-use cases, L3 cell-measurement is predicted (generated) and considered for RRM. Observation 2: Beam-level L3 predictions are considered with low priority in RAN2 FS_NR_AIML_Mob - Release 19. Proposal 1: RAN4 to focus first on L3 cell level RSRP (Point C) prediction and subsequently on L3 beam-level prediction (Point E). Observation 3: The performance of AI/ML depends on the selection of sub-use cases used to predict cell-level measurement. Observation 4: According to RAN2 observations, L3 cell level RSRP (Point C) prediction based on actual L3 cell-level measurement result(s) (i.e., Sub-use case 2) provides the highest MAE accuracy. Proposal 2: RAN4 first study performance metrics/KPIs for target cell L3 cell level RSRP prediction based on actual serving cell L3 cell-level measurement result(s) (i.e., Sub-use case 2). Observation 5: Performance of temporal prediction depends on many input/output factors, e.g., UE speed, observation window, prediction window length, number of input beams, L3 filtering, etc. Proposal 3: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of temporal domain scenarios. Observation 6: There are two implementation examples of temporal domain prediction – case B. Proposal 4: RAN4 to study and if needed, prioritize one implementation example of temporal domain prediction – case B for RRM performance/requirement evaluation. Observation 7: The input data and characteristics of predicted measurement in inter-cell inter-frequency prediction may be different from temporal domain predictions. Proposal 5: RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios. Observation 8: For measurement prediction, different accuracy metrics are considered/under discussion in RAN2 SI for AI/ML mobility and in RAN4 for AI/ML BM use case. Proposal 6: RAN4 to study prediction accuracy for the agreed scenarios. Observation 9: The impact of measurement error has been agreed to study in RAN4 AI/ML for beam management, and it is still under discussion in RAN2 AI/ML for mobility. Proposal 7: To study the impact of measurement error, RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4). Proposal 8: For RAN4#113 meeting, RAN4 to wait for further progress from RAN2 mobility (Option#3) and RAN4 AI/ML for beam management for studying the impact on core requirements (Option#4). |
| R4-2418755 | vivo | Discussion on impacts on RAN4 requirements for AI mobility Observation 1: RAN2 understanding for inter/intra-frequency is for descriping the relationship of the center frequency between the cell corresponding to AI input and the cell corresponding to AI output, which is different with RAN4 understanding. Observation 2: More information types except RSRP, e.g., BS Tx power, center frequency, co-located information, cell ID, etc., maybe also helpful as model input or other conditions to performing AI prediction in scenario 3. Proposal 1: RAN4 to take the following cases as the start point for scenarios 2,3,4 Scenario 2: - Serving cell (input) to predict the serving cell (output) - Neigbour cell i (input) to predict the neigbour cell i (output). Scenario 3: - Serving cell (input) to predict inter-frequency neigbour cell i (output) - Neigbour cell i (input) to predict co-located neigbour cell j (output), where i≠j - Neigbour cell i and neigbour cell j are inter-frequency Scenario 4: - Serving cell (input) to predict the serving cell (output) - Neigbour cell i (input) to predict the neigbour cell i (output). Proposal 2: RAN4 to take the following cases as the start point for sub-use cases Sub-use case 1: - Use A to predict A, or A1 to predict A1, or A to predict A1, and calculate C; Sub-use case 2: - Use C to predict C; Sub-use case 3: - Use A to predict C, or use A1 to predict C; Proposal 3: RAN4 only to consider the AI predict metric as the KPI and study for each cases: Sub-use case 1: - Evaluate prediction accuracy for point A/A1 Sub-use case 2: - Evaluate prediction accuracy for point C Sub-use case 3: - Study performance metrics for point C with considering: - RAN4 to find out how to obtain the ground truth value by considering the RRC configuration, e.g., L3 filtering. Proposal 4: RAN4 not to study the performance metric for L3 beam-level value (Point E). Proposal 5: RAN4 to evaluate the prediction accuracy performance with consideration of the impact of measurement error in WI . |
| R4-2417698 | CATT | Discussion on impacts of AIML mobility on RRM requirements Proposal 1: RAN4 prioritizes the discussion on performance metrics and defers the discussions on the following aspects in SI phase until RAN2’s design on AI/ML mobility is clear: - Impacts of measurement errors - Impacts on core requirements - LCM related requirements Observation 1: What level of results is outputted by AI/ML models depends on the training datasets and it is implementation-dependent. Observation 2: Defining accuracy requirements with filtering coefficients being a side condition may narrow down the applicable scope of requirements. Observation 3: Test goals also affect the determination of performance metrics. Proposal 2: RAN4 does not define accuracy requirements for point C and evaluate if the existing accuracy requirements can be reused for measurement reduction cases. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 2-1: General and Scope

Sub-topic description:

In this sub-topic the company proposals about the RRM requirements for AI/ML Mobility use cases are summarized.

#### Issue 2-1-1: Absolut and/or Relative RSRP Prediction Accuracy

- Proposals and Observations:

  - Proposal 1 (MediaTek):

    - For scenario 3 of RRM measurement prediction, further discuss whether to define the absolute and/or relative accuracy requirements of predicted/generated cell level RSRP.

  - Proposal 2 (Apple):

    - In Scenario 3 of RRM measurement prediction, further explore the decision to establish absolute and/or relative accuracy requirements for the predicted/generated L3 cell-level RSRP. Additionally, discuss methods for conducting relative measurements, such as those referencing to an anchor cell or calculating pairwise differences between cells

  - Proposal 3 (Huawei):

    - RAN4 can discuss the following two options:

      - Option 1: To verify whether the difference between the predicted measurement and the ideal measurement is within the existing relative measurement accuracy requirement.

      - Option 2: To verify whether the difference between the predicted measurement and the ideal measurement is within X dB, where X is larger than legacy relative measurement accuracy margin requirement, under the side condition of MRRT and other parameters.

- Candidate options:

  - Option 1: RAN4 to consider relative RSRP accuracy requirements only.

  - Option 2: RAN4 to consider absolute RSRP accuracy requirements only.

  - Option 3: RAN4 to consider both absolute and relative RSRP accuracy requirements.

  - For Relative RSRP accuracy

    - Option 1-a: In case of relative RSRP accuracy, the difference between the predicted measurement and the ideal measurement should be within the existing relative measurement accuracy requirement

    - Option 1-b: In case of relative RSRP accuracy, the difference between the predicted measurement and the ideal measurement can be greater than the existing relative measurement accuracy requirement

- Recommended WF

  - Discuss different options listed above. Options 1, 2 and 3 can be discussed first. Following this discussion, Sub-options for relative RSRP accuracy can be discussed.

#### Issue 2-1-2: Impacts on beam/cell level measurements

- Background:

The following agreement was achieved in RAN4#112bis meeting:

Agreement: - RAN4 to first study the impacts on requirements for the following case: - L3 cell level RSRP (Point C) prediction - FFS L1 beam-level (Point A1) - FFS L3 beam-level (Point E) - Other cases can be considered based on RAN4 further discussion and RAN2 further progress.

The figure below from TS 38.300 (Figure 9.2.4-1) should be considered as a reference for this discussion.

- Proposals and Observations:

  - Proposal 1 (Ericsson, Xiaomi, Samsung, MediaTek, vivo):

    - RAN4 to not consider studying impact on RAN4 requirement due to L3 beam-level measurement prediction at point E to align with RAN2 prioritization.

  - Proposal 2 (ZTE):

    - RAN4 should guarantee the performance of RRM measurement prediction, and define measurement prediction accuracy for AI/ML-based predicted beam level L1/L3-filtered or cell-level L3-filtered.

  - Proposal 3 (CMCC):

    - For the study on the impacts on requirements, except L3 cell level RSRP (Point C) prediction, it is proposed to consider L1 beam-level (Point A1). L3 beam-level (Point E) can be deprioritized based on RAN2 agreements

  - Proposal 4 (Apple):

    - RAN4 will need to study accuracy requirements for AI/ML predicted cell-level L3-filtered measurements for measurement reduction purpose. FFS on predicted beam level L3-filtered measurements (point E)

  - Proposal 5 (Samsung):

    - RAN4 to first study the impacts on requirements for L3 cell level RSRP (Point C) prediction

    - If RAN4 to study the impacts on requirements for L1 beam-level (Point A1)

      - Whether to/How to define an additional prediction accuracy requirement on top of the requirement at C

  - Proposal 6 (Nokia, Qualcomm):

    - RAN4 to focus first on L3 cell level RSRP (Point C) prediction and subsequently on L3 beam-level prediction (Point E).

  - Proposal 7 (Oppo): All three sub-use cases should use L3-level measurements as test objects for RAN4 testing (Point C ), rather than L1/L3 beam-level results(e.g. point A or E).

  - Proposal 8 (CATT): Do not define new accuracy requirements for point C and evaluate if the existing accuracy requirements can be reused for measurement reduction cases.

  - Proposal 9 (Huawei): For 3 sub-use cases for cell-level RRM measurement prediction, RAN4 is to define measurement prediction accuracy at point C.

  - Proposal 10 (vivo): RAN4 only to consider the AI predict metric as the KPI and study for each cases:

    - Sub-use case 1:

      - Evaluate prediction accuracy for point A/A1

    - Sub-use case 2:

      - Evaluate prediction accuracy for point C

    - Sub-use case 3:

      - Study performance metrics for point C with considering:

      - RAN4 to find out how to obtain the ground truth value by considering the RRC configuration, e.g., L3 filtering.

- Candidate options:

  - Option 1: Do not consider L3 beam-level (Point E) and L1 beam-level (Point A1) measurement prediction

  - Option 2: Consider L3 beam-level (Point E) and L1 beam-level (Point A1) measurement prediction as second priority

  - Option 3: RAN4 should consider studying the impacts on requirements for L1 beam-level (Point A1)

  - Option 4: RAN4 should consider studying the impacts on requirements for AI/ML-based predicted beam level L1/L3-filtered (both Point A1 and Point E)

  - Option 5: Consider reusing the existing accuracy requirements (at Point B) for measurement reduction cases for Point C.

- Recommended WF

  - Discuss different options listed above.

#### Issue 2-1-3: Impacts of measurement error on prediction accuracy

- Background

Following candidate options were identified in RAN4#112bis meeting’s moderator’s summary.

- Candidate options (in the TDoc summary): - Option 1: RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error. - Option 2: suggest RAN2 to take into account the measurement errors in RAN2 evaluation. - Option 3: Wait for RAN2 evaluation results and if RAN2 doesn’t consider measurement errors then RAN4 should do it after RAN2 evaluations. - Option 4: Wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - Whether and how to evaluate the impacts of measurement error on performance requirement should be considered within RAN4 discussions, do not need to let RAN2 do extra works..

  - Proposal 2 (ZTE, CMCC, Huawei, MediaTek):

    - RAN4 should study the impact of measurement error on the prediction accuracy requirements for AI mobility

  - Proposal 3 (Apple):

    - RAN4 to investigate the impact of measurement errors in the input data of AI/ML model on the accuracy of predicted measurements. Additionally, RAN4 should leverage the simulation methodologies and evaluation results (from AI/ML BM Case 1) to assess how these measurement errors affect the accuracy of predictions related to mobility.

  - Proposal 4 (Qualcomm, vivo):

    - RAN4 should not start simulation-based performance evaluation on measurement prediction due to measurement inaccuracy in the observation window during the study item phase. Instead, the group can further discuss additional aspects that might need to be taken into account in the WI phase, if approved, in addition to the outcome of the evaluations on AI/ML-based BM

  - Proposal 5 (Xiaomi):

    - RAN4 to discuss which part of procedure agreed in AI BM can be re-used for measurement error analysis of AI mobility.

  - Proposal 6 (Samsung):

    - RAN4 to study the prediction accuracy performance with consideration of the impact of measurement error

      - For FR2 temporal domain prediction, suggest to wait for more progress in AI/ML BM to study the impact of measurement error on the prediction accuracy requirements for AI mobility

      - For FR1 temporal and frequency domain predictions, suggest RAN4 to discuss how to evaluate the impacts of measurement errors on the prediction accuracy performance for AI mobility

  - Proposal 7 (Nokia):

    - To study the impact of measurement error, RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4).

  - Proposal 8 (Ericsson):

    - RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

    - Discuss and identify RAN4 specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility.

- Candidate options:

  - Option 1: RAN4 should study the impact of measurement error on the prediction accuracy during SI phase

  - Option 2: RAN4 should study the impact of measurement error on the prediction accuracy during WI phase

  - Option 3: RAN4 should wait for evaluation outcomes of RAN2 AI/ML for mobility (option3) and RAN4 AI/ML for beam management (option 4)

  - Option 4:  RAN4 to provide input to RAN2 to consider impact of error on measurement used for prediction on the accuracy of the predicted measurement/event.

    - RAN4 to discuss and identify specific simulation parameters (such as interfering cells, side conditions, RF margin etc.) that are not considered by RAN2 which may have an impact on the performance of AI/ML based mobility.

- Recommended WF

  - Discuss above listed options.

#### Issue 2-1-4: Inter-frequency prediction scenario

- Proposals and Observations:

  - Proposal 1 (Huawei):

    - For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified.

  - Proposal 2 (Xiaomi):

    - For inter-frequency inter-cell RSRP prediction, RAN4 to study the impact on CSSF reduction in measurement period.

  - Proposal 3 (Samsung):

    - RAN4 to discuss the RRM impact of measurement gap reduction in measurement period/ scheduling restriction for inter-frequency prediction

  - Proposal 4 (MediaTek):

    - For Scenario 3, gap sharing mechanism among different inter-frequency layers should be considered for both for measurement and monitoring when more than one inter-frequency layers are involved.

  - Proposal 5 (Nokia):

    - RAN4 to study the minimum requirement/characteristic of the input and output of AI/ML model for RRM and testability assessment of inter-cell inter-frequency prediction domain scenarios.

  - Proposal 6 (CMCC):

    - for RRM measurement prediction, the measurement delay requirements need to be defined/updated

      - For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements

  - Proposal 7 (Qualcomm):

    - RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g.

      - Common aspects:

        - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc.

        - Measurement and scheduling restrictions in the prediction window.

        - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation.

        - Side conditions such as requirement applicability rules.

        - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements.

      - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell

        - CSSF

        - MG

- Recommended WF

  - Discuss different proposals listed above.

#### Issue 2-1-5: LCM related requirements

- Proposals and Observations:

  - Proposal 1 (Apple):

    - RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility.

  - Proposal 2 (Samsung):

    - LCM related requirements and functional tests should to be defined based on the specific AI/ML mobility use cases and sub-use cases

      - The LCM procedures agreed in AI/ML for NR air interface RRM need to be considered as starting point

      - Prefer to use the legacy framework for RRC/MAC-CE/DCI based core requirements as the baseline for LCM procedures

  - Proposal 3 (CATT):

    - RAN4 prioritizes the discussion on performance metrics and defers the discussions on the following aspects in SI phase until RAN2’s design on AI/ML mobility is clear:

      - Impacts of measurement errors

      - Impacts on core requirements

      - LCM related requirements

  - Proposal 4 (Huawei):

    - If the functionality/model activation, deactivation, or fallback to non-AI function are indicated by network, the activation/deactivation/fallback delay and corresponding side conditions would need to be specified.

    - If the functionality/model activation, deactivation, or fallback to non-AI function are autonomously performed by UE, the requirements would be related with monitoring requirements.

- Recommended WF Candidate options / tentative agreements:

  - Option 1: RAN4 should focus first on performance metrics and relevant requirements and post-pone the discussion on LCM related issues.

  - Option 2: RAN4 should start discussion LCM related issues in parallel of discussion on performance metrics.

- Recommended WF

  - LCM related discussions should be held in RAN4 potentially at a later stage. RAN2 needs to progress on this topic.

#### Issue 2-1-6: Impact on Core requirements

- Background

- Following candidate options were identified in RAN4#112bis meeting’s moderator’s summary.

- Candidate options: - Option 1: For intra-frequency intra-cell prediction, RAN4 to study the RRM impact of observation window and prediction window defined in RAN2 - Option 2: For inter-frequency inter-cell frequency domain prediction, RAN4 to study the side condition of frequency prediction (e.g., EPRE difference). - Option 3: Wait for more progress in AI/ML BM to study the impact on measurement report delay requirements for AI mobility - Option 4: RAN4 not to discuss impacts on core requirements until RAN2 has a stable mechanism for AI/ML assisted mobility. - Option 5: RAN4 to study if HO requirements including delay, interruption, side conditions and accuracy are needed to be updated for AI/ML-assisted mobility

- Proposals and Observations:

  - Proposal 1 (Ericsson):

    - RAN4 to evaluate impact on core requirement for frequency domain prediction by at least considering the time taken by the UE to perform measurements on the measured cells

  - Proposal 2 (Nokia): For RAN4#113 meeting, RAN4 to wait for further progress from RAN2 mobility (Option#3) and RAN4 AI/ML for beam management for studying the impact on core requirements (Option#4).

  - Proposal 3 (CATT):

    - RAN4 prioritizes the discussion on performance metrics and defers the discussions on the following aspects in SI phase until RAN2’s design on AI/ML mobility is clear:

      - Impacts of measurement errors

      - Impacts on core requirements

      - LCM related requirements

  - Proposal 4 (CMCC):

- for RRM measurement prediction, the measurement delay requirements need to be defined/updated

- For intra-frequency temporal domain prediction, UE will skip some measurement occasions, which means that only subset of measurement occasions are measured so that SMTC and/or MGRP cannot be directly used in the measurement delay requirements as legacy

- For inter-frequency inter-cell frequency domain prediction, not all configured MOs need to be measuerd, which means that only the actually measured frequency layers are considered in the measurement delay requirements

- it is proposed to discuss whether to introduce RAN4 requirements about MRRT and MRRS.

  - Proposal 5 (Huawei):

    - For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

    - For intra-frequency Intra-cell temporal domain prediction case B, RSRP prediction accuracy under the side condition of MRRT with limited PW length may need to be specified.

    - For inter-frequency inter-cell frequency domain prediction, the side condition of frequency prediction (e.g., EPRE difference) and RSRP prediction accuracy may need to be specified.

    - The processing time of inference function may need to be considered in measurement reporting delay.

  - Proposal 6 (Qualcomm):

    - RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g.

      - Common aspects:

        - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc.

        - Measurement and scheduling restrictions in the prediction window.

        - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation.

        - Side conditions such as requirement applicability rules.

        - Overall budget in terms of mandatory capability on the number of cells and frequency layers for actual measurements and inference-based measurements.

      - Scenario 2: FR1 to FR1 intra-frequency temporal domain case B, Intra-cell

        - Uniform measurement approach for cells in the same frequency layer

      - Scenario 3: FR1 to FR1 inter-frequency, Inter-cell

        - CSSF

        - MG

      - Scenario 4: FR2 to FR2 intra-frequency temporal domain case A, Intra-cell

        - Uniform measurement approach for cells in the same frequency layer

- Recommended WF

  - RAN4 should wait for further progress in RAN2 before starting discussion on core requirements in RAN4.

#### Issue 2-1-7: Impacts on cell identification

- Proposals and Observations:

  - Proposal 1 (NTT Docomo):

    - RAN4 needs study the impacts on cell identification for L3 cell-level measurement prediction by AI/ML.

  - Proposal 2 (NTT Docomo):

    - For PSS/SSS sync,

      - In scenario 2 and 4, it may be achieved by prediction. However in sub-use case 2, it needs further study because the input is L3 cell-level measurement results.

      - In scenario 3, since no target cell measurement is conducted, RAN4 should discuss the feasibility on the cell detectability for all sub-use cases.

  - Proposal 3 (Qualcomm):

    - RAN4 to review the existing RRM requirement and outline the identified potential impacts at a high level, e.g.

      - Common aspects:

        - Definition of known vs. unknown target cells for mobility and/or PSCell/SCell addition and activation.

- Recommended WF

  - Discuss above listed proposals.

#### Issue 2-1-8: Impacts from observation and prediction windows

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - For each scenario, further consideration is needed on how to define it in RAN4 tests, e.g., configurations of the observation window and prediction window.

    - For temporal domain case A, following limitations have been made in RAN2 and could be considered in RAN4:

      - the ratio between observation window and prediction window is at least limited to value range {5,4,3,2,1,1/2,1/3,1/4,1/5}

      - the window length should be multiple times of sampling period or measurement period of corresponding frequency range.

  - Proposal 2 (Ericsson):

    - Measurement delay for temporal prediction cases is impacted by the duration of the observation window (OW) .

    - Study the relationship between the OW length and the PW length to understand the impact of the OW length on the accuracy of the predicted measurements for the instances inside the PW and the impact of PW length on the OW length.

  - Proposal 3 (Huawei):

    - For intra-frequency Intra-cell temporal domain prediction case A, prediction window length (or called as prediction evaluation period) may need to be discussed and specified.

  - Proposal 4 (Qualcomm):

    - RAN4 should consider identifying potential impacts from:

      - Adjustment of the length of observation and prediction windows depending on UE in DRX state and/or cycle, serving cell RSRP, etc.

      - Measurement and scheduling restrictions in the prediction window.

  - Proposal 5 (Xiaomi):

    - Observation window and prediction window defined in RAN2 will have RRM impact:

      - Observation window will have impact on measurement duration

      - Prediction window will have impact on scheduling restriction

    - For intra-frequency intra-cell prediction, suggest RAN4 to select some typical case/scenarios to study observation window and prediction window requirement.

  - Proposal 6 (Samsung):

    - RAN4 to discuss the RRM impact of OW and PW for scenarios 2 and 4 (FR1/FR2 intra-frequency temporal domain case A/B)

      - The principle of determining OW and PW based on measurement period defined in RAN2 can be applied in RAN4

    - For the side condition consideration, RAN4 to discuss whether to/how to define a certain threshold for OW length to evaluate the L3 measurement prediction accuracy for the Sub use-cases

- Recommended WF

  - Discuss above listed proposals.

#### Issue 2-1-9: Cell-level RSRP prediction accuracy definition

- Proposals and Observations:

  - Proposal 1 (Apple):

    - The proposed cell-level RSRP prediction accuracy is defined as follows: Absolute RSRP Accuracy for Cell-Level = predicted L3-RSRP of cell i - ideal received L3-RSRP of cell i

      - In FR2: The ideal received L3-RSRP is assumed to approximate the reported RSRP measurement, provided the SNR is sufficiently high for reliable RSRP accuracy.

      - In FR1: The ideal received L3-RSRP is taken to be equal to the transmitted power (assuming conducted test and AWGN channel during the test)

  - Proposal 2 (Huawei):

    - The methodology of prediction accuracy from the link level simulation perspective is,

      - Ideal RSRP at point C:

        - Input is at point A and output is at C (i.e., Input from point A, experiencing channel without noise=> ideal RSRP at point A1=> ideal RSRP at point B=> ideal RSRP at point C).

        - Ideal RSRP does not include any channel estimation noise.

      - Estimated RSRP at point C:

        - Sub-use case 1: Input from point A, experiencing channel with noise=> measured RSRP at point A1=>predicted RSRP at point A1=> generated RSRP at point B=> generated RSRP at point C

        - Sub-use case 2: Input from point A, experiencing channel with noise=> measured RSRP at point A1 => measured RSRP at point B=> measured RSRP at point C=>predicted RSRP at point C

        - Sub-use case 3: Input from point A, experiencing channel with noise=> measured RSRP at point A1 =>predicted RSRP at point C

- Recommended WF

  - Discuss above listed proposals.

# Topic #3: Testability and interoperability

Main technical topic overview. The structure can be done based on sub-agenda basis.

## Companies’ contributions summary

| T-doc number | Company | Proposals / Observations |
| --- | --- | --- |
| R4-2418675 | Huawei, HiSilicon | Discussion on testability and interoperability issues in AIML mobility Observation 1: Both cell level prediction and L3 filtered beam level prediction sub-use cases are identified in RAN2, and L3 filtered beam level prediction sub-use cases are lower priority. Proposal 1: RAN4 to discussion the testability of Case 1, Case 2 and Case 3. Proposal 2: Postpone the testability discussion of Case 1 (i.e., to predict beam level results, then generate cell level results based on the predicted beam results), Case 3 (i.e., to directly predict cell level results based on beam level results) to wait the progress of the testability study in AI-BM. Proposal 3: RAN4 to consider whether to reuse the test setup in legacy for Case 2 (i.e., to directly predict cell level results based on cell level results). |
| R4-2419187 | Nokia | Testability and Interoperability Issues for AIML Mobility Proposal 1: Each specified AI/ML-enabled Feature shall have RAN4 requirements for minimum performance and UE needs to pass at least one test for specific sub-set of applicable configurations (matching functionalities) as part of conformance testing. Proposal 2: A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML-enabled Feature. Proposal 3: For AI/ML mobility, RAN4 should consider different NW side additional conditions (i.e., NW-related parameters) for multiple target cells/sites in the same or different frequency layers. Proposal 4: RAN4 should also study how to model different time-varying characteristics per cell/site due to moving UE trajectories, and how to model different variations per test iteration. Observation 1: Majority of test cases specified in 3GPP TS 38.133 assume no application of L3 filtering (i.e., with the filter coefficient set to zero) and emulate only a limited number of beams in the test setup. Observation 2: Depending on the configuration of beam consolidation selection, L3 cell-level RSRP predictions at Point C may equal to L3 beam-level RSRP predictions at Point E. Proposal 5: RAN4 to consider L3 filtering and multi-beam scenarios when studying test cases for AI/ML mobility uses cases. Proposal 6: RAN4 should define test setups considering L1 and L3 measurements from serving and neighbouring cells as well as AI/ML model with related configuration (inputs, prediction horizon). Proposal 7: For FR2, the under-study testing setup in AI/ML BM use case should be reused as a baseline for measurement prediction use case testing. Proposal 8: RAN4 should consider new aspects for FR2 testing in AI/ML mobility such as channel model, number of AoAs, maximum number of cells and Tx beams and number of probes required in the test chamber. Proposal 9: RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI. |
| R4-2418756 | vivo | Discussion on testability and interoperability for AI mobility Observation 1: Whether and how to emulate a multi-beam environment with a fading channel of spatial characteristics is the key issue in AI BM discussion. Observation 2: There are no testability issues for the conducted testing in scenario 2 and 3. Observation 3: In AI/ML based BM case 1, all the Tx beams with spatial domain characteristics need to be emulated in OTA chamber, while the spatial characteristics are not necessarily needed in AI mobility test system. Proposal 1: RAN4 to consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell. Proposal 2: RAN4 to consider use AWGN/TDL channel model as the baseline for testing. Proposal 3: RAN4 to consider modelling reference UE trajectory for OTA testing and to emulate the dynamic channel by changing the doppler, changing the SNR, rotate UE in turnable, and change probe power. Proposal 4: RAN4 to take the potential testability setups for the 3 scenarios into consideration as the following: scenario number; Sub-scenario; Frequency Range; Test method; Channel model (as start point); Min cell number; Min probe number; SNR; UE trajectory. 2; 1; FR1; Conducted; AWGN/TDL; 1 serving cell; N/A; legacy SNR; Change doppler and SNR. 2; 2 (1 serving cell, 1 neighbour cell). 3; 1; FR1; Conducted; AWGN/TDL; 2 (1 serving cell, 1 neighbour cell); N/A; legacy SNR; Change doppler and SNR. 2; 3 (2 co-located neighbour cell, 1 serving cell). 4; 1; FR2; OTA; AWGN/TDL; 1 serving cell; 1; Post SNR of the strongest beam pair; Change doppler, SNR, UE rotation, and change probe power. 2; 2 (1 serving cell, 1 neighbour cell); 2. Observation 4: A simplified RAN4 test system would skip Tx beams and the input dimension mismatch issue may happen in the lab. Proposal 5: RAN4 to consider evaluating the impact on the model input dimensions mismatch (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance. |
| R4-2418528 | Apple | On Testability and Interoperability Issues for AIML Mobility in NR Observation 1: The primary challenge in achieving generalization for AI/ML models in AI/ML mobility is ensuring that a model trained with specific cell or inter-cell characteristics can perform effectively across different cells and inter-cell environments. A critical question is whether an AI/ML model trained in one deployment scenario can be directly applied to a different scenario without retraining. Addressing this challenge is essential, as it could reduce the need for multiple AI/ML models, thereby enhancing adaptability and robustness in diverse and realistic radio conditions (SNR). Proposal 1: RAN4 should define performance metrics/KPIs and clarify the objectives, testing properties, and methodologies for evaluating AI/ML functionalities across each mobility use case and sub-use case addressing both Goal 1 (measurement reduction) and Goal 2 (mobility optimization): - 1st goal: To assess the trade-off between measurement reduction and measurement prediction accuracy and its effect to mobility performance with legacy as a baseline with MRRT, SNR as side conditions - 2nd goal: To determine if the AI/ML model achieves measurable improvements in handover (HO) performance with PW, OW, SNR as side conditions Proposal 2: For temporal domain prediction, to investigate whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes. Proposal 3: Define multiple test iterations, each designed to reflect unique, time-varying conditions such as reception power and SNR influenced by UE trajectory, to ensure coverage of potential conditions in AI/ML-assisted mobility scenarios Proposal 4: For Rel-19 AI/ML mobility, it is essential for RAN4 to investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers. Proposal 5: RAN4 should investigate the side conditions for RSRP prediction pertinent to AI/ML mobility, along with network assistance information required to guarantee generalization and effective operation across multiple target cells and different frequency layers. Additionally, it should leverage the discussions in AI/ML beam management regarding the signaling of associated IDs to ensure consistency between primary and secondary carrier components, thereby enhancing generalization capabilities. Proposal 6: RAN4 should leverage the LCM progress and requirements discussed in R-19 WI related to AI/ML Air Interface and investigate enhancements to these LCM procedures to RRM for mobility. |
| R4-2418501 | ZTE Corporation | Discussion on the Interoperability and testability aspects Observation 1: One side model is considered in AI mobility. For one side mode, the inference is performed entirely on one side (UE side or NW side), so there should be no interoperability issues involved in the inference process. However, the interaction maybe happened between UE and NW mainly depended on signalling. Proposal 1：RAN4 to consider interoperability for one side model, including - UE sided and NW sided model for RRM measurement prediction - UE sided model for HO failure/RLF prediction - UE sided model for Measurement events prediction And detail should be further discussed based on RAN1/RAN2’s progress. Observation 2: Even the measurement results of skipped samples are predicted, the network performance based on predicted results is highly likely to differ from the performance based on actual measurements due to the vary channel condition. Proposal 2: it’s necessary to ensure the performance compared to legacy measurement and the testing goal is to verify whether the minimum performance of AI/ML Mobility can be achieved. Observation 3: the channel conditions of real environment are complex and diversified, and the channel state is not stationary and constantly changing for mobility, which can easily lead to measurement results used for inference not being applicable to the inference stage, resulting performance would degrade. Proposal 3: RAN4 should consider whether it is reliable to use the measurement results from the previous moment to infer the results from another moment under constantly changing channel conditions. Maybe some side conditions are needed. Proposal 4: Generalization discussion in RAN4 can be started if sufficient progress in RAN2 or NR_AIML_Air WI. Proposal 5: The testability discussion of sub-use-case 1 (L1-L1) and sub-use-case 3 (L1-L3) should wait the progress of the testability study in AI-BM. And RAN4 can start the discussion on sub-use-case 2 (L3-L3). Proposal 6: From the perspective of test, core part and performance part requirements should be considered, i.e. measurement delay and measurement accuracy. Proposal 7: RAN4 to discuss numbers of Tx beam/SSBs to be configured and the number of cell to be measured for AI/ML mobility. |
| R4-2417896 | MediaTek Inc. | Discussion on testability and interoperability of AI mobility Proposal 1: Use conductive test in FR1 for AI/ML mobility, there is no testability issues found. Proposal 2: Open to take a scenario as an example to further discuss the test set up details to check whether there is potential testability issues. Observation 1: Ground truth is available at TE using conductive test in FR1 for AI/ML mobility. |
| R4-2418243 | Qualcomm Incorporated | Testability and interoperability for AI based mobility Testing goals and metric Proposal 1: RAN4 to not consider the use of KPIs defined and used for assessing AI/ML-based mobility performance gains in RAN2. Based on the AI/ML-based BM WI, the following can be considered as candidates for the metric of AI/ML-based RRM prediction, as an example. However, detailed discussion should wait for further RAN2 progress: - Option 1: RRM prediction accuracy, e.g. L3 cell level RSRP (Point C), L1 beam-level (Point A1), or L3 beam-level (Point E) - Option 2: Cell [and/or beam] prediction accuracy - Top-1 (%) : the percentage of "the Top-1 strongest Cell [and/or beam]is Top-1 predicted Cell [and/or beam]” - Top-K/1 (%) : the percentage of "the Top-1 strongest Cell [and/or beam] is one of the Top-K predicted Cell [and/or beam] “ - Option 3: The successful rate for the correct prediction which is considered as maximum Cell [and/or beam] level RSRP among top-K predicted Cell [and/or beam] is larger than the RSRP of the strongest Cell [and/or beam] – x dB. Proposal 2: RAN4 to study the impacts on requirements and testability for the following case: - L3 cell level RSRP (Point C) - L3 beam-level (Point E) - L1 beam-level (Point A1) Proposal 3: Regarding the uncertainty due to the UE Rx beam gain and spherical coverage in FR2 AI/ML-based RRM prediction, RAN4 to wait and see the outcome of the simulation campaign from AI/ML-based BM and try to leverage the results and observations to the extent possible. Proposal 4: RAN4 to further study the impact of time-varying channels and SNR due to fading and UE mobility on the definition of expected RRM measurement result. Interoperability Observation 1: No specific interoperability issues have been identified. This aspect was discussed in the AI/ML air-interface item, but it was mostly limited to the two-sided AI/ML model-based CSI compression use case, which is not relevant to this study item. Data set generation and consistency Proposal 5: In order to ensure data sanity in AI/ML model training and inference and consistency between data set and measurement configuration, RAN4 should review the performance assessment assumptions and configurations when discussing the testability aspect. |
| R4-2417941 | Xiaomi | Discussion on Study of testability and interoperability for AI/ML mobility Proposal 1: For FR1 RRM measurement prediction, RAN4 can assume that the actual ground truth is available at TE. Proposal 2: For FR1 RRM measurement prediction, the configuration of 1 Tx and TDL channel model can be considered in the test. Proposal 3: For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied. Proposal 4: For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline. |
| R4-2417699 | CATT | Discussion on testability and interoperability issues for AIML mobility Proposal 1: For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS. Proposal 2: For scenario 4, RAN4 further discusses how to define HO performance metric. Proposal 3: RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs. Proposal 4: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes, which makes ground truth known to TE. The range of TX power can be FFS. Proposal 5: AWGN or TDL channel can be used in FR1 conducted test. Proposal 6: The following values can be a baseline for the number of configured cells in FR1 HO performance test: - [9] cells for intra-frequency predictions; - [5] cells for inter-frequency predictions; - And values of carrier frequencies can be determined later. Proposal 7: For measurement reduction test, less cells can be set up in test, e.g., [3] cells. Proposal 8: RAN4 to postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM. Proposal 9: Colocation can be reflected by a minor RSRP difference between two carriers. |
| R4-2417623 | OPPO | Study of testability and interoperability for AI mobility Proposal 1: For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it. Observation 1: Constraints on testability posed by the FR1 testing environment could be less than those encountered with FR2. Proposal 2: For inter-carrier scenario (inter-frequency inter-cell prediction), how to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site should be considered. Proposal 3: For inter-carrier scenario (inter-frequency inter-cell prediction), how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship should be considered. - Stable time-domain RSRP signal for the tests might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time. Proposal 4: For inter-carrier scenario (inter-frequency inter-cell prediction), to ensure the testability of measurements for inter-cell prediction, channel modelling for inter-cell prediction should be considered. - For FR1, considering the test environment is conducted testing, RAN4 should re-evaluate the selection of channel models. Option 1: UMA channel, Option 2: CDL channel. Proposal 5: Regarding L3 RSRP, compared with L1 RSRP, the key difference lies in the L3 filter which may need to be included as part of the testing side conditions. |
| R4-2418461 | CMCC | Discussion on testability and interoperability for AI/ML for mobility Proposal 1: it is proposed to reuse testing goal definition from NR_AIML_Air WI. In detail, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved. Proposal 2: for FR1 AI/ML mobility, it is proposed that the ground truth is the transmit power settled by TE. |
| R4-2419190 | Samsung | General discussion on AI mobility regarding testability and interoperability Proposal 1: In AI/ML mobility, regarding the test scope, suggest to prioritize discussions on AI/ML model inference in each use case and sub-use case. - Take Rel-18 AI/ML test principles/procedures/frameworks as much as possible for testing the AI/ML model inference functionality. Proposal 2: RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows. - RRM measurement prediction - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline - Performance gain: The measurement reduction should keep the mobility performance from degrading - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained Proposal 3: RAN4 to consider the following testability aspects for AI mobility - Consideration of serving and neighbour cells - Multi-cell setup and number of cells to measure Proposal 4: For FR2 measurement prediction testing, RAN4 made for AI/ML-BM can be reused as a baseline - RAN4 to discuss what principle on testability and interoperability for which AI/ML-PHY scenarios/cases could be reused to avoid the duplicate study |
| R4-2419364 | Ericsson | On testability issues related to AI/ML for mobility Proposal 1: Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup. Proposal 2: The aim of testing for intra-cell temporal domain prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet: - requirement for the measurements performed inside the OW, - requirement for the measurements predicted for the time instances inside the PW. Proposal 3: RAN4 to discuss considerations for deriving the ground truth value for validation of the accuracy of predicted measurements. Proposal 4: The aim of testing for inter-cell (inter-frequency) scenario should be to validate that the measurements reported by the UE: - meet the requirements for measured cell, and - meet the requirements for the target cell. Proposal 5: Existing test setup defined to validate requirements for inter-frequency measurements can be re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3. - Cell 1 in the test can be considered the cell for measurement. - Cell 2 in the test can be considered the target cell for measurement prediction. |

## Open issues summary

Before Meeting, moderators shall summarize list of open issues, candidate options and possible WF (if applicable) based on companies’ contributions.

### Sub-topic 3-1: New Testability aspects

Sub-topic description:

In this sub-topic the company proposals about the testability aspects for AI/ML Mobility use cases are summarized.

#### Issue 3-1-1: Testing Setup

- Proposals and Observations:

  - Proposal 1 (Nokia):

    - RAN4 to consider L3 filtering and multi-beam scenarios when studying test cases for AI/ML mobility uses cases.

  - Proposal 2 (ZTE):

    - RAN4 to discuss numbers of Tx beam/SSBs to be configured and the number of cell to be measured for AI/ML mobility.

  - Proposal 3 (Ericsson):

    - Test setup for testing the AI/ML model deployed at UE to support intra-cell temporal prediction scenario is based on two-cell setup.

    - Existing test setup defined to validate requirements for inter-frequency measurements can be re-used or used as a baseline for discussion on testing AI/ML based mobility for scenario 3.

      - Cell 1 in the test can be considered the cell for measurement.

      - Cell 2 in the test can be considered the target cell for measurement prediction.

  - Proposal 4 (CATT):

    - RAN4 configures one SSB per cell and the measured RSRP results are regarded as cell-level RSRPs for FR1 test.

    - AWGN or TDL channel can be used in FR1 conducted test.

  - Proposal 5 (vivo):

    - RAN4 to consider evaluate the impact of model input dimensions (e.g., the input Tx beam number for inference is not equal to the input Tx beam number for training because of the test setup) for the performance.

- Candidate Options:

    - Option 1: Study how legacy testing setup for L3 measurements can be reused

    - Option 2: Consider multi-beam scenarios

    - Option 3: Consider AI/ML BM testing setup as baseline for FR2 (Scenario 3)

- Recommended WF

    - To discuss above listed candidate options.

#### Issue 3-1-2: Prediction consistency in time domain

- Proposals and Observations:

  - Proposal 1 (Nokia):

    - RAN4 should also study how to model different time-varying characteristics per cell/site due to moving UE trajectories, and how to model different variations per test iteration.

  - Proposal 2 (Apple):

    - For temporal domain prediction, to investigate whether legacy test setups can serve as a baseline and how to introduce controlled randomness and time-domain variation/correlation to validate testing objectives comparing predicted versus actual outcomes.

  - Proposal 3 (ZTE):

    - RAN4 should consider whether it is reliable to use the measurement results from the previous moment to infer the results from another moment under constantly changing channel conditions. Maybe some side conditions are needed.

  - Proposal 4 (Qualcomm):

    - In order to ensure data sanity in AI/ML model training and inference and consistency between data set and measurement configuration, RAN4 should review the performance assessment assumptions and configurations when discussing the testability aspect.

  - Proposal 5 (CATT):

    - TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes, which makes ground truth known to TE. The range of TX power can be FFS.

- Candidate Options:

    - Option 1: Model different time-varying characteristics per cell/site due to moving UE trajectories

    - Option 2: how to introduce controlled randomness and time-domain variation/correlation

    - Option 3: TE randomly adjusts the transmitting power of SSBs from all cells to model time-variant RSRP changes

    - Option 4: Consider additional side conditions

- Recommended WF

    - To discuss above listed candidate options.

#### Issue 3-1-3: Considering multiple cells in the testing setup

- Proposals and Observations:

  - Proposal 1 (Nokia):

    - For AI/ML mobility, RAN4 should consider different NW side additional conditions (i.e., NW-related parameters) for multiple target cells/sites in the same or different frequency layers

  - Proposal 2 (Xiaomi):

    - For the scenario of FR1 to FR1 inter-frequency inter-cell frequency domain scenario, multi-cell setup in the test need to be further studied.

  - Proposal 3 (CATT):

    - The following values can be a baseline for the number of configured cells in FR1 HO performance test:

      - [9] cells for intra-frequency predictions;

      - [5] cells for inter-frequency predictions;

      - And values of carrier frequencies can be determined later.

    - For measurement reduction test, less cells can be set up in test, e.g., [3] cells.

  - Proposal 4 (Samsung):

    - RAN4 to consider the following testability aspects for AI mobility

      - Consideration of serving and neighbour cells

        - Multi-cell setup and number of cells to measure

- Candidate Options

    - Option 1: multi-cell setup in the test need to be further studied for inter-frequency predictions

    - Option 2: Consider at least two cells: serving cell and another/targe cell for measurement prediction

    - Option 3: [9] cells for intra-frequency predictions; [5] cells for inter-frequency predictions. For measurement reduction test, less cells can be set up in test, e.g., [3] cells.

- Recommended WF

    - To discuss above listed candidate options.

#### Issue 3-1-4: Ground Truth derivation

- Proposals and Observations:

  - Proposal 1 (CMCC, CATT):

    - for FR1 AI/ML mobility, it is proposed that the ground truth is the transmit power settled by TE.

  - Proposal 2 (Ericsson):

    - RAN4 to discuss considerations for deriving the ground truth value for validation of the accuracy of predicted measurements.

  - Proposal 3 (Qualcomm):

    - RAN4 to further study the impact of time-varying channels and SNR due to fading and UE mobility on the definition of expected RRM measurement result.

    - Regarding the ground truth of AI/ML-based RRM prediction, either the actual measurement report or the expected value calculated based on the test configuration, if FR1, would be feasible.

- Candidate Options

    - Option 1: the ground truth is the transmit power settled by TE, i.e., ground truth is known by the TE.

    - Options 2: RAN4 to discuss considerations for deriving the ground truth value for validation of the accuracy of predicted measurements.

    - Option 3: other

- Recommended WF

    - To discuss above listed candidate options.

### Sub-topic 3-2: Testing goals and related aspects

Sub-topic description:

In this sub-topic the company proposals about the testability and interoperability aspects for AI/ML Mobility use cases are summarized.

#### Issue 3-2-1: Testing goal

- Proposals and Observations:

  - Proposal 1 (ZTE):

    - it’s necessary to ensure the performance compared to legacy measurement and the testing goal is to verify whether the minimum performance of AI/ML Mobility can be achieved.

  - Proposal 2 (Qualcomm):

    - RAN4 to not consider the use of KPIs defined and used for assessing AI/ML-based mobility performance gains in RAN2

  - Proposal 3 (CMCC):

    - it is proposed to reuse testing goal definition from NR_AIML_Air WI. In detail, the testing goal is to verify whether the minimum performance of AI/ML functionality/feature can be achieved.

  - Proposal 4 (Samsung):

    - RAN4 would better to clarify the testing property/aim and methodology when test AI/ML functionalities for each AI/ML mobility use case and sub-use case. The alternatives are as follows.

      - RRM measurement prediction

        - 1st goal: To verify the trade-off between the measurement reduction and accuracy performance degradation, the legacy performance is as baseline

          - Performance gain: The measurement reduction should keep the mobility performance from degrading

        - 2nd goal: To verify whether the HO performance gain of AI/ML model can be obtained

  - Proposal 5 (Nokia):

    - RAN4 should have requirements for minimum performance and UE needs to pass at least one test for specific sub-set of applicable configurations (matching functionalities) as part of conformance testing.

    - A minimum set of test configurations (matching Associated ID(s)) shall be defined as mandatory for testing of AI/ML Mobility.

  - Proposal 6 (CATT):

    - For scenario 2 and 3, RAN4 to verify whether measurement results satisfy the existing accuracy requirements under certain side conditions, e.g., MRRT or MRRS.

    - For scenario 4, RAN4 further discusses how to define HO performance metric.

  - Proposal 7 (Ericsson):

    - The aim of testing for intra-cell temporal domain prediction should be to validate that the measurements reported by the UE for each cell in the test setup meet:

      - requirement for the measurements performed inside the OW,

      - requirement for the measurements predicted for the time instances inside the PW.

    - The aim of testing for inter-cell (inter-frequency) scenario should be to validate that the measurements reported by the UE:

      - meet the requirements for measured cell, and

      - meet the requirements for the target cell.

- Candidate options:

  - Option 1: RAN4 should consider reusing reuse testing goal definition from NR_AIML_Air WI. Any change in the definition in NR_AIML_Air WI will be reflected in AI/ML Mobility as well.

  - Option 2: RAN4 should define a new testing goal definition based on proposals above.

- Recommended WF

  - Discuss different options listed above.

#### Issue 3-2-2: Principle of FR2 testing

- Proposals and Observations:

  - Proposal 1 (vivo):

    - RAN4 to consider OTA testing for FR2 (scenario 4) and using one probe to emulate different Tx beams by applying different power for AI mobility FR2 beam level test for one cell.

  - Proposal 2 (Qualcomm):

    - Regarding the uncertainty due to the UE Rx beam gain and spherical coverage in FR2 AI/ML-based RRM prediction, RAN4 to wait and see the outcome of the simulation campaign from AI/ML-based BM and try to leverage the results and observations to the extent possible

  - Proposal 3 (Xiaomi, Samsung):

    - For FR2 measurement prediction, conclusions on testability that RAN4 made for AI/ML BM can be taken as a baseline.

  - Proposal 4 (Nokia):

    - For FR2, the under-study testing setup in AI/ML BM use case should be reused as a baseline for measurement prediction use case testing.

    - RAN4 should consider new aspects for FR2 testing in AI/ML mobility such as channel model, number of AoAs, maximum number of cells and Tx beams and number of probes required in the test chamber.

  - Proposal 5 (CATT):

    - RAN4 to postpone discussion on test set up in FR2 until sufficient progress is made in AI/ML BM.

  - Proposal 6 (OPPO):

    - For FR2 prediction, proposals and conclusions that RAN4 made for AI BM can be reused, e.g., whether the chamber in OTA testing can be utilized and how to utilize/revise it.

- Recommended WF

  - For AI/ML Mobility testing in FR2, use testing setup under consideration for AI/ML BM use case as a baseline.

#### Issue 3-2-3: Testing of Generalization aspects

- Proposals and Observations:

  - Proposal 1 (Apple):

    - For Rel-19 AI/ML mobility, it is essential for RAN4 to investigate generalization-related tests and requirements arising from varying propagation conditions, SNR levels, and cell-specific Tx/codebook implementations across different cells and frequency layers.

  - Proposal 2 (ZTE):

    - Generalization discussion in RAN4 can be started if sufficient progress in RAN2 or NR_AIML_Air WI.

- Recommended WF

  - Discuss above listed proposals as options.

#### Issue 3-2-4: Testing aspects of inter-carrier scenarios

- Proposals and Observations:

  - Proposal 1 (OPPO):

    - For inter-carrier scenario (inter-frequency inter-cell prediction), how to ensure/mimic that the cell to be measured and the cell to be predicted are located in the same sector of either the serving site or the same neighbouring site should be considered.

    - For inter-carrier scenario (inter-frequency inter-cell prediction), how to construct and obtain different cell’s RSRP corresponding to a specific temporal relationship should be considered.

      - Stable time-domain RSRP signal for the tests might be needed, e.g., configure DL signals of BS emulator with stable power which do not change with time

    - For inter-carrier scenario (inter-frequency inter-cell prediction), to ensure the testability of measurements for inter-cell prediction, channel modelling for inter-cell prediction should be considered.

      - For FR1, considering the test environment is conducted testing, RAN4 should re-evaluate the selection of channel models. Option 1: UMA channel, Option 2: CDL channel.

  - Proposal 2 (Ericsson):

    - The aim of testing for inter-cell (inter-frequency) scenario should be to validate that the measurements reported by the UE:

      - meet the requirements for measured cell, and

      - meet the requirements for the target cell.

  - Proposal 3 (Nokia):

    - RAN4 should study how to ensure consistency between difference carrier components in the context of AI/ML mobility SI.

  - Proposal 4 (CATT):

    - For inter-carrier scenario (inter-frequency inter-cell prediction), consider colocation scenario only and the characteristics of colocation can be reflected by a minor RSRP difference among carriers in FR1 test.

- Recommended WF

  - Above listed proposals need to be further discussed.