3GPP TSG-RAN WG4 Meeting #113	R4-2418294

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.15.2.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on FR2-1 SSB based L3 measurement delay reduction by optimizing Rx beam sweeping factor

Document for:	Discussion

- Introduction

During last meeting, RAN4 reached some agreements, but there are still many open items and it is summarized in WF [1]. This document focus on the beam sweeping factor reduction and proposes possible enhancements.

- Discussion

  - Conditions to quit optimizing Rx BSF

In the last meeting, companies proposed what should be the conditions to quit optimizing Rx BSF. Since proposals are quite diverse, it needs more discussion to reach the agreement. The summary is copied below.

Issue 1-1-2: Conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF” : - Option 1 (CATT, LGE, OPPO, MTK): use UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling. - Option 1a(LGE, NTT DCM, OPPO): For power saving for operating multi-Rx simultaneous reception, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. - Option 1b(OPPO): Excepting network configuration for FBS, UE assistance information can be beneficial, and new UAI indication is feasible and it should be up to RAN2 decision. - Option 1c(MTK): RAN4 to discuss UE indication capability to the NW whenever UE requires to deactivate multi-Rx for FR2-1 SSB based L3 measurement delay reduction (e.g., indication due to overheating resulting from activating multiple panels for long time). - Option 2 (Samsung): - Introduce NW based solution to enable the fallback for UE FBS based quick L3 measurement when it is off - Dynamic indication; Semi-static configuration - Option 3 (Samsung, Ericsson): enable FBS in a time period after FBS is triggered by the condition in issue 1-1-1 - Option 3a (Samsung): Introduce validity timer configuration with certain ranges via dedicated RRC signalling - Option 3b (Ericsson): NW indicates UE enabling FBS through L3 or lower layers signaling. To save power consumption, after receiving the indication, UE may enable FBS in a predefined time period or for predefined measurement samples. After that, the UE is allowed to disable FBS or fall back to normal beam sweeping. - Option 4 (Samsung): - Introduce/Define TTT parameters for UE using FBS (i.e., N=[reduced number]) based L3 measurement event - Option 5 (Samsung): - UE autonomously fallback based on measurement - Option 6 (HW): - For scenario 1/2/3/4, UE can switch back to normal measurement (i.e. single panel activated) when UE has reported qualified results. - Option 7 (Intel, QC): - Power consumption is not an issue in the scope since the total power consumption for a handover stays roughly the same even delay is reduced. (Intel) - Other explicit signaling (e.g. FR2 CHO, FR HO, GBBR, etc.) may disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed. (QC) [Moderator]: try to discuss the following options(wording can be revised/polished) and companies can add more details if needed. For conditions to trigger UE to quit “L3 measurement delay reduction by optimizing Rx BSF”: - For Scenario 1/2/3/4, - Option 1 (CATT, LGE, OPPO, MTK): use UE indication of Multi-Rx preference for L3 measurement and further discuss whether to reuse the existing signaling. - FFS on option 1a/1b/1c. - Option 3 (Samsung, Ericsson): enable FBS in a time period after FBS is triggered by the condition in issue 1-1-1 - FFS on option 3a/3b. - Option 5 (Samsung): UE autonomously fallback based on measurement - For Scenario 5/7, - FFS: disallow the fallback of UE Rx beam sweeping factor to the existing N value until the signaled configuration is no longer in effect or the relevant task has been completed.

As same as other issues, this is also considered case by case, thus we are basically fine with moderator’s suggestion. For scenario 1/2/3/4, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. However, the actual trigger to quit optimizing Rx BSF should be done by NW. UE autonomous quitting should be avoided. For scenario 5/7, it should be disallowed that the fallback of UE Rx beam sweeping factor to the existing N value until the signalled configuration is no longer in effect or the relevant task has been completed.

Proposal 1: For scenario 1/2/3/4, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. However, the actual trigger to quit optimizing Rx BSF should be done by NW. UE autonomous quitting should be avoided. For scenario 5/7, it should be disallowed that the fallback of UE Rx beam sweeping factor to the existing N value until the signalled configuration is no longer in effect or the relevant task has been completed.

  - Searcher assumption

In the last meeting, assumption of number of searchers occupied by optimizing beam sweeping factor did not reach agreement. The summary is copied below.

Issue 1-1-3: FFS: RRM measurement with two panels activated, two searchers are occupied by this single carrier - Option 1 (CATT, Apple, Samsung, ZTE, QC, NTT DCM): For Multi-Rx based L3 measurement, 1 searcher is assumed for single carrier. - Option 2 (vivo, LGE, OPPO): For Multi-Rx based L3 measurement, 2 searcher is assumed for single carrier. - Option 3 (CTC): Searcher occupied by single carrier is depended on UE implementation, 1 searcher or 2 searchers can be occupied by the single carrier. - Option 4 (HW): RAN4 to skip the discussion about the searcher assumption to support FR2-1 L3 measurement delay by optimizing Rx beam sweeping factor. And whether UE can support this feature can be considered with UE capability discussion. [Moderator]: FFS on option 1 and option 2.

Since the existing beam sweeping factor is not decided based on the number of searchers, the optimization can be achieved irrelevant to the number of searchers. Thus legacy assumption, i.e. one searcher for single carrier should be used.

Proposal 2: Consider one searcher for single carrier even if beam sweeping factor is optimized.

  - SSB processing delay

In the last meeting, SSB processing delay/time was discussed. The summary is copied below.

Issue 1-1-4: FFS: SSB processing delay/time for processing multiple beams received in a SMTC - Option 1 (CATT, Apple, CTC, Nokia): if option 1 is adopted in issue 1-1-3, no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 1a (CTC, Intel): UE baseband processing capabilities need to be considered. - Option 1b (Nokia): - If additional time for SSB processing is needed when UE is measuring multiple beams in one SMTC, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms - Option 2 (vivo): No need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity. - Option 3 (NTT DCM):SSB processing delay/time for this BSF optimization should be same as the legacy requirements because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB. - Option 4 (QC): Proposal 3: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - With above proposal, FR2 – FR2 HO timeline takes the following form (changes from legacy HO timeline is shown in blue): - Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + M * Tmargin ms - Where, - M is the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - Tmargin is time for SSB post-processing. Tmargin can be up to 2ms. - Tsearch is the time required to search the target cell when the handover command is received by the UE. If the target cell is a known cell, then Tsearch = 0 ms. If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)* Trs ms. If the target cell is an unknown inter-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = ceil(N/M)*3* Trs ms. N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. Proposal 4: SSB processing time does not need to be introduced to handle optimized Rx BSF in following scenarios. - Scenario 1: SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 2: SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra - Scenario 3: SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 4: SSB based Inter-frequency measurement with MG, including TPSS/SSS_sync_inter, TSSB_time_index_inter and TSSB_measurement_period_inter - Scenario 7: RRC Re-establishment/RRC Connection Release with Redirection. - Option 5 (Ericsson): - Reflecting different assumptions of searcher number, processing time is specified differently for such two configurations. - For single carrier configuration in multi-Rx reception, extra processing time=0. - For CA/DC configuration in multi-Rx reception, if applicable, extra processing time could be reviewed after completing single carrier requirements. - For single carrier configuration, no extra processing time. For CA/DC configuration, if applicable, RAN4 to check the necessity of extra processing time for handover interruption time. - No extra processing time for cell identification delay. [Moderator]: FFS the following summarized options. (CA/DC is low priority, and therefore CA/DC based options are not reflected below): - For scenario 1/2/3/4 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity, RAN4 to consider measurement delay with SSB processing as - a. Tidentify_intra_without_index = (TPSS/SSS_sync_intra + T SSB_measurement_period_intra+TSSB_processing) ms - b. Tidentify_inter_without_index = (TPSS/SSS_sync_inter + T SSB_measurement_period_inter+TSSB_processing) ms - c. where TSSB_processing = 2 ms - Option 2: no need to add additional processing time in the delay requirement. - For scenario 5 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: In the handover delay equation, SSB processing time (2 ms) can be scaled by the number of RX beams that UE employs to simultaneously receive signals from multiple directions. - For scenario 7 - Option 1: no need to add additional processing time due to multiple SSBs within one SMTC, but to consider an additional processing delay after the last SMTC periodicity - Option 2: no need to add additional processing time in the delay requirement.

The scenario discussed in this item is UE beam sweeping factor reduction by utilising two panels. As clearly stated in the WID, the objective is Rx beam sweeping factor reduction for FR2-1 SSB based L3 measurement with simultaneous reception on single carrier. Therefore measured SSB is only one at one time instance and UE uses two panels to measure it, i.e. UE simultaneously measures two spatial signal quality for one SSB. Thus, SSB processing delay/time for this BSF optimization should be same as the legacy requirements. The case that multiple SSBs within one SMTC is not needed to be considered.

Observation 1: The objective this item is Rx beam sweeping factor reduction for FR2-1 SSB based L3 measurement with simultaneous reception on single carrier. Therefore UE measures only one SSB at one time instance using two panels simultaneously.

Observation 2: Based on the UE behaviour, the case that multiple SSBs within one SMTC is not needed to be considered.

Proposal 3: SSB processing delay/time for this BSF optimization should be same as the legacy requirements for all the cases because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB.

  - DRX configuration

The applicability of optimizing Rx BSF with DRX is still under discussions. The summary is copied as follows.

Issue 1-1-5: FFS: L3 measurement delay reduction by optimizing Rx BSF with/without DRX - Option 1 (Xiaomi): The SSB based L3 measurement delay reduction based on FBS can be dependent with DRX configuration. - Option 2 (CATT, CMCC, Samsung, CTC, NTT DCM, Nokia): L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case. - Option 3 (Ericsson): - L3 measurement enhancement is applicable to UE configured with DRX. It can be studied if some DRX cases really impact performance. - When L3 measurement enhancement is enabled, DRX can be ignored during the fast L3 measurement from tech. perspective. [Moderator]: Is option 2 agreeable?

Since current L3 measurement delay already covers DRX cases, it should be considered as well. No technical reasons to prioritize/deprioritize DRX case.

Proposal 4: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

  - Solutions to apply/specify L3 measurement delay reduction by optimizing Rx BSF

In the last meeting, solutions to apply/specify L3 measurement delay reduction was discussed. The summary is copied below.

- Option 1 (Xiaomi): - In order to shorten the overall L3 measurements delay, the smaller RX beam sweeping factor for SSB index acquiring and SSB measurement can be used in comparison with that for PSS/SSS detection. - Option 2 (CATT, vivo, CMCC, Apple, ZTE, CTC, NTT DCM, Nokia, OPPO, MTK): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced according to the UE capability - Option 2a (CATT, CMCC, ZTE, CTC): For UE supporting Multi-Rx operation for L3 measurement, the Rx BSF can be reduced to 2, 4 or 6 according to the UE capability. - Option 2b (vivo): Introduce the UE capability to indicate the BSF value, the value needs to be discussed under a common assumption for the sample value, for example, sample = 4. - Option 2c (Apple, CTC): For UE supporting R19 multiple-Rx simultaneous reception, it is proposed to reduce L3 measurement delay by reducing Rx BSF, and Rel-18 Rx BSF reduction in L1 measurement can be used as baseline. - Option 2d (Nokia): At least support reduction of N from 8 to 4 in Rel-19. Support of other values such as N = 2, 6 to be discussed, and UE capability signalling to be introduced, if more than one value for reduced N is supported. - Option 2e (OPPO): Discuss the reduced scaling factors due to Rx BSF, considering both UE capabilities and applicable conditions. - Option 3 (vivo, Samsung, LGE): Introduce the UE capability to indicate the reduced M value (M=sample*BSF), - Option 3a (vivo): e.g., M=12, 14, 16, etc. - Option 3b (Samsung): suggest to take N=4, M=20 as the baseline. Whether to/How to define the other values can be based on SLS results (if necessary). - Option 3c (LGE): 3 or 4 Rx BSF value or additional scaling factor < 1 to reduce M value - Option 4 (vivo, LGE): - Introduce the UE capability to indicate the fraction value for reduction, e.g., 1/2, 1/3, etc. - Option 5 (Ericsson): - In order to cover all possible directions of neighbor cells, RX beam sweep doesn’t dynamically change, the factor can be defined as a single number, e.g., [4]. - Option 6 (Intel): - Different (or whether or not) delay reduction applies when the ratio of number of SSB within a burst and time duration of the measurement periodicity varies. - Beam sweeping factor/total sample number of measurements can be cut half as a baseline while a margin can be added to the cut factor according to different SSB/SMTC configurations. - Recommended WF - [Moderator]: discuss option 2 and option 3 together with the scenarios 1/2/3/4/5/7.

As many companies supported, the Rx BSF should be reduced according to the UE capability. For each scenario, the affected value is described as follows.

    - Scenario 1

The scenario 1 is SSB based Intra-frequency measurement without MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra. In TS 38.133, the measurement periods contain following scaling factors and they implicitly contain 8 as beam sweeping factor. More precisely, for PC3 UE, 48 = 8*6 and 24 = 8*3.

Mpss/sss_sync_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_w/o_gaps =40. For a UE supporting power class 2, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_w/o_gaps = 36. Mmeas_period_w/o_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_w/o_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_w/o_gaps =24. For a UE supporting power class 4, Mmeas_period_w/o_gaps =24. For a UE supporting power class 6, Mmeas_period_w/o_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_w/o_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_w/o_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_w/o_gaps = 36. MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72 samples. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48 samples. For a UE supporting FR2 power class 3, MSSB_index_intra = 48 samples.

Observation 3: For scenario 1, Mpss/sss_sync_w/o_gaps, Mmeas_period_w/o_gaps, MSSB_index_intra are affected.

    - Scenario 2

The scenario 2 is SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_intra and TSSB_measurement_period_intra. In TS 38.133, the measurement periods contain following scaling factors and they implicitly contain 8 as beam sweeping factor. More precisely, for PC3 UE, 48 = 8*6 and 24 = 8*3.

Mpss/sss_sync_with_gaps : For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync with_gaps=40. For a UE supporting FR2-1 power class 2, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 3, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 4, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-1 power class 6, Mpss/sss_sync with_gaps =24. For a UE supporting FR2-2 power class 1, Mpss/sss_sync with_gaps = 60. For a UE supporting FR2-2 power class 2, Mpss/sss_sync with_gaps = 36. For a UE supporting FR2-2 power class 3, Mpss/sss_sync with_gaps = 36. Mmeas_period_ with_gaps: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_ with_gaps =40. For a UE supporting FR2-1 power class 2, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 3, Mmeas_period_ with_gaps =24. For a UE supporting FR2-1 power class 4, Mmeas_period with_gaps =24. For a UE supporting FR2-1 power class 6, Mmeas_period with_gaps =24. For a UE supporting FR2-2 power class 1, Mmeas_period_ with_gaps = 60. For a UE supporting FR2-2 power class 2, Mmeas_period_ with_gaps = 36. For a UE supporting FR2-2 power class 3, Mmeas_period_ with_gaps = 36. - MSSB_index_intra: For a UE supporting FR2-2 power class 1, MSSB_index_intra = 72. For a UE supporting FR2-2 power class 2, MSSB_index_intra = 48. For a UE supporting FR2 power class 3, MSSB_index_intra = 48.

Observation 4: For scenario 2, Mpss/sss_sync_with_gaps, Mmeas_period_with_gaps, MSSB_index_intra are affected.

    - Scenario 3 and 4

The scenario 3 is SSB based Inter-frequency measurement without MG, including TPSS/SSS_sync_inter and TSSB_measurement_period_inter. The scenario 4 is SSB based Intra-frequency measurement with MG, including TPSS/SSS_sync_inter and TSSB_measurement_period_inter. In TS 38.133, the measurement periods contain following scaling factors and they implicitly contain 8 as beam sweeping factor. More precisely, for PC3 UE, 40 = 8*5 and 24 = 8*3.

Mpss/sss_sync_inter: For a UE supporting FR2-1 power class 1 or 5, Mpss/sss_sync_inter = 64 samples. For a UE supporting FR2-1 power class 2, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 3, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-1 power class 4, Mpss/sss_sync_inter = 40 samples. For a UE supporting FR2-2 power class 1, Mpss/sss_sync_inter = 96. For a UE supporting FR2-2 power class 2, Mpss/sss_sync_inter = 60. For a UE supporting FR2-2 power class 3, Mpss/sss_sync_inter = 60. MSSB_index_inter: For a UE supporting FR2-1 power class 1 or 5, MSSB_index_inter = 40 samples. For a UE supporting FR2 power class 2, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 3, MSSB_index_inter = 24 samples. For a UE supporting FR2-1 power class 4, MSSB_index_inter = 24 samples. For a UE supporting FR2-2 power class 2 or 3, MSSB_index_inter = 48 samples. For a UE supporting FR2 power class 1, MSSB_index_inter = 72 samples. Mmeas_period_inter: For a UE supporting FR2-1 power class 1 or 5, Mmeas_period_inter =64. For a UE supporting FR2-1 power class 2, Mmeas_period_inter=40. For a UE supporting FR2-1 power class 3, Mmeas_period_inter =40. For a UE supporting FR2-1 power class 4, Mmeas_period_inter = 40. For a UE supporting FR2-2 power class 1, Mmeas_period_inter = 96. For a UE supporting FR2-2 power class 2, Mmeas_period_inter = 60. For a UE supporting FR2-2 power class 3, Mmeas_period_inter = 60.

Observation 5: For scenario 3 and 4, Mpss/sss_sync_inter, Mmeas_period_inter, MSSB_index_inter are affected.

    - Scenario 5

The scenario 5 is handover. In TS 38.133, the interruption time for NR FR2-NR FR2 Handover and NR FR1-NR FR2 Handover contain beam sweeping factor N.

6.1.1.4 NR FR2- NR FR2 Handover […] 6.1.1.4.2 Interruption time The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When intra-frequency or inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - If the target cell is a known intra-frequency cell, then Tsearch = 0ms. - If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. […] Where N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2. […] 6.1.1.5 NR FR1- NR FR2 Handover […] 6.1.1.5.2 Interruption time The interruption time is the time between end of the last TTI containing the RRC command on the old PDSCH and the time the UE starts transmission of the new PRACH, excluding the RRC procedure delay. When in inter-frequency handover is commanded, the interruption time shall be less than Tinterrupt Tinterrupt = Tsearch + TIU + Tprocessing + T∆ + Tmargin ms Where: Tsearch is the time required to search the target cell when the handover command is received by the UE. Regardless of whether DRX is in use by the UE, Tsearch shall still be based on non-DRX target cell search times. - If the target cell is a known intra-frequency cell, then Tsearch = 0ms. - If the target cell is an unknown intra-frequency cell and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. […] Where N = 8 when the target cell is in FR2-1, and N = 12 when the target cell is in FR2-2.

Observation 6: For scenario 5, the beam sweeping factor N in Tsearch is affected.

    - Scenario 7

The scenario 7 is RRC Re-establishment/RRC Connection Release with Redirection. In TS 38.133, Time to identify target NR cell implicitly contains beam sweeping factor. The value 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively and 8 is beam sweeping factor [2].

6.2.1 SA: RRC Re-establishment […] Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. T Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. […] 6.2.3 SA: RRC Connection Release with Redirection […] Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell..

Observation 7: For scenario 7, Tidentify_intra_NR, Tidentify_inter_NR, Tidentify-NR are affected.

Based on above observations, the affected factors are summarized in the table below.

Proposal 5: For each scenario, the highlighted factors should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

- Conclusion

In this contribution, we propose possible enhancements.

Proposal 1: For scenario 1/2/3/4, Rel-18 UAI ‘multiRx-PreferenceFR2’ for power saving can be considered as starting point. However, the actual trigger to quit optimizing Rx BSF should be done by NW. UE autonomous quitting should be avoided. For scenario 5/7, it should be disallowed that the fallback of UE Rx beam sweeping factor to the existing N value until the signalled configuration is no longer in effect or the relevant task has been completed.

Proposal 2: Consider one searcher for single carrier even if beam sweeping factor is optimized.

Observation 1: The objective this item is Rx beam sweeping factor reduction for FR2-1 SSB based L3 measurement with simultaneous reception on single carrier. Therefore UE measures only one SSB at one time instance using two panels simultaneously.

Observation 2: Based on the UE behaviour, the case that multiple SSBs within one SMTC is not needed to be considered.

Proposal 3: SSB processing delay/time for this BSF optimization should be same as the legacy requirements for all the cases because the BSF optimization should be achieved by simultaneously measurement of two spatial signal quality for one SSB.

Proposal 4: L3 measurement delay reduction based on BSF reduction cover both non-DRX and DRX (BSF reduction is independent of DRX status), no need to prioritize/deprioritize DRX case.

Observation 3: For scenario 1, Mpss/sss_sync_w/o_gaps, Mmeas_period_w/o_gaps, MSSB_index_intra are affected.

Observation 4: For scenario 2, Mpss/sss_sync_with_gaps, Mmeas_period_with_gaps, MSSB_index_intra are affected.

Observation 5: For scenario 3 and 4, Mpss/sss_sync_inter, Mmeas_period_inter, MSSB_index_inter are affected.

Observation 6: For scenario 5, the beam sweeping factor N in Tsearch is affected.

Observation 7: For scenario 7, Tidentify_intra_NR, Tidentify_inter_NR, Tidentify-NR are affected.

Proposal 5: For each scenario, the highlighted factors should be replaced with capability value.

| Scenarios | Factors |
| --- | --- |
| 1 | Mpss/sss_sync_w/o_gaps =24 = 8*3 Mmeas_period_w/o_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 2 | Mpss/sss_sync_with_gaps =24 = 8*3 Mmeas_period_with_gaps =24 = 8*3 MSSB_index_intra = 48 = 8*6 |
| 3 | Mpss/sss_sync_inter = 40 = 8*5 Mmeas_period_inter =40 = 8*5 MSSB_index_inter = 24 = 8*3 |
| 4 | Same as scenario 3 |
| 5 | If the target cell is unknown, and the target cell Es/Iot≥-2 dB, then Tsearch = N* Trs ms. Where N = 8 when the target cell is in FR2-1. |
| 7 | RRC Re-establishment 80, 3520, 4000 come from 8*10, 8*22*20, 8*25*20 respectively Table 6.2.1.2.1-1: Time to identify target NR cell for RRC connection re-establishment to NR intra-frequency cell Serving cell; FR of target NR; Tidentify_intra_NR [ms]. SSB Ês/Iot (dB); cell; Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 5 x TSMTC); MAX (800 ms, 10 x TSMTC). ≥ -8; FR2-1; N/A; MAX (1000 ms, 80 x TSMTC)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 120 x TSMTC)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 3520Note1. < -8; FR2-2; N/A; 5280Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC > 20 ms and serving cell SSB Ês/Iot < -8 dB.. Table 6.2.1.2.1-2: Time to identify target NR cell for RRC connection re-establishment to NR inter-frequency cell Serving cell SSB Ês/Iot (dB); FR of target NR cell; Tidentify_inter_NR, i [ms]. Known NR cell; Unknown NR cell. ≥ -8; FR1; MAX (200 ms, 6 x TSMTC, i); MAX (800 ms, 13 x TSMTC, i). ≥ -8; FR2-1; N/A; MAX (1000 ms, 104 x TSMTC, i)). ≥ -8; FR2-2; N/A; MAX (1000 ms, 156 x TSMTC, i)). < -8; FR1; N/A; 800Note1. < -8; FR2-1; N/A; 4000Note1. < -8; FR2-2; N/A; 6000 Note1. Note 1: The UE is not required to successfully identify a cell on any NR frequency layer when TSMTC,i > 20 ms and serving cell SSB Ês/Iot < -8 dB.. RRC Connection Release with Redirection Table 6.2.3.2.1-1: Time to identify target NR cell for RRC connection release with redirection to NR FR of target NR cell; Tidentify-NR. FR1; MAX (680 ms, 11 x Trs). FR2-1; MAX (880 ms, 8x11 x Trs). FR2-2; MAX (880 ms, 12x11 x Trs). Note: If the UE has been provided with higher layer signaling of smtc2 specified in TS 38.331 [2] prior to the redirection command, Trs follows smtc1 or smtc2 according to the physical cell ID of the target cell.. |

References

[1] 3GPP, R4-2416853, “WF on RRM requirements for NR_RRM_Ph5_Part1”, Apple

[2] 3GPP, R4-1811709, “RRC Re-establishment Requirements without Serving Cell Timing”, Ericsson, Mediatek