3GPP TSG-RAN WG4 Meeting #113	R4-2418295

Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.17.1

Source: 		NTT DOCOMO, INC.

Title: 	Discussions on general aspects of AI/ML for NR Air Interface

Document for:	Discussion

- Introduction

During the past meetings, general aspects has been discussed and several agreements were obtained. This paper further discusses remaining issues in the summary [1].

- Discussion

  - Performance requirement for CSI prediction

In previous meeting, the definition of AI/ML performance was discussed and direction was agreed as follows.

Issue 1-4: AI/ML Performance Agreement: - For CSI prediction, AI/ML based performance requirement should not be worse than the legacy (non-AI/ML based) if a relevant equivalent legacy requirement/feature exists. - FFS which legacy requirement/feature are relevant - FFS whether AI/ML based performance should be better than the legacy - FFS on target for AI/ML based beam management

For CSI prediction, legacy PMI reporting requirements is the relevant equivalent requirement, i.e. the requirement should be defined at the relative throughput γ. The numerator is the throughput with predicted CSI, the denominator should be the throughput with random precoding, and minimum requirement should be the same as that of current specifications.

Proposal 1: For CSI prediction, legacy PMI reporting requirements is the relevant equivalent requirement.

Proposal 2: The requirement should be defined at the relative throughput γ. The numerator is the throughput with predicted CSI, the denominator should be the throughput with random precoding, and minimum requirement should be the same as that of current specifications.

  - Post deployment testing

The WF for post deployment testing in the last meeting is copied below.

Issue 1-2: Post deployment testing When operating in the field, two aspects of UE operation may impact performance: - Update or fine tuning or addition/removal of models, if applicable, resulting in a change of functionality which may impact functionality performance - Data drift / mismatch between the conditions encountered by the UE in the field and the training data, which may impact functionality performance. For dealing with potential changes in the performance of functionalities two options may be available. The options are not mutually exclusive: - Option 1: Conduct the validation of a change in AI model/functionality before its deployment/activation in already deployed UEs - Validation takes into account the UE hardware in which the model is to be deployed/activated. - FFS whether the validation takes place in the field (i.e., on each individual device) or in the lab conditions (i.e., per device type/model) - FFS whether validation can be done at the device along with inference for another active functionality(ies) - FFS on the feasibility - FFS on one possibility for consideration for option 1 is to capture model input (and if needed other test data such as ground truth) during conformance testing. This stored data can later be used to validate new or updated models. - Other aspects not precluded. - Option 2: Using performance monitoring and LCM procedures - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - FFS on monitoring can be used for managing fallback, changes in functionality, model update/model switching/model transfer, if applicable FFS: RAN4 needs to clarify whether changed functionalities that did not pass conformance testing are expected to be activated at the device. For dealing with drift / mismatch between the conditions encountered by the UE in the field and the training data for the model, which may impact functionality performance, monitoring is needed. - Performance monitoring will be designed in other groups - RAN4 may consider the need and feasibility of requirements and tests to ensure consistency and accuracy of monitoring metrics or other monitoring related data sent from the UE, and set requirements as feasible/needed. - Monitoring can be used for managing fallback, change in functionality, model update/model switching/model transfer, if applicable

According to this agreement, the direction is same as previous discussion. One is conducting as a conformance test, the other is conducting as a performance monitoring. During last meeting, one possible solution was raised, which is hybrid testing. For minor AI model functionality update, performance monitoring is used. For major AI model functionality update, conformance test is used. In order to apply this hybrid testing, followings shall be defined.

- Definition of minor and major update

  - Minor update is identical with the part of LCM, i.e. functionality/model monitoring, functionality/model transfer, and functionality/model selection, activation, deactivation, switching, and fallback operation.

  - Major update is the update of the functionality/model itself (model structure, hyper parameters, etc.)

- Concrete procedure of minor update in performance monitoring

  - According to the agreement, this will be designed in other groups

- When and how to conduct major update by conformance test

  - It is up to functionality/model provider.

Summarizing above, the proposal of post deployment testing is as follows.

Proposal 3: Post deployment testing is constructed with both conformance test and performance monitoring. For minor AI model functionality update, performance monitoring is used. For major AI model functionality update, conformance test is used.

- Minor update is identical with the part of LCM, i.e. functionality/model monitoring, functionality/model transfer, and functionality/model selection, activation, deactivation, switching, and fallback operation.

  - Concrete procedure of minor update in performance monitoring will be designed in other groups.

  - At least activation, deactivation, switching, and fallback delay are needed to be specified.

- Major update is the update of the functionality/model itself (model structure, hyper parameters, etc.)

  - Major update timing is up to functionality/model provider.

  - After passing conformance test, the updated functionality/model is allowed to be transferred.

- Conclusion

In this contribution, we proposed our views on general aspects of AI/ML for NR Air Interface.

Proposal 1: For CSI prediction, legacy PMI reporting requirements is the relevant equivalent requirement.

Proposal 2: The requirement should be defined at the relative throughput γ. The numerator is the throughput with predicted CSI, the denominator should be the throughput with random precoding, and minimum requirement should be the same as that of current specifications.

Proposal 3: Post deployment testing is constructed with both conformance test and performance monitoring. For minor AI model functionality update, performance monitoring is used. For major AI model functionality update, conformance test is used.

- Minor update is identical with the part of LCM, i.e. functionality/model monitoring, functionality/model transfer, and functionality/model selection, activation, deactivation, switching, and fallback operation.

  - Concrete procedure of minor update in performance monitoring will be designed in other groups.

  - At least activation, deactivation, switching, and fallback delay are needed to be specified.

- Major update is the update of the functionality/model itself (model structure, hyper parameters, etc.)

  - Major update timing is up to functionality/model provider.

  - After passing conformance test, the updated functionality/model is allowed to be transferred.

References

[1] 3GPP, R4-2417212, “WF on requirements for AI/ML air interface”, Qualcomm Incorporated