3GPP TSG-RAN WG4 Meeting # 113	R4-2418303

# Orlando, USA, 18th - 22th, Nov 2024

Source: 	Samsung

Title: 	Discussion on UE and NW behavior for non-collocated deployment

Agenda Item:	7.5.2.2

Document for:	Discussion

- Introduction

In RAN#103, a new WID on Non-collocated deployment is approved [1] which focus on specifying UE RF/RRM requirements for Type-4a/b UE capability and clarifying the UE behaviour for Type-4a/b supported UE.

Discussion was organized in RAN4#112bis and the agreements were captured in [2] including UE capability and UE type definition etc. We discuss the remaining issues in this paper, mainly on NW signalling associated with the UE behavior. (This paper is basically a re-submission.)

(Note that Type-1/2/3/4 in Rel-18 discussion are intended for different UE capabilities, rather than UE type.)

- Discussion

- 2.1 UE behavior

We discuss UE behavior from two aspects:

- UE initial configuration/behavior on collocated or non-collocated deployment, at the initial connection mode

- UE switching behavior between collocated and non-collocated deployment

- 2.2.1 UE switching behavior between Type-4 and Type1/Type-2/single carrier

Our thinking is illustrated in the following figure assuming UE is configured on Type-4 mode at this moment. The figure shows all the possible “switching path” from Type-4 mode to either Type-2 mode or Type-1 mode or single carrier operation. In our understanding, probably there is no need to argue which path is the most desirable one with maximum benefit, but rather focus on making everything possible and under NW full control.

(Please note that in Rel-18 design, it points to Type-2 when “nonCollocatedTypeMRDC” or “nonCollocatedTypeNR-CA” is “absent”, rather than when they are “present”.)

More explanation of above figure,

- From Type-4 to Type-2: As both Type-4 and Type-2 are non-collocated with same RF requirements and most likely same RRM requirements as well, there is no new NW signaling needed as the deployment scenario does not change, UE can just do MIMO layer re-config to become 2 layer per CC(Type-2). →No new signaling needed was agreed/confirmed in RAN4#112.

- From Type-4 to Type-1: This is from non-collocated to collocated scenario. Collocated can be either up to 4-layer/per CC or 8 layer/per CC or 2-layer per CC. We think 4 layer/CC can be the baseline for collocated Type-1 in this path, and if NW want UE to have 8-layer or 2-layer per CC, it can be achieved by further MIMO layer re-config. In our understanding new NW signaling is needed here, owing to the pre-condition/restriction of the usage of the Rel-18 NW signaling which is only present for a UE configured with maxMIMO-layers≤2. Therefore, the exiting NW IE (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) cannot be used to instruct the Type-4 to Type-1 switch because Type-4 can support up to 4 layer per CC. More details of the Rel-19 new signaling would be shared in next subclause. In addition, technically, this path maybe the least likely one, as there is no strong justification for a FWA to do switch between collocated and non-collocated. →To be further discussed in this meeting.

- From Type-4 to single carrier operation: Achieving by Scell release, and Pcell remains, which is a legacy approach. →No new signaling needed was agreed/confirmed in RAN4#112bis.

Proposal 1: New NW signaling needed between Type-4 and Type-1 switch. (more details of New NW signaling is present in Proposal 4)

- 2.2.2 At initial connection mode

We discuss the “switching behavior” in last clause, and the other case to be discussed is for the initial connection mode. How to configure the UE working under either collocated scenario or non-collocated scenario at the beginning.

In Rel-18, the definition of Rel-18 NW signaling is copied as below for reference. It is worth nothing that the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2 which is a pre-condition/restriction of the usage as mentioned in last subclause. And if maxMIMO-layers ＞2, it is collocated.

nonCollocatedTypeNR-CA This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD intra-band NR-CA. If this field is present, the UE applies MRTD according to Table 7.6.4-1 in TS 38.133 [14] and UE RF requirements for intra-band NR-CA except for 7.10A in TS 38.101-1 [15]. If this field is absent, the UE applies MTTD/MRTD requirements according to Table 7.5.4-1/Table 7.6.4-2 in TS 38.133 [14] and UE RF requirements for intra-band non-collocated NR-CA including 7.10A in TS 38.101-1 [15].

nonCollocatedTypeMRDC This field is only present for a UE configured with maxMIMO-Layers with value less than or equal to 2 for all corresponding serving cells, in case of TDD-TDD inter-band (NG) EN-DC with overlapping or partially overlapping bands. If this field is present, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.3/7.6.3 in TS 38.133 [14] and inter-band RF requirements. If this field is absent, the UE applies (NG)EN-DC MTTD/MRTD according to clause 7.5.2/7.6.2 in TS 38.133 [14] and inter-band RF requirements.

Observation 1: In Rel-18, the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2. And if maxMIMO-layers＞2, it is collocated.

In Rel-19, we think same methodology can be followed in RAN2 NW signaling design. More specific,

1) New Rel-19 NW signaling is utilized to differentiate Type-4 and Type-1 with the condition/restriction it is only present when 2＜maxMIMO-layers ≤4

2) If maxMIMO-layers ＞4, it is collocated

Proposal 2: New Rel-19 NW signaling is needed to differentiate Type-4 and Type-1 when 2＜maxMIMO-layers ≤4.

Our initial design/thinking is something like the following, in a simple “present-absent” way.

For the initial connection,

1）maxMIMO-layers＞4, it is collocated

2）maxMIMO-layers=4, it could be either collocated (Type1) or non-collocated(Type-4), pending on whether new NW signaling is present

3）maxMIMO-layers=2, it could be either collocated (Type 1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

While during the offline discussion among companies before submission deadline, there are two good counter-examples provided by MTK, which demonstrates the above Rel-19 NW signaling design is not good enough. More specific, if Rel-19 NW signaling is designed in a simple “present-absent” way, we would face the following issues, and the Rel-18 argument would repeat.

- Rel-19 type 4 capable UE behavior operate under Rel-18 network

[Samsung] Assuming a Rel-19 UE indicates both Type-4 and Type-2, then Rel-18 NW only understand Type-2 and use maxMIMO-layers=4 to control UE work under collocated if following above design, but Rel-19 UE would see "new Rel-19 NW signaling =absent + maxMIMO-layers=4", then this UE misthink Non-collocated Type-4. Discrepancy between UE and gNB happens.

(Note in our understanding a UE has no idea of the NW is of which release, and also a NW has no idea of the UE is of which release)

- Only type 2 capable Rel-19 UE behavior under Rel-19 network.

[Samsung] Seems we can just follow Rel-18 design in this case? If Type-4 is not reported, it implies this UE does not understand new Rel-19 NW signaling. So for collocated, Rel-19 NW just use " maxMIMO-layers=4". If this is common understanding, seems there would be no issue. But this needs to be clarified.

To well resolve above issues, it is proposed to have the Rel-19 NW signaling design in the “absent, present with value=0, present with value=1” way:

Proposal 3: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling and Rel-18 NW signaling:

1）maxMIMO-layers ＞4, it is collocated

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, it could be either collocated(Type-1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

More specific, “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

- 4. Conclusion

Proposal 1: New NW signaling needed between Type-4 and Type-1 switch. (more details of New NW signaling is present in Proposal 4)

Observation 1: In Rel-18, the Rel-18 NW signaling design is not like “If absent is non-collocated, if present is collocated ”, but rather the NW signaling is only present for a UE configured with maxMIMO-layers ≤2. And if maxMIMO-layers＞2, it is collocated.

Proposal 2: New Rel-19 NW signaling is needed to differentiate Type-4 and Type-1 when 2＜maxMIMO-layers ≤4.

Proposal 3: Confirm the following understanding for the usage of maxMIMO-layers and the new Rel-19 NW signaling and Rel-18 NW signaling:

1）maxMIMO-layers ＞4, it is collocated

2）maxMIMO-layers=4, if Rel-19 new BS signaling IE is present, it could be either collocated(Type-1) or non-collocated(Type-4) pending on value is 0 or 1; If Rel-19 new BS signaling IE is absent, UE follows Rel-18 specification.

3）maxMIMO-layers=2, it could be either collocated(Type-1) or non-collocated(Type-2), pending on whether existing NW signaling (nonCollocatedTypeNR-CA or nonCollocatedTypeMRDC) is present

Note: “If Rel-19 BS signaling IE is absent” intends for legacy NW or Rel-19 NW without Type-4 feature, “If Rel-19 BS signaling IE is present” intends for Rel-19 NW with Type-4 feature.

- 5. Reference

[1] RP-240838, R19 WID_Intra-band non-collocated EN-DC and NR-CA, RAN#103

[2] R4-2417115, WF on rest issues for intra-band non-collocated EN-DC,NR-CA, KDDI, RAN4#113