3GPP TSG-RAN WG4 Meeting #113	R4-2418338

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on UE RF requirements for LP-WUR

Source:	Samsung

Agenda item:	7.23.2.2

Document for:	Discussion

1.	Introduction

In RAN4#112bis meeting, the LP-WUS UE RF performance metric is confirmed and baseline architecture was agreed. Based on the agreed performance metric and baseline architecture, next REFSENS derivation will be one of the most important work [1, R4-2417112]:

Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR Agreements: - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed. - Encourage companies to also input the justification for each element.

In this contribution, we share our views on REFSENS derivation including each element.

2. 	Discussion

In the beginning of this work item RAN4 has agreed to adopt legacy equation for REFSENS derivation

REFSENS=-174 dBm+10*logBW+NF+SNR+IM,

So, the REFSENS requirement for LP-WUR will be determined by 3 elements: NF, SNR and IM. On the other hand, the LP-WUS coverage target is also expected to be considered. But the two approaches could not be combined. In addition, the channel model assumption between RAN1 and RAN4 are different (TDL vs AWGN, repetition vs no repetition), the SNR value in RAN1 could not be directly used for the REFSENS derivation. So far RAN4 has agreed on the performance metric (1% MDR and <=1% FAR) and baseline architecture (zero-IF), it is suggested to move on with the agreed RAN4 assumption and legacy equation approach to derive the REFSENS requirements and check the coverage target later before finally confirm the REFSENS value.

Proposal 1:	RAN4 to derive the REFSENS requirement for LP-WUR with the agreed RAN4 assumption and legacy equation approach firstly, and check the coverage target later before finally confirm the REFSENS value.

For NF, the outcome of TR 38.869 records the noise figure values as follows, it can be seen NF=15 is the typical value among different sources.

Table 7.1.1a-3: Relative power consumption and noise figure for OOK-1/2/4 with homodyne/zero-IF architecture with baseband envelope detection

| Source reference | [7A-1] | [7A-2] | [7A-3] | [7A-4] | [7A-5] | [7A-6] | [7A-7] | [7A-8] | [7A-9] | [7A-10] |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Power consumption (ON state) | 0.09 for single-branch, 0.01 or 0.02 for each additional branch | 0.5 | 0.1~1 | 0.1 | 0.05~ 0.5 | 0.5~1 | 0.1~0.5 | 4 | ~1 | 0.1~0.5 |
| Noise figure (dB) | 15 | 10~15 | [10-16] | 15 | 12 | 15 | 12~15 | 15 | ~15 | 12 |

For SNR, previous simulation [2, R4-2415780] based on RAN4 assumption shows that OOK-4, M=4, payload = 16bit with envelope-based receiver is the worst case and SNR= -2 dB would cover all the cases.

For IM, there has been two typical views, one is the implementation margin should be smaller than NR since the LP-WUR is a separated simple receiver, another is the implementation margin can just reuse the IM value for NR which is 2.5dB. We’d like to highlight the impact of antenna sharing between LR and MR to the IM value for REFSENS. Due to antenna sharing between LR and MR, the antenna switch component and long path from antenna to LP-WUS receiver will lead to larger insertion loss compared with MR. Given the IM for MR in NR legacy REFSENS is 2.5dB, it is reasonable that the IM for LR REFSENS should be no smaller than 3dB.

Based on above considerations, we have following proposals for the initial REFSENS value

Proposal 2:	RAN4 to derive the REFSENS of LP-WUR following the elements in below table

|  | NF (dB) | SNR (dB) | IM (dB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- |
| Envelop receiver | 15 | -2 | 3 | 10*logBW - 158 |

3. 	Conclusion

Proposal 1:	RAN4 to derive the REFSENS requirement for LP-WUR with the agreed RAN4 assumption and legacy equation approach firstly, and check the coverage target later before finally confirm the REFSENS value.

Proposal 2:	RAN4 to derive the REFSENS of LP-WUR following the elements in below table

|  | NF (dB) | SNR (dB) | IM (dB) | REFSENS (dBm) |
| --- | --- | --- | --- | --- |
| Envelop receiver | 15 | -2 | 3 | 10*logBW - 158 |

4. 	References

- R4-2417112    “WF on UE RF requirements for LP-WUS”, vivo

- R4-2415780    “Further discussions on LP-WUS REFSENS, ASCS and ACS”, vivo