3GPP TSG-RAN WG4 Meeting #113	R4-2418339

# Orlando, US, 18th – 22nd November, 2024

Title:	Discussion on testability for LP-WUR

Source:	Samsung

Agenda item:	7.23.2.4

Document for:	Discussion

1.	Introduction

Testability is one of the scopes for this work item but no much progress yet, in RAN4#112 meeting it was agreed to discuss the high-level approach on LP-WUS performance verification [1]:

Sub-topic 4-1 Testability for UE RF requirements Issue 4-1-1: General framework on LP-WUS testing Agreements: - RAN4 further discuss the high-level approach on how to achieve LP-WUS performance verification.

In RAN4#112bis meeting, most of the testability issue are FFS, including the necessity of a UE test mode

Issue 4-1-5: UE test mode for LP-WUR Agreements: - FFS A UE test mode for LP-WUR is needed.

In this contribution, we share our views on LP-WUR testability.

2. 	Discussion

In last meeting it was agreed that 1% MDR shall apply for both RF requirement and performance test. There was once proposal to adopt 1% MDR as requirement but 5% or 10% MDR for conformance test to save test time. Given 1% MDR has been agreed also for conformance test, a UE test mode to save test time is becoming more necessary.

Observation 1:	the agreement of 1% MDR for conformance test requires a UE test mode to save test time.

In addition, the MR wakeup can be triggered not only by LP-WUS signal but also by other side conditions. In RF test cases it is expected that the MR is only waken up by LP-WUS signal. Therefore, adopting test mode for LP-WUR RF testing not only saves test time, but also disable the other possibilities of MR wakeup except by LP-WUS signal.

Observation 2:	adopting test mode for LP-WUR RF testing not only saves test time, but also disable the other possibilities of MR wakeup except by LP-WUS signal.

Moreover, the REFSENS test cases of LP-WUR is likely to be more than REFSENS of NR, e.g., REFSENS of NR is only conducted under RRC connected mode, but REFSENS of LP-WUR is probably conducted in both idle mode and/or connected mode. So, test time consideration for LP-WUR testing is important.

Previously there were proposals to conduct the MDR test only in RRC connected mode to avoid the introduction of new UE test mode, however, depending on UE capability, a UE may only support LP-WUS in RRC Idle mode as the Rel-18 shows promising power saving gain in Idle mode than in connected mode.

Observation 3:	It is not feasible to test MDR in RRC connected mode for UEs only supporting LP-WUS in RRC Idle mode.

Based on above discussions, we have the following proposals as the high-level approach on how to achieve LP-WUS performance verification.

Proposal 1:	RAN4 to adopt UE test mode for LP-WUR RF testing for the sake of test time saving and test robustness.

One more issue is about whether RAN5 involvement is needed now

Issue 4-1-6: whether a LS to RAN5 on test issue Way forward: FFS

According to the WID, it is RAN4 objective to study the testability of LP-WUR RF requirements. RAN5 has no WI and no TU to handle such request. In our understanding, for a grand new feature, RAN4 takes the main role to study the testability, and when the outcome is steady RAN5 will take over for the conformance test design. LP-WUR as a grand new feature, RAN4 is the right place to discuss the testability. A LS to RAN5 is only needed when a very specific RAN5 related issue or question needs RAN5 expertise. Right now, we do not see such necessity.

Proposal 2:	RAN4 should focus on the study of LP-WUR testability as WID required, and a LS to RAN5 is not needed unless a specific RAN5 related issue or question needs RAN5 expertise.

3. 	Conclusion

Observation 1:	the agreement of 1% MDR for conformance test requires a UE test mode to save test time.

Observation 2:	adopting test mode for LP-WUR RF testing not only saves test time, but also disable the other possibilities of MR wakeup except by LP-WUS signal.

Observation 3:	It is not feasible to test MDR in RRC connected mode for UEs only supporting LP-WUS in RRC Idle mode.

Proposal 1:	RAN4 to adopt UE test mode for LP-WUR RF testing for the sake of test time saving and test robustness.

Proposal 2:	RAN4 should focus on the study of LP-WUR testability as WID required, and a LS to RAN5 is not needed unless a specific RAN5 related issue or question needs RAN5 expertise.

4. 	References

- R4-2414309    “WF on LP-WUS UE RF requirements”, vivo

- R4-2417112    “WF on UE RF requirements for LP-WUS”, vivo