3GPP TSG-RAN WG4 Meeting #113	R4-2418340

# Orlando, US, 18th – 22nd November, 2024

Title:	On OTA performance metric for handheld NTN UE

Source:	Samsung

Agenda item:	7.12.2.2

Document for:	Discussion

1.	Introduction

For the performance metric of NTN UE OTA testing, there have been many discussion in previous meetings and the trend to convergence is shown in last meeting [1, R4-2416590]:

Issue 3-2-1: Performance metric for NTN device Agreements: - For NR-NTN UEs, take a whole sphere measurement with TRP and TRS as the performance metrics for all usage modes as baseline. Other performance metric can be considered if issue is identified. - For IoT-NTN UEs, FFS performance metric.

A full sphere TRP TRS is taken as baseline performance metric for handheld NR-NTN UE, and further discussion is expected for IoT NTN. In this contribution, we share our viewpoints on the OTA performance metric for handheld NTN UE.

2. 	Discussion

It is common understanding that performance metric should be aligned with usage scenario. In last RAN4 meeting, the usage scenario of NR-NTN UE was agreed to be close to normal NR UE firstly, and then the baseline performance metric full sphere TRP TRS was agreed afterwards [1].

Issue 3-1-2: Check whether the UE orientation is arbitrary or directional during NR-NTN communication Agreements: - For NR-NTN, the target usage of NR-NTN UE is close to normal NR UE.

For the performance metric of IoT NTN UEs, we should also study the target usage scenario firstly. Note that we have agreed in previous meeting that handheld UE type is prioritized for IoT NTN and the same prioritized usage scenario as NR-NTN handheld UE [2, R4-2413603]:

Issue 3-1-4: UE type for IoT-NTN Agreements: - For IoT-NTN test method development, RAN4 can consider handheld UE type as 1st priority. Issue 3-1-5: Usage scenarios for IoT-NTN handheld UE Agreements: - Same prioritized usage scenarios as NR-NTN handheld UE

Observation 1:	for IoT-NTN handheld UE, same prioritized usage scenarios as NR-NTN handheld UE have been already agreed previously.

As mentioned in the paper from operator [3, R4-2416392], “with future higher power constellations, NB-IoT link budget would be significantly better than NR”; moreover, the narrow band properties will make the link budget for NB-IoT further better than NR. From link budget perspective, NB-IoT UEs are more applicable for a full sphere performance metric to guarantee the paging and reselection function for handheld UE in any case.

Observation 2:	From link budget perspective, NB-IoT UEs are more applicable for a full sphere performance metric to guarantee the paging and reselection function for handheld UE in any case.

In addition, dual-mode devices that adopt both NB-IoT NTN and NR NRN will share the same antenna, it will be difficult to optimize the antenna design if different performance metric are required for NB-IoT NTN and NR NRN.

Observation 3:	different performance metric for NB-IoT NTN and NR NRN respectively is not friendly for IoT&NR NTN dual-mode devices.

Based on above observations, it seems better to adopt the same performance metric for IoT NTN handheld UE as that of NR NTN handheld UE.

Proposal 1:	For both IoT NTN handheld UE and NR NTN handheld UE, RAN4 to adopt full sphere TRP TRS as performance metric.

Another issue regarding performance metric is whether performance metric should be different for GEO and NEGO for handheld UE [1].

Issue 3-2-2: Whether performance metric should be different for GEO and NGEO for handheld UEs Agreements: - FFS whether different metrics for GEO and NGEO would be needed.

This issue has been kept open for many meeting cycles and no conclusion yet. Based on above discussion and proposal 1, a full sphere TRP TRS as baseline performance metric for handheld UE indicates that there is no need to differentiate between GEO and NGEO.

Proposal 2:	For handheld NTN UE (both NR NTN and IoT NTN), the performance metric should be the same for GEO and NEGO.

3. 	Conclusion

Observation 1:	for IoT-NTN handheld UE, same prioritized usage scenarios as NR-NTN handheld UE have been already agreed previously.

Observation 2:	From link budget perspective, NB-IoT UEs are more applicable for a full sphere performance metric to guarantee the paging and reselection function for handheld UE in any case.

Observation 3:	different performance metric for NB-IoT NTN and NR NRN respectively is not friendly for IoT&NR NTN dual-mode devices.

Proposal 1:	For both IoT NTN handheld UE and NR NTN handheld UE, RAN4 to adopt full sphere TRP TRS as performance metric.

Proposal 2:	For handheld NTN UE (both NR NTN and IoT NTN), the performance metric should be the same for GEO and NEGO.

4. 	References

- R4-2416590    “Way Forward for [112bis][323] TRP_TRS_Ph3”, vivo

- R4-2413603    “WF for [112][333] TRP_TRS_MIMO_OTA”, vivo

- R4-2416392    “Operator views on Usage Modes for NTN Smartphones”, Inmarsat, Viasat