# 3GPP TSG-RAN WG4 Meeting #113	R4-2418361

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters for NTN less than 5MHz

# Agenda item:	7.8.3.1

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on enhanced requirements and conductive test methodology for NR NTN and IoT NTN with the following RAN4 objective. In this contribution, we want to share some initial views about less than 5MHz for NTN from system parameters perspective.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

# 2 Discussion

Issue 1-1: Whether 3MHz channel bandwidth for NR-NTN in FR1-NTN bands is mandatory or optional? - Way forward: - Option 1: Mandatory - Option 2: Optional Issue 1-2: Clarification on maximum transmission bandwidth configuration - Agreement: - Follow the previous agreement in WF R4-2413528 as: Issue 1-1-3: Maximum transmission bandwidth configuration(NRB) - Agreement - Reuse 15 RBs for 3MHz channel bandwidth with 15kHz SCS for NR NTN.. Issue 1-3: Signalling of SS raster - Way forward: - Option 1: The exiting signalling of support3MHz-ChannelBW-Symmetric-r18 and support3MHz-ChannelBW-Asymmetric-r18 are reused for NTN operations. - Option 2: Defer the UE capability discussion for 3MHz in FR1-NTN after all the requirements are clear. - Option 3: Synchronization raster shall not be signaled for 3MHz channel bandwidth for NR-NTN.

The above are agreements and way forward in last meeting. The remaining issue are whether 3MHz channel bandwidth is optional or mandatory for NR-NTN and whether signalling introduced for NR less than 5MHz in Rel-18 can be reused. For the first remaining issue, the UE capability of supporting 3MHz channel bandwidth introduced in Rel-18 is per band and optional. However, it can be mandatory from Rel-19 based on operators’ need.

Proposal 1: 3MHz channel bandwidth can be mandatory from Rel-19 based on operators’ need.

The following are signallings introduced for NR less than 5MHz in Rel-18. The former three features are all per band and optional with capability signaling. The latter two features are per UE, which are defined specially for band n100. From our understanding, there is no corner use case of 12 RB and 20 RB transmission bandwidth, so there is no need to support the latter two features in NR-NTN. Considering “add less than 5MHz channel bandwidth, 3MHz with the RAN1 design in Rel-18” in the objective and operators’ needs, the existing signallings of support12PRB-CORESET0-r18, support3MHz-ChannelBW-Asymmetric-r18 and support3MHz-ChannelBW-Symmetric- r18 should be supported for NR-NTN. However, in current TS 38.331 spec, these signallings are only related to TS 38.101-1, thus, some updates are needed to these UE features.

Proposal 2: The existing signallings of support12PRB-CORESET0-r18, support3MHz-ChannelBW-Asymmetric-r18 and support3MHz-ChannelBW-Symmetric-r18 are reused for NR-NTN.

Proposal 3: In current TS 38.331 spec, these signallings for less than 5MHz channel bandwidth are only related to TS 38.101-1, thus, some updates are needed for these UE features.

| Definitions for parameters | Per | M | FDD- TDD DIFF | FR1- FR2 DIFF |
| --- | --- | --- | --- | --- |
| support12PRB-CORESET0-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block that is located according to Table 5.4.3.1-2 in TS 38.101-1 [2]. A UE supporting this feature shall also indicate support of support-3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15kHz SCS only. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 12 PRB BWP operation. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Asymmetric-r18 Indicates whether the UE supports 3 MHz channel bandwidth in uplink with larger than 3 MHz channel BW in DL, including short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and applies to bands where the UE indicates support for asymmetricBandwidthCombinationSet with 3 MHz UL according to clause 5.3.6 of TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE 1: The UE supporting this feature supports configuration of 15 PRB UL BWP operation. NOTE 2: If the UE indicates support in asymmetricBandwidthCombinationSet for a 3MHz UL in a band according to clause 5.3.6 of 38.101-1 [2], this feature shall be indicated for the band. | Band | No | FDD only | FR1 only |
| support3MHz-ChannelBW-Symmetric-r18 Indicates whether the UE supports 3 MHz symmetric channel bandwidth in DL and UL, including the following functional components: - Reception of 12 PRB PBCH based on RB-level puncturing; - Short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS; - Reception of 15 PRB CORESET0. This feature is supported for 15kHz SCS only. It is applicable only to single-carrier operation and when an associated SS/PBCH block is located according to Table 5.4.3.3-2 in TS 38.101-1 [2]. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this capability supports configuration of 15 PRB BWP operation in DL and UL. | Band | No | FDD only | FR1 only |
| support5MHz-ChannelBW-20PRB-CORESET0-r18 Indicates whether the UE supports short RACH preamble formats with 15kHz SCS, and long PRACH formats with 1.25kHz SCS, and the reception of 20 PRB CORESET0. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41638 of Table 5.4.3.1-3 in TS 38.101-1 [2]. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. NOTE: The UE supporting this feature supports configuration of 20 PRB BWP operation. | UE | No | FDD only | FR1 only |
| support12PRB-CORESET0-GSCN-41637-r18 Indicates whether the UE supports reception of 12 PRB CORESET0 with an associated SS/PBCH block located at GSCN 41637. A UE supporting this feature shall also indicate support of support3MHz-ChannelBW-Symmetric-r18. This feature is supported for 15 kHz SCS only. This feature is only applicable when an associated SS/PBCH block is located in band n100 at GSCN 41637 of Table 5.4.3.1-3 in TS 38.101-1 [2]. NOTE: The UE supporting this FG supports configuration of 12 PRB BWP operation. This feature is only applicable to single-carrier operation. This feature is not applicable to UEs indicating supportOfRedCap-r17 or supportOfERedCap-r18. | UE | No | FDD only | FR1 only |

# 3 Conclusion

In this contribution, we shared some views on system parameters for NTN less than 5MHz and the proposals are made as follows:

Proposal 1: 3MHz channel bandwidth can be mandatory from Rel-19 based on operators’ need.

Proposal 2: The existing signallings of support12PRB-CORESET0-r18, support3MHz-ChannelBW-Asymmetric-r18 and support3MHz-ChannelBW-Symmetric-r18 are reused for NR-NTN.

Proposal 3: In current TS 38.331 spec, these signallings for less than 5MHz channel bandwidth are only related to TS 38.101-1, thus, some updates are needed for these UE features.

# 4 Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, Huawei