# 3GPP TSG-RAN WG4 Meeting #113	R4-2418362

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on UE RF requirements for NTN less than 5MHz

# Agenda item:	7.8.3.2

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on enhanced requirements and conductive test methodology for NR NTN and IoT NTN with the following RAN4 objective. In this contribution, we want to share some initial views about less than 5MHz for NTN from UE RF requirements perspective.

Less than 5MHz for NTN - Specify the system parameters for channel bandwidth of less than 5 MHz for NR-NTN (NR based Non-Terrestrial Networks) in FR1-NTN bands - Consider the NR-NTN bands n255, n256, and n254 - Add less than 5MHz channel bandwidth - 3MHz with the RAN1 design in Rel-18 - Investigate and if necessary, specify the additional synchronization (sync) raster(s) for the additional channel bandwidth - Leverage the work on less than 5MHz for TN - Note: no RAN1 impact is expected - Specify the necessary SAN RF core requirements for NR-NTN in FR1-NTN bands - Specify the necessary NR-NTN UE RF core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Focus on power class 3 (PC3) for Tx RF requirements - Specify the necessary RRM core requirements for less than 5 MHz for NR-NTN in FR1-NTN bands - Specify the necessary signaling to support the above objectives

# 2 Discussion

- UE Tx requirements

Issue 2-1-3: A-MPR - Way forward - RAN4 to evaluate A-MPR for 3MHz channel BW for NS_02N, NS_03N, NS_04N, NS_05N and NS_24; Issue 2-1-9: General spurious emission - Way forward - FFS Issue 2-1-10: Additional spurious emissions - Way forward - Option 1: New requirements need define for NS_02N, 03N, 04N, 05N - Option 2: For the additional spurious emissions requirements, add 3 MHz CBW into - Table 6.5.3.3.2-1: Additional requirements for "NS_02N"; - Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N"; - Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N". - Add Table 6.5.3.3.5-1 Additional out-of-band requirements for "NS_24" - Option 3: Introduce 3 MHz channel bandwidth for all network signalling values

- A-MPR

For NS_01 and NS_02N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, but no A-MPR is needed to be defined. For NS_03N, NS_04N and NS_05N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, and A-MPR requirements for 3 MHz channel bandwidth need to be defined for each NS value in the corresponding clause. For NS_24, 3 MHz channel bandwidth also needs to be introduced. But, the corresponding clause for NS_24 is specified in clause 6.2.3.15 in TS 38.101-1. Hence, it is better specify A-MPR requirements for 3 MHz channel bandwidth for band n256 in a new clause in TS 38.101-5.

Proposal 1:

- For NS_01 and NS_02N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, but no A-MPR is needed to be defined.

- For NS_03N, NS_04N and NS_05N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, and A-MPR requirements for 3 MHz channel bandwidth need to be defined for each NS value in the corresponding clause.

- For NS_24, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1. Specify A-MPR requirements for 3 MHz channel bandwidth for band n256 in a new clause 6.2.3.X in TS 38.101-5.

- General spurious emission

- For boundary between NR out of band and general spurious emission domain, 6MHz can be reused for NTN less than 5MHz.

Proposal 2: For boundary between NR out of band and general spurious emission domain, 6MHz can be reused for NTN less than 5MHz.

- Additional spurious emissions

- For the additional spurious emissions requirements, add 3 MHz CBW into:

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N";

- Add Table 6.5.3.3.5-1: Additional out-of-band requirements for "NS_24".

- Proposal 3: For the additional spurious emissions requirements, add 3 MHz CBW into:

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N";

- Add Table 6.5.3.3.5-1: Additional out-of-band requirements for "NS_24"

- UE Rx requirements

Issue 2-2-1: Reference sensitivity - Way forward - Option 1: 2.2dB improvement compared with 5 MHz CBW; - Option 2: 1.7dB improvement compared with 5 MHz CBW; Issue 2-2-2: Maximum input level - Way forward - FFS on the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm) Issue 2-2-3: ACS - Way forward - Option 1: Reuse the requirements for 5MHz, 10MHz of NTN FR1 UE. - Option 2: 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset.

- Reference sensitivity

- For REFSENS, we propose to use 2.2dB improvement compared with 5 MHz CBW. REFSENS for 3MHz can be defined as Table 2-1 and uplink configuration for REFSENS can be defined as Table 2-2.

Table 2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -102.2 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | 752 503 |  |
| n255 | 30 |  |  | 24 | 36 | 362 243 | FDD |
|  | 60 |  |  | 10 | 18 | 182 103 |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |  |

- Proposal 4: REFSENS for 3MHz can be defined as Table 2-1 and uplink configuration for REFSENS can be defined as Table 2-2.

Maximum input level

- For maximum input level, reuse the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm).

- Proposal 5: For maximum input level, reuse the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm).

- ACS

- For ACS, 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset, which are shown in the following tables.

Table 7.5-1: ACS for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20 |
| ACS | dB | 33 | 30 | 27 |

Table 7.5-2: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm | REFSENS + 45.5 dB |  | REFSENS + 42.5 dB | REFSENS + 39.5 |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset)2 | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | -71.5 |  | -68.5 | -65.5 |
| Pinterferer | dBm | -40 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

- Proposal 6: For ACS, 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset, which are shown in the following tables.

# 3 Conclusion

In this contribution, we shared some views on UE RF requirements for NTN less than 5MHz and the proposals are made as follows:

Proposal 1:

- For NS_01 and NS_02N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, but no A-MPR is needed to be defined.

- For NS_03N, NS_04N and NS_05N, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1, and A-MPR requirements for 3 MHz channel bandwidth need to be defined for each NS value in the corresponding clause.

- For NS_24, 3 MHz channel bandwidth shall be introduced to Table 6.2.3.1-1. Specify A-MPR requirements for 3 MHz channel bandwidth for band n256 in a new clause 6.2.3.X in TS 38.101-5.

Proposal 2: For boundary between NR out of band and general spurious emission domain, 6MHz can be reused for NTN less than 5MHz.

- Proposal 3: For the additional spurious emissions requirements, add 3 MHz CBW into:

- Table 6.5.3.3.2-1: Additional requirements for "NS_02N";

- Table 6.5.3.3.3-1: Additional out-of-band requirements for "NS_03N";

- Table 6.5.3.3.4-1: Additional out-of-band requirements for "NS_04N" and "NS_05N";

- Add Table 6.5.3.3.5-1: Additional out-of-band requirements for "NS_24"

- Proposal 4: REFSENS for 3MHz can be defined as Table 2-1 and uplink configuration for REFSENS can be defined as Table 2-2.

Table 2-1: Two antenna port reference sensitivity QPSK PREFSENS for FDD bands

| Operating band / SCS / Channel bandwidth |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS kHz | 3 MHz (dBm) | 5 MHz (dBm) | 10 MHz (dBm) | 15 MHz (dBm) | 20 MHz (dBm) | 25 MHz (dBm) | 30 MHz (dBm) | 35 MHz (dBm) | 40 MHz (dBm) | 45 MHz (dBm) | 50 MHz (dBm) |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 | -93.3 |  |  |  |  |  |  |
| n256 | 30 |  |  | -96.6 | -94.6 | -93.5 |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 | -93.7 |  |  |  |  |  |  |
|  | 15 | -102.2 | -100.0 | -96.8 | -95.0 | -93.8 |  |  |  |  |  |  |
| n255 | 30 |  |  | -97.1 | -95.1 | -94.0 |  |  |  |  |  |  |
|  | 60 |  |  | -97.5 | -95.4 | -94.2 |  |  |  |  |  |  |
|  | 15 | -101.7 | -99.5 | -96.3 | -94.5 |  |  |  |  |  |  |  |
| n254 | 30 |  |  | -96.6 | -94.6 |  |  |  |  |  |  |  |
|  | 60 |  |  | -97.0 | -94.9 |  |  |  |  |  |  |  |
| NOTE：The transmitter shall be set to PUMAX as defined in clause 6.2.4 of 3GPP TS 38.101-1 [5]. |  |  |  |  |  |  |  |  |  |  |  |  |

Table 2-2: Uplink configuration for reference sensitivity

| Operating band / SCS (kHz) / Channel bandwidth (MHz) / Duplex mode |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Operating Band | SCS | 3 | 5 | 10 | 15 | 20 | Duplex Mode |
|  | 15 | 15 | 25 | 50 | 75 | 100 |  |
| n256 | 30 |  |  | 24 | 36 | 50 | FDD |
|  | 60 |  |  | 10 | 18 | 24 |  |
|  | 15 | 15 | 25 | 50 | 75 | 752 503 |  |
| n255 | 30 |  |  | 24 | 36 | 362 243 | FDD |
|  | 60 |  |  | 10 | 18 | 182 103 |  |
|  | 15 | 15 | 25 | 50 | 75 |  |  |
| n254 | 30 |  |  | 24 | 36 |  | FDD |
|  | 60 |  |  | 10 | 18 |  |  |
| NOTE: UL resource blocks shall be located as close as possible to the downlink operating band but confined within the transmission bandwidth configuration for the channel bandwidth in Table 5.3.2-1. |  |  |  |  |  |  |  |

- Proposal 5: For maximum input level, reuse the requirements for 5MHz, 10MHz, 15MHz and 20MHz of NTN FR1 UE (-40dBm).

- Proposal 6: For ACS, 3 MHz shall be added to table 7.5-1 into the same column as for 5 and 10 MHz channel BW. For tables 7.5-2 and 7.5-3, 3 MHz channel BW shall also be added but considering a 3 MHz interferer BW and 3/-3 interferer offset, which are shown in the following tables.

Table 7.5-1: ACS for NR satellite bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz

| RX parameter | Units | Channel bandwidth (MHz) |  |  |
| --- | --- | --- | --- | --- |
|  |  | 3, 5, 10 | 15 | 20 |
| ACS | dB | 33 | 30 | 27 |

Table 7.5-2: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 1

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | REFSENS + 14 dB |  |  |  |
| Pinterferer4 | dBm | REFSENS + 45.5 dB |  | REFSENS + 42.5 dB | REFSENS + 39.5 |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset)2 | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 4 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

Table 7.5-3: Test parameters for NR bands with FDL_high < 2700 MHz and FUL_high < 2700 MHz, case 2

| RX parameter | Units | Channel bandwidth (MHz) |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | 3 | 5, 10 | 15 | 20 |
| Power in transmission bandwidth configuration | dBm | -71.5 |  | -68.5 | -65.5 |
| Pinterferer | dBm | -40 |  |  |  |
| BWinterferer | MHz | 3 | 5 |  |  |
| Finterferer (offset) | MHz | 3 /- 3 | BWChannel /2 + 2.5 / -(BWChannel /2 + 2.5) |  |  |
| NOTE 1: The transmitter shall be set to 24 dB below PCMAX_L,f,c at the minimum UL configuration specified in clause 7.3.2 with PCMAX_L,f,c defined in clause 6.2.4. NOTE 2: The absolute value of the interferer offset Finterferer (offset) shall be further adjusted to MHz with SCS the sub-carrier spacing of the wanted signal in MHz. The interferer is an NR signal with 15 kHz SCS. NOTE 3: The interferer consists of the NR interferer RMC specified in 3GPP TS 38.101-1 [5] Annex A.3.2.2 with one sided dynamic OCNG Pattern OP.1 FDD for the DL-signal as described in 3GPP TS 38.101-1 [5] Annex A.5.1.1. NOTE 4: Pinterferer shall be rounded to the next higher 0.5dB value. |  |  |  |  |  |

# 4 Reference

- RP-240857, New WID: Enhanced requirements and test methodology for NR and IoT NTN, Huawei