# 3GPP TSG-RAN WG4 Meeting #113	R4-2418366

# Orlando, US, 18th – 22nd November, 2024

# Source: 	ZTE Corporation, Sanechips

# Title: 	Discussion on system parameters for NR NTN Ku band

# Agenda item:	7.9.3

# Document for:	Approval

# 1 Introduction

In RAN#104 meeting, a new Work Item [1] was approved on introduction of Ku Band for NR NTN for NR NTN with the following RAN4 objective. In this contribution, we want to share some initial views about NR NTN Ku band from system parameters perspective.

The following assumptions are taken as the baseline for this work: - GSO and NGSO - NTN capable UE type 1 to 5 (38.101-5 Table 9.2.1.0-1) for above 10 GHz (no handheld devices) - FDD Mode - GNSS Receiver capability The objectives are: - Use the regulation requirements in ITU Regions 1, 2 and 3 to identify relevant adjacent band co-existence scenarios for NTN Ku Band covering the following frequency ranges, considering targeted deployment scenarios: - For ITU regions 1 and 3: - Downlink 10.70 – 12.75 GHz - Uplink 13.75-14.5 GHz - For ITU regions 2 excluding US: - Downlink 10.70 – 12.70 GHz - Uplink 13.75-14.5 GHz - For ITU regions 1 and 3 - Downlink 10.70 – 12.75 GHz - Uplink 12.75 - 13.25 GHz - For ITU regions 2 excluding US - Downlink 10.70 – 12.70 GHz - Uplink 12.70 - 13.25 GHz - Note: Appropriate TX/Rx separation for uplink downlink pairing will be required. - Conduct coexistence study. Reuse Ka band coexistence study assumptions as much as possible, with modifications where necessary, e.g. for frequency range and subcarrier spacing. Conduct new co-existence studies based on adjacent services . - Specify RF requirements for satellite access node and NTN VSAT types in 38.101-5 also considering existing regulations on antenna sizes for certain parts of the Ku band. - Priority 1: for all ITU regions, define Ku band #1a and #1b for the downlink block and uplink block B to enable the future possibility of supporting satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths based on existing channel bandwidths. Potential approaches for achieving this shall be studied, from which one shall be selected including but not limited to: - Ku band #1a and #1b as FR1-NTN supporting 20 MHz, 35 MHz, 50 MHz, 70 MHz and 100 MHz channel bandwidths with 30 kHz subcarrier spacing - Ku band #1a and #1b as FR2-NTN supporting 50 MHz, 100 MHz, 200 MHz, 400 MHz (120kHz SCS only) channel bandwidths with 120 kHz subcarrier spacing - RAN4 will select which one of the above approaches for DL and UL will be pursued by RAN #106. - Priority 2: for all ITU regions excluding the US in region 2, using the approach in Priority 1, define Ku band #2a and #2b as appropriate for the downlink block and uplink block A to support satellite services in 125 MHz (UL) and 250 MHz (DL) aggregated bandwidths. - Specify RRM requirements Note: Support for NTN CA is not in the scope of this WID

# 2 Discussion

- Sub-topic 3-4: Phase noise target SNR Further analysis on the impact of phase noise on FR1/FR2 numerology selection should assume a maximum of 12 dB SNR and maximum 16QAM modulation. This is just for analysis and is not intended to limit support of higher MCS. - Sub-topic 3-5: Efficient use of spectrum for FR1 – native bandwidths Assume 10 MHz and 25 MHz native bandwidths are included in the analysis of spectrum use efficiency . - Sub-topic 3-12: SCS channel raster SCS-based channel raster is assumed. - Sub-topic 3-13: Flexible TX Rx spacing Flexible TX Rx spacing is assumed. - Sub-topic 3-14: Sync raster The existing sync raster design is reused for Ku band(s).

The above is agreement and way forward we reached in last meeting [2], and we need to continue the feasibility study of FR1 and FR2 numerology. Here we provide some analysis based on the following technical criteria.

- UL timing synchronization

The wider SCS is, the shorter CP is, which in turn diminishes the capacity to accommodate uncertainty in uplink timing. Based on the analysis in [3], it can be observed that the margin left for implementation of NW UL reception to accommodate timing drift, channel delay spread,etc is very limited for 120kHz SCS. Since UL timing synchronization is critical for NTN, the longer CP is helpful for requiring the transmission from all users to arrive within the CP. From UL timing synchronization aspect, FR1 numerology should be selected.

Proposal 1: From UL timing synchronization aspect, FR1 numerology should be selected.

- Phase noise

Phase noise is another important aspect when considering SCS. High frequency range tends to use wider SCS to reduce the impact of phase noise, and the least SCS in FR2 is 60kHz. Since Ku band is a brand new frequency range will be introduced in 3GPP, we need to evaluate whether narrower SCS than 60kHz is feasible for Ku band. In last meeting, we initially compare the demodulation performance between 30kHz, 60kHz and 120kHz through relative throughput simulation with the assumption of 64QAM modulation. Furthermore, way forward in last meeting states that further analysis on the impact of phase noise on FR1/FR2 numerology selection should assume a maximum of 12 dB SNR and maximum 16QAM modulation. Accordingly, the simulation result from our side is shown in Figure 1. The phase noise we selected is based on Example 2 of mmWave SSB phase noise model in TS38.803 with the principle that PN could increase by 6 dB every time when f0 doubles (assuming FoM and other things do not change), which is given in Figure 2. From relative throughput simulation result, it can be observed that 30kHz without PTRS has slightly worse performance than 60kHz with PTRS and 120kHz with PTRS. PTRS also has been introduced for FR1, which may be used to mitigate the impact of phase noise. Consequently, from our understanding, FR1 and FR2 numerology are all feasible for Ku band from the impact of SCS on susceptibility to phase noise aspect.

Proposal 2: FR1 and FR2 numerology are all feasible for Ku band from the impact of SCS on susceptibility to phase noise aspect.

Figure 1: Relative throughput between different SCS      Figure 2: PSD of proposed phase noise model for Ku band

- Efficient channel utilization

According to [3] and [4], the channel bandwidth of existing legacy and advance Ku band satellites in service are 8MHz, 27MHz, 36 MHz, 54 MHz, 72 MHz, 108 MHz, 144 MHz, 216 MHz, 450 MHz and 662 MHz. For satellite channel bandwidth less than 100MHz, using FR1 numerology can maximize channel utilization. For satellite channel bandwidth more than 100MHz, we can use carrier aggregation to achieve flexible deployment. FR2 numerology may cause higher spectrum wastage compared to FR1 numerology.

Proposal 3: From efficient channel utilization, FR1 numerology should be used for Ku band to maximize spectrum utilization.

- Based on the above analysis, we suggest to use FR1 numerology for Ku band from UL timing synchronization, phase noise and efficient channel utilization aspect.

Proposal 4: From UL timing synchronization, phase noise and efficient channel utilization aspect, it is proposed to use FR1 numerology for Ku band.

# 3 Conclusion

In this contribution, we shared some views on system parameters for NTN Ku band and the proposal is made as follows:

- Proposal 1: From UL timing synchronization, phase noise and efficient channel utilization aspect, FR1 numerology is feasible for Ku band.

Proposal 2: FR1 and FR2 numerology are all feasible for Ku band from the impact of SCS on susceptibility to phase noise aspect.

Proposal 3: From efficient channel utilization, FR1 numerology should be used for Ku band to maximize spectrum utilization.

Proposal 4: From UL timing synchronization, phase noise and efficient channel utilization aspect, it is proposed to use FR1 numerology for Ku band.

# 4 Reference

- RP-241690, New WID on Introduction of Ku Band for NR NTN, Intelsat et al.

- R4-2416593, Way Forward for [112bis][313] NR_NTN_Ku_Band_General, Moderator (Eutelsat Group)

- R4-2415436, Discussions on system parameters of Ku Band for NR NTN, MediaTek Inc.