3GPP TSG-RAN WG4 Meeting #113 	R4-2418378

Orlando, US, Nov 18th – Nov 22nd, 2024

Source:	Nokia

Title:	(LTE_NR_DC_enh2-Core) Alignment of RAN4 requirements with RAN2 procedures

Agenda item:	4.5.2

Document for:	Approval

Introduction

This topic has been discussed in several meetings [e.g. in 1, 3, 4, 6, 8, 10, 12]. In RAN4#112 meeting this was discussed as part of legacy maintenance. RAN4 almost reached a conclusion during the meeting. However, one issue was raised related to that there can be more TCI states indicated in the tci-ActivatedConfig. A question was raised which one of the TCI states the UE is to select?

This discussion paper continues the discussion from earlier papers. Additionally, we state our view related to the issue raised in RAN4#112 based on the RAN2 specification.

Notice, we’re proposing that RAN4 solves the misalignment between RAN4 and RAN2 by either of the following two options:

- Option 1: Update the RAN4 UE requirements capturing that tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

- Option 2: If proposal 1 is not directly agreeable, we suggest RAN4 to send an LS to RAN2 clarifying the RAN2 understanding of the applicability of tci-ActivatedConfig.

The very basic issue at hand is that tci-ActivatedConfig has not been accounted fully into the RAN4 requirements. However, from network point of view it is important to know what can be expected from the UE if network does configure tci-ActivatedConfig e.g. together with SCell configuration which includes direct SCell activation.

Currently, UE requirements when the tci-ActivatedConfig is included into the SCell configuration is left open.

In this paper we discuss the applicability issue of tci-ActivatedConfig and in our CR [14] we have a proposal how to capture the requirements (this CR is based on the outcome from the discussions in RAN4#112).

As alternative we have also provided a drat LS to RAN2 in the appendix.

Discussion

tci-ActivatedConfig Configuration

tci-ActivatedConfig is introduced in Rel-17 by RAN2. tci-ActivatedConfig may be configured together with ServingCellConfig [2]:

#### –	ServingCellConfig

The IE ServingCellConfig is used to configure (add or modify) the UE with a serving cell, which may be the SpCell or an SCell of an MCG or SCG. The parameters herein are mostly UE specific but partly also cell specific (e.g. in additionally configured bandwidth parts). Reconfiguration between a PUCCH and PUCCHless SCell is only supported using an SCell release and add.

The detailed parameter description [2]:

The actual tci-ActivatedConfig:

More specifically, looking at the parts that would be of interest for defining the UE requirements:

tci-ActivatedConfig

- If configured for an SCell, or

- if configured for the PSCell when the SCG is being activated upon the reception of the containing message,

  - the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell.

- If configured for the PSCell when the SCG is indicated as deactivated in the containing message:

- the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD.

When this field is absent for the PSCell and the SCG is being deactivated:

- the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

tci-ActivatedConfig and UE requirements

As we see from the RAN2 description, the tci-ActvatedConfig can be configured for:

- an SCell, or

- a PSCell when the SCG is activated upon configuration.

From the RAN2 description it is apparent that the tci-ActvatedConfig can be configured for an SCell unconditionally while for a PSCell tci-ActvatedConfig can be configured only when the PSCell is activated upon configuration. Based on the current RAN2 specification we conclude following:

tci-ActivatedConfig can be configured for any SCell unconditionally.

tci-ActvatedConfig ActvatedConfig can only be configured for a PSCell which is activated when configured.

Hence, from the current RAN2 wording and procedure, tci-ActivatedConfig can be configured for an SCell no matter if the SCell is activated not activated when configured. The tci-ActivatedConfig can be configured for a deactivated SCell as well as for an SCell being configured as activated upon configuration (direct activated).

tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

Currently, the RAN4 specifications are not clear about the UE requirements when tci-ActivatedConfig is configured and included into the ServingCellConfig. The UE requirements for this scenario would need to be clarified.

Update the RAN4 UE requirements capturing the UE requirements when tci-ActivatedConfig is configured for a deactivated SCell and a direct activated SCell.

tci-ActivatedConfig including multiple TCI states

In RAN4#112, one ‘new’ issue was raised related to the fact that the tci-ActivatedConfig can include one or more TCI states indicated in the tci-ActivatedConfig. The question raised was: which one of the TCI states the UE is to select?

The question is already addressed and considered by RAN2. RAN2 has captured following in [2] in ServingCellConfig field descriptions:

tci-ActivatedConfig

If configured for an SCell, or if configured for the PSCell when the SCG is being activated upon the reception of the containing message, the UE shall consider the TCI states provided in this field as the activated TCI states for PDCCH/PDSCH reception on this serving cell.

If configured for the PSCell when the SCG is indicated as deactivated in the containing message:

- the UE shall consider the TCI states provided in this field as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the TCI states provided in this field for PDCCH as RS for RLM, respectively for BFD.

When this field is absent for the PSCell and the SCG is being deactivated:

- the UE shall consider the previously activated TCI states as the TCI states to be activated for PDCCH/PDSCH reception upon a later SCG activation in which tci-ActivatedConfig is absent

- if bfd-and-RLM is configured and no RS is configured in RadioLinkMonitoringConfig for RLM, respectively for BFD, the UE shall use the previously activated TCI states for PDCCH as RS for RLM, respectively for BFD.

Hence, this clearly addresses the open question. UE shall consider the one or more TCI states provided in the tci-ActivatedConfig as activated TCI states.

UE shall consider the one or more TCI states provided in the tci-ActivatedConfig as activated TCI states.

In [14] we have provided a CR capturing the UE requirements accounting tci-ActivatedConfig.

Clarifying the procedure with RAN2

In earlier meetings, views have been diverse although now somehow converging. Hence, it seems unclear how the RAN2 specification was to be interpreted and our proposed alignment changes in our CRs has not been agreed.

Based on this we have also provided the option to ask RAN2 to clarify the relevant parts of the RAN2 procedure and specification quoted above. This would help clarifying UE requirements and RAN4 specification.

In the appendix we have provided a draft LS TP as follows:

RAN4 has been discussing the requirements related to PSCell and SCell activation delays when the PSCell or SCell configuration (ServingCellConfig) includes tci-ActivatedConfig IE.

RAN4 would like to ask RAN2 if the tci-ActivatedConfig IE can be configured for an SCell which is not activated upon configuration (hence, the SCell is configured in deactivated state)?

Send LS to RAN2 clarifying the RAN2 understanding of the tci-ActivatedConfig IE, if proposal 1 is not agreeable.

Conclusion

In this paper we discussed the applicability issue of tci-ActivatdConfig and observe:

- tci-ActivatedConfig can be configured for any SCell unconditionally.

tci-ActvatedConfig can only be configured for a PSCell which is activated when configured.

tci-ActivatedConfig can be configured for a deactivated SCell and a direct activated SCell.

UE shall consider the one or more TCI states provided in the tci-ActivatedConfig as activated TCI states.

Based on the discussion and observations we propose:

- Update the RAN4 UE requirements capturing the UE requirements when tci-ActivatedConfig is configured for a deactivated SCell and a direct activated SCell.

However, if proposal 1 is not agreeable we suggest clarifying the issue with RAN2

Send LS to RAN2 clarifying the RAN2 understanding of the tci-ActivatedConfig IE, if proposal 1 is not agreeable.

In CR [14] we have a proposal how to capture the requirements and we have provided a Draft LS in the appendix.

- References

- R4-2313357, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- 38.331.

- R4-2313356, LTE_NR_DC_enh2-Core Alignment of RAN4 requirements with RAN2 procedures, Nokia, Nokia Shanghai Bell.

- R4-2320284, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- R4-2320286, Draft LS on TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell

- R4-2320283, LTE_NR_DC_enh2-Core Alignment of RAN4 requirements with RAN2 procedures, Nokia, Nokia Shanghai Bell

- R4-2401727, LTE_NR_DC_enh2-Core CR correcting TCI state activation command at SCell activation, Nokia, Nokia Shanghai Bell.

- R4-2401726, (LTE_NR_DC_enh2-Core) Discussion on correcting TCI state activation command at SCell activation, Nokia

- R4-2408154, [LTE_NR_DC_enh2-Core] CR on alignment of RAN4 requirements with RAN2 procedures, Nokia

- R4-2408153, [LTE_NR_DC_enh2-Core] Alignment of RAN4 requirements with RAN2 procedures, Nokia

- R4-2408154, [LTE_NR_DC_enh2-Core] CR on alignment of RAN4 requirements with RAN2 procedures, Nokia

- R4-2411962, (LTE_NR_DC_enh2-Core) Alignment of RAN4 requirements with RAN2 procedures, Nokia

- R4-2411961, (LTE_NR_DC_enh2-Core) Alignment of RAN4 requirements with RAN2 procedures, Nokia

- R4-2418379, (LTE_NR_DC_enh2-Core) CR on alignment of RAN4 requirements with RAN2 procedures, Nokia

Appendix

3GPP TSG RAN WG 4 Meeting #113		TDoc R4-24xxxxx

Orlando, USA, Nov 18th – Nov  22nd, 2024

Title:	LS on alignment of RAN4 requirements with RAN2 procedures

Response to:

Release:	Rel-17

Work Item:	LTE_NR_DC_enh2-Core

Source:	RAN4

To:	RAN2

Cc:

Contact person:	Lars Dalsgaard

lars.dalsgaard@nokia.com

Attachments:	None

# 1	Overall description

RAN4 has been discussing the requirements related to PSCell and SCell activation when the PSCell or SCell configuration includes tci-ActivatedConfig.

RAN4 would like to ask RAN2 if the tci-ActivatedConfig can be configured for an SCell which is not activated upon configuration (hence, the SCell is configured in deactivated state)?

# 2	Actions

To RAN2

ACTION: 	RAN4 asks RAN2 to provide guidelines on whether the tci-ActivatedConfig can be configured for an SCell which is not activated upon configuration

# 3	Dates of next TSG RAN WG 4 meetings

TSG-RAN4 Meeting #114	17 February – 21 February, 2025	Athens, Greece

TSG-RAN4 Meeting #114bis	07 April – 11 April, 2025		China, China