3GPP TSG-RAN4 Meeting #113	R4-2418391

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

Source:	LG Electronics

Title:	Discussion for LP-WUR REFSENS requirement

Agenda item:	7.23.2.2

Document for:	Discussion

Introduction

In the RAN4#112bis meeting,  the WF on LP-WUS was approved. In this discussion, we share our point of view for the ΔTRxSRS requirement based on the approved WF[1].

Discussion

Table 1 WF on LP-WUS REFSENS requirements in the RAN4#112bis [1]

- Sub-topic 2-2 NF and REFSENS requirements Issue 2-2-1: Baseline architecture for FR1 OFDM-based LP-WUS Agreements: - use zero-IF receiver as a baseline RF architecture for OFDM based LP-WUR. Issue 2-2-3: REFSENS, NF, IM, SNR aspects for FR1 OOK-based and OFDM-based LR Agreements: - Collect input on REFSENS with values for each element（NF, IM, SNR） in the table next meeting. The value for each element can be discussed. - Encourage companies to also input the justification for each element. Issue 2-2-10: Baseline architecture for FR2 OOK-based and OFDM-based LP-WUR Agreements: - Companies to study baseline architecture for FR2 LP-WUS.

To specify the REFSENS requirement for LP-WUS, it is necessary to determine the values for NF, IM, and SNR. Our perspective on how to define NF, IM, and SNR individually is outlined below.

NF

The NF value for LP-WUR may vary depending on the architecture of the LP-WUR. There is a trade-off between power consumption and NF; the higher the NF, the lower the power consumption can be. This was discussed during the LP-WUS study phase, and the OOK-based and OFDM-based NF values can be defined by referring to the table 2.

Table 2 NF study for LP-WUR in TR38.869 [2]

For OOK For OFDM

Proposal 1: OOK-based NF and OFDM-based NF can be defined as shown below.

- OOK-based NF: 16 dB as a starting point

- OFDM-based NF: 12 dB as a starting point

IM

In LTE, the IM value assumed when defining REFSENS was based on the target SINR. If the SINR is below 7 dB, an IM of 2.5 dB was assumed, and for SINR values above 7 dB, IM was assumed to be either 3 dB or 4 dB.

Figure 1 Example of REFSENS computation for band 1 [3]

Observation: In LTE, the IM value assumed when defining REFSENS was based on the target SINR. If the SINR is below 7 dB, an IM of 2.5 dB was assumed, and for SINR values above 7 dB, IM was assumed to be either 3 dB or 4 dB.

Proposal 2: Use a 2.5 dB IM value assuming the target SNR for LP-WUS is below 7 dB.

SNR

The REFSENS target SNR for LP-WUS can be defined based on the LLS results.

Proposal 3: Determine the target SNR based on companies LLS simulation results on LP-WUS

Conclusion

Observation: In LTE, the IM value assumed when defining REFSENS was based on the target SINR. If the SINR is below 7 dB, an IM of 2.5 dB was assumed, and for SINR values above 7 dB, IM was assumed to be either 3 dB or 4 dB.

Proposal 1: OOK-based NF and OFDM-based NF can be defined as shown below.

- OOK-based NF: 16 dB as a starting point

- OFDM-based NF: 12 dB as a starting point

Proposal 2: Use a 2.5 dB IM value if the target SNR for LP-WUS is below 7 dB.

Proposal 3: Determine the target SNR based on companies LLS simulation results on LP-WUS

- References

# [1] R4-2417112, WF on UE RF requirements for LP-WUS (vivo)

# [2] TR38.869

# [3]Stefania Sesia et al., 2011, The UMTS Long Term Evolution: From Theory to Practice