3GPP TSG-RAN4 Meeting #113	R4-2418400

# Orlando, US, 18th Nov 2024 – 22nd Nov 2024

Source:	LG Electronics

Title:	Discussion for IoT NTN HPUE MPR

Agenda item:	7.8.2.3.1

Document for:	Discussion

Introduction

In the RAN4#112bis meeting,  the WF on IoT NTN HPUE was approved. In this discussion, we share our point of view for the ΔTRxSRS requirement based on the approved WF[1]

Discussion

Table 1 WF on IoT NTN HPUE in the RAN4#112bis [1]

Issue 1-2-4: MPR - Proposals - Proposal 1: If IoT-NTN PC3 ACLR value could be reused for IoT-NTN HPUE ACLR and relaxing SEM would be possible under HPUE operation, discuss whether reusing IoT-NTN PC3 MPR values for PC2 MPR would be one possible way - Proposal 2: Others - Recommended WF - Encourage more input next meeting.

The discussion on NTN HPUE is progressing as a Rel-19 Work Item, and it’s likely that PC2, PC1.5, and PC1 will be specified. Therefore, MPR requirements for NTN HPUE need to be defined. FR1 NTN UE is assumed to have an antenna gain of 0 dBi and that the distance between the UE and NTN is very large, there is a significant lack of link margin, particularly on the uplink. Introducing HPUE can help mitigate this shortage in the uplink link margin. However, as power class increases, the MPR requirement tends to increase as well, which could diminish the effectiveness of NTN HPUE. Therefore, to effectively address the uplink link margin deficiency in NTN UEs, it would be beneficial to consider a lower MPR value for NTN HPUE compared to that of legacy TN HPUE.

Observation: To effectively address the uplink link margin deficiency in NTN UEs, it would be beneficial to consider a lower MPR value for NTN HPUE compared to that of legacy TN HPUE.

The MPR values have different limiting factors depending on the modulation order and the location of the RB allocation, with possible limiting factors including ACLR, IBE, EVM, and SEM.. Therefore, to consider a lower MPR value than that of existing HPUE, it is necessary to identify the limiting factors that were used in defining the MPR for  TN HPUE, and Study will be needed to determine how to adjust these limiting factors when defining the MPR for NTN HPUE.

Proposal: It is necessary to identify the limiting factors that were used in defining the MPR for  TN HPUE, and study will be needed to determine how to adjust these limiting factors when defining the MPR for NTN HPUE.

Conclusion

Observation: To effectively address the uplink link margin deficiency in NTN UEs, it would be beneficial to consider a lower MPR value for NTN HPUE compared to that of legacy TN HPUE.

Proposal: It is necessary to identify the limiting factors that were used in defining the MPR for  TN HPUE, and study will be needed to determine how to adjust these limiting factors when defining the MPR for NTN HPUE.

- References

# [1] R4-2417205, WF on HPUE for IoT NTN (vivo)