3GPP TSG-RAN WG4 Meeting#113		R4-2418413

Orlando, United States, Nov 18th -22nd, 2024

Agenda item:	7.23.2.1

Source: 	Spreadtrum Communications

Title: 	Discussion on systems parameters for LP-WUR

Document for:	Discussion

Introduction

In RAN4#112meeting, a WF [1] was approved, in which there were remaining open issues as listed. This contribution continues to be discussed as follows.

Discussion

Channel raster for LP-WUR

Issue 1-2-1: Channel raster for LP-WUR Agreements: - Companies directly bring the specification impact if any and those will be discussed instead of whether a channel raster is required or not.

The channel raster defines a subset of RF reference frequencies that can be used to identify the RF channel position in the uplink and downlink. The centre of the channel bandwidth configured by gNB for UE should be aligned with the channel raster. Therefore, it is necessary for gNB to know the channel raster. According to the previous agreement, we focus on IMT bands. No matter whether LR and MR use the same frequency band or different bands, these bands belong to the IMT bands. Based on the above analysis, gNB could know the channel raster based on existing channel raster for IMT bands. Thus, the existing channel raster could be reused for LP-WUR. No new channel raster is needed for LP-WUR. In addition, from RAN1’s perspective, the same grid of serving cells can be shared by MR and LP-WUR. Hence, so no new channel raster is needed for LP-WUR. There is no specification impact.

Proposal 1: There is no specification impact for channel raster.

With regard to obtaining the frequency domain position of LP-WUS, the indication process of RAN1 is as follows. It is similar to the indication process of CORESET0.By indicating the starting position of LP-WUS frequency domain and an offset of the lower boundary of LP-WUS frequency domain in SIB1, the frequency domain position of LP-WUS can be obtained. The frequency position of LP-WUS is composed of the initial RB position and the number of RB.

Observation 1: RAN1 does not indicate the centre operation frequency of LP-WUS.

Observation 2: The frequency position of LP-WUS is composed of the initial RB position and the number of RB, which is indicated by SIB1.

Conclusions

In this paper, we provide our views on system parameters for LP-WUR, we have made the following proposals and observations.

Proposal 1: There is no specification impact for channel raster.

Observation 1: RAN1 does not indicate the centre operation frequency of LP-WUS.

Observation 2: The frequency position of LP-WUS is consist of initial RB which is indicated by SIB1 and RB number.

- Reference

- R4-2417112, WF on LP-WUS UE RF requirements, vivo, TSG-RAN WG4 112bis meeting