3GPP TSG-RAN WG4 Meeting # 113	R4-2418414

# Orlando, US, 18th – 22nd November, 2024

Source:	LG Electronics

Title:	Discussion on RRM requirements for (e)RedCap in NTN

Agenda item:	7.26.4.1

Document for:	Discussion

Introduction

In the last meeting, RAN4 discussed and agreed on applicability clarification and RRM requirements to be furthee defined for (e)RedCap NTN. In this contritubtion, we provide our views on open issues for RRM requirements based on the approved WF [1].

Discussion

General consideration on RRM requirements

Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN

<Agreement in RAN4#112bis> - The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands. - Both time-based and location-based cell reselection measurement criteria will be defined. - For both earth fixed and earth moving scenario - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined. - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement. <Agreement in RAN4#112> - For the requirements that have only been defined for (e)Redcap and not been defined for FR1-NTN, RAN4 NOT to define the following requirements for NTN (e)RedCap in Rel-19: - NR measurements for positioning in RRC_INACTIVE state - NR Handover to other RATs - Note: For NTN (e)RedCap, it refers to, for example, NTN to E-UTRA handover - RRC connection release with redirection to E-UTRAN - deriveSSB-IndexFromCell tolerance - Uplink spatial relation switch delay - Inter-RAT measurements in idle/inactive/connected mode - PRS-RSRPP measurements - NR measurements with autonomous gaps

In RAN4#112 meeting, there had agreement with a misclassification of the requirements for inter-RAT measurement in IDLE/INACTIVE mode for (e)RedCap and FR1-NTN. In IDLE and INACTIVE mode, inter-RAT E-UTRAN cells measurement has been defined for both NTN and (e)RedCap UE. Therefore, inter-RAT E-UTRA TN cells measurement requirements should be defined for NTN (e)RedCap UE in Rel-19.

- Proposal 1: Intorduce inter-RAT (NTN to E-UTRA TN) measurement in IDLE/INACITVE mode for NTN (e)RedCap UE in Rel-19.

The Requirements to be further determined

<Agreement> - Define NR Rel-17 and Rel-18 Conditional Handover requirements. - Include location-based CHO and time-based CHO. - Support earth moving cell scenario - Define RACH-less Handover requirements. - Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state. - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN. - FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

For SDT requirements in INACTIVE mode, there was FFS whether to consider TA validation for NTN (e)RedCap UE. Basically, UE to support NTN can pre-compensate the feeder and service link delay, i.e., NTA,adjcommon and NTA,adjUE. Therefore, before transmission, if UE updates the NTA,adjcommon and NTA,adjUE, TA validation procedure could be removed. Or, if NTN (e)RedCap UE support RACH-less HO, TA validation procedure could be removed in SDT requirements.

- Proposal 2: TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission.

eDRX configuration

<Agreement> - For GSO: - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN. - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used. - Further discuss for LEO.

In NR NTN, RAN4 has considered {0.32, 0.64, 1.28, 2.56}sec DRX cycle for RRM requirements for LEO deployment, and for 2.56s DRX cycle, UE is not required to meet the requirmeents for earth-moving LEO deployment. It implies that DRX cycle larger than 2.56s at least for earth-moving LEO based NTN scenario is meaningless. eDRX cycle defined in RRM requiqrements is larger than 2.56s, so eDRX configuration for earth-moving LEO deployment should be precluded. For quasi-earth fixed LEO deployment, considering (e)RedCap, limited eDRX cycle could be considered such as 5.12s.

- Proposal 3: For eDRX configuration for RedCap NTN,

  - for quasi-earth fixed cell LEO deployment, the limited eDRX cycle (e.g.,≤5.12s) could be considered.

  - for earth-moving LEO deployment, eDRX configuration should be precluded.

Multiple SMTCs

In NR NTN, multiple SMTCs are supported by UE capability to measure SSBs having different timing on a single frequency carrier. However, considering (e)RedCap UE as reduced capability, multiple SMTC on a single frequency carrier would introduce high implementation complexity. Therefore, (e)RedCap UE with NTN is not to support multiple SMTC.

- Proposal 4: RAN4 not to consider multiple SMTC capability for (e)RedCap UE with NTN

Requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

Basically, for 2Rx (e)RedCap NTN, exsiting NR NTN requirmeents can be reused. However, depending on supporting UE capability (e.g., multiple SMTC) of NR NTN for (e)RedCap NTN, existing NR NTN requirements could be revised.

- Proposal 5: For 2Rx (e)RedCap NTN, exsiting NR NTN requirmeents can be reused, and depending on supporting UE capability (e.g., multiple SMTC) of NR NTN for (e)RedCap NTN, existing NR NTN requirements sould be revised.

For 1Rx RedCap NTN, in RAN4 specification, extended measurement /evaluation period and handover delay for RRM requirements for 1Rx RedCap UE have already considered, so RRM requirements for 1Rx RedCap NTN can be defined based on existing 1Rx RedCap RRM requirements. RAN4 needs to consider the following requirements that are not defined on 1Rx RedCap

- NR SAN Satellite switching with re-synchronization

- Pathloss reference signal switching delay

- Network verified UE location

- Conditional Handover / RACH-less Handover

- SDT (if needed)

- Proposal 6: For 1Rx RedCap NTN, existing 1Rx RedCap RRM requirements can be reused for 1Rx RedCap NTN, and RAN4 only needs to define the following requirements that are not defined on 1Rx RedCap

- NR SAN Satellite switching with re-synchronization

- Pathloss reference signal switching delay

- Network verified UE location

- Conditional Handover / RACH-less Handover

- SDT (if needed)

Conclusion

In this contribution, we provide views on (e)RedCap NTN, and we propose

- Proposal 1: Intorduce inter-RAT (NTN to E-UTRA TN) measurement in IDLE/INACITVE mode for NTN (e)RedCap UE in Rel-19.

- Proposal 2: TA validation procedure can be skipped for SDT requirements if NTN (e)RedCap UE supports RACH-less HO or UE updates the NTA,adjcommon and NTA,adjUE before transmission.

- Proposal 3: For eDRX configuration for RedCap NTN,

  - for quasi-earth fixed cell LEO deployment, the limited eDRX cycle (e.g.,≤5.12s) could be considered.

  - for earth-moving LEO deployment, eDRX configuration should be precluded.

- Proposal 4: RAN4 not to consider multiple SMTC capability for (e)RedCap UE with NTN

- Proposal 5: For 2Rx (e)RedCap NTN, exsiting NR NTN requirmeents can be reused, and depending on supporting UE capability (e.g., multiple SMTC) of NR NTN for (e)RedCap NTN, existing NR NTN requirements sould be revised.

- Proposal 6: For 1Rx RedCap NTN, existing 1Rx RedCap RRM requirements can be reused for 1Rx RedCap NTN, and RAN4 only needs to define the following requirements that are not defined on 1Rx RedCap

- NR SAN Satellite switching with re-synchronization

- Pathloss reference signal switching delay

- Network verified UE location

- Conditional Handover / RACH-less Handover

- SDT (if needed)

Reference

[1] R4-2416865, “WF on RRM requirements for NR_NTN_Ph3_Part1,” Moderator (CATT)