3GPP TSG-RAN WG4 Meeting # 113		R4-2418421

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	5.6.4

Source: 	CMCC

Title: 			  Discussion on demodulation requirements for ATG UE

Document for:	Discussion

- Introduction

For TDD pattern 30D4S6U, in RAN4#112 meeting, the ATG k1 size and range issue has been addressed and captured in the latest spec for UEs that support outOfOrderOperationDL-r16, while for UEs that don’t support outOfOrderOperationDL-r16, how to perform HARQ-ACK and which test case to apply are still unclear now.

In this contribution, we provide our proposal about how to address the issue.

- Discussion

We figure out two ways for the HARQ-ACK feedback for UE not supporting outOfOrderOperationDL-r16:

- HARQ partially disabled

- k1 size extend

- Firstly, extending the k1 size can fundamentally address this HARQ-ACK issue. However, this work need the support from RAN1 and RAN2, which seems impossible in R18 maintenance phase.

- Hence, we think the HARQ partially disabled method can be a starting point. With eight k1 value and in order HARQ restriction, we think one way of mapping relationship between PDSCH and HARQ-ACK is shown as follows:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 |
| --- | --- | --- |

- By this way, 13 of 30 HARQ process are enabled, 17 of 30 HARQ process are disabled. We run the simulation for 13/30 HARQ process enabled case and all HARQ process enabled case, the simulation results are:

- Table 1. 13/30 HARQ process enabled

| TDD SCS=30kHz AWGN+500Hz doppler |  |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna | BWP(MHz) | PRBs | MCS | SNR（70%TP） |
| 2T2R | 40 | 106 | Table1 13 | 2.90 |
|  |  |  | Table1 22 | 10.29 |
|  |  |  | Table2 24 | 19.39 |
| 2T4R | 40 | 106 | Table1 13 | -0.06 |
|  |  |  | Table1 22 | 7.28 |
|  |  |  | Table2 24 | 16.39 |

- Table 2. All HARQ process enabled

| TDD SCS=30kHz AWGN+500Hz doppler |  |  |  |  |
| --- | --- | --- | --- | --- |
| Antenna | BWP(MHz) | PRBs | MCS | SNR（70%TP） |
| 2T2R | 40 | 106 | Table1 13 | 2.87 |
|  |  |  | Table1 22 | 10.27 |
|  |  |  | Table2 24 | 19.41 |
| 2T4R | 40 | 106 | Table1 13 | -0.08 |
|  |  |  | Table1 22 | 7.26 |
|  |  |  | Table2 24 | 16.40 |

- Form the simulation results, we can see that the SNR gap between 13/30 HARQ process enabled and all HARQ process enabled is about 0.03-0.01dB, which can be ignored. This is because the BLER is quite low in AWGN scenario.

Observation 1: The ideal SNR performance gap between 13/30 HARQ process enabled and all HARQ process enabled is about 0.03-0.01, which can be ignored.

- Proposal 1: For UE not supporting outOfOrderOperationDL-r16, mapping relationship between PDSCH and HARQ-ACK as follows:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 |
| --- | --- | --- |

- Proposal 2: For UE not supporting outOfOrderOperationDL-r16, the current SNR requirement can be reused.

Regarding the spec impact, there are two ways to capture the test case for UEs that don’t support outOfOrderOperationDL-r16

- Option 1: Use the current test case, but update the TDD UL-DL configuration, for example:

Table A.1.2-2c: TDD UL-DL configuration for SCS 30 kHz for ATG

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.30-8 |
| TDD Slot Configuration pattern (Note 1) |  |  | 30D4S6U |
| Special Slot Configuration (Note 2) |  |  | 14G |
| referenceSubcarrierSpacing |  | kHz | 30 |
| Pattern 1 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots | slot | 20 |
|  | nrofDownlinkSymbols | symbol | 0 |
|  | nrofUplinkSlot | slot | 0 |
|  | nrofUplinkSymbols | symbol | 0 |
| Pattern 2 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots |  | 10 |
|  | nrofDownlinkSymbols |  | 0 |
|  | nrofUplinkSlot |  | 6 |
|  | nrofUplinkSymbols |  | 0 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5 28 if mod(i,40) = 6, 7, 8, 9, 10, 11 22 if mod(i,40) = 12, 13, 14, 15, 16, 17 16 if mod(i,40) = 18, 19, 20, 21, 22, 23 10 if mod(i,40) = 24, 25, 26, 27, 28, 29 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE not supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index of all slots in every 20ms, i = {0,…,39} |  |  |  |

- Option 2: Add new test case and new TDD UL-DL configuration

- Since the SNR requirement can be reused, we slightly prefer Option 1 which has less spec impact.

- Proposal 3: Regarding specification documentation, we prefer to reuse the current test case, but update the TDD UL-DL configuration as follows:

Table A.1.2-2c: TDD UL-DL configuration for SCS 30 kHz for ATG

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.30-8 |
| TDD Slot Configuration pattern (Note 1) |  |  | 30D4S6U |
| Special Slot Configuration (Note 2) |  |  | 14G |
| referenceSubcarrierSpacing |  | kHz | 30 |
| Pattern 1 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots | slot | 20 |
|  | nrofDownlinkSymbols | symbol | 0 |
|  | nrofUplinkSlot | slot | 0 |
|  | nrofUplinkSymbols | symbol | 0 |
| Pattern 2 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots |  | 10 |
|  | nrofDownlinkSymbols |  | 0 |
|  | nrofUplinkSlot |  | 6 |
|  | nrofUplinkSymbols |  | 0 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5 28 if mod(i,40) = 6, 7, 8, 9, 10, 11 22 if mod(i,40) = 12, 13, 14, 15, 16, 17 16 if mod(i,40) = 18, 19, 20, 21, 22, 23 10 if mod(i,40) = 24, 25, 26, 27, 28, 29 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE not supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index of all slots in every 20ms, i = {0,…,39} |  |  |  |

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Observation 1: The ideal SNR performance gap between 13/30 HARQ process enabled and all HARQ process enabled is about 0.03-0.01, which can be ignored.

- Proposal 1: For UE not supporting outOfOrderOperationDL-r16, mapping relationship between PDSCH and HARQ-ACK as follows:

| The number of slots between PDSCH and corresponding HARQ-ACK information (Note 3) |  | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 |
| --- | --- | --- |

- Proposal 2: For UE not supporting outOfOrderOperationDL-r16, the current SNR requirement can be reused.

- Proposal 3: Regarding specification documentation, we prefer to reuse the current test case, but update the TDD UL-DL configuration as follows:

Table A.1.2-2c: TDD UL-DL configuration for SCS 30 kHz for ATG

| Parameter |  | Unit | UL-DL pattern |
| --- | --- | --- | --- |
|  |  |  | FR1.30-8 |
| TDD Slot Configuration pattern (Note 1) |  |  | 30D4S6U |
| Special Slot Configuration (Note 2) |  |  | 14G |
| referenceSubcarrierSpacing |  | kHz | 30 |
| Pattern 1 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots | slot | 20 |
|  | nrofDownlinkSymbols | symbol | 0 |
|  | nrofUplinkSlot | slot | 0 |
|  | nrofUplinkSymbols | symbol | 0 |
| Pattern 2 | dl-UL-TransmissionPeriodicity | ms | 10 |
|  | nrofDownlinkSlots |  | 10 |
|  | nrofDownlinkSymbols |  | 0 |
|  | nrofUplinkSlot |  | 6 |
|  | nrofUplinkSymbols |  | 0 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0, 1, 2, 3, 4, 5 28 if mod(i,40) = 6, 7, 8, 9, 10, 11 22 if mod(i,40) = 12, 13, 14, 15, 16, 17 16 if mod(i,40) = 18, 19, 20, 21, 22, 23 10 if mod(i,40) = 24, 25, 26, 27, 28, 29 |
| The number of slots between PDSCH and corresponding HARQ-ACK information for UE not supporting outOfOrderOperationDL-r16 (Note 3) |  |  | 34 if mod(i,40) = 0 30 if mod(i,40) = 4, 5 26 if mod(i,40) = 9,10 22 if mod(i,40) = 14,15 18 if mod(i,40) = 19,20 14 if mod(i,40) = 24,25 11 if mod(i,40) = 28 10 if mod(i,40) = 29 |
| Note 1: D denotes a slot with all DL symbols; S denotes a slot with a mix of DL, UL and guard symbols; U denotes a slot with all UL symbols. The field is for information. Note 2: D, G, U denote DL, guard and UL symbols, respectively. The field is for information. Note 3: i is the slot index of all slots in every 20ms, i = {0,…,39} |  |  |  |