3GPP TSG-RAN WG4 Meeting # 113		R4-2418430

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.1

Source: 	CMCC

Title: 			  Discussion on RRM requirement of OD-SSB for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed some general measurement procedure and principles for requirement definition about objective OD-SSB in NES enhancement WI. The WF has been approved in [1]. In this paper, we further discuss OD-SSB related issues, and identify RAN4 impacts according to the progress of other working groups.

- General On-demand SSB(OD-SSB) requirements

Issue 1-1-1: RAN1 LS

In RAN1#118 meeting, RAN1 sent the LS to RAN4 for asking if the working assumption of T_min=m+3Nslotsubframe,μ+1 can be confirmed. In RAN1#118bis meeting, RAN1 further revised the related agreement as below:

Agreement The previous RAN1 agreement is partly confirmed and further revised as follows. - For SSB burst(s) indicated by on-demand SSB SCell operation via a MAC CE, UE expects that on-demand SSB burst(s) is transmitted from time instance A which is determined as follows. - Alt 3-1: Time instance A is the beginning of the first slot containing [candidate SSB index 0 or the first actually transmitted SSB index] ofwithin the first “possible” on-demand SSB burst which is at least T slots after the slot where UE receives a signalling from gNB to indicate on-demand SSB transmission - The SSB time domain positions of on-demand SSB burst are configured by gNB. - The location(s) (e.g., SFN offset, half frame index) in the time domain of “possible” on-demand SSB burst and SSB position within the burst should be configured by the gNB - Note: The value of T is not less than existing timeline required for UE’s MAC CE processing for SCell activation - (Working assumption): T is not less than T_min=m+3Nslotsubframe,μ+1 where slot n+m is a slot indicated for PUCCH transmission with HARQ-QCK information when the UE receives MAC CE signaling to indicate on-demand SSB transmission ending in slot n, and Nslotsubframe,μ is as defined in current specification. - RAN4 to confirm that T_min can be equal to m+3Nslotsubframe,μ+1 - (Working assumption) T=T_min - Above applies at least for the case where SCell with on demand SSB transmission and cell with signalling transmission have the same numerology.

After UE receiving OD-SSB indication by MAC-CE in slot n, UE at least need m slots for feedback HARQ-ACK and 3ms = 3Nslotsubframe,μ for MAC-CE processing. Then, we think UE is able to receive OD-SSB as long as OD-SSB is in the active BWP.

Observation 1: UE is able to receive the OD-SSB after m+3Nslotsubframe,μ+1 slots.

However, if UE need to perform measurement on OD-SSB, as mentioned by companies in last meeting,  additional processing time for re-allocating the measurement searcher resource may be needed. In Case 1 that no AO-SSB, due to the number of frequency carriers to be measured may be changed, UE should re-allocate it’s measurement searcher resource. In Case 2 that AO-SSB is transmitted, if OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB, then additional processing time may needed. For other cases, we think the additional processing time is not necessary.

Observation 2: For following cases, additional processing time may be needed for UE to measure the OD-SSB:

- No AO-SSB on the SCell (Case 1)

- AO-SSB is transmitted on the SCell (Case 2), while OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB

Based on our understanding, the main reason RAN1 need the T_min value is to assist OD-SSB scheduling. For example, network will not send the OD-SSB before the slot n+T_min. Therefore, we prefer to reply the LS with all our observations about HARQ-ACK delay, MAC-CE processing delay, and additional processing time for measurement searcher resource re-allocation in some cases, as well as the UE behavior after each time point. RAN1 can make the final decision with these information.

Proposal 1: Reply the LS with all observations about HARQ-ACK delay, MAC-CE processing delay, and additional processing time for measurement searcher resource re-allocation in some cases, as well as the UE behavior after each time point. For example:

- UE is able to receive the OD-SSB after m+3Nslotsubframe,μ+1 slots.

- UE is able to measure the OD-SSB after m+3Nslotsubframe,μ+1 +T_meas_process slots for following two cases. For other cases, UE is able to measure the OD-SSB after m+3Nslotsubframe,μ+1 slots:

  - No AO-SSB on the SCell (Case 1)

  - AO-SSB is transmitted on the SCell (Case 2), while OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB

Regarding how long the additional processing time will take, considering the measurement resource re-allocation updating scheme is not complicated, we think 1 slot can be the starting point.

Proposal 2: For the value of T_meas_process, take 1 slot as the starting point.

Issue 1-1-4: L1 measurement for deactivated SCell

So far, RAN1 achieved following agreements about L1 measurement:

Agreement from RAN1#117 - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB. Agreement from RAN1#118 - Update the previous RAN1 agreement as follows. - At least support L1 measurement based on on-demand SSB - For L1 measurement based on on-demand SSB, periodic, semi-persistent, [and aperiodic] L1 measurement reports based on existing CSI framework are supported. - FFS on potential enhancements of CSI report configuration and/or triggering/activation mechanisms for L1 measurement based on on-demand SSB - The support of LTM is a separate discussion point Agreement from RAN1#118bis - For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), consider only one or both of the following options for UE to perform L1 measurement based on on-demand SSB. - Option 1: A CSI report configuration is associated with both of on-demand SSB and always-on SSB - Option 2: A CSI report configuration is associated with one of always-on SSB and on-demand SSB - FFS: Whether OD-SSB and always on SSB have same beam or not

For L1 measurement during SCell activation, as RAN4 agreed in last meeting, OD-SSB will be used as the baseline.

Issue 1-4-5: Which type of SSB is used during SCell activation Agreement: - In Case 1, UE is required to always use OD-SSB for Scell activation. - In Case 2, when both OD-SSB and always-on SSB are transmitted, - RAN4 assumes UE to use OD-SSB to perform SCell activation as a baseline - The agreement can be revisited if different with other WG

During SCell activation, if UE need to perform L1-RSRP measurement, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB. As for CSI reporting, the current definition of TCSI_reporting and TL1-RSRP, report can be reused.

Proposal 3: For L1 measurement during SCell activation, from RAN4 requirement point of view, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB.

For the L1 measurement after SCell activation period, regarding whether OD-SSB or AO-SSB or both will be used for measurement, and when/how to perform the measurement, we think the unified scheme should be designed between L1 and L3 measurement. We can discuss the L3 measurement first.

Proposal 4: For L1 measurement before and after SCell activation, regarding whether OD-SSB or AO-SSB or both will be used for measurement, and when/how to perform the measurement, unified design between L1 and L3 measurement is preferred.

Issue 1-1-5: CSSF

In FR1, the searcher assumption is that PCC will occupy one searcher, SCC and inter-frequency MO occupy the other searcher with same priority.

In FR2, the searcher assumption is that PCC will occupy one searcher, SCC where neighbour cell measurement is required occupy half of the other one searcher, SCC where neighbour cell measurement is not required and inter-frequency MO occupy half of the other one searcher with same priority.

One issue is whether the priority will be increased for SCC with OD-SSB. Based on our understanding, OD-SSB has higher priority than AO-SSB in one SCell, but the priority among SCells should not be changed by OD-SSB triggering.

The second issue is whether both OD-SSB and AO-SSB are expected to be measured when they are not on the same frequency layer. Considering the measurement results combination between OD-SSB and AO-SSB on different frequency layer is not straight-forward, we think UE will not perform L3 measurement on both AO-SSB and OD-SSB which on different frequency layers.

With our consideration on these two issues, we think the CSSF assumption and algorithm can be kept as existing way.

Proposal 5: The legacy CSSF can be reused.

Issue 1-1-6: OD-SSB accuracy

The legacy measurement accuracy should also applied for OD-SSB based measurement.

Proposal 6: Apply the legacy measurement accuracy for OD-SSB based measurement

- OD-SSB based deactivated SCell L3 measurement (Case 1)

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1)

Agreements:

Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window.

- After the time window, UE can perform deactivated SCell measurement with a slow mode.

  - Option 2-1: fallback legacy requirement

  - Option 2-2: stop measurement

  - Option 2-3: No requirement

- Time window means the time duration starting from receiving OD-SSB activation indication

  - FFS the time window configuration or pre-defined

- FFS the fast L3 measurement requirement

- FFS the relation between time window and OD-SSB deactivation command in the time domain

When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

In order to avoid UE power consumption waste at some extreme scenarios, for example, network transmit OD-SSB activation indication a long time before SCell activation command, and OD-SSB keeps activation during this period. RAN4 agreed to introduce a time window, UE only required to perform fast mode L3 measurement in this time window. However, the time window length, fast L3 measurement requirement, UE behavior after the time window are still FFS.

For time window length, it should equal or longer than the time period for measurement reporting delay i.e., Tidentify_intra_with_index to let UE finish at least one shot measurement. Besides, it should also consider the deactivation time of OD-SSB which has been agreed by RAN1, due to UE is not required to measure the deactivated SCell when UE receives  OD-SSB deactivation indication. Therefore, we propose the pre-defined time window, which length is min(m+3Nslotsubframe,μ+1 +T_meas_process + Tidentify_intra_with_index, TOD-SSB_activation), TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication.

Proposal 7: Pre-define the time window length as min(m+3Nslotsubframe,μ+1 +T_meas_process + Tidentify_intra_with_index, TOD-SSB_activation), TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication.

For the fast L3 measurement requirement, UE perform the measurement following the OD-SSB periodicity can be the baseline, that UE could finish the measurement with the shortest time duration, then further help network power saving.

Proposal 8: UE perform the fast L3 measurement following the OD-SSB periodicity.

For the UE behavior after the time window, we prefer to fall back the legacy requirement, which means UE will continue the measurement following the measCycleSCell if OD-SSB is not deactivated.

Proposal 9: After the time window, UE fallback to legacy requirement.

- OD-SSB based deactivated SCell L3 measurement (Case 2)

Issue 1-3-1: Always-on SSB and OD-SSB time/frequency domain relation

- Background

From moderator’s understanding, the following scenarios for always-on SSB and OD-SSB relation are possible based on RAN1’s current agreements.

  - Scenario 1: OD-SSB and always-on SSB are with same frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different frequencies.

Agreement:

- In OD-SSB Case 2, RAN4 to first discuss the deactivated SCell requirement for Scenario 1. RAN4 to further discuss whether to define the requirement for the scenario 2 and 3 if scenario 2 and 3 are supported in RAN1 based on RAN1’s agreement.

  - Scenario 1: OD-SSB and always-on SSB are with same SSB carrier frequency, same offset but different periodicities.

  - Scenario 2: OD-SSB and always-on SSB are with same SSB carrier frequency, different offsets and periodicities.

  - Scenario 3: OD-SSB and always-on SSB are within different SSB carrier frequencies.

- Send the above information to RAN1, further discuss the text including the questions to RAN1 in the LS.

- RAN4 will inform RAN1 if any further agreement.

Based on the agreement from RAN1, all Scenario 1, 2 and 3 are valid. We are fine to first figure out the behavior in Scenario 1, but when we design the procedure, the unified design is preferred by us, let UE have common  behavior for each scenario.

Agreement from RAN1#118bis For a cell supporting on-demand SSB SCell operation and for Case #2 (i.e., Always-on SSB is periodically transmitted on the cell), study at least the following Mux-Cases. - Mux-Case #1: No time-domain overlap between always-on SSB and on-demand SSB. - Mux-Case #2: Always-on SSB and on-demand SSB overlap at least in time or frequency domain

Observation 3: All Scenario 1, 2 and 3 are valid based on RAN1’s agreement.

Proposal 10: Prefer the unified design for Scenario 1, 2 and 3.

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

Agreements:

OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2)

  - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE)

  - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO)

  - Option 3:  RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE)

  - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm)

  - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

For Case 2, we prefer to use the same design as Case 1, that introducing a time window for UE. During the time window, RAN4 requirement is defined based on OD-SSB periodicity. Due to UE has measured AO-SSB before time window, there is no need for UE to do the cell identification for sure. Therefore, the time window duration can be updated as min(m+3Nslotsubframe,μ+1 +T_meas_process + TSSB_measurement_period_intra, TOD-SSB_activation), TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication.

Proposal 11: UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window, which duration is pre-defined as min(m+3Nslotsubframe,μ+1 +T_meas_process + TSSB_measurement_period_intra, TOD-SSB_activation).

Issue 1-3-5: Transition period between measurement

If UE perform the legacy L3 measurement follows ‘measCycleSCell’ before on-demand SSB triggering, and perform new OD-SSB based L3 measurement after OD-SSB triggering, there will be a transition period between measurement.

With the time-window design, we think UE shall be allowed to restart the ongoing evaluation using the second SSB type when the measured SSB type changes.

Proposal 12: With the design of time window, UE shall be allowed to restart the ongoing evaluation using the second SSB type when the measured SSB type changes.

- OD-SSB based SCell activation

Issue 1-4-3: known cell condition design – principle

Agreement:

- The legacy known condition principle can be reused, which is defined based on if the UE has sent a valid measurement report during a ‘period’ before the reception of the SCell activation command.

- When OD-SSB transmission is before/together with the SCell activation command(Scenario #2/Scenario #2A), UE follows the principles of legacy known/unknown SCell activation requirement once the known/unknown condition is met.

  - FFS how known/unknown condition is defined i.e. based on always-on SSB and/or OD-SSB

- Other principles are not precluded.

About how known/unknown condition is defined, in legacy FR1, the known case requires UE to sent a valid measurement report during a period before the reception of the SCell activation command, the period equal to max(5*measCycleSCell,  5*DRX cycles), which related to the legacy L3 measurement requirement.

In other words, as long as UE reported the measurement report during max(5*measCycleSCell,  5*DRX cycles) before the SCell activation command, UE could maintain the known states of this deactivated SCell. From this point of view, we think this known/unknown condition can be reused even for OD-SSB case, and it fits well for the extreme scenarios we mentioned before, that network transmit OD-SSB activation indication a long time before SCell activation command. We illustrate the cases in the figure below.

Proposal 13: Reuse the legacy known/unknown cell condition.

- Conclusion

Based on the discussion above, the following proposals and observations are concluded.

Observation 1: UE is able to receive the OD-SSB after m+3Nslotsubframe,μ+1 slots.

Observation 2: For following cases, additional processing time may be needed for UE to measure the OD-SSB:

- No AO-SSB on the SCell (Case 1)

- AO-SSB is transmitted on the SCell (Case 2), while OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB

Proposal 1: Reply the LS with all observations about HARQ-ACK delay, MAC-CE processing delay, and additional processing time for measurement searcher resource re-allocation in some cases, as well as the UE behavior after each time point. For example:

- UE is able to receive the OD-SSB after m+3Nslotsubframe,μ+1 slots.

- UE is able to measure the OD-SSB after m+3Nslotsubframe,μ+1 +T_meas_process slots for following two cases. For other cases, UE is able to measure the OD-SSB after m+3Nslotsubframe,μ+1 slots:

  - No AO-SSB on the SCell (Case 1)

  - AO-SSB is transmitted on the SCell (Case 2), while OD-SSB and AO-SSB are within different frequencies and UE is expected to perform measurement on both OD-SSB and AO-SSB

Proposal 2: For the value of T_meas_process, take 1 slot as the starting point.

Proposal 3: For L1 measurement during SCell activation, from RAN4 requirement point of view, the current requirement can be reused with the clarification of TSSB in TL1-RSRP_Measurement_Period_SSB is the SSB periodicity of OD-SSB.

Proposal 4: For L1 measurement before and after SCell activation, regarding whether OD-SSB or AO-SSB or both will be used for measurement, and when/how to perform the measurement, unified design between L1 and L3 measurement is preferred.

Proposal 5: The legacy CSSF can be reused.

Proposal 6: Apply the legacy measurement accuracy for OD-SSB based measurement

Proposal 7: Pre-define the time window length as min(m+3Nslotsubframe,μ+1 +T_meas_process + Tidentify_intra_with_index, TOD-SSB_activation), TOD-SSB_activation is the time duration between OD-SSB activation indication and OD-SSB deactivation indication.

Proposal 8: UE perform the fast L3 measurement following the OD-SSB periodicity.

Proposal 9: After the time window, UE fallback to legacy requirement.

Observation 3: All Scenario 1, 2 and 3 are valid based on RAN1’s agreement.

Proposal 10: Prefer the unified design for Scenario 1, 2 and 3.

Proposal 11: UE performs the OD-SSB based fast L3 measurement following OD-SSB periodicity for deactivated SCell within a time window, which duration is pre-defined as min(m+3Nslotsubframe,μ+1 +T_meas_process + TSSB_measurement_period_intra, TOD-SSB_activation).

Proposal 12: With the design of time window, UE shall be allowed to restart the ongoing evaluation using the second SSB type when the measured SSB type changes.

Proposal 13: Reuse the legacy known/unknown cell condition.

- Reference

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson