3GPP TSG-RAN WG4 Meeting #113		R4-2418431

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.22.2.2

Source: 	CMCC

Title: 			  Discussion on RRM requirement of signal adaptation for NES enhancement

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed SSB signal adaptation, and some initial conclusions have been agreed in WF. [1] In this paper, we further discuss SSB adaptation related issues, and identify RAN4 impacts according to the progress of other working group.

- SSB Adaptation

Cell DTX extension to SSB adaptation

For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, RAN1 achieved following agreements:

Agreement from RAN1#118bis

- For Cell DTX extension to SSBs not on sync-raster for connected mode UEs, select Option 3, i.e. Cell DTX does not impact UE assumption on SSB transmissions (i.e. legacy behavior).

  - No spec impact

Therefore, when RAN4 define requirement with SSB adaptation, no need to consider the Cell DTX impact.

Proposal 1: No Cell DTX impact on SSB adaptation related requirements.

Transition period

For the mechanism of SSB adaptation in time-domain, RAN1 concluded following agreements[4]:

Agreement For adaptation of SSB in time-domain, Option 1 is supported - Option 1: Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) - Note: Using Option 2 to realize Option 1 is not precluded - Option 2: Adaptation based on two SSB configurations [where up to two configurations can be active] - FFS: details of the differences between the two SSB configurations, e.g. two different periodicities - FFS: Details including applicable scenarios - FFS: Support of Cell DTX for connected mode UEs for SSB

Adaptation of SSB burst periodicity using one or more SSB burst periodicity value(s) is supported, which means there will be SSB periodicity transitions during the measurement.

For delay requirement during the transition period, UE is unable to use the minimum requirements corresponding to the first SSB configuration/periodicity and the second SSB configuration/periodicity, due to the SSB resource itself is adapted.

Therefore, for both L1 and L3 requirements during the transition period, requirements apply based on the longer delay before or after the transition.

Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity.

Proposal 2: For both L1 and L3 requirements during the transition period, requirements apply based on the longer delay before or after the transition. Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity.

- Conclusion

Based on the discussion above, the following proposals are concluded.

Proposal 1: No Cell DTX impact on SSB adaptation related requirements.

Proposal 2: For both L1 and L3 requirements during the transition period, requirements apply based on the longer delay before or after the transition. Subsequent to the transition period, requirements apply based on the second SSB configuration/periodicity.

- Reference

- R4-2416860, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei