3GPP TSG-RAN WG4 Meeting #113		R4-2418433

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.23.4.2

Source: 	CMCC

Title: 			 Discussion on RRM impact of LP-WUR

Document for:	Discussion

- Introduction

In last meeting, RAN4 further discussed some general issues and the requirement assumption. The WF has been approved in [1]. In this contribution, we further analyze the RRM impact of LP-WUR based on the progress of RAN1 and RAN2, and provide our proposals.

- Discussion

Issue 1-1-1: Cases/states to be considered for RRM relaxation and serving cell measurement offloading

Background:

Case #1 and #3 have been agreed

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #1 Fully offloading case | Off | Off: FFS the condition and the details | ON |
| #3 Relaxed case b | On with relaxation measurement | On with relaxation measurement | ON |

RAN4 to further discuss case #2

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #2 Relaxed case a | On with relaxation measurement | Off | ON |

Case #4 is covered by issue for higher priority frequency layer

| RRM measurement case index | MR serving cell measurement | MR neighboring cell measurement | LR measurement |
| --- | --- | --- | --- |
| #4 Relaxed case c | Off | On, FFS the condition and the details | ON |

The legacy cell re-selection procedure in TS 38.304 are:

- If the serving cell fulfils Srxlev > SIntraSearchP and Squal > SIntraSearchQ, the UE may not perform intra-frequency measurements.

- If the serving cell fulfils Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ,  the UE may choose not to perform measurements of NR inter-frequency cells of equal or lower priority, or inter-RAT frequency cells of lower priority.

These two procedures should also be valid in scenario with LP-WUS/WUR.

- Observation 1: The legacy cell re-selection measurement rules should also be valid in the scenario with LP-WUS/WUR.

Then, as long as network configures Case#3 entry condition lower than SnonIntraSearchP and/or SIntraSearchP, and Case#1 entry condition higher than SIntraSearchP, after UE entry in Case 3 and the Srelev becomes higher than SIntraSearchP and lower than the entry condition of Case#1, only MR serving cell relaxed measurement need to be performed, which is Case#2. Hence, Case#2 is a possible case, and its existence depends on network configuration.

- Proposal 1: Case#2 is a possible case, its existence depends on network configuration.

Issue 1-1-2: Measurement requirements to be specified for LP-WUR

Agreement from RAN4#110bis

- At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following:

  - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

  - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Other related requirements are FFS

Agreement from RAN1#118bis

Support overlaid OFDM sequence(s) for LP-SS:

- LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS.

- Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable.

  - Applicable at least for OOK-1 and FFS for OOK-4

- From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1 agreed that overlaid OFDM sequence over LP-SS is supported, but RAN4 requirements are not intended to be introduced. Therefore, RAN4 can only focus on the measurement requirement specification of following two cases:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Proposal 2: RAN4 specifies measurement requirements for the following two cases:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Issue 1-1-4: Threshold for switch between case 1 and case 3

Agreement from RAN2#127:

- For serving cell measurement offloading (i.e., there is no serving cell measurement by MR):

  - The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered.

  - The exit condition is based on the LR measurement results.

Agreement from RAN2#127bis: Working assumption

- For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’.

- FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Which type of threshold will be used for RRM measurement relaxation and offloading criteria should be decided by RAN2, it has already been under discussion. RAN4 should wait for more output from RAN2.

- Proposal 3: RAN2 is responsible for the discussion of which type of threshold will be used for RRM measurement relaxation and offloading criteria.

Issue 1-1-8: LP-WUR operating carrier frequency

Agreement:

- RAN4 will further discuss whether any technical issues regarding the LS from RAN1.

- From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2.

Issue for further discussion based on contribution driven:

- For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

LS from RAN1#118 R1-2407559

- In RAN1, the common understanding is that UE may not support LP-WUS reception on all the bands supported by the UE. - RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs. To RAN2 and RAN4: ACTION: RAN1 respectfully asks RAN2 and RAN4 to check if there is any issue and specification support needed for IDLE/INACTIVE UEs.

- Firstly, from LR RF capability, low-cost, and low power consumption point of view, we also think UE LR may not support LP-WUS reception/LP-SS measurement on all the bands supported by the UE MR. The common understanding from RAN1 is confirmed.

- If MR camps on one serving cell, which operating band can not be supported by LR, we think there are two kinds of reasonable UE behavior, and their respective pros and cons are as follows:

- The LR based LP-WUS reception/LP-SS measurement can not be enabled

  - Pros: No additional specification support is needed.

  - Cons: Network should configure LP-WUS/LP-SS on each carrier frequency, or else, LR may not be workable in some areas, which will cause large network overhead.

- The LR based LP-SS measurement can not be enabled, but LR based LP-WUS reception can be performed on the carrier frequency which LR supported.

  - Pros: Network only be required to configure LP-WUS on the carrier frequency of bottom network, or the carrier frequency widely supported by UE’s LR. The network overhead can be saved a lot. Compared to the first UE behavior, no additional UE power consumption is needed.

  - Cons: Additional specification support may be needed.

- Considering the second UE behavior can bring benefits to network overhead, meanwhile no additional UE effort and power consumption needed, we support to further investigate, and feedback RAN4’s observation to RAN1.

- Proposal 4: When LR doesn’t support the serving cell’s operating band, LP-WUS reception can be performed on the other carrier supported by LR. The details e.g., where and how LR monitor the LP-WUS may need additional specification support. This part should be studied in RAN1 first.

Issue 1-1-10: Considerations on higher priority frequency layer

Currently, higher priority frequency layer can be configured by network, to carry the capacity. Whether exist high priority frequency layer is highly related with the operator’s deployment strategy considering different network scenario.

Since high priority frequency layer is always the inter-frequency layer, thus, it can only be monitored by MR. Then, when MR fully offloading to LR, i.e., only LR perform the measurement, how does UE perform the higher-layer frequency measurement should be further discussed.

In our perspective, as long as network configure such higher priority layer, UE should monitor it for possible cell re-selection. In current spec, UE is required to monitor per higher priority layer within 60s if no relaxation applied, and within 1h if relaxation applied. When fulling offloading case happens (Case#1), in order to guarantee UE power saving gain, we think the relaxed higher priority layer measurement requirement can be applied. When partial offloading case happens (Case#3), we are open to discuss which higher priority layer measurement requirement should be applied.

- Proposal 5: As long as network configure such higher priority layer, UE should monitor it for possible cell-re selection

- When fulling offloading case happens (Case#1), the relaxed higher priority layer measurement requirement can be applied, i.e., per higher priority layer within 1h

- When partial offloading case happens (Case#3), open to further discuss which higher priority layer measurement requirement should be applied

Issue 1-2-1: Accuracy requirements

About the IDLE/Inactive mode measurement accuracy, we support to reuse the legacy method, that reflect the accuracy performance as a margin in the core requirement, no need to define it in the performance section.

- Proposal 6: For IDLE/Inactive mode measurement accuracy, reflect the accuracy performance as a margin in the core requirement.

Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation

For the entry/exit criteria evaluation, we think the evaluation period can be the same as MR and/or LR measurement period, no need to define dedicated evaluation period.

- Proposal 7: For the entry/exit criteria evaluation, the evaluation period can be the same as MR and/or LR measurement period.

Issue 1-2-3: On interruption related requirements for LP-WUR

Due to MR will go into ultra-deep sleep mode in RRC IDLE/Inactive mode, it can’t receive paging immediately after LR receiving LP-WUS with paging indication.

RAN1 also discussed this issue, how does UE report the wake up delay capability, and how does network configure the offset value(s) between an LO and a reference PO/PF, following agreements have been achieved:

Agreement For the offset value(s) between an LO and a reference PO/PF, consider the following options: - Definition: The gap between an LO and a PO is considered to be no less than the wake-up delay a UE supports if the gap between the end of the last LP-WUS MO the UE monitors in the LO and the start of the PO is no less than the wake-up delay. - Option 1: gNB configures a single offset value. - If the gap between an LO and the PO associated with the offset is no less than the wake-up delay a UE supports, the UE monitors the PO associated with the offset after receiving a wake-up indication in a LP-WUS. - Otherwise, - Option 1-1: the UE follows the legacy paging monitoring procedure. - Option 1-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - Option 2: gNB configures one or multiple offset values. - For the same PO, each offset corresponds to a LO. - This does not preclude the possibility that the same LO may correspond to different POs with different offset values. - Option 2A: A UE does not expect that the gap between the LO associated with the largest offset and the corresponding PO is less than the wake-up delay the UE supports. The UE monitors the LO associated with one offset that has a gap between the LO and the corresponding PO no less than the wake-up delay. - This implies that the gNB needs to configure at least one offset value that is no less than the largest wake-up delay supported by the UEs. - FFS exactly how to choose the offset - Option 2B: - If the gap between the LO associated with the largest offset and the corresponding PO is no less than the wake-up delay a UE supports, the UE monitors the LO associated with one offset that has a gap between the LO and the PO associated with the offset no less than the wake-up delay. - FFS exactly how to choose the offset - Otherwise, - Option 2B-1: the UE follows the legacy paging monitoring procedure. - Option 2B-2: the UE monitors LP-WUS. If it receives a wake-up indication in a LP-WUS, it monitors the first PO after its reported wake-up delay. - FFS exactly how to choose the offset - FFS the UE shall monitor the LO associated with additional offset(s) Note: The PO mentioned above refers to legacy PO configured for the UE. Agreement Support 3 candidate values for the wake-up delay capability a UE reports, two values for ultra-deep sleep state and one value for deep sleep state. - FFS the values - Note: Ultra-deep sleep state and deep sleep state are mentioned above solely for the purpose of RAN1 discussions and it is not intended to be captured in specifications from RAN1 perspective

If RAN1 will specify the offset values and related UE behaviors, RAN4 don’t need to define the duplicate interruption requirement. Hence, we can postpone the discussion and wait for more RAN1 progress.

- Proposal 8: For the interruption between LP-WUS (with paging indication) reception and MR to start paging monitoring, postpone the discussion and wait for more RAN1 progress.

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor

Issue 1-3-3: Accuracy for relaxed MR measurement

Since case#3 has been agreed, then MR based serving cell measurement relaxation requirement and neighbouring cell measurement relaxation requirement should be specified.

The first key point is the scaling factor, which should guarantee the mobility performance and power saving gain simultaneously. Based on the output observation from TR38.869 clause 8.1.1.5.2, if X<8, then very limited power saving gain will be achieved (average: 4%, range: -18%~50%). Therefore, the MR should at least be relaxed 8 times. We propose to use 8 as the baseline.

The second key point is the measurement accuracy. The MR measurement result will be used to determine serving cell quality and neighbouring cell quality for cell selection and cell re-selection, and whether to trigger the measurement offloading, the accuracy is rather important among these procedure to guarantee UE mobility and camp on. The legacy accuracy should be reused.

- Proposal 9: MR based serving cell measurement and neighbouring cell measurement requirement should be specified. The scaling factor should guarantee the mobility performance, measurement accuracy and power saving gain simultaneously.

- Proposal 10: For the measurement based on both MR with RRM relaxation (X time relaxation) and LP-WUR, X =8 can be the starting point.

- Proposal 11: The legacy accuracy for relaxed MR measurement should be reused for Case#3.

Issue 1-5-2: LR based RRM with EMR in IDLE/inactive mode

- Proposals

- P1: For a UE which supports idleInactiveNR-MeasReport-r16 or idleInactiveEUTRA-MeasReport-r16, and serving cell configures carriers for idle mode CA/DC measurement reporting with T331 running, UE shall keep the MR ON for EMR measurement regardless of the MR offloading condition is met or not. (Apple)

This issue is similar as the issue 1-1-10, that network configures one measurement which can only be performed by MR. Then when UE enters Case#1, whether the measurement need to be done by MR or not should be further investigate.

In case#1, we deem that the condition of Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ will be valid. Based on existing spec, the UE shall search for inter-frequency layers configured for idle mode CA/DC measurements at least every Thigher_priority_search

We think the importance of EMR measurement is lower than high priority measurement. Therefore, if only relaxed higher priority layer measurement requirement can be applied in case 1, then the measurement requirement for EMR also need to be relaxed at this case. We prefer to discuss the high priority measurement first, and then discuss this issue.

- Proposal 12: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition.

- Conclusion

Based on the discussion above, the following proposals are concluded.

- Observation 1: The legacy cell re-selection measurement rules should also be valid in the scenario with LP-WUS/WUR.

- Proposal 1: Case#2 is a possible case, its existence depends on network configuration.

- Proposal 2: RAN4 specifies measurement requirements for the following two cases:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

- Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

- Proposal 3: RAN2 is responsible for the discussion of which type of threshold will be used for RRM measurement relaxation and offloading criteria.

- Proposal 4: When LR doesn’t support the serving cell’s operating band, LP-WUS reception can be performed on the other carrier supported by LR. The details e.g., where and how LR monitor the LP-WUS may need additional specification support. This part should be studied in RAN1 first.

- Proposal 5: As long as network configure such higher priority layer, UE should monitor it for possible cell-re selection

- When fulling offloading case happens (Case#1), the relaxed higher priority layer measurement requirement can be applied, i.e., per higher priority layer within 1h

- When partial offloading case happens (Case#3), open to further discuss which higher priority layer measurement requirement should be applied

- Proposal 6: For IDLE/Inactive mode measurement accuracy, reflect the accuracy performance as a margin in the core requirement.

- Proposal 7: For the entry/exit criteria evaluation, the evaluation period can be the same as MR and/or LR measurement period.

- Proposal 8: For the interruption between LP-WUS (with paging indication) reception and MR to start paging monitoring, postpone the discussion and wait for more RAN1 progress.

- Proposal 9: MR based serving cell measurement and neighbouring cell measurement requirement should be specified. The scaling factor should guarantee the mobility performance, measurement accuracy and power saving gain simultaneously.

- Proposal 10: For the measurement based on both MR with RRM relaxation (X time relaxation) and LP-WUR, X =8 can be the starting point.

- Proposal 11: The legacy accuracy for relaxed MR measurement should be reused for Case#3.

- Proposal 12: The methodology of EMR requirement definition under each Cases can refer to the methodology of high priority layer measurement requirement definition.

- Reference

- R4-2416861, WF on RRM requirements for NR_LPWUS, vivo