3GPP TSG-RAN WG4 Meeting # 113		R4-2418434

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.26.4.1

Source: 	CMCC

Title: 			   Discussion on the RRM requirement for NTN phase3

Document for:	Discussion

- Introduction

In last meeting, RAN4 discussed (e)RedCap UEs over NR FR1-NTN. The WF has been approved in [1].

In this contribution, we further discuss the RRM impact and specific requirements.

- General consideration on RRM requirements

Issue 2-2-1: Further discussion on cell reselection requirement for (e)RedCap UE with FR1-NTN

<Agreement>:

- The existing cell reselection requirements in RRC_IDLE/RRC_INACTIVE state defined for Rel-17 and Rel-18 (e)NTN can be the baseline for (e)RedCap 2Rx UE with FR1-NTN bands.

  - Both time-based and location-based cell reselection measurement criteria will be defined.

    - For both earth fixed and earth moving scenario

  - Both NTN-NTN and NTN-TN (from NTN to TN and from TN to NTN) cell reselection requirements will be defined.

  - FFS for the measurement requirements for inter-RAT (NTN to E-UTRA TN) measurement.

Considering some areas may not been covered by 5G network yet, e.g. desert, coastal, and forest area edge, which are typical NTN service cases, we support to also define the cell re-selection requirement for inter-RAT (NTN to E-UTRA TN) measurement, to guarantee the service consistency between NTN and TN network.

Proposal 1: Define the cell re-selection requirement for inter-RAT (NTN to E-UTRA TN) measurement.

Issue 2-2-3: The Requirements to be further determined

<Agreement>:

- Define NR Rel-17 and Rel-18 Conditional Handover requirements.

  - Include location-based CHO and time-based CHO.

  - Support earth moving cell scenario

- Define RACH-less Handover requirements.

- Define SDT requirements (including CG-SDT and RA-SDT) in RRC_INACTIVE state.

  - FFS TA validation based on the RSRP change criterion for RedCap in TN shall be removed and the requirements may refer to the PUR requirements for IoT for NTN.

- FFS: MDT requirements in RRC_IDLE and RRC_INACTIVE state.

Regarding CG-SDT, in TN network, the procedure may rely on TA validation based on the RSRP change criterion. However, in NTN network, due to in-apparent near-far-effect, the cg-SDT-RSRP-ChangeThreshold for TA validation methodology may not valid anymore.

Similar cases happens in IOT-NTN, at that time,  RAN4 agreed to use pur-TimeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE to determine the TA validation, the specification is as follows:

The UE is allowed to transmit using PUR provided that TA is valid provided that pur-TimeAlignmentTimer is not configured or pur-TimeAlignmentTimer is running as defined in [2] The UE shall determine the uplink timing, TTA, for transmitting on PUR using the valid values of NTA, NTA,offset, NTA,adjcommon and NTA,adjUE where TTA is defined in [16] which are the latest acquired values before PUR transmission.

We think similar approach can be referred for CG-SDT, that using cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE to determine the TA validation.

Proposal 2: For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN.

Regarding MDT, we think it is beneficial to support MDT capability for Redcap UE over NTN.

Firstly, For NTN network with large/moving cell coverage, it is barely impossible to use the traditional drive test, which requires support from both operation and testing equipment, the difficulty and cost of network optimization will increase a lot. While MDT could help network to optimize the scheduling with low cost.

Secondly, the link performance seen between handheld UE and Redcap UE may be different, especially for 1Rx Redcap and HD-FDD Redcap. Therefore, although NTN network has already have MDT report from handheld UE, the MDT report from Redcap UE can still provide more information to network.

Lastly, as mentioned in the summary of last meeting, TS 38.306 states that all other feature groups or components of the feature groups as captured in TR 38.822 [24] as well as capabilities specified in this specification remain applicable for RedCap UEs same as non-RedCap UEs, unless indicated otherwise. We think the MDT can be supported for RedCap UE, but if companies have different view on this, we are open to sent LS to RAN2 to check the capability.

- Proposal 3: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN, if the related features are supported in RAN1/2 spec.

Issue 2-3-1: RedCap R17 relaxed measurement

<Agreement>:

- FFS: R17 NR NTN relaxations in GSO (corresponding to the TN R16 relaxation) for normal UE are applicable for (e)RedCap UE with FR1-NTN bands.

- FFS: RAN4 to define Rel-17 RedCap relaxed measurement requirements for cell reselection for (e)RedCap UE with FR1-NTN bands, and the corresponding requirements defined for RedCap UE can be the baseline.

  - If defined, the requirements apply provided that target cell is GSO

We think both R17 NR NTN relaxation in GSO and R17 Redcap relaxed measurement requirement for (e)RedCap UE can be both supported in GSO scenario, with super large cell coverage and long service time, the power saving advantage can be maximally utilized.

Proposal 4: Support both R17 NR NTN relaxation in GSO and R17 Redcap relaxed measurement requirement for (e)RedCap UE in GSO deployment scenario.

Issue 2-3-2: eDRX configuration

<Agreement>:

- For GSO:

  - Define eDRX related requirements in RRC_Idle / RRC_Inactive mode for (e)RedCap UE with FR1-NTN.

    - For GSO, All DRX and eDRX cycle can be supported, and the DRX and eDRX related requirements for RedCap UE in TN are re-used.

- Further discuss for LEO.

In R17 Redcap WI, requirements were introduced for eDRX_IDLE up to 10485.76s and eDRX_INACTIVE cycles which up to 10.24 seconds. In R18 eRedcap WI, enhanced requirements were introduced for eDRX_INACTIVE up to 10485.76s.

In R17 and R18 NTN related WI, requirements were introduced for R16 DRX cycle, and 2.56s DRX cycle is not applicable for earth-moving LEO deployment due to limited serving time.

In our understanding, at least in GEO deployment, all eDRX cycle can be supported. In earth-fixed LEO deployment, considering about 300s service time per satellite, requirement are applicable for up to 5.12s eDRX cycle. In earth-moving LEO deployment, requirement are not applicable for eDRX cycle.

Proposal 5: Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario:

- For GEO deployment, all DRX and eDRX cycle can be supported.

- For earth-fixed LEO deployment, requirements are applicable for up to 5.12s eDRX cycle

- For earth-moving LEO deployment, requirements are not applicable for eDRX cycle

Issue 2-3-3: Multiple SMTCs

<Way Forward>:

- Proposal 1 (HW): RAN4 to discuss support of multiple SMTCs for (e)Redcap UE with FR1-NTN bands.

  - Option 1: make it optional for RedCap UE to support 2 SMTC per MO.

  - Option 2: change the requirements so that measurement for different SMTCs are not parallel.

- Proposal 2 (ZTE): Multiple SMTCs shall be reused for RedCap UEs in NTN scenario.

  - Proposal 2a (Ericsson): RedCap for NTN supports (at the least) 2 SMTC.

- Proposal 3 (OPPO): RAN4 to not consider the support of multiple SMTCs for (e)RedCap UE with FR1-NTN bands.

- Proposal 4 (QC): NTN RRM requirements defined for simultaneous measurements on multiple satellites are not applicable for RedCap NTN UE.

In R17 NTN, the number of target satellites UE needs to monitor per carrier is 2 including serving LEO satellite. Due to the propagation delay between LEO satellites are typically different, UE shall be capable of performing RRM measurements with at least 2 SMTCs with different offsets and same periodicity.

For Redcap over NTN, since the network scenario is unchanged, we think it is still necessary to guarantee the UE capability of monitor 2 target satellites including serving LEO satellite.

There are three ways to define the requirement:

- Alt1: Same as R17 NR-NTN: UE capable of performing RRM measurements with at least 2 SMTCs with different offsets and same periodicity, optionally support 4 SMTCs per MO.

- Alt2: New method: UE support 1 SMTC per MO as legacy TN, optionally support 2/4 SMTCs with different offsets and same periodicity per MO.

- Alt3: Same as R18 IOT-NTN: UE not capable of performing RRM measurements with SMTC-s, UE treat the intra-frequency neighbor NGSO satellite measurement as inter-frequency measurement.

- Considering the capability of Redcap UE is equal/lower than handheld UE, we are fine with using Alt1 or Alt2 to define the requirements.

Proposal 6: Define the measurement requirement with either of SMTC capability assumption as below:

- Alt1: Same as R17 NR-NTN: UE capable of performing RRM measurements with at least 2 SMTCs with different offsets and same periodicity, optionally support 4 SMTCs per MO.

- Alt2: New method: UE support 1 SMTC per MO as legacy TN, optionally support 2/4 SMTCs with different offsets and same periodicity per MO.

Issue 2-3-4: Concurrent gap

<Way Forward>:

- Proposal 1 (CMCC, HW): Consider concurrent gap for Redcap over NTN.

  - Proposal 1a (HW): Concurrent gaps are supported for (e)Redcap UE with FR1-NTN bands, and existing requirements for normal UE in NTN are re-used as baseline.

- Proposal 2 (CATT, QC): NTN RRM requirements defined for concurrent measurement gaps are not applicable for RedCap NTN UE.

- Proposal 3 (Ericsson): RedCap for NTN supports (at the least) 1 MG.

In R17 NTN, it is optional for UE to support 2 MGs, and one frequency layer can be associated to both concurrent measurement gaps with the same gap type. No need to define additional NTN UE capability for this association. This design could help measurement efficiency enhancement, since propagation delay difference between cells are typically quite large.

Although concurrent gap may need stronger UE capability, considering the network scenario is unchanged and it is an optional feature, we are fine to consider same design for Redcap over NTN. But to be mentioned, for the Redcap UE supporting 2 concurrent gaps, it shall mandatory to support 2 SMTCs per MO.

Proposal 7: Redcap UE optionally support 2 concurrent MGs, and one frequency layer can be associated to both concurrent measurement gaps with the same gap type. No need to define additional NTN UE capability for this association.

Proposal 8: For the Redcap UE supporting 2 concurrent gaps, it shall mandatory to support 2 SMTCs per MO.

Issue 2-3-5: The number of searchers and CSSF

<Way Forward>:

- Proposal 1 (CMCC, CATT): Single searcher assumption and the CSSF defined for (e)RedCap UEs in TN will be reused for (e)RedCap UE with FR1-NTN.

The CSSF is a fundamental parameter for further defining the measurement requirements. For Redcap UE, it has been defined in Clause 9.1A.5 TS 38.133, which only considering NR SA single carrier mode. We think it can be the starting point for CSSF of Redcap over NTN.

Proposal 9: Define the CSSF for Redcap over NTN, the legacy CSSF for Redcap can be the baseline.

Issue 2-3-7: NCD-SSB

In the last meeting, RAN4 has the following agreements for NCD-SSB according to the WF:

#### Issue 6-6-1: The impact of NCD-SSB for (e)RedCap UEs with FR1-NTN Online agreement <Agreement> - For (e)RedCap UEs with FR1-NTN, there is no need to define NCD-SSB specific measurement requirements.

Therefore, for this issue, suggest RAN4 to further focus on how to understand ‘no need to define NCD-SSB specific measurement requirements’ agreed in the last meeting and how to draft the CR.

<Way Forward>:

- Option 1: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, and add a clarification in a separate clause on this aspect (i.e, the requirements are only applicable to CD-SSB).

- Option 2: Use the terminology of CD-SSB explicitly for (e)RedCap with FR1-NTN requirements.

- Note: Other options are not precluded.

Based on our understanding, NCD-SSB is not the key feature for Redcap over NTN, because the bandwidth of NTN bands are less than 30MHz. There is no need to introduce NCD-SSB specific requirements, for example, following NCD-SSB specific requirements are specified in Clause 6.1D TS 38.133:

The requirements in this clause apply for the following handover scenarios: Handover to a target cell’s initial BWP associated with CD-SSB; Handover to a target cell’s specific Redcap BWP associated with NCD-SSB besides to the initial BWP associated with CD-SSB (i.e. UE directly sync to the NCD-SSB and perform RACH on that BWP). - Trs is the SMTC configured in the measObjectNR having the same SSB frequency and subcarrier spacing as NCD-SSB indicated by nonCellDefiningSSB-r17 if the first active DL BWP included in handover command is configured with nonCellDefiningSSB-r17, otherwise, as CD-SSB indicated by absoluteFrequencySSB in frequencyInfoDL in handover command.

Instead, we propose to use the terminology of SSB without differentiating CD-SSB/NCD-SSB as R17 NTN, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

Proposal 10: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

Issue 2-3-8: Other features

<Way Forward>:

- Proposal 1 (CMCC): For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed.

For R17 and R18 NTN, RAN1 to RAN4 introduced a lot of new UE features and related requirements for network access, measurement, mobility, and so on. For the optional NTN features below which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether define RAN4 requirements should be further discussed.

In current stage, we think Set 1 features can be optionally supported by Redcap UE, and details can be further discussed. Set 2 features need further discussion.

Set 1:

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

- The number of target LEO satellites the UE can monitor per carrier including serving satellite

  - Further discuss the max number

Set 2:

- Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell

- Parallel measurements on multiple NGSO satellites within a SMTC

Proposal 11: For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed.

Set 1: propose to be optionally supported by Redcap over NTN

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

- The number of target LEO satellites the UE can monitor per carrier including serving satellite

  - Further discuss the max number

Set 2: FFS

- Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell

- Parallel measurements on multiple NGSO satellites within a SMTC

- Reduction in the number of UE Rx branches

In RAN4#112, RAN4 has agreed that the same principle for (e)RedCap UE in TN network for 1Rx and 2 Rx (e)Redcap UE will be followed as baseline, in the following part, we will discuss how to define the specific requirements by following this principle.

Issue 3-2-1: RRC_IDLE/RRC_INACTIVE mode mobility for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, firstly to discuss the following proposals one by one:

  - Proposal 1: RAN4 to consider the offset for the reletive cell-specific RSRP thresholds for 1Rx (e)RedCap UEs with FR1-NTN bands, including Qrxlevmin and Qqualmin:

    - -1 dB offset can be reused.

  - Proposal 2: RAN4 to reduce the number of NR inter-frequency carriers that UE shall at least be capable of monitoring for 1Rx (e)RedCap UEs with FR1-NTN bands.

    - 6 NR inter-frequency carriers can be reused.

  - Proposal 3: The paremeters of Nserv_RedCap, Tdetect,NR_Intra/Inter_RedCap, Tmeasure,NR_Intra/Inter_RedCap and Tevaluate,NR_Intra/Inter_RedCap defined for cell re-selection requirements will be not distinguished between 1Rx and 2 Rx (e)Redcap UE, and the requirements will be the same.

  - Proposal 4: For the measurements of intra- and inter-frequency NR cells,

    - +1 dB offset on the threshold of absThreshSS-BlocksConsolidation can be reused as a baseline for 1Rx(e)RedCap UEs with FR1-NTN bands.

    - For the cases of rangeToBestCell is not configured and rangeToBestCell is configured, +1 dB offset can be reused for better cell ranked for 1Rx (e)RedCap UEs with FR1-NTN bands.

  - Proposal 5: In RRC_IDLE / RRC_INACTIVE state, +1 dB offset can be reused for a margin in FR1 for reselections when UE evaluating whether a newly detectable inter-frequency cell meets the reselection criteria for 1Rx (e)RedCap with FR1-NTN:

    - For 1 Rx RedCap by a margin of at least

      - 6 dB in FR1 for reselections based on ranking or

      - 7 dB in FR1 for SS-RSRP reselections based on absolute priorities or

      - 5 dB in FR1 for SS-RSRQ reselections based on absolute priorities.

  - Further discuss SDT requirements.

  - MDT requirements are related to issue 2-2-3, RAN4 to discuss them based on the conclusions on issue 2-2-3 later.

For RRC_IDLE/RRC_INACTIVE mode mobility, except the impact from some optional features which haven’t discussed or agreed so far, for instance, SMTC adjustment in idle/inactive, Enhanced RRM requirements for measurements in IDLE and INACTIVE modes:

For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs.

For 1Rx (e)RedCap UEs with FR1-NTN bands, the proposal 1 to proposal 5 exactly follow the same principle for (e)RedCap UE in TN network, we support all of them.

Proposal 12: For RRC_IDLE/RRC_INACTIVE mode mobility, except the impact from some optional features which haven’t discussed or agreed so far, for instance, SMTC adjustment in idle/inactive, Enhanced RRM requirements for measurements in IDLE and INACTIVE modes:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support the proposal 1 to proposal 5 from RAN2#112bis WF.

For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 can be reused for 1 Rx RedCap UE for determining whether to perform SDT procedure.

Proposal 13: For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

Issue 3-2-2: NR HO requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For NR RACH-based HO requirements,

  - For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RACH-based HO requirements for normal UEs.

  - For 1Rx (e)RedCap UEs with FR1-NTN bands, RACH-based handover delays should be extended compared to those defined for 2Rx (e)RedCap UEs.

    - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

      - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

      - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

- Further discuss the requirements for RACH-less HO, Time/location-based CHO with/without L3 measurement and Satellite switching with re-sync.

For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-sync:

For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy corresponding requirements for normal UEs.

For 1Rx (e)RedCap UEs with FR1-NTN bands, requirements should be extended compared to those defined for 2Rx (e)RedCap UEs.

- Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

  - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

  - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

Proposal 14: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-sync:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy corresponding requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, requirements should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

    - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

For Time/location-based CHO with L3 measurement, the 1Rx/2Rx Tidentify time defined in Section 9 shall be referred for 1Rx/2Rx case respectively.

Proposal 15: For Time/location-based CHO with L3 measurement, the 1Rx/2Rx Tidentify time defined in Section 9 shall be referred for 1Rx/2Rx case respectively.

Issue 3-2-3: RRC Re-establishment requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC re-establishment delay requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, UE RRC re-establishment delay (TUE_re-establish_delay) should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tidentify for known and unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax the Tidentify_intra_NR and Tidentify_inter_NR, i by 1 more samples for FR1 known and unknown intra/inter-frequency cell.

Issue 3-2-4: RRC Connection Release with redirection requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RRC Connection Release with redirection requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, the delay of RRC connection release with redirection to NR (Tconnection_release_redirect_NR) should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tidentify-NR for FR1, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax the Tidentify-NR by 1 more samples for FR1.

Issue 3-2-5: RA requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy random access requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to consider the offset for the following relative cell-specific RSRP thresholds for random access requirements

  - +1 dB offset can be reused as a baseline for rsrp-ThresholdSSB, msgA-RSRP-ThresholdSSB and msgA-RSRP-Threshold.

  - -1 dB offset can be reused as a baseline for rsrp-ThresholdMsg3.

Issue 3-3-1: Timing requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- The existing timing requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands.

  - UE transmit timing

  - UE timer accuracy

  - Timing Advance

- The timing requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Issue 3-4-1: RLM/BFD/CBD requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN RLM/BFD/CBD requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands,

  - For SSB based and CSI-RS based RLM/BFD requirements, some of hypothetical PDCCH parameters including aggregation level (CCE) and bandwidth (PRBs) are defined as follows:

    - Aggregation level (CCE) of 16.

    - Bandwidth (PRBs) of 48.

  - For SSB based and CSI-RS based RLM/BFD requirements, SCS=60kHz is not applicable for (e)RedCap UEs with FR1-NTN.

  - The SSB and CSI-RS based RLM OOS and BFD evaluation periods should be extended for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - For RLM evaluation periods,

      - RAN4 not to extend the TEvaluate_in_SSB/CSI-RS for FR1.

      - RAN4 to extend the TEvaluate_out_SSB/CSI-RS by factor 2 for FR1.

    - For BFD evaluation periods,

      - RAN4 to extend the TEvaluate_BFD_SSB/CSI-RS by factor 2 for FR1.

Issue 3-4-2: Other requirements in signaling characteristics for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- The existing following requirements defined for NTN can be reused for (e)RedCap UEs with FR1-NTN bands:

  - Active BWP switch delay,

  - active TCI state switch delay

  - UE specific CBW change requirements

  - Pathloss reference signal switching delay

- The above requirements defined for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same, and there is no need to differentiate 1Rx/2Rx requirements.

Issue 3-6-1: Measurement accuracy relaxation for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, RAN4 to reuse the measurement accuracy requirements defined in NTN.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support to relax the measurement accuracy for SS-RSRP, SS-RSRQ, SS-SINR and L1-RSRP compared to those defined for 2Rx (e)RedCap UEs.

  - The same relaxation defined in TN for 1Rx (e)RedCap UEs can be reused.

    - SS-RSRP, SS-RSRQ, SS-SINR: relax by 1dB.

    - L1-RSRP: relax by 3dB.

- RAN4 to reuse the conditions defined in NTN for a corresponding Band, and the values for SSB_RP and SSB Ês/Iot are the same for 1Rx (e)RedCap UEs and 2Rx (e)RedCap UEs with FR1-NTN bands.

For RRC Re-establishment requirements, RRC Connection Release with redirection requirements,RA requirements, timing requirements, RLM/BFD/CBD requirements, Measurement accuracy requirement and Other requirements in signaling characteristics including Active BWP switch delay, active TCI state switch delay, UE specific CBW change requirements, Pathloss reference signal switching delay, the way forward from last meeting exactly follow the same principle for (e)RedCap UE in TN network, we support all of them.

Proposal 16: For RRC Re-establishment requirements, RRC Connection Release with redirection requirements,RA requirements, timing requirements, RLM/BFD/CBD requirements, Measurement accuracy requirement and Other requirements in signaling characteristics including Active BWP switch delay, active TCI state switch delay, UE specific CBW change requirements, Pathloss reference signal switching delay, support the way forward from last meeting.

Issue 3-5-1: Measurement requirements for 1Rx/2Rx (e)RedCap UEs with FR1-NTN

<Way Forward>:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN measurement requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, to discuss the following proposals one by one:

  - The time period for PSS/SSS detection defined for intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Increase the number of attempts by 2 SSB samples for TPSS/SSS_sync_intra and TPSS/SSS_sync_inter.

      - Not to extend the lower boundary.

  - The time period for SSB index identification (PBCH decoding) defined in intra-/inter-frequency measurement requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Increase number of attempts by 3 SSB samples for TSSB_time_index_intra and TSSB_time_index_inter.

      - Extend the lower boundary from ‘120 ms’ to ‘160ms’.

  - The measurement period defined in inter-frequency measurement with measurement gaps requirements should be relaxed for 1Rx (e)RedCap UEs with FR1-NTN bands compared to those defined for 2Rx (e)RedCap UEs.

    - Reuse the same extensions for 1Rx RedCap UE in TN network.

      - Extend the lower boundary from ‘200 ms’ to ‘400ms’

      - Keep the same number of samples for T SSB_measurement_period_inter.

  - The measurement period defined in intra-frequency measurement with/without gap and inter-frequency measurement without gaps requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

  - The SSB and CSI-RS based L1-RSRP measurement requirements for 1Rx and 2Rx (e)RedCap UEs with FR1-NTN bands will be the same.

For measurement requirements RRC_CONNECTED mode, except the impact from some optional features which haven’t discussed or agreed so far, for instance, parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell, and Parallel measurements on multiple NGSO satellites within a SMTC, the way forward from last meeting exactly follow the same principle for (e)RedCap UE in TN network, we support all of them.

Proposal 17: For measurement requirements RRC_CONNECTED mode, except the impact from some optional features which haven’t discussed or agreed so far, for instance, parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell, and Parallel measurements on multiple NGSO satellites within a SMTC, support the way forward from last meeting.

- The impact of HD-FDD

Issue 4-1-1: General HD-FDD related impact

<Way Forward>:

- Proposal 1 (Xiaomi, CATT, QC, ZTE, CMCC):

  - RAN4 to check whether existing HD-FDD related requirements and applicable conditions can be reused or not after RAN1 reaching further conclusions.

  - Proposal 1a (CATT):

    - To following the TN, at least the following requirements will be affected by HD-FDD for (e)RedCap UE with FR1-NTN:

      - Paging reception requirements in RRC_IDLE/ RRC_INACTIVE state

      - Handover interruption time

      - Random access

      - SA: RRC Connection Release with Redirection

      - Minimum requirement for L1 indication for RLM and LR

      - MAC-CE/DCI/RRC based uplink spatial relation switch delay

      - Scheduling availability of UE performing intra/inter measurements

      - Scheduling availability of UE during L1-RSRP measurement

  - Proposal 1b (QC): RAN4 to wait for further RAN1 process on the enhancements for HD RedCap and eRedCap UEs which may potentially impact the following aspects:

    - Additional latencies, Scheduling/Measurement restrictions, etc. due to DL/UL collisions.

Firstly, we think the same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

Proposal 18: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

For NTN specific aspects, the DL and UL collision error due to mismatch between actual TA used by UE and assumed TA at the gNB may need to further considered. RAN1 is discussing the solutions and TA reporting enhancement, which may impact Scheduling availability requirement, we can wait for more progress. The current progress is listed for information:

Agreement The collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) can’t be assumed as error case. When the collision happens at UE side for collision case 3, one of the following options on priority rules is supported. - Option A: the priority of keeping one direction and overriding another direction is based on different use cases (e.g. the collision happening in different channel types or different scheduling cases) without network signalling - It is not precluded that some or all use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - Note: it is not precluded that the same direction is prioritized for all different use cases - Option B: network indicates the priority configuration to the UE - It is not precluded that some use cases are left to UE implementation regarding whether DL reception or UL transmission is dropped - It is not precluded to define default rules, if necessary Agreement To mitigate the collisions of case3 and case4, the following TA reporting enhancements for Rel-19 NTN HD-FDD (e)Redcap UE can be further studied: - Finer TA report granularity - Smaller TA offset threshold - Trigger a TA report when the collision is detected at the UE - TA reporting with a new triggering mechanism from gNB - TA drifting rate reporting Note: The benefit, complexity, power consumption and signaling overhead of each scheme is to be further investigated. Note: other solutions can also be studied Conclusion The different use cases (e.g. the collision happening in different channel types or different scheduling cases) from RAN1#118 agreement for collision case 3 (Semi-statically configured DL reception collides with semi-statically configured UL transmission) consist of pairs of one semi-statically configured DL reception and one semi-statically configured UL transmission, among: - Semi-statically configured DL reception includes Type-0/0A/0B/1/2-PDCCH CSS and dedicated semi-statically configured PDCCH USS/PDSCH SPS/CSI-RS/PRS/[Type 3 PDCCH CSS] - Semi-statically configured UL transmission includes semi-statically configured PUSCH/PUCCH/SRS

Proposal 19: Wait for further RAN1 progress which may impact Scheduling availability requirement.

Issue 4-2-2: The impact on NR Satellite switching with re-synchronization due to HD-FDD

<Way Forward>:

- Proposal 1 (vivo): For requirements on NR Satellite switching with re-synchronization for Red-cap NTN, RAN4 to reuse the existing Satellite switching with re-synchronization requirements for NTN with adding the following applicability rules to adapt HD-FDD case:

  - For RedCap NTN UE with HD-FDD, the Satellite switching with re-synchronization requirements are met provided that

    - SSB is available at the UE once every SMTC period during Tsearch

    - One SSB is available during T∆

    - One SSB is available during TIU.

- For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

- For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 20: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 21: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU

- Others

Issue 5-2-1: How to declare the capabilities for (e)RedCap UE with FR1-NTN bands?

<Way Forward>:

- Proposal 1 (Nokia):

  - RAN4 to discuss whether a new capability is needed to declare RedCap support of NTN or whether this support is implicit from declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) AND declare support of NTN (nonTerrestrialNetwork-r17)

For (e)Redcap UE capability declaring, both way as stated in proposal 1 is workable. Considering the principle of no need to introduce new capability if existing capability can be used, we slightly prefer the implicit way to save the workload of RAN2.

Proposal 22: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Conclusion

Based on the discussion above, the following observations and proposals are concluded.

Proposal 1: Define the cell re-selection requirement for inter-RAT (NTN to E-UTRA TN) measurement.

Proposal 2: For CG-SDT TA validation, define the requirement based on cg-SDT-timeAlignmentTimer and valid values for NTA, NTA,offset, NTA,adjcommon and   NTA,adjUE , specific approach can refer to the PUR requirements for IOT-NTN.

- Proposal 3: Define the MDT requirements in RRC_IDLE and RRC_INACTIVE state for Redcap over NTN, if the related features are supported in RAN1/2 spec.

Proposal 4: Support both R17 NR NTN relaxation in GSO and R17 Redcap relaxed measurement requirement for (e)RedCap UE in GSO deployment scenario.

Proposal 5: Support eDRX configuration for Redcap over NTN, and define following applicability rule for each deployment scenario:

- For GEO deployment, all DRX and eDRX cycle can be supported.

- For earth-fixed LEO deployment, requirements are applicable for up to 5.12s eDRX cycle

- For earth-moving LEO deployment, requirements are not applicable for eDRX cycle

Proposal 6: Define the measurement requirement with either of SMTC capability assumption as below:

- Alt1: Same as R17 NR-NTN: UE capable of performing RRM measurements with at least 2 SMTCs with different offsets and same periodicity, optionally support 4 SMTCs per MO.

- Alt2: New method: UE support 1 SMTC per MO as legacy TN, optionally support 2/4 SMTCs with different offsets and same periodicity per MO.

Proposal 7: Redcap UE optionally support 2 concurrent MGs, and one frequency layer can be associated to both concurrent measurement gaps with the same gap type. No need to define additional NTN UE capability for this association.

Proposal 8: For the Redcap UE supporting 2 concurrent gaps, it shall mandatory to support 2 SMTCs per MO.

Proposal 9: Define the CSSF for Redcap over NTN, the legacy CSSF for Redcap can be the baseline.

Proposal 10: Use the terminology of SSB without differentiating CD-SSB/NCD-SSB, no need to add additional requirement applicability about CD-SSB/NCD-SSB.

Proposal 11: For the optional NTN features which have impact on RAN4 requirements and need enhanced UE capability, whether Redcap UE could optionally support it and whether RAN4 need to define requirements should be further discussed.

Set 1: propose to be optionally supported by Redcap over NTN

- SMTC adjustment in idle/inactive

- Location-based measurement report trigger

- Enhanced RRM requirements for measurements in IDLE and INACTIVE modes

- The number of target LEO satellites the UE can monitor per carrier including serving satellite

  - Further discuss the max number

Set 2: FFS

- Parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell

- Parallel measurements on multiple NGSO satellites within a SMTC

Proposal 12: For RRC_IDLE/RRC_INACTIVE mode mobility, except the impact from some optional features which haven’t discussed or agreed so far, for instance, SMTC adjustment in idle/inactive, Enhanced RRM requirements for measurements in IDLE and INACTIVE modes:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy FR1-NTN requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, support the proposal 1 to proposal 5 from RAN2#112bis WF.

Proposal 13: For SDT requirements, +1 dB offset on the threshold of sdt-RSRP-Threshold-r17 for 1 Rx RedCap UE.

Proposal 14: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-sync:

- For 2Rx (e)RedCap UEs with FR1-NTN bands, reuse the legacy corresponding requirements for normal UEs.

- For 1Rx (e)RedCap UEs with FR1-NTN bands, requirements should be extended compared to those defined for 2Rx (e)RedCap UEs.

  - Define the relaxation on Tsearch for unknown intra/inter-frequency cell, and the same relaxation for 1 Rx RedCap UE in TN network can be reused.

    - Relax by 1 sample: Tsearch = 2*Trs for intra-frequency HO

    - Relax by 2 samples: Tsearch = 5* Trs for inter-frequency HO

Proposal 15: For Time/location-based CHO with L3 measurement, the 1Rx/2Rx Tidentify time defined in Section 9 shall be referred for 1Rx/2Rx case respectively.

Proposal 16: For RRC Re-establishment requirements, RRC Connection Release with redirection requirements,RA requirements, timing requirements, RLM/BFD/CBD requirements, Measurement accuracy requirement and Other requirements in signaling characteristics including Active BWP switch delay, active TCI state switch delay, UE specific CBW change requirements, Pathloss reference signal switching delay, support the way forward from last meeting.

Proposal 17: For measurement requirements RRC_CONNECTED mode, except the impact from some optional features which haven’t discussed or agreed so far, for instance, parallel measurements on cells belonging to a different NGSO satellite than a serving satellite without scheduling restrictions on normal operations with the serving cell, and Parallel measurements on multiple NGSO satellites within a SMTC, support the way forward from last meeting.

Proposal 18: Same principle for HD-FDD (e)RedCap UE in TN network will be followed as baseline.

Proposal 19: Wait for further RAN1 progress which may impact Scheduling availability requirement.

Proposal 20: For RACH-based HO, RACH-less HO, Time/location-based CHO without L3 measurement and Satellite switching with re-synchronization, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - SSB is available at the UE once every SMTC period during Tsearch

        - One SSB is available during T∆

        - One SSB is available during TIU.

Proposal 21: For Time/location-based CHO with L3 measurement, following applicability rules to adapt HD-FDD case should be added:

      - For RedCap NTN UE with HD-FDD, the requirements are met provided that

        - One SSB is available during T∆

        - One SSB is available during TIU

Proposal 22: Declare the capabilities for (e)RedCap UE with FR1-NTN bands through implicitly by declaring support of RedCap (supportOfRedCap-r17 or supportOfERedCap-r18) and support of NTN (nonTerrestrialNetwork-r17).

- Reference

- R4-2416865, WF on RRM requirements for NR_NTN_Ph3_Part1, Moderator (CATT)