# 3GPP TSG RAN WG4 Meeting#113	R4-2418438

# Orlando, US, Nov 18 – Nov 22, 2024

Source: 	Qualcomm Incorporated

Title: 	Discussion on NTN testing for NGSO

Agenda item:	7.8.4

Document for:	Discussion

# 1. Introduction

During the last meeting, RAN4 discussed issues related to propagator models and test procedures. In this paper, we share our views on the remaining open issues.

2. TE Emulated Channel Model

2.1 Time Varying Doppler and Delay Shift Modelling

Methodology for Time varying Doppler and Delay shifts modelling

- RAN4 aims to introduce TE-emulated time varying doppler shift and propagation delay model matching a satellite motion with following option:

    - Satellite motion emulated using the two-body Keplerian propagator model, [i.e., spherical gravity model]

- RAN4 further discuss how to derive the Doppler shift and propagation delay variation based on Keplerian model with the necessary parameters and details procedure

- Companies are encouraged to provide the two-body Keplerian propagator model to be implemented in the TE, to derive the Doppler shift and propagation delay.

During the last meeting, RAN4 discussed several proposals from companies in regard to modelling the time varying Doppler and delay shift for satellite mobility. The relevant way forward is presented below [1].

At the last RAN4 meeting, we observed a strong desire from companies to see the procedure and formulas from the step of obtaining initial ephemeris data (in both formats supported in SIB19/31) to the step of generating propagation delay and Doppler shift at any point in time. Below, we present the details for each step for generating time-varying propagation delay and Doppler shift based on satellite motion and UE location.

  - Step A: The initial ephemeris for each orbit is generated by either GMAT or any numerical method

  - Step B: The orbit information is generated based on the two-body Keplerian model

  - Step C: Calculate the Doppler shift and propagation delay based on the generated orbit and UE location

For the obtained initial ephemeris, we consider both formats provided in Table 1 and Table 2. And the detailed steps from Step B and Step C, we follow the symbols defined in the tables.

Table 1. Initial ephemeris information in format of position and velocity state vector in ECEF (aka PVT)

| Parameters | Symbols | Unit | Note |
| --- | --- | --- | --- |
| positionX-r17 | rx | meter | 26 bits with 1.3 m resolution, i.e. actual value = field value*1.3 |
| positionY-r17 | ry | meter | 26 bits with 1.3 m resolution, i.e. actual value = field value*1.3 |
| positionZ-r17 | rz | meter | 26 bits with 1.3 m resolution, i.e. actual value = field value*1.3 |
| velocityVX-r17 | vx | meter/second | 18 bits with 0.06 m/s resolution, i.e. actual value = field value*0.06 |
| velocityVY-r17 | vy | meter/second | 18 bits with 0.06 m/s resolution, i.e. actual value = field value*0.06 |
| velocityVZ-r17 | vz | meter/second | 18 bits with 0.06 m/s resolution, i.e. actual value = field value*0.06 |
| Note: The symbol represents the actual value derived from the value for the corresponding to the parameter |  |  |  |

Table 2: Initial ephemeris information in format of orbital parameters in ECI (aks Keplerian orbital elements)

| Parameters | Symbols | Unit | Note |
| --- | --- | --- | --- |
| semiMajorAxis-r17 | a | meter | 33 bits with 4.249*10-3 m resolution, i.e. actual value = 6500000+field value*4.249*10-3 |
| eccentricity-r17 | e | NA | 20 bits with 1.431*10-8 resolution, i.e. actual value = field value*1.431*10-8 |
| periapsis-r17 | ω | radian | 28 bits with 2.341*10-8 rad resolution, i.e. actual value = field value*2.341*10-8 |
| longitude-r17 | Ω | radian | 28 bits with 2.341*10-8 rad resolution, i.e. actual value = field value*2.341*10-8 |
| inclination-r17 | i | radian | 27 bits with 2.341*10-8 rad resolution, i.e. actual value = field value*2.341*10-8 |
| meanAnomaly-r17 | M | radian | 28 bits with 2.341*10-8 rad resolution, i.e. actual value = field value*2.341*10-8 |
| Note: The symbol represents the actual value derived from the value for the corresponding to the parameter |  |  |  |

In Step B where we derive satellite’s position and velocity vectors, we use two-body Keplerian model. We note that the mean anomaly M can be computed from the eccentric anomaly E and the eccentricity e with Kepler’s equation as below:

M = E – e sinE

The classical method of finding the position of an object in an elliptical orbit from a set of orbital elements is to calculate the mean anomaly M from mean angular motion n, and then to solve Kepler’s equation for the eccentric anomaly E. Therefore, the true anomaly can be computed from the eccentric anomaly E and the eccentricity e as below:

v =2tan-11+e1-etanE2

We note that numerous methods of propagation exist that cater to varying degrees of modelling and accuracy requirements. The following describes an algorithm for elementary Keplerian propagation (2-body gravity). Trading off the cost of accuracy, this algorithm is simple to implement and is expected to be sufficient to the needs of Satellite motion emulation at TE.

With this, we follow the below procedure in Table 3 to generate the propagation delay and Doppler shift.

- Step 1: Conversion of the ephemeris format from the ephemeris data into Kepler elements. - From the conversion, the initial Kepler element vector can be denoted as (a,e,ω,Ω,i,M0) at epoch time T0 - Note: The two formats in Table 1 and Table 2 are interchangeable. If the format is based on Table 1, this step can be skipped, and the values in Table 2 can directly replace the above Kepler element vector. - Step 2: Calculate mean angular rate n. - n=μEa3 where μE is Earth’s gravitational parameter and equal to 3.986*1014 m3s-2 - Step 3: Obtain the propagated mean anomaly M at the time instance t. - M=M0+n*(t-T0) - Step 4: Solve Kepler’s equation for new eccentric anomaly M. - A numerical method is required to solve the equation, and Newton-Raphson method is chosen in this example. - Input: (M, e, ϵ), Output: E - Initialize E as E=M. Repeat the below calculation while E-E0>ϵ, and for e being not larger than 0.015 as per TS38.133 the below can converge with an accuracy requirement of ϵ=10-8 with 4 iterations. - E0=E, E=E0+M-E0+esinE01-ecosE0 - Step 5: Obtain new true anomaly. - ν=2tan-11+e1-etanE2 - Step 6: Obtain position and velocity state vectors corresponding to the Kepler elements (a,e,ω,Ω,i,ν) - Step 7: Rotate the position vector r=[rx,ry,rz]T and velocity vector v=(vx,vy,vz)T in ECI to the vectors r' and v’ in ECEF coordinates. - Step 8: Calculate propagation delay and delay rate at T0 and T1=T0+1 sec - Calculate propagation delay τ(T0) and τ(T1) based on the following equation: - τt=d'relc-we<k , r'tx(ttx)×r'rxttx>d'rel where d'rel=r'rxttx-r'tx(ttx) - Calculate propagation delay rate τ(T0) and τ(T1) based on the following equation: - τt=<v'rel+wek×d'rel ,d'rel>c where d'rel=d'rel/d'rel - Calculate Doppler shift fD(T0) and fD(T1) based on the following equation: - fDt=-fc*τt - Step 9: Apply cubic spline interpolation to obtain propagation delay and Doppler shift at any time T0<t<T1

Note that in order to avoid unnecessarily heavy computational complexity overhead, we have considered an interval of 1 second for solving the Kepler equation, followed by cubic spline interpolation. With this approach, the maximum errors in delay and delay rate during the 1-second interval with cubic spline interpolation are limited as follows:

- Max|delay-error| ~ 3e-13 s

- Max|delay-rate error| ~ 3.28e-12

Proposal 1: RAN4 to adopt the following for the emulation of satellite motion-based time-varying propagation delay and Doppler shift:

- Use a two-body Keplerian propagator model to generate the satellite position/velocity vectors from the initial ephemeris data.

- Follow the steps provided in Table 3 to derive the time-varying propagation delay and Doppler shift based on the generated satellite position/velocity vectors and UE location.

2.2 Assumption of UE Motion and UE Position

Assumption of UE Motion and UE Position

- Same UE location information should be set for both DUT and TE before the test start with the following initial position

- Assuming UE is stationary during the demodulation and RF requirement testing

- FFS on the assuming UE is in motion during the RRM requirement testing

Several assumptions were discussed for the UE motion and position, The related way forward is copied below. As in Rel-17 NR NTN assumptions, we think that the UE location should be stationary for RF and demod tests. For RRM tests, UE may be stationary or mobile depending on the test purposes, e.g., distance-based measurement report/mobility for earth-fixed cell.

Proposal 2: For RRM tests, consider stationary or mobile UE depending on the test purposes.

2.3 Assumption of Elevation Angle Range

The related way forward for this issue is copied below. We note that the minimum angle may differ for demodulation and RRM test cases depending on the test purposes.

Parameters for TE-emulated Channel Model

  - Option 1: In the configuration for NGSO test cases, the satellite information for the serving cell shall be initiated at a position where the elevation angle measured by the UE at the chosen GNSS position used for the test is below [35] degrees and the satellite is moving away from the UE.

    - This configuration applies regardless if the UE position is obtained via AT command or via GNSS simulator.

    - The initial elevation angle might be increased if needed for guaranteeing the GNSS is within the limit of 30 degrees throughout the entire duration of the test.

- Option 2: To align with Rel-17 NR NTN channel model discussion, the elevation angle with 30 degrees and 90 degrees can be considered for testing. Large testing period is required for IoT NTN testing

- Other options are not precluded.

Proposal 3: Consider potentially different minimum elevation angle for demodulation and RRM test cases.

We note that in previous releases, e.g., in Rel-17 NR NTN, 30-degree elevation was considered for both demodulation and RRM test cases. Therefore, we prefer to consider 30-degree elevation angle, unless it can be justified otherwise.

Proposal 4: Consider 30-degree elevation angle.

2.4 Assumption of Multiple Satellite Scenario

The related way forward for this issue is copied below. We note that for RRM test cases, e.g., inter-satellite handover/measurements, it may be necessary to consider multiple satellite scenario. Therefore, in general, we support considering multiple satellite scenario.

Assumption of Multiple satellite scenario

- RAN4 focuses TE emulated channel modeling for one satellite

- FFS on the TE emulated channel modelling for multiple satellite scenario, considering the serving satellite changes.

Proposal 5: Consider multiple satellite scenario after TE emulated channel modelling for one satellite has been understood and settled.

2.4 Test Procedure

  - RAN4 further discuss on general test principle and how to generate channel model to match with satellite motion trajectory with the following options

    - Option 1: The entire N orbits within a test time of X seconds are generated as follow

      - The initial ephemeris for each orbit is generated by either GMAT or any numerical method.

      - The initial ephemeris for each orbit should be chosen such that the elevation angle from the UE perspective is not smaller than Z degrees. The value of Z may vary for different test cases. And if multiple satellites (i.e. N > 1) are needed in RRM mobility/measurement test cases, the value of Z may be different for different satellites.

      - The ephemeris data after the initial data is generated based on the two-body Keplerian model at every Y sec. The value of Y may differ for NR NTN and IoT NTN.

      - The propagation delay and Doppler shift at every Y sec are derived based on the projected ephemeris data and given UE location

      - In case Y is larger than the slot length of interest in the test case due to, e.g., test equipment/channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying, e.g., linear interpolation between the propagation delays and Doppler shifts at Y sec intervals.

    - Other options are not precluded

During the last RAN4 meeting, test procedure related to TE-emulated channel model was discussed. The way forward for this issue is copied below.

A diagram showing the test procedure is presented below.

Figure 1: Pictorial view of the test procedure for the two-body Keplerian-based propagator model

As seen in Fig. 1, we consider a test time of X seconds containing a total of N orbits. The initial ephemeris for each orbit is generated by either GMAT or any numerical method. While doing so, it is also necessary to make sure that the elevation angle is not very small, as otherwise UE will not be satellite’s view. The subsequent ephemeris data after the initial ephemeris will be generated based on the two-body Keplerian model at a certain interval, at which point the propagation delay and Doppler shift will be derived based on the predicted ephemeris. If the prediction window is larger than the slot length of interest in the test case, e.g., due to test equipment or channel emulator complexity, the propagation delay and Doppler shift at the beginning of each slot can be generated by applying linear interpolation between the propagation delays and Doppler shifts for that interval. Such a proposal is captured in option 1.

Note that in Table 3, we considered 1 second for the value of X and used cubic spline interpolation to obtain the propagation delay and Doppler shift at the beginning of each slot length.

Proposal 6: Support option 1

3. RAN4 Requirements and Tests

3.1 UE Assumption for the Propagator Model

- Option 1: Consider Keplerian model for UE assumption on the propagator model

- Option 2: Evaluate the performance based on the agreed TE emulated channel model first. It is not necessary to have assumptions on the propagator model for satellite motion at the UE side if the simulation results from companies based on the agreed TE emulated channel model are within some span.

During the last meeting, RAN4 briefly discussed the assumptions on the UE for the propagation model. The related way forward is copied below.

To align results across companies during the propagator model evaluation phase, it would be useful to consider a certain propagator model at the UE side. To this end, our preference is to consider Keplerian propagator model as it offers a good trade-off between implementation complexity and performance. We note that this does not prevent the UE from implementing a more accurate propagator model than Keplerian.

Proposal 7: Support option 1 (Keplerian model for UE assumption on the propagator model)

4. Conclusions

In this paper, we share our views on the performance requirements for NTN testing for UE supporting NGSO scenarios. The following have been proposed.

Proposal 1: RAN4 to adopt the following for the emulation of satellite motion-based time-varying propagation delay and Doppler shift:

- Use a two-body Keplerian propagator model to generate the satellite position/velocity vectors from the initial ephemeris data.

- Follow the steps provided in Table 3 to derive the time-varying propagation delay and Doppler shift based on the generated satellite position/velocity vectors and UE location.

Proposal 2: For RRM tests, consider stationary or mobile UE depending on the test purposes.

Proposal 3: Consider potentially different minimum elevation angle for demodulation and RRM test cases.

Proposal 4: Consider 30-degree elevation angle.

Proposal 5: Consider multiple satellite scenario after TE emulated channel modelling for one satellite has been understood and settled.

Proposal 6: Support option 1

Proposal 7: Support option 1 (Keplerian model for UE assumption on the propagator model)

5. References

[1] R4-2416556, “WF for NTN testing for NGSO channel model,” 3GPP TSG RAN WG Meeting #112-bis, Hefei, CN, Oct 14 – 18, 2024.