# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418447

# Orlando, US, 18th – 22nd November, 2024

Agenda Item:	7.20.3.2

Source: 	CMCC

Title: 			 Discussion on RRM impacts for SBFD operation

Document for:	Discussion

- Introduction

In RAN #104 meeting, the revised WID on SBFD was approved [1]. According to the WID, the objectives having RAN4 impact are duplicated as following.

- Specify enhancements for CLI handling [RAN1, RAN2, RAN3, RAN4] - Information exchange among gNBs, including [RAN3] - Semi-static cell-specific SBFD time and frequency location configuration - Measurement resource configuration, i.e., SSB and/or periodic NZP CSI-RS - Strongest DL beam information - CLI-mitigation request - UL resource muting for PUSCH, including [RAN1, RAN2, RAN4] - Indication/determination of UL resource muting for PUSCH based on semi-static configuration, assuming comb-2 for both DFT-S-OFDM and CP-OFDM in each allocated PRB and up to 2 symbols in time domain - PUSCH resource mapping, i.e., rate-matching around the muted REs - UCI resource determination in symbols with muted REs - L1 based UE-to-UE CLI measurement and reporting based on existing CSI framework, including [RAN1, RAN2, RAN4] - Periodic, semi-persistent, or aperiodic measurement resource (set), i.e., SRS-RSRP resource or CLI-RSSI resource, and configuration/determination of ‘typeD’ QCL assumptions for the CLI measurement resource - At least aperiodic reporting - New report quantities, e.g., L1-SRS-RSRP, L1-CLI-RSSI and/or measurement resource indices - UCI bits generation - Priority rules for multiple CSI reporting - CLI measurement accuracy requirement - Note: Without dedicated optimization for dynamic/flexible TDD. - Specify BS RF requirements for SBFD operation at gNB [RAN4] - Specify applicable RRM core requirements for CLI handling mechanisms [RAN4] - Specify other RRM core requirements for SBFD operation, if identified [RAN4]

In last meeting, there was discussion on SBFD, and a WF was agreed [2]. This contribution provides discussion on RRM impacts for SBFD operation.

- Discussion

#### Issue 1-2-1: Requirements for SSB based measurement - Agreements (from online session) - Down-select one of the two options: - Option 1: - As baseline, for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission. - This can be revisited if it proves to be conflicting with further RAN1 agreements. - Option 2: - For inter-frequency measurement without gap, UL transmission is prioritized. - Follow option 1 in other cases.

In August meeting, it was agreed that for SBFD-aware UE, existing requirements apply for SSB-based serving cell measurement. And further discuss for SSB based neighbour cell measurement in RAN4 [3]. The reason is that RAN1 has agreed that SSB is prioritized over configured UL transmission and dynamically scheduled UL transmission. RAN1 agreements are mainly for L1 measurement for serving cell. For L3 measurement for neighbour cell, it is normally discussed in RAN4. According to legacy scheduling availability of UE performing measurements in TDD bands, UE is not expected to transmit PUCCH/PUSCH/SRS on SSB symbols to be measured, RSSI measurement symbols, and on 1 data symbol before each consecutive SSB to be measured/RSSI symbols and 1 data symbol after each consecutive SSB to be measured/RSSI symbols within SMTC window duration. In our view, legacy approach, i.e. SSB measurement is prioritized over UL transmission, can be reused for neighbour cell neasurement for SBFD-aware UE. As for whether to differentiate inter-frequency measurement without gap from other cases, as proposed in Option 2, we do not see the necessity.

Proposal 1: for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

Issue 1-2-3: Requirements for scheduling restriction - Proposals - Proposal 1 (CATT): - Scheduling restrictions should take DL/UL transition time into account and the specific value can be FFS based on RAN1 design, if any. - Proposal 2 (Samsung): - Further discuss scheduling availability or measurement restriction when neighbour cell SSB/CSI-RS based L3 measurement is not completely contained in active DL sub-bands on SBFD symbols/slots - Proposal 3 (ZTE): - RAN4 needs to identify the scheduling restriction on the SBFD symbols for SSB based RRM measurements and CSI-RS based RRM measurements respectively. - For SSB based, reusing the existing scheduling restriction is preferred. - For CSI-RS based, some updated solution is needed. - Proposal 4 (HW, E///): - For SSB based L1 measurement of the serving cell collision with UL transmission, the scenario is covered by RAN1 agreement, no RAN4 scheduling restriction requirement is needed.

There is no scheduling restriciton for legacy L1 measurement, since there is guard period for the DL-to-UL transition. However, guard period is absent during the DL to UL transition. The potential issue is whether scheduling restriction is needed.

RAN1 is discussing similar issue, as duplicated as following. To avoid duplicated discussion, the issue can be left to RAN1.

Agreements in RAN1#116 meeting For SBFD-aware UEs, collisions between DL reception in DL subband(s) and UL transmission in UL subband in a SBFD symbol may be addressed or alleviated with proper scheduling. The following cases of potential collisions, [if link direction indication is not supported or provided], can be further studied to see if any change to the current specs is necessary: - Case 1: Dynamically scheduled DL reception vs. semi-statically configured UL transmission - e.g., dynamic PDSCH or CSI-RS collides with configured SRS, PUCCH, or CG PUSCH - Case 2: Semi-statically configured DL reception vs. dynamically scheduled UL transmission - e.g., PDCCH or SPS PDSCH collides with dynamic PUSCH or PUCCH - Case 3: Semi-statically configured DL reception vs. semi-statically configured UL transmission - Case 4: Dynamically scheduled DL reception vs. dynamic scheduled UL transmission - Case 5: SSB vs. dynamically scheduled or configured UL transmission - e.g., PUSCH, PUCCH, PRACH, SRS - Case 6: Dynamic or semi-static DL vs. valid RO Note: In addition to collision between UL transmission and DL reception in the same SBFD symbol(s), collision between UL transmission and DL reception in different symbol(s) due to lack of sufficient transition time between Tx/Rx at UE side is also included.

Proposal 2: for SSB based L1 measurement of the serving cell collision with UL transmission, the scenario is covered by RAN1 discussion, no RAN4 scheduling restriction requirement is needed.

- Conclusion

This contribution provides discussion on RRM requirements for SBFD. The proposals are:

Proposal 1: for SBFD-aware UE, existing requirements apply for SSB based neighbour cell measurement, i.e. SSB measurement is prioritized over UL transmission.

Proposal 2: for SSB based L1 measurement of the serving cell collision with UL transmission, the scenario is covered by RAN1 discussion, no RAN4 scheduling restriction requirement is needed.

- Reference

- RP-241614, Revised WID: Evolution of NR duplex operation: Sub-band full duplex (SBFD)

- R4-2416858, WF on RRM requirements for NR_duplex_evo, Huawei, HiSilicon

- R4-2413900, WF on SBFD RRM, Huawei, HiSilicon

- R4-2416910, Simulation assumption for evaluate L1- SRS-RSRP measurement requirements, Ericsson