# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418452

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.17.4.2

Source: 	CMCC

Title: 			 Discussion on testability and interoperability issues for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on testability and interoperability issues for beam management.

- Discussion

Issue 2-8: Channel model for beam prediction testing - Proposals - Option 1: simplified CDL - Option 2: TDL - Option 3: others

Spatial/time domain prediction require to learn the sufficient spatial/time domain characteristic, which means that sufficient randomness and variation in time and spatial domain in the test is necessary. Compared with TDL, CDL channel reflects field conditions accurately, and is more suitable for beam prediction testing.

Proposal 1: for beam prediction testing, simplified CDL is prefered.

Issue 2-11: Beam prediction testing process - Proposals - Option 1: Test needs to have at least 2 parts: - 1. inference part of the test (UE has to make the actual prediction) - 2. predicted beam measurement (UE has to measure and report “the ground truth”) - Option 2: others

Since reported RSRP measurement is impacted by RX beamforming gain which is up to UE implementation, it was agreed in RAN4 #111 meeting that the ground truth for FR2 is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam. And assumption is that SNR is high enough for sufficient accuracy of reported RSRP.

Agreements in RAN4 #111 meeting Agreement: - The ground truth for the predicted RSRP is the ideal measurement of RSRP on the predicted Tx beam - In RAN4, the ground truth is the approximate as the reported RSRP measurement under the certain SNR on the predicted Tx beam - FFS on SNR level to ensure that SNR is high enough for sufficient accuracy of reported RSRP - FFS on impact of multiple-AoA test setup - FFS on the channel condition - FFS on whether the ground truth will be changing or not - Other solutions are not precluded

With above agreements, during the test, it is necessary to have the procedure to get the ground truth. And it is straightforward that there is a part for prediction. Of course, the final KPI for BM is not conclided, if absolute RSRP accuracy is agreed as test metric, Option 1 can be used as baseline that test for BM at least include two parts: the part to get ground truth, and the part for prediction performance verification.

Proposal 2: If the absolute RSRP is agreed to be metric for the beam management prediction, it is proposed that tests for BM at least include two parts: the part to get ground truth, and the part for prediction verification.

- Conclusion

This contribution provides discussion on testability and interoperability issues for beam management. The proposals are:

Proposal 1: for beam prediction testing, simplified CDL is prefered.

Proposal 2: If the absolute RSRP is agreed to be metric for the beam management prediction, it is proposed that tests for BM at least include two parts: the part to get ground truth, and the part for prediction verification.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface