# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418453

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.17.4.1

Source: 	CMCC

Title: 			 Discussion on RRM requirements for beam management

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on RRM requirements for beam management.

- Discussion

Issue 2-3: RSRP Absolute accuracy Agreement: - If the absolute RSRP is agreed to be metric for the beam management prediction, the absolute RSRP accuracy for AI/ML based beam prediction = predicted L1-RSRP of beam index i – groundtruth of L1-RSRP of beam index i. The index i may be any beam index [in top-K beams based on groundtruth L1-RSRP].

As for the part with square brackets in the agreement, we do not think it is necessary. In our view, the accuracy requirements should apply to all predicted beams, no need to limit to a subset of predicted beams, which are selected predefined side conditions. There are two reasons to suggest to apply the accuracy requirements to all predicted beams. On one hand, all the predicted/reported beams are benificial for network, for example, can be used for load balancing. On the other hand, the final reported beam index and/or related L1-RSRP are based on ranking of all the predicted beams. If the accuracy of some predicted beams are not guaranteed, it will have impact on the final decision of the reported beams.

Proposal 1: it is proposed that the RSRP absolute accuracy requirements apply to all predicted beams.

Different metrics/KPIs have been discussed and were captured in the TR: For metrics for beam management requirements/tests, the following test metrics are identified and could be considered - Option 1: RSRP accuracy - Option 2: Beam prediction accuracy -Top-1 (%) : the percentage of "the Top-1 strongest beam is Top-1 predicted beam" -Top-K/1 (%) : the percentage of "the Top-1 strongest beam is one of the Top-K predicted beams" -Top-1/K (%) : the percentage of "the Top-1 predicted beam is one of the Top-K strongest beams" - Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB, -Related measurement accuracy can be considered to determine x - Option 4: combinations of above options Issue 2-7: Metrics/KPIs for beam prediction Proposals - Option 1: Use Option 1 - Option 2: Use Option 2 - Option 3: Use combined Option 2 and 3: - Option-New: The successful rate for the correct prediction which is considered as "the Top-1 predicted beam is one of the Top-K strongest beams, and the Top-1 predicted beam’s ground truth RSRP value is larger than the RSRP of the strongest beam – x dB” - FFS: K = 2 and x = 3 as baseline - Option 4: Neither Option 1, 2, 3 is appropriate, a new metric is needed - Option 5: Consider reliability metric based on relative difference in between the predictions due to change in the conditions/inputs. - consider defining multiple tests under different conditions but with the same outcome(same predicted beam) - Option 6: Other

For beam management, both spatial-domain DL beam prediction and temporal DL beam prediction are considered. For metrics for beam management requirements/tests, the following test metrics are identified [6] and be further discussed in the WID phase .

- Option 1: RSRP accuracy

- Option 2: Beam prediction accuracy

  - Top-1 (%) : the percentage of “the Top-1 strongest beam is Top-1 predicted beam”

  - Top-K/1 (%) : the percentage of “the Top-1 strongest beam is one of the Top-K predicted beams”

  - Top-1/K (%) : the percentage of “the Top-1 predicted beam is one of the Top-K strongest beams”

- Option 3: The successful rate for the correct prediction which is considered as maximum RSRP among top-K predicted beams is larger than the RSRP of the strongest beam – x dB,

  - Related measurement accuracy can be considered to determine x

- Option 4: combinations of above options

From our point of view, both RSRP accuracy and beam prediction accuracy are proposed to be considered as metrics for requirements/tests, since the two metrics verify the inference performance from different perspective. Beam prediction accuracy (absolute or relative) is similar like existing L1-RSRP measurement accuracy. For example, the absolute prediction accuracy is the difference between predict L1-RSRP and ideal L1-RSRP.  Beam prediction accuracy is used to reflect the percentage that the predict beam is the “genie-aided” beam. Having good RSRP accuracy does not mean that the beam prediction accuracy is good, vice versa.

Proposal 2: it is proposed that both the RSRP accuracy and beam prediction accuracy are the metrics/KPIs for beam prediction.

- Conclusion

This contribution provides discussion on RRM requirements for beam management. The proposals are:

Proposal 1: it is proposed that the RSRP absolute accuracy requirements apply to all predicted beams.

Proposal 2: it is proposed that both the RSRP accuracy and beam prediction accuracy are the metrics/KPIs for beam prediction.

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface