# 3GPP TSG-RAN WG4 Meeting #113                                R4-2418456

# Orlando, US, 18th - 22nd November, 2024

Agenda Item:	7.17.2.2

Source: 	CMCC

Title: 			 Discussion on AI/ML two-sided model logistical issues

Document for:	Discussion

- Introduction

In RAN #102 meeting, the WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [1]. In RAN #105 meeting, the revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface was approved [2]. The objectives for RAN4 are duplicated as following.

- Core requirements for the above two use cases for AI/ML LCM procedures and UE features [RAN4]: - Specify necessary RAN4 core requirements for the above three use cases. This includes the test setup and methodology for the beam management use case. - Specify necessary RAN4 core requirements for LCM procedures including performance monitoring. - Testability and interoperability [RAN4]: - Finalize the testing framework and procedure for one-sided models and further analyse the various testing options for two-sided models, in collaboration with RAN1, and including at least: - Relation to legacy requirements - Performance monitoring and LCM aspects considering use-case specifics - Generalization aspects - Static/non-static scenarios/conditions and propagation conditions for testing (e.g., CDL, field data, etc.) - UE processing capability and limitations - Post-deployment validation due to model change/drift

In last meeting, there is discussion on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface, and a WF was approved [3]. This contribution provides discussion on AI/ML two-sided model logistical issues.

- Discussion

Agreement on Dataset format: Use NumPy for dataset sharing Do not use pickled data(compression mechanism) Use npy – single array in each file Agreement on Training Data file format: Dataset/model input file format: - N (samples) X 2 (IQ) X nSB (number of subbands) X nPorts (number of CSI-RS ports) X nLayers (number of layers) - In other words, dataset should content following info: - 1st dimension: Number of samples - 2nd dimension: Real and imaginary - 3rd dimension: Number of sub-bands - 4th dimension: Number of antenna ports - 5th dimension: Number of layers – present only if >1 layer - Note: Each element of the dataset will be a float32 real number The dataset includes eigenvectors calculated based on average covariance channel matrix for the given MIMO layer for each sub-band based on pre-processing assumptions in WF R4-2414447 Single file for fixed scalar quantization(only dataset), TBD whether additional file is needed for variable/trainable quantization Dataset files can be split into multiple files to enable easier upload. - Use the 2 digits for split files starting from 00, increment for each additional file Split files and then archive each file

The dataset format and training data file format are discussed and agreed. The discussion focus on training data,  while inference data and testing data also need to be considered.

According to the email discussion on data sharing for AI/ML after RAN4 #112 meeting, the common understanding is that testing and inference data should be a subset of the training dataset. From this point of view, no need to have dedicated file to share testing/inference data seperately. But it is necessary to discuss which part of training dataset are used for testing and/or inference so that companies could obtain the alligned testing data and inference data. According to the email discussion, most companies propose that testing dataset should be 10-25% of the training dataset, but this is noe enough, at least it also need to allign the location, i.e. it is first 10%~25% or last 10%~25% of training detaset are for testing and/or inference.

Proposal 1: it is proposed to discuss and allign on which part of training dataset are used for testing and/or inference .

Another issue is about the “lifetime” of the data/model. This can be discussed case by case. For the data/model captured in the TS, the lifetime could be indefinite. For the data/model uploaded as references for requirements definition, the lifetime could be one or several releases. For the data/model uploaded for companies allignement, the lifetime could be one or several meetings.

Proposal 2: it is proposed to discuss the “lifetime” of the data/model in a case by case manner. At least following cases are considered:

- data/model captured in the TS

- data/model uploaded as references for requirements definition

- data/model uploaded for companies allignement

- Conclusion

This contribution provides discussion on AI/ML two-sided model logistical issues. The proposals are:

Proposal 1: it is proposed to discuss and allign on which part of training dataset are used for testing and/or inference .

Proposal 2: it is proposed to discuss the “lifetime” of the data/model in a case by case manner. At least following cases are considered:

- data/model captured in the TS

- data/model uploaded as references for requirements definition

- data/model uploaded for companies allignement

- Reference

- RP-234039, New WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- RP-242399, Revised WID on Artificial Intelligence (AI)/Machine Learning (ML) for NR Air Interface

- R4-2417207, WF on Data Sharing framework, Qualcomm Incorporated

- R4-2417212, WF on requirements for AI/ML air interface, Qualcomm Incorporated

- R4-2414323, WF on AI/ML, Qualcomm Incorporated

- R4-2410570, WF on AI/ML, Qualcomm Incorporated

- R4-2406617, WF on AI/ML, Qualcomm Incorporated

[5] 3GPP TR 38.843, Study on Artificial Intelligence (AI)/Machine Learning (ML) for NR air interface