# 3GPP TSG-RAN WG4 Meeting #113	R4-2418471

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.22.2.1

Source:	China Telecom

Title:	Discussion on On-demand SSB SCell operation for enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, On-demand SSB SCell operation for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on On-demand SSB SCell operation for enhancements of network energy savings for NR issues captured in the WF [1] and summary [2].

- Discussion

General OD-SSB requirements

Status in the summary [2]:

Issue 1-1-4: L1 measurement for deactivated SCell - Proposals - Option 1: Xiaomi, Samsung, China Telecom, Intel - RAN4 shall define both L1 and L3 measurement requirements based on OD-SSB. - Option 2: Qualcomm, OPPO, Nokia - L1 measurements are not supported for deactivated SCell - Option 3: Nokia - Wait further RAN1 clarification on the applicable scenario - Recommended WF - Notes: - From moderator’s understanding, RAN1’s agreement is applied to the L1 measurement after receiving SCell activation command other than before SCell activation. - RAN4 only needs to clarify the L1 related requirement can also applied to OD-SSB in SCell. - Suggest to further discuss the scenarios below. The proponent companies need to clarify which of the following scenarios are supported by your proposal. - L1 measurement before SCell activation: - L1 measurement during SCell activation: - L1 measurement after SCell activation:

In our understanding, for Case #1, there is no always-on SSB on the cell, UE activates the SCell based on OD-SSB, SCell activation procedures including L1 measurement need to be performed based on OD-SSB. Therefore, it’s proposed to define L1 measurement requirements based on OD-SSB during SCell activation.

Proposal 1: It’s proposed to define L1 measurement requirements based on OD-SSB during SCell activation.

Status in the summary [2]:

Issue 1-1-6: OD-SSB accuracy - Proposals - Option 1: Qualcomm - No new measurement accuracy requirements should be defined. - Recommended WF - Moderator suggests the group to check whether the following proposal can be agreed. - No new measurement accuracy requirements should be defined.

For OD-SSB accuracy, we support that no new measurement accuracy requirements should be defined.

Proposal 2: No new measurement accuracy requirements should be defined.

OD-SSB based deactivated SCell L3 measurement (Case 1)

Status in the WF [1]:

Issue 1-2-1: OD-SSB based deactivated SCell measurement(Case 1) Agreements: Once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. - After the time window, UE can perform deactivated SCell measurement with a slow mode. - Option 2-1: fallback legacy requirement - Option 2-2: stop measurement - Option 2-3: No requirement - Time window means the time duration starting from receiving OD-SSB activation indication - FFS the time window configuration or pre-defined - FFS the fast L3 measurement requirement - FFS the relation between time window and OD-SSB deactivation command in the time domain When UE receives OD-SSB deactivation, UE is not required to measure the deactivated SCell.

In last meeting, it was agreed that once UE receives the OD-SSB activation indication, UE performs the OD-SSB based fast L3 measurement for deactivated SCell within a time window. After the time window, considering UE power consumption, UE can perform deactivated SCell measurement with legacy requirement.

Proposal 3: After the time window, considering UE power consumption, UE can perform deactivated SCell measurement with legacy requirement.

For the OD-SSB based fast L3 measurement requirement for deactivated SCell, considering network energy savings gains, it’s proposed to follow the OD-SSB periodicity.

Proposal 4: For the OD-SSB based fast L3 measurement requirement for deactivated SCell, considering network energy savings gains, it’s proposed to follow the OD-SSB periodicity.

OD-SSB based deactivated SCell L3 measurement (Case 2)

Status in the WF [1]:

Issue 1-3-4: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) Agreements: OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered(Case 2) - Option 1: RAN4 requirement is defined based on OD-SSB periodicity.(CATT, Samsung, CMCC, ZTE) - Option 2: RAN4 requirement is defined based on legacy measCycleSCell (Apple, Xiaomi, vivo, OPPO) - Option 3: RAN4 requirement is defined based on combining the always-on SSB and OD-SSB. (Ericsson, LG, ZTE) - Option 4: RAN4 for further discussion (Nokia, LG, CMCC, ZTE, Qualcomm) - Option 5: RAN4 requirement is defined based on the indicated measurement periodicity (either OD-SSB periodicity or measCycleSCell for always-on SSB) (vivo)

For Case 2, always-on SSB is periodically transmitted on the cell, before UE receives OD-SSB transmission indication, deactivated SCell measurement can be performed based on always-on SSB. After UE receives OD-SSB transmission indication and when OD-SSB is transmitted on the cell, UE can perform deactivated SCell measurement based on OD-SSB, or based on both OD-SSB and always-on SSB. In our views, in order to maximum network energy gains and avoid complex implementation, we support that RAN4 requirement is defined based on OD-SSB periodicity for OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2.

Proposal 5: In order to maximum network energy gains and avoid complex implementation, it’s proposed that RAN4 requirement is defined based on OD-SSB periodicity for OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2.

OD-SSB based SCell activation

Status in the summary [2]:

Issue 1-4-4: The detail update of known cell condition - Proposals - Option 1: CMCC - RAN4 to update the ‘period’ in known condition based on the new L3 deactivated SCell measurement requirement for the case of OD-SSB is activated before SCell activation command(Scenario#2). - Option 2: Xiaomi, Samsung, Ericsson - RAN4 to discuss the known cell condition related to the OD-SSB based SCell activation. - Option 3: Qualcomm - FFS: a new known condition can be defined for OD-SSB following the measurement periodicity - Option 4: Nokia - The SCell shall be considered as known if the UE has sent a measurement report for either regular SSB or OD-SSB on the deactivated SCell. - Option 5: NTT - In Case #1 of scenario #2, known cell condition should be evaluated based on the time period between OD-SSB triggering command and SCell activation command. - In Case #1 of scenario #2A, SCell to be activated should be always treated as unknown. - In Case #2 of scenario #2, known cell condition should be evaluated based on both always-on SSB and OD-SSB - In Case #2 of scenario #2A, known cell condition should be evaluated as same manner as that of current specification. - Recommended WF - From moderator’s understanding, this issue is pending on the issue 1-2-1. - Postpone discussion

L3 measurement for deactivated SCell can be performed based on OD-SSB or always-on SSB in Case 2, in our views, the SCell can be considered as known if UE has sent a valid measurement report based on either OD-SSB or always-on SSB for the SCell being activated. Whether the ‘period’ in known condition to be updated or not can be further discussed.

Proposal 6: The SCell can be considered as known if UE has sent a valid measurement report based on either OD-SSB or always-on SSB for the SCell being activated.

- Conclusion

The contribution presents our views on On-demand SSB SCell operation for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: It’s proposed to define L1 measurement requirements based on OD-SSB during SCell activation.

Proposal 2: No new measurement accuracy requirements should be defined.

Proposal 3: After the time window, considering UE power consumption, UE can perform deactivated SCell measurement with legacy requirement.

Proposal 4: For the OD-SSB based fast L3 measurement requirement for deactivated SCell, considering network energy savings gains, it’s proposed to follow the OD-SSB periodicity.

Proposal 5: In order to maximum network energy gains and avoid complex implementation, it’s proposed that RAN4 requirement is defined based on OD-SSB periodicity for OD-SSB based deactivated SCell L3 measurement when OD-SSB transmission is triggered in Case 2.

Proposal 6: The SCell can be considered as known if UE has sent a valid measurement report based on either OD-SSB or always-on SSB for the SCell being activated.

- References

- R4-2416916, WF on RRM requirements for Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.

- R4-2415662, Topic summary for [112bis][215] Netw_Energy_NR_enh_Part1, Ericsson, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.