# 3GPP TSG-RAN WG4 Meeting #113	R4-2418472

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.22.2.2

Source:	China Telecom

Title:	Discussion on adaptation of common signal/channel transmission for enhancements of network energy savings for NR

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, adaptation of common signal/channel transmission for enhancements of network energy savings for NR was discussed and the WF was approved [1].

In this paper, we present our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR issues captured in the WF [1] and summary [2].

- Discussion

SSB adapation – Requirements impacts

Status in the summary [2]:

Issue 1-2-2: Whether and how to define requirements for “transition period” - Proposals - Option 1: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied. (CTC, CMCC, CATT) - Option 1a: For SSB periodicity adaptation in time-domain, the existing minimum requirement at transitions for L1 measurement can still apply. (CATT) - Option 2: For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements. (Vivo) - Option 3: RAN4 to discuss whether and how to define requirements when SSB adaption happens during RRM procedures (Huawei, Samsung) - Option 3a: FFS Requirements on interrupts at state transitions. (Samsung) - Option 4: When SSB periodicity changes upon SSB periodicity adaptation, the cell identification, measurement period and L1 requirements apply based on the longer delay before or after the adaptation. (Apple, MTK) - Option 4a: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact. (Nokia) - Option 5: In L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. (Ericsson) - Option 6: (QC) - RAN4 should investigate what requirements are impacted by those periodicity transitions by SSB adaptation in time domain - Only if clear benefits of measurement combining can be shown, RAN4 should further analyse how such requirements for measurement combining could be defined. - RAN4 should further discuss which of the two options, i.e., combing of measurements or applying the maximum of the two periodicities, is more feasible and trade-off benefits versus impacts. - Option 7: (Samsung) UE behavior under NES scenario based on two SSB configurations - If NES only cell case is considered. The NES-capable UE shall measure the NES-based SSB resources always - If NES cell case is considered. The candidate UE behaviours can be - NES-capable UEs to measure these additional NES-based SSB resources with the highest priority - Both SSB resources (legacy SSBs and adapted SSBs) the NES-capble UEs can to measure. New measurement rules and the measurement results combination rules shall be discussed in RAN4 - Based on UE implementation - Moderator: Based on the contributions about “transition period” of SSB adaptation, some proposals are for both L1 and L3 measurement and some proposals are only for L1 measurement, while L3 measurement may related to SMTC. Thus, to facilitate the discussion, the issue is only to discuss the requirements impacts for “transition period” of SSB adaptation for L1 measurement, and L3 measurement will be discussed in separate issues. - Recommended WF: - For L1 measurement requirements, RAN4 to define RRM requirements for “transition period” of SSB adaptation, considering following approach: - Option 1: Reuse exiting transition requirements - Option 2: longer delay before or after SSB adaption - Option 3: Combine SSB measurement before and after SSB adaptation - Other options are not precluded.

For L1 measurement requirements, RAN4 to define RRM requirements for transition period of SSB adaptation, we support to reuse exiting transition requirements for L1 measurement.

Proposal 1: For L1 measurement requirements, RAN4 to define RRM requirements for transition period of SSB adaptation, it’s proposed to reuse exiting transition requirements for L1 measurement.

Status in the summary [2]:

Issue 1-2-3: L1 Requirements after “transition period” - Proposals - Option 1: After SSB adaptation, for SSB based measurement based on SSB periodicity, UE shall perform measurement based on “new SSB configurations” (e.g. new SSB periodicity). (Huawei, Ericsson) - Option 1a: In L1 measurement, UE shall combine the SSB measurements before and after SSB adaptation with different periodicities. Subsequent to the transition measurement period, the cell identification and measurement period requirements will follow the SSB periodicity after adaptation. (Ericsson) - Option 1b: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. (ZTE) - Option 1c: For SSB periodicity adaptation in time-domain, the existing L1/L3 measurement requirements can still apply. (CATT) - Recommended WF: - Discuss above proposal

After transition period for L1 measurement, we suggest to follow legacy logic that UE shall perform measurement based on new SSB configurations, e.g. new SSB periodicity.

Proposal 2: After transition period for L1 measurement, UE shall perform measurement based on new SSB configurations, e.g. new SSB periodicity.

Status in the summary [2]:

Issue 1-2-4: SCell activation requirement for SSB adaptation - Proposals - Option 1: After SSB adaptation, For L1 and L3 measurement during SCell activation, UE shall perform SCell activation based on “new SSB configurations” (e.g. new SSB periodicity). (Huawei, OPPO) - Option 1b: Don’t pursue SMTC-based L3 measurement for SCell activation. (OPPO) - Option 2: RAN4 to study the SCell activation and deactivation delay impact. (Samsung) - RAN4 to focus on MAC CE based SCell activation case as starting point for SSB adaptation SCell operation - Combine the RAN1 conclusions of on-demand SSB and SSB adaptation to define the corresponding SCell requirements - Recommended WF: - Discuss above proposal

After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity. For L3 measurement during SCell activation, since the LS related to SMTC impact was sent to other WGs and whether there is any SMTC impact due to SSB adaptation is not concluded, we can wait for the conclusions on SMTC impact from other WGs.

Proposal 3: After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity.

Proposal 4: After SSB adaptation, for L3 measurement during SCell activation, wait for the conclusions on SMTC impact from other WGs.

Status in the summary [2]:

Issue 1-2-5: L3 requirements for “transition period” - Proposals - Option 1: For the measurement requirement during SSB periodicity transitions period, the legacy transition requirement can be applied. (CMCC, CTC) - Option 2: For SSB periodicity adaptation, RAN4 further check whether existing transition period requirements for L3 measurement can be re-used for BFD and L1/L3 measurements. (Vivo) - Option 2a: RAN4 to investigate whether a transition period with no or relaxed RRM requirements can serve as starting point when changing SSB burst periodicity for defining the RRM impact. (Nokia) - Option 3: When the UE transitions when SSB periodicity changes, the L1/L3 measurement requirements apply based on the longer delay before or after the transition. (MTK) - Option 4: In L3 measurement, RAN4 to wait other WGs’ input on whether new SMTC after SSB adaptation will be applied. (Ericsson) - Option 5: (OPPO) - don’t support L3 measurement after the SCell has been activated without relying on SMTC - RAN4 not to introduce new SMTC due to SSB adaptation - Option 5a: For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC. (Huawei) - Option 6: Deprioritize the minimum requirement at transitions for L3 measurement until RAN1 has further progress on SMTC configuration. (CATT) - Option 7: When defining the requirement with adaptation SSB, RAN4 needs only consider the case in which there is no any SSB adaptation period changed within one reporting interval. (Xiaomi) - Option 8: The impact caused by the SSB adaptation in time domain is to identify the relevant L1/L3 measurement requirements based on the adapted SSB periodicity. (ZTE) - Moderator: - Based on the proposals above, the first question needs to be discussed is whether L3 measurement is still based on configured SMTC with SSB adaptation. - Recommended WF: - Discuss following question: - Q1: Whether L3 measurement is still based on configured SMTC with SSB adaptation. - Q1-1: If the answer is “yes”, whether RAN4 needs to discuss new SMTC for SSB adaptation? - Q1-2: If the answer is “NO”, what is the detailed UE measurement behaviour? Issue 1-2-6: L3 requirements after “transition period” - Proposals - Option 1: For SSB periodicity adaptation in time-domain, the existing L1/L3 measurement requirements can still apply. (CATT) - Option 1a: (Huawei) - After SSB adaptation, for SSB based measurement based on SMTC: - For L3 measurement in clause 9, UE is still expected to perform measurement based on configured SMTC. - Recommended WF: - Depends on the conclusion of issue 1-2-5 about UE measurement behaviour of L3 measurement.

For L3 requirements for transition period and after transition period, similarly, since the LS related to SMTC impact was sent to other WGs and whether there is any SMTC impact due to SSB adaptation is not concluded, we can wait for the conclusions on SMTC impact from other WGs.

Proposal 5: For L3 requirements for transition period and after transition period, wait for the conclusions on SMTC impact from other WGs.

- Conclusion

The contribution presents our views on adaptation of common signal/channel transmission for enhancements of network energy savings for NR, with the following proposals:

Proposal 1: For L1 measurement requirements, RAN4 to define RRM requirements for transition period of SSB adaptation, it’s proposed to reuse exiting transition requirements for L1 measurement.

Proposal 2: After transition period for L1 measurement, UE shall perform measurement based on new SSB configurations, e.g. new SSB periodicity.

Proposal 3: After SSB adaptation, for L1 measurement during SCell activation, UE shall perform SCell activation based on new SSB configurations, e.g. new SSB periodicity.

Proposal 4: After SSB adaptation, for L3 measurement during SCell activation, wait for the conclusions on SMTC impact from other WGs.

Proposal 5: For L3 requirements for transition period and after transition period, wait for the conclusions on SMTC impact from other WGs.

- References

- R4-2416860, WF on RRM requirements for Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.

- R4-2415663, Topic summary for [112bis][216] Netw_Energy_NR_enh_Part2, Huawei, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.