# 3GPP TSG-RAN WG4 Meeting #113	R4-2418473

# Orlando, US, 18th – 22nd November, 2024

Agenda item:	7.23.4.2

Source:	China Telecom

Title:	Discussion on RRM core requirements for LP-WUS/WUR

Document for:	Discussion

- Introduction

In RAN4 #112bis meeting, RRM core requirements for LP-WUS/WUR was discussed and the WF was approved [1].

In this paper, we present our views on RRM core requirements for LP-WUS/WUR issues captured in the WF [1] and summary [2].

- Discussion

General aspects

Status in the summary [2]:

Issue 1-1-5: Criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation - Proposals - P1: Up to RAN2/other group’s decision (xiaomi vivo Apple CT Huawei) - P1-1: The criteria for MR RRM measurement relaxation and fully offloading Up to RAN2/other group’s decision. Do not define entry/exit conditions for LP-WUR measurement. (CATT) - P2: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with RAN1 and RAN2 and further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighboring cell measurement and LP-WUR measurement as a package (Samsung) - P3: Calibration between MR measurement and LR measurement maybe required which also pending on the progress of UE RF session on LP-WUR RF architecture, NF assumption (Samsung) - P4: RAN4 shall wait for RAN2 to define the relaxed measurement condition for serving cell measurement if it has. (ZTE) - P5: RAN4 to discuss whether and how to differentiate the entry/exit conditions for Case#1 and Case#3. The following options can be considered (MTK) - Option 1: Multiple thresholds: separate entry/exit thresholds are defined for different cases, Case#1 and Case#3. - Option 2: Single threshold: single entry/exit threshold is defined for both cases, Case#1 and Case#3, in which UE enters Case#3 (RRM relaxation) first, then enters Case#1 (RRM offloading) after some confidence period T. Recommendations: Criteria are up to RAN2’s consideration Issue 1-1-6: Criteria (entry/exit conditions) for LP-WUS monitoring - Proposals - P1: For the entry/exit condition for LP-WUS monitoring, follow RAN1/2 conclusions or can be triggered by other groups if necessary (xiaomi vivo,CATT Apple ) - P2: For LP-WUS monitoring, RAN4 to discuss whether RSRP and RSRQ measurement from LR can reflect the impact of the inter-carrier interference (Huawei) - P3: As agreed by RAN2, LP-WUS monitoring thresholds, which allow UE to stop monitoring legacy PO, are configured independently from MR offloading and relaxation thresholds. RAN4 shall define LP-WUS monitoring requirements independently from the MR measurement relaxation and offloading. (Nokia) Recommendations: Criteria are up to RAN2’s consideration Issue 1-1-7: On jointly consideration on issue 1-1-5, 1-1-6 and 1-1-7 - Proposals - P1-1: Define same entry conditions for LP-WUS monitoring and RRM measurement relaxed cases (case 3). (vivo Huawei). - P1-2: Define same entry/exit condition for LP-WUS monitoring and LP-WUR serving cell measurements (xiaomi Huawei MTK) - P1-3：the threshold to turn on LR or to involve LR for measurement can be equivalent to the threshold of entry condition for LP-WUS monitoring.(Apple) - P2: The LR-based LP-WUR measurement threshold can be less than or equal to the LR-based LP-WUS monitoring threshold. (LG) - P3: The threshold for MR fully offloading to LR can be same as the threshold for neighbor cell measurement triggering (highest one between SIntraSearchP/SIntraSearchQ and SnonIntraSearchP/SnonIntraSearchQ), or at least offloading threshold can be not lower than threshold for neighbour cell measurement triggering. (Apple): - P5: RAN4 shall be involved on criterion and scenario (entry/exit condition) design for LP-WUR measurement and MR RRM measurement relaxation together with RAN1 and RAN2. (Samsung) - RAN4 shall further discuss the applicable side condition /scenario with corresponding assumption on MR serving cell measurement, MR neighbouring cell measurement and LP-WUR measurement as a package - P6: (Ericsson) - When UE checks the LR entry threshold - UE is required to evaluate both MR and LR RRM quality if both configured from NW side; - UE is optionally to only evaluate MR quality if only MR evaluation criteria is configured from NW side. - P7: RAN4 to work on the following issues once RAN2 has made progress (Nokia) - Criteria (entry/exit conditions) for LP-WUR serving cell measurement - Criteria (entry/exit conditions) for MR RRM measurement relaxation - Criteria (entry/exit conditions) for LP-WUS monitoring Recommendations: Focus on: The entry point between the start of LP-WUS monitoring and the start of case 3 [case 1] The exit point between the exit of LP-WUS monitoring and the exit of case 3 [case 1] The relationship between the threshold for MR fully offloading to LR and the threshold for neighbor cell measurement triggering has been covered by Issue 1-1-4.

Regarding criteria (entry/exit conditions) for fully offloading case and MR RRM measurement relaxation and for LP-WUS monitoring, we support that RAN2 is the main group for criteria (entry/exit conditions) design.

Proposal 1: RAN2 is the main group for criteria (entry/exit conditions) design for fully offloading case and MR RRM measurement relaxation and LP-WUS monitoring.

Status in the summary [2]:

Issue 1-1-10: Considerations on higher priority frequency layer - Proposals - P1: MR perform relaxed higher priority frequency layer search in RRM relaxation case#1. Open to further discuss which higher priority layer measurement requirement should be applied for case#3 (CMCC) - P2: MR perform relaxed higher priority frequency layer search in RRM relaxation case#1 and case#3 (xiaomi vivo CT) - P3: Higher priority layer neighbor cell measurement shall also be OFF for case #1 (Apple vivo) - P4: UE should perform measurement for higher priority frequency layers, if configured (Huawei) - If serving cell is below threshold for inter-frequency neighbor cell measurement (Case #3), UE meet the relaxed measurement requirements - If serving cell is above threshold for inter-frequency neighbor cell measurement (Case #4), FFS whether existing requirements (60 * Nlayers) are re-used or further relaxed. - P5: High priority frequency layer measurement requirements shall use the corresponding requirements of the legacy Rel-16 RRM relaxation WI as the base (ZTE) - P5-1: RAN4 shall study the relaxed scaling factor or additional scenarios (ZTE) - P6: No need to define new requirements for higher priority frequency layer (Nokia, Ericsson) Recommendations: Check firstly for case 1, whether higher priority frequency layer search will be performed or not.

In our views, higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Proposal 2: Higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Detail LP-WUR requirements at RRC_IDLE/INACTIVE state

Status in the summary [2]:

Issue 1-2-1: Accuracy requirements - Proposals - P1: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement. (xiaomi LG Apple CT Huawei Ericsson MTK) - P2: Define LP-WUR measurement accuracy in dedicated section. Whether/how to reflect it in the core requirements can be further discussed after the simulation by taking the results into account (CATT) Recommendations: Check whether P1 is agreeable

For LP-WUR measurement requirements at RRC_IDLE/INACTIVE state, the measurement delay, measurement accuracy, and measurement filtering can be discussed to specify. For accuracy requirements, since there is no measurement reporting to network for LP-WUR measurement at RRC_IDLE/INACTIVE state, it’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Proposal 3: It’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Status in the summary [2]:

Issue 1-2-2: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation - Proposals - P1: Define evaluation period for evaluating entry/exit threshold for LP-WUS monitoring; Define evaluation period for evaluating entry/exit threshold for fully offloading (case 1) or MR RRM relaxation (case 3). (vivo CATT LG MTK) - P1-1: The evaluation period is the same as that of the corresponding measurement period. (vivo) - P1-2: The evaluation period is based on LP-WUR measurement period, and further discuss whether to define measurement filtering (e.g., number of samples). (CATT) - P1-3: RAN4 to discuss the delay requirements to evaluate the entry criteria by using MR measurement results only and to evaluate exit criteria by using LR measurement results only. (MTK) - P2: RAN4 specify following requirements for MR RRM relaxation and offloading and existing requirements for S criteria or relaxed measurement criterion can be considered as starting point (Samsung) - LR measurement period and accuracy - MR measurement relaxation and the evaluation period for entry/exit criteria threshold evaluation under different states (case 1 fully offloading, and case 3 MR relaxation) - P3-1: Do not need a dedicated evaluation delay requirement (e.g., in how long time the MR/LR measurement results can meet the condition). Use the MR measurement period and/or LR measurement period to check the criteria for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, i.e., MR measurement delay or LR measurement delay are sufficient for criteria checking; (Apple) - P3-2: RAN4 to define requirements for serving cell evaluation against the entry/exit condition for LP-WUS monitoring, including Measurement delay (number of samples); Measurement accuracy (margin for evaluation); Measurement filtering (number of samples and separation between samples) (Huawei) - P4: RAN4 specifies the following for entry/exit criteria evaluation for WUS paging monitoring: (QC) - The number of measurement samples needed for filtering. - The number of consecutive times the UE needs to meet the entry/exit criteria Background: The measurement period of LR has already been agreed to be specified as issue 1-1-2. The question is whether to define a dedicated evaluation delay requirement other than measurement delay requirement to check the threshold is fulfilled or not. Previous case: At Rel-16 UE power saving WI, the evaluating period on whether the threshold of the “not at cell edge criteria” is fulfilled or not is the serving cell measurement period. Recommendations: For both MR and LP-WUR, for checking whether the thresholds for entry/exit WUS paging monitoring/LP-WUR measurement/MR RRM relaxation are fulfilled or not, Option 1: Use the measurement period of LR or MR (Note: the measurement period of LR has already been agreed to be specified as issue 1-1-2) Option 2: Define another evaluation delay period. Evaluation is longer or the same as that of measurement period.

On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, it’s proposed to define evaluation period for evaluating entry/exit threshold for LP-WUS monitoring and for fully offloading (case 1) or MR RRM relaxation (case 3), and use the measurement period of LR or MR for evaluation period, that is, the evaluation period is the same as that of the corresponding measurement period.

Proposal 4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, it’s proposed to define evaluation period for evaluating entry/exit threshold for LP-WUS monitoring and for fully offloading (case 1) or MR RRM relaxation (case 3), and the evaluation period is the same as that of the corresponding measurement period.

MR RRM relaxation

Status in the summary [2]:

Issue 1-3-1: MR RRM relaxation for serving cell/neighbour cell - Proposals - P1: Use relaxation/scaling factor for MR serving/neighbour cell relaxation(xiaomi vivo CMCC vivo Huawei Samsung) - P1-1: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor (Samsung CT ZTE MTK) - P1-2: Relaxation factor =8 as the starting point (CMCC) - P1-3: Relaxation factor >=16 (Apple vivo Huawei MTK) - P1-4: At least the scaling factor should be larger than that of legacy Rel-16 UE power saving WI. (vivo) - P1-5: The scaling factor can be further discussed after RAN4 have concrete agreements on the MR RRM relaxation cases. (xiaomi) - P2: For MR neighbor cell measurement relaxation, the existing relaxed requirements in 4.2.2 can be used as baseline. For MR serving cell measurement relaxation, a scaling factor can be introduced. Further discuss whether to use same scaling factor as neighbor cell measurement (CATT, Ericsson) - P3: RAN4 to further discuss scaling factor for MR RRM relaxation for serving and/or neigbhor cell after scenario and detailed criterion for measurement relaxation have been clarified (LG) - P4: The existing relaxation for scaling factor can be the starting point for MR RRM relaxation (NTT Docomo) - P6: The legacy intra-/inter-frequency and inter-RAT neighbour cell measurement requirements can be the baseline and RAN4 can study the relaxed scaling factor (ZTE) - P7: Multiple values can be considered depending on conditions. (Samsung MTK) Recommendations: Check “Use relaxation/scaling factor for MR serving/neighbour cell relaxation, relaxation factor >=8 as the starting point” agreeable

For MR RRM relaxation, MR RRM relaxation factor for serving cell/neighbor cell is an important issue to be discussed in RAN4. Referring to analysis of RRM for LP-WUR duty-cycled monitoring in TR 38.869, the average of power saving gain provided by LP-WUS is 4% when MR relaxed < 8 times, which is very limited. While the gain is 40% when 8 times<= MR relaxed <=16 times, and the gain is 60% when MR relaxed > 16 times. Therefore, it’s proposed to discuss relaxation factor equals to 8 as the starting point for the relaxation factor for MR RRM relaxation.

Proposal 5: It’s proposed to discuss relaxation factor equals to 8 as the starting point for the relaxation factor for MR RRM relaxation.

Status in the summary [2]:

Issue 1-3-2: On Neighbour cell and serving cell relaxation factor - Proposals - P1: Neighbour measurements have the more relaxation than or equivalent relaxation as serving cell measurement. (Apple) - P2: Same relaxation factor applies to serving and neighbor cell measurements (vivo CT Huawei MTK) - P3: Serving cell measurement can be more relaxation than neighbour cell measurement. (Ericsson) Recommendations:

On neighbor cell and serving cell relaxation factor, in our views, it’s preferred to define same relaxation factor for MR RRM relaxation for serving and neighbor cell measurements.

Proposal 6: Define same relaxation factor for MR RRM relaxation for serving and neighbor cell measurements.

- Conclusion

The contribution presents our views on Rel-19 RRM core requirements for LP-WUS/WUR, with the following proposals:

Proposal 1: RAN2 is the main group for criteria (entry/exit conditions) design for fully offloading case and MR RRM measurement relaxation and LP-WUS monitoring.

Proposal 2: Higher priority frequency layer search can be performed in relaxed mode for case #1/ case #3.

Proposal 3: It’s proposed to reflect the accuracy performance as a margin in the core requirement, no dedicated accuracy requirement is defined in the performance section for LP-WUR based RRM measurement in Idle/inactive states.

Proposal 4: On requirements for entry/exit criteria evaluation for WUS paging monitoring/LP-WUR measurement/MR RRM relaxation, it’s proposed to define evaluation period for evaluating entry/exit threshold for LP-WUS monitoring and for fully offloading (case 1) or MR RRM relaxation (case 3), and the evaluation period is the same as that of the corresponding measurement period.

Proposal 5: It’s proposed to discuss relaxation factor equals to 8 as the starting point for the relaxation factor for MR RRM relaxation.

Proposal 6: Define same relaxation factor for MR RRM relaxation for serving and neighbor cell measurements.

- References

- R4-2416861, WF on RRM requirements for NR_LPWUS, vivo, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.

- R4-2415664, Topic summary for [112bis][217] NR_LPWUS, vivo, RAN WG4 Meeting #112bis, 14 – 18 October, 2024.