# 3GPP TSG-RAN WG4 Meeting #113				                    R4-2418479

# Orlando, USA, 18th - 22nd November, 2024

Source:	ZTE Corporation, Sanechips

Title:	Discussion on core parts on LP-WUS

Agenda item:	7.23.4.2

Document for:	Approval

Introduction

In RAN #103 meeting the latest WID has been agreed [1] on Low-power wake-up signal and it is the first meeting for RAN4 to discuss the core part in this meeting. The WID can be seen as below:

The objectives of the work item are the following: - To specify an LP-WUS design commonly applicable to both IDLE/INACTIVE and CONNECTED modes (RAN1, RAN4) - Specify OOK (OOK-1 and/or OOK-4) based LP-WUS with overlaid OFDM sequence(s) over OOK symbol - The LP-WUS design shall ensure that for IDLE/INACTIVE operation, the same information is delivered irrespective of LP-WUR type. The OFDM sequence can carry information. - At least duty-cycled monitoring of LP-WUS is supported - For IDLE/INACTIVE modes - Specify procedure and configuration of LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring (RAN2, RAN1, RAN3, RAN4) - Specify LP-SS with periodicity with Yms for LP-WUR, for synchronization and/or RRM for serving cell. (RAN1, RAN4) - LP-SS is based on OOK-1 and/or OOK-4 waveform with or without overlaid OFDM sequences. Further down selection between with and without overlaid OFDM sequences is to be done within WI. - Note: For LP-WUR that can receive existing PSS/SSS, existing PSS/SSS can be used for synchronization and RRM instead of LP-SS. - Y will be decided within WI. 320ms is the start point. - Specify further RRM relaxation of UE MR for both serving and neighbor cell measurements, and UE serving cell RRM measurement offloaded from MR to LP-WUR, including the necessary conditions (RAN4, RAN2) - For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR - Note: The target coverage of LP-WUS and LP-SS shall be the coverage of PUSCH for message3. - Note: The optimization of LP-WUS signal design for idle/inactive mode is prioritized over the optimization for connected mode. - Specify the necessary RAN4 core requirement(s) to support the feature (RAN4). - Specify UE low-power wake-up receiver requirements, at least REFSENS, ACS and ASCS requirements with consideration of possible new methodology to assess the low-power wake-up receiver performance - Define guard RBs for ACS and ASCS cases - Study testability of above requirements - Consider impacts of different architecture and impairments, and set requirements that enable all types of reasonable implementation - Study and if necessary specify or support by declaration, the corresponding BS requirements, e.g., dynamic range for LP-WUS/LP-SS. - Current NR BS requirements is baseline - Specify necessary RRM requirements

In this paper we will continue to discuss the related issues and provide our views.

Discussion

General aspects

In previous meetings, the basic scenarios were discussed with a few agreements. In this section, we mainly discuss the LP-WUS monitoring and the MR/LR measurements based on RAN1 and RAN2’s agreements.

Whether to keep Case#2

In previous meetings, RAN4 has already agreed the case#1 (MR Fully offload to LR) without consideration of configured high priority frequency, that is, only LR performs the serving cell measurement and MR is OFF. In the last meeting, we also agreed that MR and LR are in ON state (MR partially offload to LR, MR+LR).

For case#1, the measurement only performed by LR, that is, the LR measurement result will decide the MR is ON or OFF.

For case#3, the remaining issue is whether to combine the measurements across the two radios or not since MR and LR both perform the serving cell measurement, only  MR performs neighbour cell measurement.

There are still case#2 and case#4 (measurements with high priority frequency) are not discussed and we will firstly analyze the remaining scenarios and then specify the RRM requirements for all cases including the remaining issue for case#3.

Firstly, we shall focus on the feasibility of Case#2 and the following figure can be seen:

Figure 1 Different cases

Based on the above figure, we find that the MR and LR is ON and both of them perform the serving cell measurement for Case#2. However, the main difference between Case#2 and Case#3 is that there is no neighbour cell measurement for Case#2. In general, Case#2 and Case#3 depends on the serving cell measurement results. If the service of the current cell is better than the threshold defined in existing spec, the UE will not do the neighbour cell measurement, however, the measurement result is below the threshold which can enter into Case#1, the UE is still in the MR+LR state (Case#2). Otherwise, the UE shall perform the neighbour cell measurement (Case#3). We deem that that is not a corner case and Case#2 is needed. However, we shall have a comprehensive understanding if we decide to study Case#2. To my understanding, whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

We know that if UE stays in MR+LR state, the scaling factor for MR measurement time period is needed. The MR measurement is more infrequent than LR measurement. And the MR measurement will cost more power. Some companies think that the accuracy of LR measurement is lower than MR measurement. However, there is no evidence can prove that and that is RAN1’s evaluation work, we shall wait for RAN1’s agreements. Also, the periodicity of LR is under discussion in RAN1 which maybe 160ms, 320ms etc, that is, LR will measure more samples than MR, the measurement results may not necessarily be inaccurate. If the accuracy of LR measurement is the key concern, we only need to consider how to keep similar accuracy between LR and MR, such as introduce the offset for LR measurement instead of studying another case. Thus, the Case#2 is not necessary to consider.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

Measurement requirements to be specified for LP-WUR

In RAN4 #110bis meeting, RAN4 has already agreed the agreement as below:

At Rel-19 LP-WUR WI, for LP-WUR measurement, RAN4 specifies measurement requirements for the following: - Measurement requirements for LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state - Other related requirements are FFS

We agreed that the measurement requirements shall be considered based on exixting PSS/SSS and OOK-based LP-SS. However, some companies deem that the OFDM-based LP-SS shall also be considered to define RRM measurement requirements. RAN1 agreed the following contents in the last meeting:

Agreement Support overlaid OFDM sequence(s) for LP-SS: - LP-SS reuses the overlaid OFDM sequence(s) specified for LP-WUS. The design on overlaid OFDM sequence(s) specified for LP-WUS doesn’t target for sync and RRM measurement performance based on overlaid OFDM sequence for LP-SS. - Whether to transmit LP-SS by using a specified overlaid OFDM sequence is configurable. - Applicable at least for OOK-1 and FFS for OOK-4 From RAN1 perspective, it is not intended to introduce new RAN4 requirements specific to overlaid sequences

RAN1 has already reached the conclusion that the the design on overlaid OFDM sequences doesn’t target for sync and RRM measurement. Thus, we shall also focus on the previous agreement. From my perspective, the legacy agreements could be modified as below:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Observation 2: RAN1 has already clarified that OFDM-based LP-SS doesn’t target for sync and RRM measurement.

Proposal 2: RAN4 shall not define requirements for OFDM-based LR serving cell measurement based on LP-SS.

Proposal 3: RAN4 shall only consider the following cases:

- Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Threshold for switch between Case#1 and Case#3

For entry and exit conditions based on LR and MR measurement, RAN2 has already reached the following conclusion in previous meetings:

127 meeting: For serving cell measurement offloading (i.e., there is no serving cell measurement by MR): The entry conditions for serving cell measurement offloading can be defined as at least MR greater than a certain RSRP threshold, and LR could also be considered. The exit condition is based on the LR measurement results. 127bis meeting: If NW configure thresholds for both MR and LR measurements, then the entry condition is met when all the measured results are above the configured threshold(s).

For the entry condition, there are two cases which from Case#5 to Case#3 (5-->3) and from Case#1 or Case#2 which is not confirmed to Case#3 (1-->3 or 2-->3). For the first case, only MR measurement in Case#5 and if MR measurements result can be satisfied, UE can enter into Case#3. For the second case, only LR performs serving cell measurement, and the LR based measurement can be the only criteria to enter into Case#3, and the threshold can be the legacy neighbour cell triggering.

For the exit condition, Case#3 shall enter into Case#5 or Case#1 (3-->5 and 3-->1). For exit condition, the MR and LR perform the serving cell measurement together. If network configures two different thresholds which are for MR based measurement and LR based measurement, the two measurements can compare to the different thresholds. In Case#5, MR does not perform serving cell measurement with relaxation, however, in Case#3 the MR shall perform the serving cell measurement with relaxation. Thus, we think the LR measurement can be trusted since LR can measure more samples without relaxation compared to MR based measurement. Based on RAN2’s agreements, if the MR based measurement and LR based measurement can be used and each measurement result shall satisfy each threshold, the UE can exit Case#3.

From RAN4 perspective, the threshold can be discussed based on RAN2’s agreements. We deem that the threshold between Case#3 and Case#1 is the whether neighbour cell measurement is triggered. Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement. RAN1 has agreed introduced the LP-RSRP and LP-RSRQ in previous meetings, thus, the new threshold for LP-RSRP and LP-RSRQ could be considered. Also, some companies proposed an offset for LR measurement threshold, that is, we could continue to use the existing threshold with an offset for LP-RSRP and LP-RSRQ.

Observation 3:

- For the entry condition, there are two cases which from Case#5 to Case#3 and from Case#1 or Case#2 which is not confirmed to Case#3.

- For the exit condition, Case#3 shall enter into Case#5 or Case#1.

Observation 4: Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement.

Proposal 4: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ.

Proposal 5: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ.

Criteria for LP-WUS monitoring/ LR measurement/ MR measurement relaxation

First of all, we shall clarify these three different criteria:

- LP-WUR measurement: LR starts serving cell measurement.

- MR measurement relaxation: MR performed relaxed measurement and LR is ON.

- LP-WUS monitoring: LR starts monitoring LP-WUS and MR may stop the legacy PO monitoring before LR receives LP-WUS indication.

We can see the criteria on LR measurement before and the threshold for MR relaxed measurement can be seen in RAN2:

126 meeting: - RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise. RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’. 127bis meeting: Working assumption - For neighbor cell measurement relaxation for UEs capable of LP-WUS, do not define additional MR-based criterion over the R16 criteria. RAN2 assume ‘UE not at cell edge’ is reused, FFS on ‘UE with low mobility’. - FFS (if needed) on enhancements based on R16 criteria (e.g., based on the LR measurements) for the case when MR serving cell measurement results are not available.

Although there is no specific conclusion on criteria on MR measurement relaxation, we see that the R16 criteria could be the baseline and whether enhancements are needed is still under discussion in RAN2.

For LR measurements, RAN2 has already reached several agreements and the details are still discussing in RAN2.

For LP-WUS monitoring, the agreements in RAN2 can be seen:

127 meeting: Working assumption (can revisit if R1/R4 reached different conclusions): If the entry/exit conditions are configured, besides MR-based thresholds, LP-WUS monitoring entry condition can also include LR-based thresholds. The metrics for serving cell quality measured by MR/LR for entry condition includes (LP-)RSRP and optional (LP-)RSRQ. The metrics for serving cell quality measured by LR for exit condition includes (LP-)RSRP and optional (LP-)RSRQ. 127bis meeting: The LPWUS monitoring exit condition does not include MR measurements.

From my perspective, the criteria for LR measurement/ MR RRM measurement relaxation/ LP-WUS monitoring shall be continued to discuss in RAN2. RAN4 can wait for more details and then define the related requirements.

Proposal 6: RAN4 shall wait for more details on the criteria for LR measurement/ MR RRM measurement relaxation/ LP-WUS monitoring which are under discussion in other working groups.

LP-WUR operating carrier frequency

In the last meeting, RAN4 discussed the LS from RAN1 and the agreements can be seen as below:

Agreement: - RAN4 will further discuss whether any technical issues regarding the LS from RAN1. - From Rel-19 RAN4 RRM requirement of MR offloading and relaxation perspective, RAN4 assumed LR and MR are operating on the same carrier frequency as baseline. FFS for the case of MR and LR working on different carrier frequencies if it is supported in RAN1/2. Issue for further discussion based on contribution driven: - For WUS monitoring assumption, further discuss whether or not the same carrier frequency is assumed for LR and MR.]

Since we know that the LR measurement only depends on LP-RSRP/RSRQ or OFDM based existing PSS/SSS. Thus, LR measurement requirements are not related to MR working band. The following figures shall be the different options:

Figure 2 WUS monitoring operating different carrier frequency

Firstly, we shall clarify what is the definition of the separated band is for LP-WUR. There are two options:
Option 1: LP-SS and WUS are in the different bands, LP-SS and MR measurement are in the same band.

Option 2: LP-SS and WUS are in the same band , LP-SS and MR measurement are in different bands.

To my understanding, one prerequisite is that if LR operates on one band, then the LP-SS and LP-WUS shall also operate on the same band. Option 1 means that LP-SS and WUS will stay in different band, this can not realize for LR receiver. In TR 38.869, we find the following:

RF envelop detection architecture is more appropriate for single-band operation.

For RF ED based WUR, it is appropriate for single-band operation, that is, LP-SS and LP-WUS shall be in the same band together. Thus, separated band in Option 1 is not appropriate for us.

For option 2, the LP-SS and LP-WUS are in the same band. In the last meeting, the LR and MR measurement are operating in same carrier frequency is the baseline. We shall discuss whether the separate band can be realized. Some companies deem that if LR measurement can help the band which is different to the LR operating band perform MR measurement, how to guarantee the measurement accuracy for MR since the channel conditions may be different. From my perspective, this can be achieved by accuracy threshold, that is, if the difference between LP-SS measurement and MR measurement is lower than one configured accuracy threshold, then MR and LR can work on different band. However, this can be discussed in the further meeting. The remaining issue is whether MR in the other band can be woke up to perform paging monitoring after UE receiving the WUS. To my understanding, this is possible for WUS to wake up MR in the other band in option 2, this is different from RRM measurement.

Observation 5: RF envelop detection architecture is more appropriate for single-band operation.

Proposal 7: For the discussion on separate band, there are two options, and RAN4 shall consider option 2:

Option 1: LP-SS and WUS are in the different bands, LP-SS and MR measurement are in the same band.

Option 2: LP-SS and WUS are in the same band , LP-SS and MR measurement are in different bands.

Proposal 8: For RRM measurement, LR and MR could work on different band. The accuracy threshold may be considered.

Proposal 9: For WUS monitoring, LP-WUS could wake up MR in different band to perform paging monitoring.

Higher priority frequency layer

When UE stays in center area of the current cell and the service of the cell is good enough, the UE will not perform neighbour cell measurement without configured high priority frequency. However, the UE shall search for inter-frequency and inter-RAT layers of high priority if high priority frequency information is provided by the serving cell based on TS 38.133:

If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-frequency layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.7. If Srxlev > SnonIntraSearchP and Squal > SnonIntraSearchQ then the UE shall search for inter-RAT E-UTRAN layers of higher priority at least every Thigher_priority_search where Thigher_priority_search is described in clause 4.2.2.

- Observation 6: The different cases can be seen as following table with high priority frequency:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 (to be confirmed) | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

- For Case#4:

UE is in fully LR state and the MR shall be OFF, that is , UE is in the center area of the current cell and the service is good enough. In this case, serving cell has configured the high priority frequency information and UE shall search for high priority frequency layers and perform the neighbour cell measurement. Thus, MR shall be ON from OFF since UE search for the high priority frequency layer at Thigher_priority_search. But only LR performs serving cell measurement. As mentioned before, RAN2 is discussing the neighbour cell measurement relaxation condition and if the condition is the same as legacy, we deem that the legacy relaxed requirement can be as baseline and RAN4 shall only discuss the relaxed scaling factor.

- Observation 7: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 10: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Detailed LP-WUR requirements at RRC IDLE/INACTIVE state

Accuracy requirements

In previous meetings, we have proposed several times that no dedicated accuracy requirements is defined in performance section for LP-WUR based RRM measurement in Idle/Inactive states since the accuracy is implicitly defined as margin as below:

For an intra-frequency cell that has been already detected, but that has not been reselected to, the filtering shall be such that the UE shall be capable of evaluating that the intra-frequency cell has met reselection criterion defined in TS38.304 [1] within Tevaluate,NR_Intra when Treselection = 0 as specified in table 4.2.2.3-1, table 4.2.2.3-2, table 4.2.2.3-3, table 4.2.2.3-4 or table 4.2.2.3-5 provided that: when rangeToBestCell is not configured: - the cell is at least 3dB better ranked in FR1 or 4.5dB better ranked in FR2.

Proposal 11: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Interruption for LP-WUR

In order to reduce the power consumption, the LP-WUR has been introduced. According to WI, “Specify procedure and configuration design for LP-WUS indicating paging monitoring triggered by LP-WUS, including at least configuration, sub-grouping and entry/exit condition for LP-WUS monitoring” involves RAN4. When DRX is configured and there is no traffic transmission, the MR (main receiver) enters a sleep state, only waking up at corresponding moments (DRX ON) to perform paging monitoring. If the specific conditions for entering LR coverage are satisfied, LR allows the UE MR to enter a deep sleep state prior to monitoring the LP-WUS (Low Power Wake-Up Signal). When the UE receives SIB message, the UE initiates monitoring of LP-WUS, subsequently triggering the awakening of the UE MR to monitor PO (paging occasion), this process aims to further mitigate power consumption:

Figure 3 UE behavior upon detecting LP-WUS with paging indication in IDLE/INACTIVE state

If the entry threshold is satisfied, UE will enter the LP-WUS coverage and turn off MR for paging monitoring at the same time. If the SIB message has been received, UE would like to monitor the LP-WUS and wake up MR to monitor the legacy PO.

On the contrary, if the exit threshold is met, UE will exit the LR coverage and turn on the MR receiver for paging monitoring.

Based on the above analysis, it is crucial that UE shall not miss paging when entry/exit conditions for LP-WUS monitoring are met. RAN4 shall study and define the related requirements on timing from UE receiving SIB message to waking up MR to monitor legacy PO when the entry condition is met. It also can be seen that the time window LP-WUS occasions (LOs) are defined for LP-WUS monitoring, each LO has one or more LP-WUS occasions.

From my perspective, the following shall be considered so that the UE will not miss the paging:

Figure 4 Time delay for paging monitoring

MR wake-up time: UE detects the LP-WUS and decodes it, then LP-WUS wakes up MR to monitor the legacy PO.

When the entry condition for LP-WUS monitoring is met, the time delay shall contain MR wake-up time TMR_wake-up . This time will decide whether LP-WUS can timely wake up MR and ensure that it does not miss the paging monitoring.

On the contrary, UE will stop LP-WUS monitoring and turn on the MR to monitor legacy PO if MR turns off before when the exit condition is met. As we all know, the time delay for MR wake-up also exists during this process.  However, network seldom send the paging message to UE in this scenario, and the network does not page the UE in most cases since UE MR will stay deep sleep state and only LR is ON before UE exits LR coverage, there is even no paging. Thus, this time delay will not cause the bad service or communication delay.

Proposal 12: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO.

Core requirements to be specified for LP-WUR measurement in IDLE/INACTIVE state

According to WI, it specifies that the RAN4 should define the core requirements to support the feature. In this part, we mainly analyze the UE camps on different receivers and related core requirements combining with the progress and agreements in other groups. In this part, we primarily study the different cases under entry/exit conditions and specify the related RRM requirements, especially MR relaxation requirements for serving/neighbour cell and LR measurement requirements.

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

Cases/States without high priority frequency

We shall clarify one key point that only MR can perform the neighbour cell measurement and LR only performs serving cell measurement based on RAN2 agreements:

- RAN2 understand that the RRM measurement of the neighboring cell can only be performed by MR. Can discuss again if RAN1 inform us otherwise.

- Observation 8: MR and LR shall both perform serving cell measurement. Only MR performs neighbour cell measurement.

We shall clarify the whole procedure based on the serving cell measurement and the entry/exit condition. To my understanding, the whole procedure can be:

Fully MR state (including the relaxation) → MR+LR → Fully LR → MR+LR → Fully MR (including the relaxation)

In different states, UE will perform corresponding measurements and the related measurement requirements shall be discussed and defined.

- Proposal 13: RAN4 shall specify the whole procedure based on serving cell measurement and specify the UE behaviour when it satisfies the entry/exit condition:

- When the entry condition is satisfied:

- Option 1: Fully MR →MR+LR →Fully LR

- Option 2: Fully MR →Fully LR

- When the exit condition is satisfied:

- Option 1: Fully LR →MR+LR→ Fully MR

- Option 2: Fully LR→ Fully MR

- For Case#3 (MR+LR):

In this case, UE satisfies the entry condition to enter into LR coverage. UE stays in MR+LR state, that is, MR and LR is ON. We have already agreed this case without the detailed requirements. In this section, we mainly analyze the RRM requirements.

- Serving cell measurement (including MR RRM relaxation)

In this case, we find that the serving cell measurement is performed by LR and MR. Thus, we shall consider the MR measurement relaxation to reduce the power consumption. However, RAN2 only defines the relaxed measurement criterion for neighbour cell not for serving cell in IDLE/INACTIVE state in legacy. Thus, RAN4 shall wait for RAN2 progress about the relaxed measurement criteria for serving cell. But RAN4 can study the mobility performance to quantify the relaxation such as scaling factor for measurement time period.

Observation 9: In case#3, MR and LR are ON. The serving cell measurement is performed by LR and MR, MR measurement relaxation is aimed at reducing power consumption.

Observation 10: RAN4 only defines the relaxed measurement requirements for neighbour cell not for serving cell.

Proposal 14: RAN4 shall wait for RAN2 to define the relaxed measurement condition for serving cell measurement if it has.

Proposal 15: When UE stays in MR+LR state, RAN4 shall consider the MR serving cell measurement relaxation and also study the mobility performance to quantify the relaxation such as scaling factor for time period.

- Neighbour cell measurement (MR RRM relaxation)

If  Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ, it means the service of serving cell is not good. Then MR will perform the neighbour cell measurement. RAN4 shall also consider the MR neighbour cell measurement relaxation. In legacy, RAN2 defined the neighbour cell relaxation condition and RAN4 defined the relevant RRM requirements. In this case, RAN2 is discussing the related relaxed measurement condition as below:

RAN2 will further discuss the neighbor cell measurement relaxation criteria (if the UE is using LR to measure the serving cell), e.g., considering reuse Rel-16 criteria for ‘not at cell edge’ and ‘low mobility’.

If the legacy condition can be reused, we deem that the legacy relaxed measurement requirements for neighbour cell shall be as baseline and RAN4 can discuss the suitable scaling factor.

- Proposal 16: RAN4 shall wait for RAN2 to define the final relaxed measurement conditions for neighbour cell measurement.

- Proposal 17: The legacy intra-/inter-frequency and inter-RAT neighbour cell measurement requirements can be the baseline and RAN4 can study the relaxed scaling factor.

If Srxlev > S(non)IntraSearchP or Squal > S(non)IntraSearchQ, it means the service of serving cell is better and UE is near the center of the current cell. Then MR will not perform neighbour cell measurement without configured high priority frequency.

Observation 11: When UE camps on MR+LR state, MR and LR shall be ON. MR performs serving cell measurement and neighbour cell measurement based on the service of current cell. If the service of serving cell is good, UE MR will not perform the neighbour cell measurement. On the contrary, if service of serving cell is bad, UE MR will perform neighbour cell measurement without high priority frequency.

- For Case#5 (legacy MR state):

In this case, UE stays in fully MR state, that is, only MR is ON and LR is OFF. From UE perspective, UE is far away from the center of the current cell and UE MR will perform the serving cell measurement based on SSB. There is no doubt that the measurement results SS-RSRP and SS-RSRQ will be smaller than the threshold ( Srxlev ≤ S(non)IntraSearchP or Squal ≤ S(non)IntraSearchQ) specified in TS38.304 and TS38.133. In this way, MR will not only measure the current serving cell but the neighbour cell. For neighbour cell measurement, UE shall perform intra-frequency measurement, inter-frequency measurement and inter-RAT measurement . Thus, the MR shall be always ON in case 1.

To my understanding, the legacy requirements shall be reused for case 1 since there is little different from legacy situation.

Observation 12: When UE camps on fully MR state, only MR is ON and LR is OFF. MR performs serving cell measurement and also neighbour cell measurement.

Proposal 18: RAN4 shall reuse the legacy cell re-selection requirements when UE stays in fully MR state.

Core requirements to be specified for MR RRM relaxation

As indicated in the latest WID, RAN4 shall specify further RRM relaxation of UE MR for both serving and neighbor cell measurements. Legacy RRM relaxation at Idle/Inactive state have been specified since Rel-16 in the WID of power saving. Here we believe similar solution can be reused for MR RRM relaxation, i.e. introduce relaxation scaling factor.

Issue 2-3-1: MR RRM relaxation factor for serving cell/neighbour cell - Proposals - P1: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation (vivo) - P2-1: Equal or larger than 8 (CMCC) - P2-2: >=16 (Huawei) - P2: RAN4 should consider relaxation on higher priority frequency layer search requirements based on “conditions” defined by RAN2. (vivo) - P3: Neighbour cell evaluation can be discussed after further input from RAN2 (Samsung)

Based on the TR 38.869, there are two kinds of LP-WUS monitoring which are RRM with duty-cycle LP-WUS monitoring and RRM with continues LP-WUS monitoring. The power saving gain can be seen as the following table:

Table1 Power saving gain based on TR38.869

|  | RRM with duty-cycle LP-WUS | RRM with continues LP-WUS |
| --- | --- | --- |
| No MR RRM relaxed | average: -211% | average: -644% |
| MR relaxed < 8 times | average: 4% | average: 7% |
| 8 times<=MR relaxed<=16 times | average: 40% | average: 32% |
| MR relaxed > 16 times | average: 60% | average: 57% |
| MR offload RRM to LR | average: 83% | average: 74% |

For scenario 1 (No MR RRM relaxed), we find no power saving gain if no MR RRM relaxed. For scenario 3 (MR offload RRM to LR), we can infer that the propagation condition is the best and the MR will stop any RRM measurement and the power saving gain is the best during the whole table. For scenario 2, there are 3 cases can be seen, the different relaxation factors have the different power saving gain and the larger relaxation factors the larger power saving gain.

The above table shows us if the factor is less than 8 times the power saving gain is average 4%. However, if the relaxation factor is equal or larger than 8, the power saving gain has a sharp increase whatever with duty-cycle or continues LP-WUS. Thus, from my perspective, the factor within the range from 8 to 16 times can be the starting point and baseline since the power saving gain in that range is close to half the gain.

Observation 13: If the factor is less than 8 times the power saving gain is average 4%. However, if the relaxation factor is equal or larger than 8, the power saving gain has a sharp increase.

Proposal 19: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

LP-WUR at connected mode

We mainly focus on the measurement requirements in the above. For the connected mode, in the latest WID, we can see that no RAN4’s involvement is required:

- For CONNECTED mode, specify procedures to allow UE MR PDCCH monitoring triggered by LP-WUS including activation and deactivation procedure of LP-WUS monitoring (RAN2, RAN1) - Check in RAN#105 for potential TU adjustment in RAN2 - Note: In CONNECTED mode, UE MR ultra-deep sleep is not considered, and UE RRM/RLM/BFD/CSI measurements are performed by MR

It is obviously the CONNECTED state is not in the scope of the WID.

Proposal 20: RAN4 shall not define the measurement requirements at CONNECTED state.

Conclusion

In this paper we provided our views on LPWUS.

Observation 1: Whether to study Case#2 or not shall consider the following reasons:

- MR measurement relaxation and power consumption

- The accuracy of LR measurement

Proposal 1: Case#2 is not necessary to consider.

Observation 2: RAN1 has already clarified that OFDM-based LP-SS doesn’t target for sync and RRM measurement.

Proposal 2: RAN4 shall not define requirements for OFDM-based LR serving cell measurement based on LP-SS.

Proposal 3: RAN4 shall only consider the following cases:

  - Measurement requirements for OOK-based LP-WUR serving cell measurement based on LP-SS at Idle/Inactive state

  - Measurement requirements for OFDM-based LP-WUR serving cell measurement based on existing PSS/SSS at Idle/Inactive state

Observation 3:

- For the entry condition, there are two cases which from Case#5 to Case#3 and from Case#1 or Case#2 which is not confirmed to Case#3.

- For the exit condition, Case#3 shall enter into Case#5 or Case#1.

Observation 4: Whatever from Case#3 to Case#1 or from Case#1 to Case#3, the LR is always ON and the LR measurement result is also key point to decide whether to switch or not refer to RAN2. However, in current spec, the conditions for neighbour cell measurement is the RSRP of current serving cell which performed by MR measurement.

Proposal 4: Use existing thresholds for SS-RSRP and SS-RSRQ, and define new thresholds for LP-RSRP and LP-RSRQ.

Proposal 5: Add an offset for existing threshold which is compared to MR based measurement for LP-RSRP and LP-RSRQ.

Proposal 6: RAN4 shall wait for more details on the criteria for LR measurement/ MR RRM measurement relaxation/ LP-WUS monitoring which are under discussion in other working groups.

Observation 5: RF envelop detection architecture is more appropriate for single-band operation.

Proposal 7: For the discussion on separate band, there are two options, and RAN4 shall consider option 2:

Option 1: LP-SS and WUS are in the different bands, LP-SS and MR measurement are in the same band.

Option 2: LP-SS and WUS are in the same band , LP-SS and MR measurement are in different bands.

Proposal 8: For RRM measurement, LR and MR could work on different band. The accuracy threshold may be considered.

Proposal 9: For WUS monitoring, LP-WUS could wake up MR in different band to perform paging monitoring.

- Observation 6: The different cases can be seen as following table with high priority frequency:

| RRM measurement case index | MR serving cell measurement | MR neighbouring cell measurement | LR measurement |
| --- | --- | --- | --- |
| Case#1 | OFF | OFF | ON |
| Case#2 (to be confirmed) | ON, with measurement relaxation | OFF | ON |
| Case#3 | ON, with measurement relaxation | ON, with measurement relaxation | ON |
| Case#4 | OFF | ON, with measurement relaxation | ON |
| Case#5 | ON | ON | OFF |

- Observation 7: MR will perform inter-frequency measurement with high priority frequency and inter-RAT measurement with high priority frequency if high priority frequency information is provided by the serving cell even if the service of current cell is good enough.

- Proposal 10: RAN4 shall study the MR neighbour cell measurement relaxation when the high priority frequency is configured. If the legacy relaxed conditions are reused, the legacy relaxed measurement requirements can be the baseline and RAN4 shall study the relaxed scaling factor.

Proposal 11: No dedicated accuracy requirement is defined in the performance section for LR-WUR based RRM measurement in Idle/inactive states, and reflect the accuracy performance as a margin in the core requirement.

Proposal 12: RAN4 shall clarify the timeline from decoding LP-WUS to MR wake-up in order to monitor legacy PO.

- Observation 8: MR and LR shall both perform serving cell measurement. Only MR performs neighbour cell measurement.

- Proposal 13: RAN4 shall specify the whole procedure based on serving cell measurement and specify the UE behaviour when it satisfies the entry/exit condition:

- When the entry condition is satisfied:

- Option 1: Fully MR →MR+LR →Fully LR

- Option 2: Fully MR →Fully LR

- When the exit condition is satisfied:

- Option 1: Fully LR →MR+LR→ Fully MR

- Option 2: Fully LR→ Fully MR

Observation 9: In case#3, MR and LR are ON. The serving cell measurement is performed by LR and MR, MR measurement relaxation is aimed at reducing power consumption.

Observation 10: RAN4 only defines the relaxed measurement requirements for neighbour cell not for serving cell.

Proposal 14: RAN4 shall wait for RAN2 to define the relaxed measurement condition for serving cell measurement if it has.

Proposal 15: When UE stays in MR+LR state, RAN4 shall consider the MR serving cell measurement relaxation and also study the mobility performance to quantify the relaxation such as scaling factor for time period.

- Proposal 16: RAN4 shall wait for RAN2 to define the final relaxed measurement conditions for neighbour cell measurement.

- Proposal 17: The legacy intra-/inter-frequency and inter-RAT neighbour cell measurement requirements can be the baseline and RAN4 can study the relaxed scaling factor.

Observation 11: When UE camps on MR+LR state, MR and LR shall be ON. MR performs serving cell measurement and neighbour cell measurement based on the service of current cell. If the service of serving cell is good, UE MR will not perform the neighbour cell measurement. On the contrary, if service of serving cell is bad, UE MR will perform neighbour cell measurement without high priority frequency.

Observation 12: When UE camps on fully MR state, only MR is ON and LR is OFF. MR performs serving cell measurement and also neighbour cell measurement.

Proposal 18: RAN4 shall reuse the legacy cell re-selection requirements when UE stays in fully MR state.

Observation 13: If the factor is less than 8 times the power saving gain is average 4%. However, if the relaxation factor is equal or larger than 8, the power saving gain has a sharp increase.

Proposal 19: Relaxation factors within the range from 8 to 16 as the starting point for the relaxation factor for the MR RRM relaxation.

Proposal 20: RAN4 shall not define the measurement requirements at CONNECTED state.

References

[1] RP-221018, New WID: Low-power wake-up signal and receiver for NR (LP-WUS/WUR)

[2] TR 38.869